3GPP TSG-CT WG3 Meeting #132	C3-240173

# E-Meeting, 22nd – 24th January, 2024												(revision of C3-240092)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0224 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Define API of SS_AADRF_DataManagement API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2024-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The A-ADRF related services defined in clause 9.3 of TS 23.436 need to be supported in stage 3. CT3 considers to merge these A-ADRF APIs into one general data management API in stage 3 for the following reasons: - These five APIs are all used for data retrieval from the A-ADRF which means they belong to the same overall functionality. - If the consumer needs to retrieve several types of data from the A-ADRF simultaneously, it only needs to invoke one service instead of invoking several services. - There are a lot of duplicated information in the different subscriptions/requests and notifications/responses, and merging these APIs will make it much easier for us to maintain them in the future. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the resource structure and data model of SS_AADRF_DataManagement API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 requirement is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10.8(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

7.10.8	SS_AADRF_DataManagement API

7.10.8.1	API URI

The SS_AADRF_DataManagement service shall use the SS_AADRF_DataManagement API.

The request URIs used in HTTP requests from the ADAE server towards the A-ADRF shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-aadrf-datamanagement".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.10.8.2

7.10.8.2	Resources

7.10.8.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.8.2.1-1 depicts the resource URIs structure for the SS_AADRF_DataManagement API.

Figure 7.10.8.2.1-1: Resource URI structure of the SS_AADRF_DataManagement API

Table 7.10.8.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.8.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| A-ADRF Data Management Subscriptions | /subscriptions | POST | Creates a new Individual A-ADRF Data Management Subscription resource. |
| Individual A-ADRF Data Management Subscription | /subscriptions/{subscriptionId} | DELETE | Deletes an Individual A-ADRF Data Management Subscription identified by {subscriptionId}. |

7.10.8.2.2	Resource: A-ADRF Data Management Subscriptions

7.10.8.2.2.1	Description

The A-ADRF Data Management Subscriptions resource represents all the subscriptions stored at A-ADRF. The resource allows an NF service consumer to create a new Individual A-ADRF Data Management Subscription resource.

7.10.8.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-aadrf-datamanagement/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 7.10.8.2.2.2-1.

Table 7.10.8.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

7.10.8.2.2.3	Resource Standard Methods

7.10.8.2.2.3.1	POST

This method shall support the URI query parameters specified in table 7.10.8.2.2.3.1-1.

Table 7.10.8.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.8.2.2.3.1-2 and the response data structures and response codes specified in table 7.10.8.2.2.3.1-3.

Table 7.10.8.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataManageSub | M | 1 | Create a new Individual A-ADRF Data Management Subscription resource. |

Table 7.10.8.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataManageSub | M | 1 | 201 Created | The creation of an Individual A-ADRF Data Management Subscription resource is confirmed and a representation of that resource is returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.8.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-aadrf-datamanagement/<apiVersion>/subscriptions/{subscriptionId} |

7.10.8.2.2.4	Resource Custom Operations

None in this release of the specification.

7.10.8.2.3	Resource: Individual A-ADRF Data Management Subscription

7.10.8.2.3.1	Description

The Individual A-ADRF Data Management Subscription resource represents a single subscription to the SS_AADRF_DataManagement Service at a given A-ADRF.

7.10.8.2.3.2	Resource definition

Resource URI: {apiRoot}/aadrf-datamanagement/<apiVersion>/subscriptions/{subscriptionId}

The <apiVersion> shall be set as described in clause 7.10.8.2.3.2-1.

This resource shall support the resource URI variables defined in table 7.10.8.2.3.2-1.

Table 7.10.8.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| subscriptionId | string | Identifies a subscription to the SS_AADRF_DataManagement Service. |

7.10.8.2.3.3	Resource Standard Methods

7.10.8.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 7.10.8.2.3.3.1-1.

Table 7.10.8.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.8.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.8.2.3.3.1-3.

Table 7.10.8.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.8.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Individual A-ADRF Data Management Subscription resource matching the subscriptionId was deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative A-ADRF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative A-ADRF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.8.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

Table 7.10.8.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

7.10.8.2.3.4	Resource Custom Operations

None in this release of the specification.

7.10.8.3	Custom Operations without associated resources

None.

7.10.8.4	Notifications

7.10.8.4.1	General

Table 7.10.8.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event Notification | {notifUri} | POST | Report the subscribed data. |

7.10.8.4.2	Event Notification

7.10.8.4.2.1	Description

The Event Notification is used by the A-ADRF to report the subscribed data to an NF service consumer that has subscribed to such Notifications.

7.10.8.4.2.2	Operation Definition

Callback URI: {notifUri}

The operation shall support the callback URI variables defined in Table 7.10.8.4.2.2-1, the request data structures specified in table 7.10.8.4.2.2-2 and the response data structure and response codes specified in Table 7.10.8.4.2.2-3.

Table 7.10.8.4.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | The Notification Uri is assigned within the Individual A-ADRF Data Management Subscription Resource and described within the DataManageSub type. |

Table 7.10.8.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataManageNotification | M | 1 | Provides information about the requested data. |

Table 7.10.8.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during the event notification. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during the event notification. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |  |

Table 7.10.8.4.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification is redirected. |

Table 7.10.8.4.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification is redirected. |

7.10.8.5	Data Model

7.10.8.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.10.8.5.1-1 specifies the data types defined specifically for the SS_AADRF_DataManagement API service.

Table 7.10.8.5.1-1: SS_AADRF_DataManagement API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AadrfEvent | 7.10.8.5.3.1 | Indicates the subscribed events. |  |
| ApiLogInfo | 7.10.8.5.2.7 | Represents the API log data. |  |
| ApiLogReq | 7.10.8.5.2.5 | Represents the API log request requirement. |  |
| DataManageNotification | 7.10.8.5.2.6 | Represents the notification to the consumer. |  |
| DataManageSub | 7.10.8.5.2.2 | Represents the events subscription. |  |
| EdgeInfo | 7.10.8.5.2.8 | Represents the EDGE related data. |  |
| EdgeReq | 7.10.8.5.2.4 | Represents the EDGE data request requirement. |  |
| EventSubscription | 7.10.8.5.2.3 | Represents the event subscription. |  |
| ExposureLevel | 7.10.8.5.3.2 | Indicates the exposure level. |  |

Table 7.10.8.5.1-2 specifies data types re-used by the SS_AADRF_DataManagement API service.

Table 7.10.8.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Dnai | 3GPP TS 29.571 [21] | Identifies a user plane access to one or more DN(s). |  |
| Dnn | 3GPP TS 29.571 [21] | Used to Identify a DNN. |  |
| Float | 3GPP TS 29.571 [21] | Used to represent the fractional part of the proximity range in the reference UE details. |  |
| Snssai | 3GPP TS 29.571 [21] | Used to Identify the S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the applicability of optional features. |  |
| Uinteger | 3GPP TS 29.571 [21] | Used to represent integer attributes within MeasurementData and ReportingRequirements data structures. |  |
| Uri | 3GPP TS 29.571 [21] | Used to indicate the notification URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to indicate either VAL User ID or VAL UE ID. |  |
| ValidityConditions | 7.5.1.4.2.13 | Represents the temporal and/or spatial conditions applied for the events to be monitored. |  |

7.10.8.5.2	Structured data types

7.10.8.5.2.1	Introduction

7.10.8.5.2.2	Type: DataManageSub

Table 7.10.8.5.2.2-1: Definition of type DataManageSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubscriptions | array(EventSubscription) | M | 1..N | Subscribed events. |  |
| notifUri | Uri | M | 1 | Identifies the recipient of Notifications sent by the A-ADRF. |  |
| notifCorrId | string | O | 0..1 | Notification correlation identifier. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | List of Supported features used as described in clause 7.10.8.7. This parameter shall be supplied by NF service consumer in the POST request that request the creation of an A-ADRF Event Subscriptions resource and shall be supplied by the A-ADRF in the reply of corresponding request. |  |

7.10.8.5.2.3	Type: EventSubscription

Table 7.10.8.5.2.3-1: Definition of type EventSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AadrfEvent | M | 1 | The event that is subscribed. |  |
| dataCollectReq | string | O | 0..1 | The requirements for data collection. |  |
| dataProducerIds | array(string) | O | 1..N | The list of Data Producer IDs. |  |
| valUes | array(ValTargetUe) | O | 1..N | The target VAL UE(s) identifiers. |  |
| valServerId | string | O | 0..1 | Identifies the target VAL server for which the data collection subscription applies. |  |
| valServiceId | string | O | 0..1 | The VAL service ID of the VAL application. |  |
| profileCriteria | string | O | 0..1 | The characteristics of the data producers to be used. |  |
| validConds | ValidityConditions | O | 0..1 | Represents the temporal and/or spatial conditions applied for the request. |  |
| edgeReq | EdgeReq | O | 0..1 | The EDGE data collection requirement. This attribute may be present if the subscribed event is "EDGE_DATA". |  |
| snssais | array(Snssai) | C | 1..N | Identification(s) of network slice(s) to which the subscription applies. This attribute shall be present if the subscribed event is "NETWORK_SLICE". |  |
| apiLogReq | ApiLogReq | O | 0..1 | The historical service API logs requirement. This attribute may be present if the subscribed event is "HISTORICAL_SERVICEAPI". |  |

Editor's Note:	The data type of "profileCriteria" and "dataCollectReq" attributes, and whether new features are needed for different events are FFS.

7.10.8.5.2.4	Type: EdgeReq

Table 7.10.1.4.2.4-1: Definition of type EdgeReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnais | array(Dnai) | C | 1..N | Identifiers the DN Access Identifier representing location of the service flow. (NOTE) |  |
| destEasId | string | C | 0..1 | Identifier for the destination EAS, for which the edge load analytics subscription or the edge load data collection subscription, is requested. (NOTE) |  |
| destEesId | string | C | 0..1 | Identifier for the destination EES, for which the edge load analytics subscription or the edge load data collection subscription, is requested. (NOTE) |  |
| dnns | array(Dnn) | C | 1..N | Identifiers of the DNNs. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |  |

7.10.8.5.2.5	Type: ApiLogReq

Table 7.10.8.5.2.5-1: Definition of type ApiLogReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiRequestorInfo | string | M | 1 | Identifier of the originated application querying service API log request. |  |
| apiId | string | O | 0..1 | String identifying the API invoked. |  |
| apiName | string | O | 0..1 | Name of the API which was invoked, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [3]. |  |
| apiVersion | string | O | 0..1 | Version of the API which was invoked. |  |
| inputParameters | ANY TYPE | O | 0..1 | List of input parameters. |  |
| result | string | O | 0..1 | For HTTP protocol, it contains HTTP status code of the invocation. |  |
| apiInvokerId | string | O | 0..1 | Identity of the API invoker which invoked the service API. |  |
| exposureLvl | ExposureLevel | O | 0..1 | The level of exposure requirement for the logs to be exposed. |  |
| apiAggreInfo | string | O | 0..1 | The aggregation or abstraction/filtering requirement needs to be applied. |  |

Editor's Note:	The data type of "apiAggreInfo" attribute is FFS.

7.10.8.5.2.6	Type: DataManageNotification

Table 7.10.1.4.2.6-1: Definition of type DataManageNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AadrfEvent | M | 1 | The event that is subscribed. |  |
| notifCorrId | string | O | 0..1 | Notification correlation identifier. |  |
| valServerId | string | O | 0..1 | Identifies the target VAL server for which the data collection subscription applies. This attribute shall be present if the subscribed event is "HISTORICAL_SERVICEAPI". |  |
| valServiceId | string | O | 0..1 | The VAL service ID of the VAL application. |  |
| valUeIds | array(ValTargetUe) | O | 1..N | The target VAL UE(s) identifiers. |  |
| locAccuracy | string | O | 0..1 | The requested location analytics/data. |  |
| validConds | ValidityConditions | O | 0..1 | Represents the temporal and/or spatial conditions applied for the request. |  |
| apiLog | ApiLogInfo | O | 0..1 | The API log information. This attribute may be present if the subscribed event is "HISTORICAL_SERVICEAPI". |  |
| rttDeviation | Float | O | 0..1 | Indicates the deviation value of RTT. This attribute may be present if the subscribed event is "NETWORK_SLICE". |  |
| snssais | array(Snssai) | O | 1..N | Identification(s) of network slice(s) to which the subscription applies. This attribute may be present if the subscribed event is "NETWORK_SLICE". |  |
| edgeInfo | EdgeInfo | O | 0..1 | The EDGE related information. This attribute may be present if the subscribed event is "EDGE_DATA". |  |

Editor's Note:	The data type of "locAccuracy" attribute is FFS.

7.10.8.5.2.7	Type: ApiLogInfo

Table 7.10.1.4.2.7-1: Definition of type ApiLogInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiId | string | M | 1 | String identifying the API invoked. |  |
| apiName | string | O | 0..1 | Name of the API which was invoked, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [3]. |  |
| failNum | Uinteger | O | 0..1 | Indicates the number of failure API invocations. |  |
| apiAvailableInd | boolean | O | 0..1 | Indicates the API availability. Set to "true" to indicate that the API is available. Default value is "false" if omitted. |  |
| apiVerChgNum | Uinteger | O | 0..1 | Indicates the number of API version changes. |  |
| apiLocChg | boolean | O | 0..1 | Indicates the API location changes for the target API. Set to "true" to indicate that the API location has changed. Default value is "false" if omitted. |  |
| apiThrottlingEvents | string | O | 0..1 | Indicates the API throttling events. |  |
| invokeNum | Uinteger | O | 0..1 | Indicates the number of API invocations. |  |
| apiVersion | string | O | 0..1 | Version of the API which was invoked. |  |

Editor's Note:	The data type of "apiThrottlingEvents" attribute is FFS.

7.10.8.5.2.8	Type: EdgeInfo

Table 7.10.8.5.2.8-1: Definition of type EdgeInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| edgeReq | EdgeReq | O | 0..1 | Indicates the DNAI, EAS and EES information which the data applies. |  |
| output | string | O | 0..1 | The reported data. |  |

Editor's Note:	The data type of "output" attribute is FFS.

7.10.8.5.3	Simple data types and enumerations

7.10.8.5.3.1	Enumeration: AadrfEvent

Table 7.10.8.5.3.1-1: Enumeration AadrfEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| HISTORICAL_SERVICEAPI | The event for historical service API logs. |  |
| NETWORK_SLICE | The event for the network slice data. |  |
| EDGE_DATA | The event for the EDGE related data. |  |
| LOCATION_ACCURACY | The event for the location accuracy data. |  |

7.10.8.5.3.2	Enumeration: ExposureLevel

Table 7.10.8.5.3.2-1: Enumeration ExposureLevel

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| READ | The exposure level for the logs to be exposed is read. |  |
| WRITE | The exposure level for the logs to be exposed is write. |  |
| DELETE | The exposure level for the logs to be exposed is delete. |  |

7.10.8.6	Error Handling

7.10.8.6.1	General

HTTP error handling shall be supported as specified in clause 6.7.

7.10.8.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_AADRF_DataManagement API.

7.10.8.6.3	Application Errors

The application errors defined for SS_AADRF_DataManagement API are listed in table 7.10.8.6.3-1.

Table 7.10.8.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

7.10.8.7	Feature negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.10.8.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

# *** End of Changes ***