3GPP TSG-CT WG3 Meeting #132	C3-240036

# Online, 22 - 24 January, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0216 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location accuracy performance analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2023-12-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.436 adds stage 2 for location accuracy performance analytics for the ADAE service which is a SEAL service. Stage 3 of location accuracy performance analytics needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The added changes are: - Added new SEAL service in the table. - Added SS_ADAE_LocationAccuracyAnalytics API - Added resource for individual location accuracy performance event subscription - Modified data types for LocAccurSubs and LocAccurNoti - Added yaml for SS_ADAE_LocationAccuracyAnalytics API |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 of SS_ADAE_LocationAccuracyAnalytics API does not exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.X.4 (new), 7.10.4.2.3 (new), 7.10.4.4.1, 7.10.4.4.2.2, 7.10.4.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 5.1	Introduction of SEAL services

The table 5.1-1 lists the SEAL server APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 5.1-1: List of SEAL Service APIs

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting | Request/ Response | VAL server |
|  | Fetch_Location_Report_Trigger | Request/Response | VAL server |
|  | Update_Trigger_Location_Reporting | Request/ Response | VAL server |
|  | Cancel_Trigger_Location_Reporting | Request/ Response | VAL server |
| SS_LocationInfoEvent | Subscribe_Location_Info | Subscribe/Notify | VAL server |
|  | Unsubscribe_Location_Info |  | VAL server |
|  | Notify_Location_Info |  | VAL server |
| SS_LocationInfoRetrieval | Obtain_Location_Info | Request/ Response | VAL server |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | Request/ Response | VAL server |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | Subscribe/Notify | VAL server |
|  | Unsubscribe_Location_Monitoring |  |  |
|  | Notify_Location_Monitoring_Events |  |  |
| SS_LocationAreaMonitoring | Subscribe_Location_Area_Monitoring | Subscribe/Notify | VAL server |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | Request/Response | VAL server |
|  | Obtain_VAL_Service_Area | Request/Response | VAL server |
|  | Update_VAL_Service_Area | Request/Response | VAL server |
|  | Delete_VAL_Service_Area | Request/Response | VAL server |
| SS_GroupManagement | Query_Group_Info | Request/ Response | VAL server |
|  | Update_Group_Info | Request/ Response | VAL server |
|  | Create_Group | Request/ Response | VAL server |
|  | Delete_Group | Request/Response | VAL server |
| SS_GroupManagementEvent | Subscribe_Group_Info_Modification | Subscribe/Notify | VAL server |
|  | Notify_Group_Info_Modification |  | VAL server |
|  | Notify_Group_Creation |  | VAL server |
| SS_UserProfileRetrieval | Obtain_User_Profile | Request/ Response | VAL server |
| SS_VALServiceData | Obtain_VAL_Service_Data | Request/Response | SEAL server |
| SS_UserProfileEvent | Subscribe_User_Profile_Update | Subscribe/Notify | VAL server |
|  | Notify_User_Profile_Update |  | VAL server |
| SS_NetworkResourceAdaptation (NOTE 3) | Reserve_Network_Resource | Request/Response | VAL server |
|  | Request_Unicast_Resource | Request/Response | VAL server |
|  | Update_Unicast_Resource | Request/Response | VAL server |
|  | Request_Multicast_Resource | Request/Response | VAL server |
|  | Notify_UP_Delivery_Mode | Subscribe/Notify | VAL server |
|  | Discover_TSC_Stream_Availability | Request/Response | VAL server |
|  | Create_TSC_Stream | Request/Response | VAL server |
|  | Delete_TSC_Stream | Request/Response | VAL server |
|  | Create_MBS_Resource | Request/Response | VAL server |
|  | Update_MBS_Resource | Request/Response | VAL server |
|  | Delete_MBS_Resource | Request/Response | VAL server |
|  | Activate_MBS_Resource | Request/Response | VAL server |
|  | Deactivate_MBS_Resource | Request/Response | VAL server |
| SS_EventsMonitoring | Subscribe_Monitoring_Events | Subscribe/Notify | VAL server |
|  | Notify_Monitoring_Events |  |  |
| SS_Events | Subscribe_Event | Subscribe/Notify | VAL server |
|  | Notify_Event |  | VAL server |
|  | Unsubscribe_Event |  | VAL server |
|  | Update_Subscription |  | VAL server |
| SS_KeyInfoRetrieval | Obtain_Key_Info | Request/Response | VAL server |
| SS_NetworkSliceAdaptation | Request_Network_Slice_Adaptation | Request/Response | VAL server |
| SS_NetworkResourceMonitoring | Subscribe_Unicast_QoS_Monitoring_Data | Subscribe/Notify | VAL server |
|  | Unsubscribe_Unicast_QoS_Monitoring_Data |  | VAL server |
|  | Notify_Unicast_QoS_Monitoring_Data |  | VAL server |
|  | Obtain_Unicast_QoS_Monitoring_Data | Request/Response | VAL server |
|  | Update_Unicast_QoS_Monitoring_Subscription |  | VAL server |
| SS_IdmParameterProvisioning | Provide_Configuration | Request/Response | VAL server |
| SS_ADAE_LocationAccuracyAnalytics | Subscribe_Location_Accuracy_Analytics | Subscribe/Notify | VAL server |
|  | Notify_Location_Accuracy_Analytics |  |  |
|  | Unsubscribe_Location_Accuracy_Analytics |  |  |
| NOTE 1: The service operations of SS_Events API are reused by the SS_LocationInfoEvent, SS_LocationMonitoring, SS_LocationAreaMonitoring, SS_GroupManagementEvent, SS_UserProfileEvent and SS_EventsMonitoring for events related services. NOTE 2: The service APIs exposed by the SEALDD Server and the corresponding service operations, operation semantics and service consumers are specified in clause 5 of 3GPP TS 29.548 [35]. NOTE 3: The "Create_MBS_Resource", "Update_MBS_Resource", "Delete_MBS_Resource", "Activate_MBS_Resource" and "Deactivate_MBS_Resource" service operations correspond to the stage 2 "Request_Multicast/Broadcast_Resource", "Update_Multicast/Broadcast_Resource", "Delete_Multicast/Broadcast_Resource", "Activate_Multicast_Resource" and "Deactivate_Multicast_Resource" service operations defined in clause 14.4.2 of 3GPP TS 23.434 [2]. |  |  |  |

Table 5.1-2 summarizes the corresponding APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| SS_LocationReporting | 7.1 | Report Location Information Service. | TS29549_SS_LocationReporting.yaml | ss-lr | A.2 |
| SS_GroupManagement | 7.2 | Group Management Service | TS29549_SS_GroupManagement.yaml | ss-gm | A.3 |
| SS_UserProfileRetrieval | 7.3 | User Profile Retrieval Service | TS29549_SS_UserProfileRetrieval.yaml | ss-upr | A.4 |
| SS_NetworkResourceAdaptation | 7.4 | Network Resource Adaptation Service | TS29549_SS_NetworkResourceAdaptation.yaml | ss-nra | A.5 |
| SS_Events | 7.5 | Events Notify Service | TS29549_SS_Events.yaml | ss-events | A.6 |
| SS_KeyInfoRetrieval | 7.6 | Key Information Retrieval Service | TS29549_SS_KeyInfoRetrieval.yaml | ss-kir | A.7 |
| SS_LocationAreaInfoRetrieval | 7.1 | Location Area Info Retrieval Service | TS29549_SS_LocationAreaInfoRetrieval.yaml | ss-lair | A.8 |
| SS_NetworkSliceAdaptation | 7.7 | Network Slice Adaptation Service | TS29549_SS_NetworkSliceAdaptation.yaml | ss-nsa | A.9 |
| SS_NetworkResourceMonitoring | 7.4 | Network Resource Monitoring | TS29549_SS_NetworkResourceMonitoring.yaml | ss-nrm | A.10 |
| SS_VALServiceData | 7.3 | VAL Service Data Service | TS29549_SS_VALServiceData.yaml | ss-vsd | A.11 |
| SS_VALServiceAreaConfiguration | 7.1 | VAL Service Area Configuration Service | TS29549_SS_VALServiceAreaConfiguration.yaml | ss-vsac | A.12 |
| SS_ADAE_LocationAccuracyAnalytics | 7.10 | ADAE location accuracy performance analytics service | TS29549_SS_ADAE_LocationAccuracyAnalytics.yaml | ss-adaelaa | A.17 |
| NOTE: The APIs exposed by the SEALDD Server are specified in clause 5 of 3GPP TS 29.548 [35]. |  |  |  |  |  |

* * * Next Change * * * *

### 5.X.4	SS_ADAE_LocationAccuracyAnalytics API

#### 5.X.4.1	Service Description

##### 5.X.4.1.1	Overview

The SS_ ADAE_LocationAccuracyAnalytics API, as defined 3GPP TS 23.436 [38], allows the VAL server via ADAE-S reference point to subscribe to location accuracy performance analytics event.

#### 5.X.4.2	Service Operations

##### 5.X.4.2.1	Introduction

The service operation defined for SS_ ADAE_LocationAccuracyAnalytics API is shown in the table 5.X.4.2.1-1.

Table 5.X.4.2.1-1: Operations of the SS_ADAE_LocationAccuracyAnalytics API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Location_Accuracy_Analytics | This service operation is used by VAL server to subsribe to the event of the location accuracy performance analytics. | VAL Server |
| Notify_Location_Accuracy_Analytics | This service operation is used by ADAE server to notify about the location accuracy performance analytics. | ADAE server |
| Unsubscribe_Location_Accuracy_Analytics | This service operation is used by VAL server to unsubsribe from the event of the location accuracy performance analytics. | VAL server |

##### 5.X.4.2.2	Subscribe_Location_Accuracy_Analytics

###### 5.X.4.2.2.1	General

This service operation is used by the VAL server for location accuracy performance analytics event subscription to the ADAE server.

###### 5.X.4.2.2.2	Subscribing to location accuracy performance analytics event using Subscribe_Location_Accuracy_Analytics service operation

To subscribe to location accuracy performance analytics event, the VAL server shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy" and with a body containing data type LocAccurSubs as defined in clause 7.10.4.4.2.2 with the following attributes:

1.	VAL server identifier;

2.	analytics identifier for the location accuracy performance event;

3.	type of analytics;

4.	one or more VAL UEs;

5.	accuracy level for the location accuracy performance analytics;

6.	identifier for VAL service;

7.	optionally, the confidence level for predictive analytics;

8.	optionally, the service area;

9.	optionally, the time interval; and

10.	optionally, the mobility and route information.

Upon receipt of the HTTP POST request, the ADAES shall:

1.	verify the identity of the VAL server and determine if the VAL server is authorized to subscribe to the location accuracy performance analytics event; and

2.	if the VAL server:

a.	is not authorized, the ADAE server shall respond to the VAL server with an appropriate error status code; or

b.	is authorized, the ADAE server shall create a new "Subscription to the event of location accuracy performance analytics" resource and respond to the VAL server with an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Subscription to the event of location accuracy performance analytics" and the response body including the LocAccurSubs data structure containing a representation of the created resource as defined in clause 7.10.4.2.

##### 5.X.4.2.3	Notify_Location_Accuracy_Analytics

###### 5.X.4.2.3.1	General

This service operation is used by the ADAE server to send notification to the VAL server with the location accuracy performance analytics event subscription to the ADAE server.

###### 5.X.4.2.3.2	Notifying location accuracy performance analytics event using Notify_Location_Accuracy_Analytics service operation

To notify location accuracy performance analytics event, the ADAE server shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/ss-adae-laa/<apiVersion>/ location-accuracy and with a body containing data type LocAccurNotif as defined in clause 7.10.4.4.2.3 with the following attributes:

1.	identity of the location accuracy performance analytics;

2.	predictive or statistical slice-specific application performance analytics;

3.	optionally, type of analytics; and

4.	optionally, the confidence level for predictive analytics.

Upon receipt of the HTTP POST request, the VAL server shall process the event notification.

##### 5.X.4.2.4	Unsubscribe_Location_Accuracy_Analytics

###### 5.X.4.2.4.1	General

This service operation is used by the VAL server to unsubscribe from the location accuracy performance analytics event.

###### 5.X.4.2.4.2	Unsubscribing from location accuracy performance analytics event using Unsubscribe_Location_Accuracy_Analytics service operation

To unsubscribe from location accuracy performance analytics event, the VAL server shall send an HTTP DELETE request to the resource representing the event in the ADAE server as specified in clause 7.10.4.2.3.3.1.

Upon receiving the HTTP DELETE request, the ADAE sever shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to unsubscribe from the location accuracy performance analytics event associated with the resource URI "{apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy/{locAccId}"; and

2.	if the VAL server is authorized to unsubscribe from the location accuracy performance analytics event, the ADAE server shall delete the resource pointed by the resource URI "{apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy/{locAccId}".

* * * Next Change * * * *

##### 7.10.4.2.3	Resource: Individual location accuracy event subscription

###### 7.10.4.2.3.1	Description

The individual location accuracy event subscription resource represents an individual event subscription of the VAL server.

###### 7.10.4.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy/{locAccId}

This resource shall support the resource URI variables defined in the table 7.10.4.2.3.2-1.

Table 7.10.4.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| locAccId | string | Identifies a location accuracy event subscription |

###### 7.10.4.2.3.3	Resource Standard Methods

7.10.4.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 7.10.4.2.3.3.1-1.

Table 7.10.4.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.4.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.4.2.3.3.1-3.

Table 7.10.4.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.4.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual location accuracy event subscription matching the locAccId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server or an alternative data producer such as A-ADRF, A-DCCF, VAL server, or SEALDD. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server or an alternative data producer such as A-ADRF, A-DCCF, VAL server, or SEALDD. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.4.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server or an alternative data producer such as A-ADRF, A-DCCF, VAL server, or SEALDD. |

Table 7.10.4.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server or an alternative data producer such as A-ADRF, A-DCCF, VAL server, or SEALDD. |

###### 7.10.4.2.3.4	Resource Custom Operations

None.

* * * Next Change * * * *

##### 7.10.4.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.4.4.1-1 specifies the data types defined specifically for the SS_ADAE_LocationAccuracyAnalytics API service.

Table 7.10.4.4.1-1_SS_ADAE_LocationAccuracyAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| LocAccurSubs | 7.10.4.4.2.2 | Subscription to the location accuracy analytics event |  |
| LocAccurNotif | 7.10.4.4.2.3 | Notification information of the location accuracy analytics event |  |

Table 7.10.4.4.1-2 specifies data types re-used by the SS_ADAE_LocationAccuracyAnalytics API service:

Table 7.10.4.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.3.3 | Type of analytics for the event of the VAL application performance analytics. |  |
| DurationSec | 3GPP TS 29.122 [3] | Represents a period of time in units of seconds. |  |
| LocationArea | 3GPP TS 29.122 [3] | Represents location information. |  |
| UeMobility | 3GPP TS 29.520 [33] | Represents UE Mobility and route information |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Indicate either VAL User ID or VAL UE ID. |  |

* * * Next Change * * * *

###### 7.10.4.4.2.2	Type: LocAccurSubs

Table 7.10.4.4.2.2-1: Definition of type LocAccurSubs

| Attribute name | Data type | P |  | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| subscriberId | string | M | 1 |  | Identity of the VAL server subscribing to the location accuracy performace analytics event. |  |
| analyticsId | string | M | 1 |  | Identity of the analytics if the subsctipion is for location accuracy performance analytics event. |  |
| analyticsType | AnalyticsType | M |  | 1 | Identity the type of the location accuracy analytics |  |
| valUesIds | array(ValTargetUe) | M |  | 1..N | A list of identities of one or more VAL UEs, whose location accuracy analytics are subscribed to. |  |
| accuracyLevel | string | M |  | 1 | Represents the desired level of accuracy of the requested location information. The value shall be between 0.01 and 1.00 with a step size of 0.01, represented as string. If not present, accuracy level 1.00 applies. Pattern: '^[0]\.[0-9]{2}¦[1.00]$' |  |
| valServiceId | string | O |  | 0..1 | The identifier of the VAL service for which location accuracy analytics is requested. |  |
| confidenceLevel | string | O |  | 0..1 | Defines the accuracy level for the location analytics analytics if the location accuracy analytics is for prediction. The value shall be between 0.01 and 1.00 with a step size of 0.01, represented as string. If not present, confidence level 1.00 applies. Pattern: '^[0]\.[0-9]{2}¦[1.00]$' |  |
| area | LocationArea | O |  | 0..1 | The geographical or service area, to which the location accuracy analytics subscription is applied. |  |
| timeInterval | DurationSec | O |  | 0..1 | The time interval as the start and the end time, to which the location accuracy analytics subscription is applied. |  |
| ueMobs | array(UeMobility) | O |  | 1..N | Mobility and route information on the one or more target VAL UE. |  |

Editor's Note:	Detailed definitions for data types are FFS.

* * * Next Change * * * *

###### 7.10.4.4.2.3	Type: LocAccurNotif

Table 7.10.4.4.2.3-1: Definition of type LocAccurNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| outputId | string | M | 1 | Identifier of the event output. |  |
| analyticsOutputs | array(string) | M | 1..N | Location accuracy analytics for prediction or statistics depending on the type. |  |
| analyticsType | AnalyticsType | O | 0..1 | Identity the type of the location accuracy analytics |  |
| confidenceLevel | string | O | 0..1 | Provides accuracy level if the location accuracy analytics is prediction. The value shall be between 0.01 and 1.00 with a step size of 0.01, represented as string. If not present, confidence level 1.00 applies. Pattern: '^[0]\.[0-9]{2}¦[1.00]$' |  |

Editor's Note:	Detailed definitions for data types are FFS.

* * * End of Changes * * * *