3GPP TSG-CT3 Meeting #132e	C3-240047

# e-meeting, 22nd – 24th November 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on defining the openAPI of the NSCE_NSDiagnostics API

Spec:	3GPP TS 29.435 V 0.1.1

Agenda item:	18.49 (NSCALE)

Document for:	Agreement

1. Introduction

As specified in clause 9.14 of TS 23.435, the NSCE_NSDiagnostics Service API was defined in order to support the functionality of receiving Network Slice Diagnostics for specified events by the VAL server.

The stage 3 definition of this API in this specification needs hence to be started.

2. Reason for Change

Update the definition of the open API clause of the new NSCE_NSDiagnostics Service API in the new TS 29.435.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.435 V 0.1.1.

# A.15	NSCE_NSDiagnostics API

openapi: 3.0.0

info:

title: NSCE Server Network Slice Diagnostics Service

description: |

API for NSCE Server Network Slice Diagnostics Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.435 V0.1.1; Service Enabler Architecture Layer for Verticals (SEAL);

Network Slice Capability Exposure (NSCE) Server Service(s); Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.435/

servers:

- url: '{apiRoot}/nsce-nsd/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request:

post:

summary: request the network slice diagnostics.

operationId: RequestNSDiagnostics

tags:

- Network Slice Diagnostics Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NwSliceDiagReq'

responses:

'200':

description: The operation is successful.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/NwSliceDiagRep'

minItems: 1

description: Network slice diagnostics reporting.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

NwSliceDiagReq:

description: >

Represents the information associated with requested network slice diagnostics.

type: object

properties:

nwSliceDiagId:

type: string

servDgradInfos:

type: array

items:

$ref: '#/components/schemas/ServDgradInfo'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- nwSliceDiagId

- servDgradInfos

NwSliceDiagRep:

description: >

Represents the requested network slice diagnostics report.

type: object

properties:

nwSliceDiagId:

type: string

startTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

dataReport:

type: array

items:

$ref: '#/components/schemas/DataReport'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- nwSliceDiagId

ServDgradInfo:

description: >

Represents the degraded service information.

type: object

properties:

nwSliceDiagId:

type: string

errors:

type: array

items:

$ref: '#/components/schemas/Error'

minItems: 1

snssai:

$ref: 'TS29435_NSCE_PolicyManagement.yaml#/components/schemas/NetSliceId'

ueIds:

type: array

items:

type: string

minItems: 1

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

startTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- nwSliceDiagId

- errors

- snssai

- startTime

- stopTime

DataReport:

description: >

Represents the reported data.

type: object

properties:

error:

$ref: '#/components/schemas/Error'

dataType:

type: string

dataOutput:

type: array

items:

type: string

minItems: 1

#

# ENUMS

#

Error:

anyOf:

- type: string

enum:

- COMMUNICATION_ERROR

- RTT_ABOVE_LIMIT

- QOS_DOWNGRADE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the errors causing service degradation.

Possible values are:

- COMMUNICATION_ERROR: Indicates that the service degradation is due to detected

communication error.

- RTT_ABOVE_LIMIT: Indicates that the service degradation is due to packet round trip

time exceeding an upper threshold limit.

- QOS_DOWNGRADE: Indicates that the service degradation is due to Quality of Service

being downgraded.

* * * * End of changes * * * *