3GPP TSG-CT3 Meeting #132e	C3-240049

# e-meeting, 22nd – 24th November 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on defining the resources and data model clauses of the NSCE_NSAllocation API

Spec:	3GPP TS 29.435 V 0.1.1

Agenda item:	18.49 (NSCALE)

Document for:	Agreement

1. Introduction

As specified in clause 9.14 of TS 23.435, the NSCE_NSAllocation_Request Service API was defined in order to support the functionality of Network slice allocation in a NSaaS framework by the NSCE server based on the VAL server provided Network slice service profile.

The stage 3 definition of this API in this specification needs hence to be started.

2. Reason for Change

Update the definition of the new NSCE_NSAllocation_Request API resources and data model in the new TS 29.435.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.435 V 0.1.1.

* * * * Start of changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[8]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	IETF RFC 9113: "HTTP/2".

[11]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[12]	IETF RFC 9457: "Problem Details for HTTP APIs".

[13]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[14]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[15]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification".

[17]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

* * * * Next changes * * * *

## 6.18	NSCE_NSAllocation API

### 6.18.1	Introduction

The request URI used in each HTTP request from the VAL server towards the NSCE server shall have the structure as defined in clause 6.5 of 3GPP TS 29.549 [15] with the following clarifications:

-	The <apiName> shall be "nsce-nsa".

-	The <apiVersion> shall be "v1".

-	The <custOpName> shall be set as described in clause 6.18.1.3.

### 6.18.2	Usage of HTTP

### The provisions of clause 6.3 of 3GPP TS 29.549 [15] shall apply for the NSCE_NSAllocation API.

#### 6.18.3	Resources

There are no resources defined for this API in this release of the specification.

#### 6.18.4	Custom Operations without associated resources

##### 6.18.4.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.18.4.1-1 depicts the resource URIs structure for the NSCE_NSAllocation API.

Figure 6.18.4.1-1: Custom operation URI structure of the NSCE_NSAllocation API

Table 6.18.4.1-1 provides an overview of the custom operation and applicable HTTP methods.

Table 6.18.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Request | /request | POST | Request the network slice allocation. |

##### 6.18.4.2	Operation: Request

###### 6.18.4.2.1	Description

The custom operation allows a VAL server to request network slice allocation from the NSCE server.

###### 6.18.4.2.2	Operation Definition

This operation shall support the request and response data structures and response code specified in table 6.18.4.2.2-1 and table 6.18.4.2.2-2.

Table 6.18.4.2.2-1: Data structures supported by the POST Request Body for this operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NwSliceAllocReq | M | 1 | Parameters to request network slice allocation |

Table 6.18.4.2.2-2: Data structures supported by the POST Response Body for this operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NwSliceAllocResp | M | 1 | 200 OK | The successful response to the request, including the network slice allocation information. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing an alternative NSCE server to which the request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing an alternative NSCE server to which the request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.18.4.2.2-3: Headers supported by 307 Response Code for this operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing an alternative NSCE server to which the request should be redirected. |

Table 6.18.4.2.2-4: Headers supported by 308 Response Code for this operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing an alternative NSCE server to which the request should be redirected. |

#### 6.18.5	Notifications

None.

#### 6.18.6	Data Model

##### 6.18.6.1	General

This clause specifies the application data model supported by the API.

Table 6.18.6.1-1 specifies the data types defined specifically for the NSCE_NSAllocation API service.

Table 6.18.6.1-1: NSCE_NSAllocation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| NwSliceAllocReq | 6.18.6.2.2 | Represents the information associated with requested network slice allocation. |  |
| NwSliceAllocResp | 6.18.6.2.3 | Represents the network slice allocation information. |  |

Table 6.18.6.1-2 specifies data types re-used by the NSCE_NSAllocation API service.

Table 6.18.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| LocationArea5G | 3GPP TS 29.122 [2] | Represents a location area. |  |
| NetSliceId | 6.3.6.2.15 | Represents the identification information of a network slice. |  |
| Snssai | 3GPP TS 29.571 [16] | Represents the S-NSSAI. |  |
| ServiceProfile | 3GPP TS 28.541 [17] | Represents the Network slice service profile. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Used to negotiate the applicability of the optional features. |  |
| NOTE: Properties marked with a feature as defined in clause 5.14.6 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [2]. If no feature is indicated, the related property applies for all the features. |  |  |  |

##### 6.18.6.2	Structured Data Types

###### 6.18.6.2.1	Introduction

###### 6.18.6.2.2	Type: NwSliceAllocReq

Table 6.18.6.2.2-1: Definition of type NwSliceAllocReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServiceId | string | M | 1 | Represents the VAL service identifier. |  |
| valUeIds | array(string) | O | 1..N | Represents the list of VAL UEs ID. |  |
| locArea | LocationArea5G | M | 1 | Identification of location area to which the request applies. (NOTE) |  |
| sliceId | NetSliceId | O | 0..1 | Represents the requested slice identifier. |  |
| nwSliceServProf | ServiceProfile | O | 0..1 | Represents the requested Network slice service requirements. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.18.8. This attribute shall be present only when feature negotiation needs to take place. |  |
| NOTE: The network area information within the “locaArea” attribute shall not be included. |  |  |  |  |  |

###### 6.18.6.2.3	Type: NwSliceAllocResp

Table 6.18.6.2.3-1: Definition of type NwSliceAllocResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| snssai | snssai | M | 1 | Represents the allocated slice identifier |  |
| nwSliceAllocProf | ServiceProfile | M | 1 | Represents the allocated Network slice attributes |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.18.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 6.18.6.3	Simple data types and enumerations

None.

#### 6.18.7	Error Handling

##### 6.18.7.1	General

HTTP error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [15].

In addition, the requirements in the following clauses shall apply.

##### 6.18.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the NSCE_NSAllocation API.

##### 6.18.7.3	Application Errors

The application errors defined for NSCE_NSAllocation API are listed in table 6.18.1.6.3-1.

Table 6.18.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.18.8	Feature Negotiation

The optional features listed in table 6.18.8-1 are defined for the NSCE_NSAllocation API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [15].

Table 6.14.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.14.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [15] shall apply for the NSCE_NSAllocation API.

* * * * End of changes * * * *