3GPP TSG CT WG3 Meeting #132e	C3-240062r1

# Electronic, 22 - 24 January, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.122 | CR | 0788 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of feature dependency on QoSMonitoring_5G for EnQoSMon |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2024-1-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the procedure description in 4.4.9.2 of 29.522, the support of QoSMonitoring_5G feature is the pre-condition of the support of EnQoSMon feature. Furthermore, QOS_MONITORING event is only applicable to QoSMonitoring_5G feature as defined in Table 5.14.2.2.3-1, and QosMonitoringInformation and QosMonitoringReport data types are only applicable for QoSMonitoring_5G feature as defined in Table 5.14.2.1.1-2. Therefore, EnQoSMon feature has dependency on QoSMonitoring_5G feature. This dependency needs to be indicated in 5.14.4. In addtion, the description of reqQosMonParams attribute in QosMonitoringInformation data type needs to be extended to cover congestion information and date rate monitoring. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indicate in 5.14.4 that EnQoSMon feature requires that QosMonitoring_5G feature is supported. Update the description of reqQosMonParams attribute in 5.14.2.1.6 and 5.14.2.1.6 to cover congestion information and date rate monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification may lead to implementation mistakes and interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.2.1.6, 5.14.2.1.7, 5.14.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have any impact in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.14.2.1.6	Type: QosMonitoringInformation

Table 5.14.2.1.6-1: Definition of type QosMonitoringInformation

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| reqQosMonParams | array(RequestedQosMonitoringParameter) | 1..N | Indicates the QoS information to be measured, e.g.UL packet delay, DL packet delay and/or round trip packet delay between the UE and the UPF is to be monitored when the QoS Monitoring for packet delay is enabled for the service data flow. If the "EnQoSMon" feature is supported, the indication of QoS monitoring for congestion (e.g., the UL and/or the DL congestion indication) or data rate (e.g., the UL and/or the DL data rate indication) may also be provided. |  |
| repFreqs | array(ReportingFrequency) | 1..N | Indicates the frequency for the reporting, such as event triggeredand/or periodic. |  |
| repThreshDl | Uinteger | 0..1 | Unsigned integer identifying a threshold in units of milliseconds for DL packet delay for packet delay or packet delay variation measurement reports. It shall be present when the "reqQosMonParams" attribute includes "DOWNLINK". |  |
| repThreshUl | Uinteger | 0..1 | Unsigned integer identifying a threshold in units of milliseconds for UL packet delay for packet delay or packet delay variation measurement reports. It shall be present when the "reqQosMonParams" attribute includes "UPLINK". |  |
| repThreshRp | Uinteger | 0..1 | Unsigned integer identifying a threshold in units of milliseconds for round trip packet delay for packet delay or packet delay variation measurement reports. It shall be present when the "reqQosMonParams" attribute includes "ROUND_TRIP". |  |
| conThreshDl | Uinteger | 0..1 | Indicates the downlink threshold for congestion reporting, i.e. for the reporting of the received ECN marking percentage for DL. Only applicable when the "repFreqs" attribute is not supplied or the "repFreqs" is set to "EVENT_DETECTION". Minimum = 0%. | EnQoSMon |
| conThreshUl | Uinteger | 0..1 | Indicates the uplink threshold for the congestion reporting, i.e. for the reporting of the received ECN marking percentage for UL. Only applicable when the "repFreqs" attribute is not supplied or the "repFreqs" is set to "EVENT_DETECTION". Minimum = 0%. | EnQoSMon |
| waitTime | DurationSec | 0..1 | Indicates the minimum waiting time between subsequent reports. It shall be present when the "repFreqs" attribute includes "EVENT_TRIGGERED". |  |
| repPeriod | DurationSec | 0..1 | Indicates the time interval between successive reporting. It shall be present when the "repFreqs" attribute includes "PERIODIC". If the feature "PacketDelayFailureReport" is supported, it also indicates the time interval at which a measurement failure needs to be reported if no measurement result is provided. It shall be present when the "repFreqs" attribute includes "PERIODIC" or "EVENT_TRIGGERED". |  |
| repThreshDatRateDl | BitRate | 0..1 | Indicates the bit rate threshold for the DL. It shall be present when the "reqQosMonParams" attribute includes "DOWNLINK_DATA_RATE". | EnQoSMon |
| repThreshDatRateUl | BitRate | 0..1 | Indicates the bit rate threshold for the UL. It shall be present when the "reqQosMonParams" attribute includes "UPLINK_DATA_RATE". | EnQoSMon |
| consDataRateThrDl | BitRate | 0..1 | Indicates the Downlink Consolidated Data Rate Threshold. | ListUE_5G |
| consDataRateThrUl | BitRate | 0..1 | Indicates the Uplink Consolidated Data Rate Threshold. | ListUE_5G |

Editor’s Note: It is FFS whether the event reporting frequency applies for congestion (ECN marking percentage) information.

# *** 2nd Change ***

5.14.2.1.7	Type: QosMonitoringInformationRm

This type represents a QoS Monitoring Information which is defined in clause 5.14.2.1.7 but defined with "nullable: true" property so it can be removed in "JSON Merge Patch", as defined in IETF RFC 7396 [39]. It shall comply with the provisions defined in table 5.14.2.1.7-1.

Duration and volume are also removable in "JSON Merge Patch".

Table 5.14.2.1.7-1: Definition of type QosMonitoringInformationRm

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| reqQosMonParams | array(RequestedQosMonitoringParameter) | 0..N | Indicates the QoS information to be measured, e.g.UL packet delay, DL packet delay and/or round trip packet delay between the UE and the UPF is to be monitored when the QoS Monitoring for packet delay is enabled for the service data flow. If the "EnQoSMon" feature is supported, the indication of QoS monitoring for congestion (e.g., the UL and/or the DL congestion indication) or data rate (e.g., the UL and/or the DL data rate indication) may also be provided. |  |
| repFreqs | array(ReportingFrequency) | 0..N | Indicates the frequency for the reporting, such as event triggeredand/or periodic. |  |
| repThreshDl | UintegerRm | 0..1 | Unsigned integer identifying a threshold in units of milliseconds for DL packet delay for packet delay or packet delay variation measurement reports. It shall be present when the "reqQosMonParams" attribute includes "DOWNLINK". |  |
| repThreshUl | UintegerRm | 0..1 | Unsigned integer identifying a threshold in units of milliseconds for UL packet delay for packet delay or packet delay variation measurement reports. It shall be present when the "reqQosMonParams" attribute includes "UPLINK". |  |
| repThreshRp | UintegerRm | 0..1 | Unsigned integer identifying a threshold in units of milliseconds for round trip packet delay for packet delay or packet delay variation measurement reports. It shall be present when the "reqQosMonParams" attribute includes "ROUND_TRIP". |  |
| conThreshDl | UintegerRm | 0..1 | Indicates the downlink threshold for congestion reporting, i.e. for the reporting of the received ECN marking percentage for DL. Only applicable when the "repFreqs" attribute is not supplied or the "repFreqs" is set to "EVENT_DETECTION". Minimum = 0%. | EnQoSMon |
| conThreshUl | UintegerRm | 0..1 | Indicates the uplink threshold for congestion reporting, i.e. for the reporting of the received ECN marking percentage for UL. Only applicable when the "repFreqs" attribute is not supplied or the "repFreqs" is set to "EVENT_DETECTION". Minimum = 0%. | EnQoSMon |
| waitTime | DurationSecRm | 0..1 | Indicates the minimum waiting time between subsequent reports. It shall be present when the "repFreqs" attribute includes "EVENT_TRIGGERED". |  |
| repPeriod | DurationSecRm | 0..1 | Indicates the time interval between successive reporting. It shall be present when the "repFreqs" attribute includes "PERIODIC". If the feature "PacketDelayFailureReport" is supported, it also indicates the time interval at which a measurement failure needs to be reported if no measurement result is provided. It shall be present when the "repFreqs" attribute includes "PERIODIC" or "EVENT_TRIGGERED". |  |
| repThreshDatRateDl | BitRateRm | 0..1 | Indicates the bit rate threshold for the DL. It shall be present when the "reqQosMonParams" attribute includes "DOWNLINK_DATA_RATE". | EnQoSMon |
| repThreshDatRateUl | BitRateRm | 0..1 | Indicates the bit rate threshold for the UL. It shall be present when the "reqQosMonParams" attribute includes "UPLINK_DATA_RATE". | EnQoSMon |
| consDataRateThrDl | BitRateRm | 0..1 | Indicates the Downlink Consolidated Data Rate Threshold. | ListUE_5G |
| consDataRateThrUl | BitRateRm | 0..1 | Indicates the Uplink Consolidated Data Rate Threshold. | ListUE_5G |

Editor’s Note: It is FFS whether the event reporting frequency applies for congestion (ECN marking percentage) information.

# *** 3rd Change ***

5.14.4	Used Features

The table below defines the features applicable to the AsSessionWithQoS API. Those features are negotiated as described in subclause 5.2.7.

Table 5.14.4-1: Features used by AsSessionWithQoS API

| Feature Number | Feature | Description |
| --- | --- | --- |
| 1 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 5.2.5.4. This feature requires that the Notification_test_event featute is also supported. |
| 2 | Notification_test_event | The testing of notifications connections is supported according to clause 5.2.5.3. |
| 3 | EthAsSessionQoS_5G | Setting up required QoS for Ethernet UE. This feature may only be supported in 5G. |
| 4 | MacAddressRange_5G | Indicates the support of a set of MAC addresses with a specific range in the traffic filter. This feature may only be supported in 5G. |
| 5 | AlternativeQoS_5G | Indicates the support of alternative QoS requirements and the QoS notification (i.e. whether the QoS targets for SDF(s) are not guaranteed or guaranteed again). This feature may only be supported in 5G. |
| 6 | QoSMonitoring_5G | Indicates the support of QoS Monitoring functionality and the report for packet delay monitoring. This feature may only be supported in 5G. |
| 7 | DisableUENotification_5G | Indicates the support of disabling QoS flow parameters signalling to the UE when the SMF is notified by the NG-RAN of changes in the fulfilled QoS situation. This feature may only be supported in 5G. This feature requires that the AlternativeQoS_5G feature is also supported. |
| 8 | TSC_5G | Indicates the support of Time Sensitive Communication. This feature may only be supported in 5G. |
| 9 | AppId | Indicates the support of dynamically providing the Application Identifier via the API. |
| 10 | ExposureToEAS | This feature indicates the support of direct notification in 5GC. This feature requires that the QoSMonitoring_5G feature is also supported. |
| 11 | enNB | Indicates the support of enhancements to the northbound interfaces. |
| 12 | AltQosWithIndParams_5G | This feature indicates the support of provisioning Alternative Service Requirements with individual QoS parameters. This feature requires that the AlternativeQoS_5G feature is also supported. |
| 13 | EnEthAsSessionQoS_5G | Indicates the support of required QoS for Ethernet UE, allowing to indicate separately different UL and/or DL Ethernet flows. This feature may only be supported in 5G. |
| 14 | enNB_5G | Indicates the support of enhancements to the northbound interfaces and only applicable to 5G. |
| 15 | PacketDelayFailureReport | Indicates the support of packet delay failure report as part of QoS Monitoring procedures. This feature requires that QoSMonitoring_5G is supported. This feature may only be supported in 5G. |
| 16 | ToSTC_5G | Indicates the support of Type of Service or Traffic Class. This feature may only be supported in 5G. |
| 17 | EnTSCAC | Indicates the support of extensions to TSCAC and the RAN feedback for BAT offset and adjusted periodicity. This feature may only be supported in 5G, and requires that the TSC_5G feature is also supported. |
| 18 | AltQoSProfilesSupportReport | This feature indicates the support of the report of whether Alternative QoS parameters are supported by the access network. This feature requires that AlternativeQoS_5G and/or AltQosWithIndParams_5G features are also supported. |
| 19 | ExtQoS_5G | This feature indicates the support of extended QoS parameters. This feature may only be supported in 5G. |
| 20 | MultiMedia | Indicates the support for multi-modal or multimedia flows for single UE and multiple UE. This feature may only be supported in 5G. This feature may be used in eXtend Reality (XR) use cases. |
| 21 | ExtErrors | Indicates the support of additional application errors related to authorization or PDU Session availability. |
| 22 | QoSTiming_5G | This feature indicates the support of QoS timing information for the transfer and support of data transmission (e.g., AI/ML transmission). This feature may only be supported in 5G. |
| 23 | ListUE_5G | Indicates the support for the list of UEs This feature may only be supported in 5G. |
| 24 | GMEC_5G | This feature indicates the support of Generic Group Management Exposure and Communication related enhancements. The following functionalities are supported: - Support AF requested QoS for a UE or group of UE(s) not identified by the UE address. This feature may only be supported in 5G. |
| 25 | PDUSetHandling | This feature indicates the support of PDU Set handling. This feature may be used for eXtended Reality (XR) and interactive media services. This feature may only be supported in 5G. |
| 26 | RTLatency | This feature indicates the support of Round-Trip latency. This feature may be used for eXtended Reality (XR) and interactive media services. This feature may only be supported in 5G. |
| 27 | EnQoSMon | This feature indicates the support of enhanced QoS monitoring functionality, i.e. the report of the congestion information, and/or, the RTT delay over two QoS flows, and/or, the data rate information, and/or, the Packet Delay Variation monitoring. This feature requires that QoSMonitoring_5G is supported. This feature may only be supported in 5G. |
| 28 | PowerSaving | This feature indicates the support of the Power Saving for different traffic measurement. This feature may only be supported in 5G. |
| 29 | L4S | This feature indicates the support of the AF indication of ECN marking for L4S support. This feature may only be supported in 5G. |
| Feature: A short name that can be used to refer to the bit and to the feature, e.g. "Notification". Description: A clear textual description of the feature. |  |  |

Editor's Note:	Whether and/how to indicate the support of end of burst indication, and provision the flow periodicity information within the Power Saving feature is FFS.

*** End of Changes ***