3GPP TSG-CT WG4 Meeting #132	C4-255096

Dallas, US; 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0388 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Convey the updated UE user plane positioning capabilities in UP Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP TSG CT WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS 24.501, the UE may update its user plane positioning capabilities (e.g. support for multiple LCS secured user plane connections) during mobility or periodic registration updates by setting the MLCSUP and LCS-UPP bits in the 5GMM capability IE of the REGISTRATION REQUEST message. "5.5.1.3.2 Mobility and periodic registration update initiation The UE in state 5GMM-REGISTERED shall initiate the registration procedure for mobility and periodic registration update by sending a REGISTRATION REQUEST message to the AMF, a) when the UE detects that the current TAI is not in the list of tracking areas that the UE previously registered in the AMF; b) when the periodic registration updating timer T3512 expires in 5GMM-IDLE mode and the UE is not registered for emergency services (see subclause 5.3.7); c) when the UE receives a CONFIGURATION UPDATE COMMAND message indicating "registration requested" in the Registration requested bit of the Configuration update indication IE as specified in subclauses 5.4.4.3; d) when the UE in state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE either receives a paging or the UE receives a NOTIFICATION message with access type indicating 3GPP access over the non-3GPP access for PDU sessions associated with 3GPP access; NOTE 1: As an implementation option, MUSIM UE is allowed to not respond to paging based on the information available in the paging message, e.g. voice service indication. e) upon inter-system change from S1 mode to N1 mode and if the UE previously had initiated an attach procedure or a tracking area updating procedure when in S1 mode; f) when the UE receives an indication of "RRC Connection failure" from the lower layers and does not have signalling pending (i.e. when the lower layer requests NAS signalling connection recovery); g) when the UE changes the 5GMM capability or the S1 UE network capability or both; " As agreed in C4-254359, after AMF change, the target AMF shall inform the serving LMF using the UP Configuration procedure. However, TS 29.572 currently does not define any means for the AMF to convey the updated UE user plane positioning capabilities to the LMF during the UP Configuration service operation. This misalignment prevents the LMF from being aware of the UE’s latest positioning capabilities after mobility or capability update, which can impact LCS-UPP operation continuity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the Nlmf_Location_UPConfig Request data type to include an optional IE conveying the updated UeUpPositioningCapabilities of the UE. Specify that the AMF shall include this IE when updated UE user plane positioning capabilities are available (e.g. after registration update). Clarify that the LMF receiving the UPConfig shall update the UE context accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | the LMF operates using outdated UE user plane capability information after AMF relocation or UE capability update, leading to mismanagement of LCS-UPP connections and failure to support multiple user plane connections as intended. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.9.2, 6.1.6.2.52, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29572_Nlmf_Location.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 5.2.2.9.2	Configure LCS-UP connection

This procedure allows a consumer NF to set up, modify or terminate a secure LCS-UP connection for a target UE.

Figure 5.2.2.9.2-1: LCS-UP connection

1.	The NF service consumer shall send an HTTP POST request to the resource URI associated with the "configure-up" custom operation to set up, modify or terminate LCS-UP connection. The request content shall include a UP notify callback URI, a notification correlation ID, the UE identity, and, if available, the UE user plane positioning capabilities.

2a.	On success, "204 No Content" shall be returned.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.4.7.2-2 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.4.7.2-2.

==============Next change==============

##### 6.1.6.2.52	Type: UpConfig

Table 6.1.6.2.52-1: Definition of type UpConfig

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upNotifyCallBackUri | Uri | M | 1 | Callback URI of the NF Service Consumer |
| notifCorrelationId | CorrelationID | M | 1 | Notification correlation ID. |
| supi | Supi | C | 0..1 | SUPI (NOTE) |
| gpsi | Gpsi | C | 0..1 | GPSI (NOTE) |
| amfReallocationInd | boolean | O | 0..1 | Indicates AMF reallocation indication. When present, it shall be set as follows: - true: AMF reallocated. - false (default): AMF not reallocated. |
| lcsUpConnectionInd | LcsUpConnectionInd | O | 0..1 | LCS-UP connection indication |
| targetLMFId | LMFIdentification | O | 0..1 | Target LMF identifier. |
| ueUpPosCaps | array(UeUpPositioningCapabilities) | O | 1..N | This IE shall be present if available and indicate the user plane positioning capabilities supported by the UE. See clause 5.5.1.3 of 3GPP TS 24.501 [22]. |
| NOTE: At least one of the supi or gpsi shall be present in the UpConfig structure. |  |  |  |  |

==============Next change==============

# A.2	Nlmf_Location API

openapi: 3.0.0

info:

version: '1.4.0-alpha.5'

title: 'LMF Location'

description: |

LMF Location Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.572 V19.4.0; 5G System; Location Management Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.572/'

servers:

- url: '{apiRoot}/nlmf-loc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

[Skipped for clarity]

UpConfig:

description: UP Config

type: object

required:

- upNotifyCallBackUri

- notifCorrelationId

anyOf:

- required: [supi]

- required: [gpsi]

properties:

upNotifyCallBackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrelationId:

$ref: '#/components/schemas/CorrelationID'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

amfReallocationInd:

type: boolean

default: false

lcsUpConnectionInd:

$ref: '#/components/schemas/LcsUpConnectionInd'

targetLMFId:

$ref: '#/components/schemas/LMFIdentification'

ueUpPosCaps:

type: array

items:

$ref: '#/components/schemas/UeUpPositioningCapabilities'

minItems: 1

[Skipped for clarity]

==============End of change==============