3GPP TSG-CT WG4 Meeting #133	C4-260111

Goa, India; 09th – 13th February 2026

Source:	NTT DOCOMO

Title:	Enhancement of Solution #3 with post-failure phase consideration

Spec:	3GPP TR 29.867 v0.4.0

Agenda item:	21.1

Document for:	Approval

1. Introduction

<Introduction part (optional)>

2. Reason for Change

2.1 Background

Solution #3 (SMF-initiated PDU session release) provides a mechanism to suppress IMS registration attempts by releasing the PDU session with a back-off timer.

However, when it comes to using "back-off timer" mechanism, a critical issue exists in the post-failure phase. The current specification (TS24.501) defines the UE's re-connection behavior upon timer expiry as a "may" requirement. The real-world implementation data shows the significant discrepancies where UE does not consistently initiate the recovery upon back-off timer expiry. This lack of deterministic behaviour prevents network operators from managing a predictable restoration of services, making it impossible to build a reliable network operation plan for post-failure scenarios.

2.2 Solution overview

To complement Solution #3, this solution introduces a new configuration parameter within the NAS Management Object (MO) to enforce deterministic UE behaviour.

By configuring the UE with an "Autonomous Re-establishment" parameter, the network can ensure that the UE must initiate the PDU session re-establishment immediately upon the expiry of the back-off timer received in the PDU SESSION RELEASE COMMAND (with e.g., 5GSM cause #67).

With this solution, network operator can provide their operator policy to the UE and transforms current implementation-dependent UE's re-connection action into a controllable action, allowing operators to manage both congestion suppression (failure phase) and network recovery (post-failure phase) as a comprehensive set of operations.

NOTE that delivery of 5GSM cause #67 is not only via PDU SESSION RELEASE COMMAND, but also other messages according to existing TS24.501. And also, the SM congestion control has three types of congestion control which are DNN based congestion control, DNN and S-NSSAI based congestion control, and S-SSSAI based congestion control and each has a dedicated 5GSM cause. This solution should be applicable to the scenario for other 5GSM causes also (not mentioned here specifically).

2.2 Procedure outline

- Configuration via NAS MO: The operator pre-configures the UE with the IMS Resiliency exceptional handling parameter (e.g., /<X>/IMSResil) via the NAS MO. When this parameter is set to "1" (active), the UE is instructed to perform autonomous re-establishment upon back-off timer expiry

- Failure Detection and Suppression: Following the detection of P-CSCF failure, the SMF initiates e.g., a PDU session release for SIP signaling, including e.g., the 5GSM cause #67 and a Back-off timer, as described in Solution #3.

- Back-off Period: The UE starts the back-off timer and refrains from re-trying the PDU session establishment for the specified duration.

- Deterministic Recovery: Upon expiry of the back-off timer, the UE checks the NAS MO setting:

- If the exceptional handling (IMSResil) is active, the UE shall autonomously initiate a PDU SESSION ESTABLISHMENT REQUEST for the same DNN/S-NSSAI for the P-CSCF discovery and the IMS Initial Registration follows.

- If the setting is inactive, the re-establishment remains dependent on upper-layer application triggers (default legacy behaviour).

Note that the outline above assumes that this exceptional handling (autonomous re-establishment) is applied when the UE has no available P-CSCF locally (UE exhausted all P-CSCF addresses available locally), but the operator may further refine the behaviour by using specific MO leaves (which may be different to above) to differentiate whether this handling applies regardless of the P-CSCF list usage status e.g., when the UE has not exhausted all P-CSCF addresses available locally.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 29.867 v0.4.0.

* * * First Change * * * *

6.z	Solution #z: Enhancement of Solution #3 with MO-based Recovery

6.z.1	General

Solution #3 (SMF-initiated PDU session release) provides a mechanism to suppress IMS registration attempts by releasing the PDU session with a back-off timer. However, a critical issue exists in the post-failure phase. The current specification (3GPP TS 24.501 [6]) defines the UE's re-connection behaviour upon timer expiry as a "may" requirement.

Real-world implementation data shows significant discrepancies where UEs do not consistently initiate recovery upon back-off timer expiry. This lack of deterministic behaviour prevents network operators from managing a predictable restoration of services and building a reliable network operation plan for post-failure scenarios.

This solution enhances Solution #3 by introducing a controllable recovery mechanism based on UE configuration.

6.z.2	Description

6.z.2.1	P-CSCF failure detection

The UE detects the failure of all locally stored P-CSCF addresses based on the mechanisms defined in 3GPP TS 24.229 [3] (e.g., receipt of a 504 Server Time-out, 305 Use Proxy, or the expiry of Timer F). The network (SMF) detects the P-CSCF failure as described in clause 6.3.2.1.

6.z.2.2	Configuration of Autonomous Re-establishment via NAS MO

To ensure deterministic behaviour, the UE is configured with an "Autonomous Re-establishment" parameter within the NAS Management Object. This parameter (e.g., /<X>/IMSResil) defines whether the UE is requested to initiate recovery procedures upon the expiry of a back-off timer received from the network.

The operator can set this parameter to "active" to transform the recovery phase from an implementation-dependent optional action into a controllable procedure.

6.z.2.3	Mechanism for post-failure phase

The mechanism for suppression phase is as described in Solution#3 that following the detection of P-CSCF failure, the SMF initiates a PDU session release for SIP signalling, including a 5GSM cause (e.g., #67) and a Back-off timer. The UE starts the back-off timer and refrains from re-trying the PDU session establishment for the specified duration.

The mechanism for post-failure phase is as follows:

-	About inter-system change: The UE can choose to temporarily suppress the disabling of N1 mode if the MO described above is configured or new indication in the PDU session release indicating "S1 mode recovery is not allowed" is received. This is particularly effective in such scenario where a voice-centric UE might otherwise disable N1 mode and fall back to EPS during a period of simultaneous IMS congestion in EPS, which results in unnecessary inter-system changes and delay service restoration. With this mechanism, the network operator can prevent the unnecessary inter-system change and ensure the UE remains in 5GS to perform the re-establishment upon timer expiry if necessary.

NOTE:	Detail protocol definition e.g., MO, indication, or etc can be determined during technical specification.

-	About re-establishment: Upon expiry of the back-off timer, the UE checks the NAS MO setting. If the "Autonomous Re-establishment" is active, the UE shall autonomously initiate a PDU SESSION ESTABLISHMENT REQUEST for the same DNN/S-NSSAI to trigger P-CSCF discovery and subsequent IMS Initial Registration. If the setting is inactive, the re-establishment remains dependent on upper-layer application triggers, maintaining compatibility with legacy behaviour.

6.z.3	Impacts on services, entities, and interfaces

UE:

-	Needs to support the MO parameters and implementing the logic to trigger autonomous PDU session establishment upon back-off timer expiry.

SMF:

-	No additional changes beyond the existing Solution #3 capabilities.

6.z.4	Pros

The pros of the solution in addtion to the ones captured for Solution #3:

-	Eliminates the uncertainty caused by UE's unpredictable recovery behaviour, allowing operators to execute a controllable recovery plan.

6.z.5	Cons

The cos of the solution:

-	Requires maintaining and updating MO configurations across the target UEs.

* * * Next Change * * * *

7.1	Evaluation of Solutions for Key Issue#1

A group of Solutions have been proposed for Key Issue#1 to address to prevent UE SIP Registration attempts during IMS Failures.

Solution #1 proposes leveraging existing timeout mechanism during SIP Registration procedure to trigger a registration retry delay timer in the UE. Specifically, when Timer F expires for all P-CSCFs that are in the list of P-CSCFs for a UE, the retry delay timer is activated. This delay is computed using the algorithm specified in subclause 4.5 of RFC 5626 [9]. This solution builds upon existing procedures in 3GPP TS 24.229 [3]. Recovery detection similarly relies on the retry delay mechanism. The UE detects P-CSCF recovery when it sends a new SIP REGISTER after the expiry of the retry delay time and receives a positive SIP response.

Solution #2 proposes notifying P-CSCF failure status of registered UE with PCO/ePCO. To detect P-CSCF failure and recovery, several options are introduced, leveraging either existing standardized mechanisms or implementation-specific approaches. For P-CSCF failure notification, a new IE, Request Suppression Timer is introduced within PCO/ePCO. This IE indicates that the P-CSCF is unavailable and specifies the duration during which the UE should refrain from sending SIP Registration requests. During this period, the UE does not perform IMS PDN Detach or bearer re-establishment procedures such as those triggered by a Back-Off Timer or Retry-After header. Instead, the IMS PDN session is maintained. Additionally, the solution proposes defining a new IE during PDU Session Establishment to indicate whether the UE supports this feature. The SMF can be configured with specific criteria as to when to send the failure notification to the UE taking into account, e.g., unavailability of all or a specified number of P-CSCFs. For P-CSCF recovery notification, the solution reuses the P-CSCF restoration mechanism as defined in 3GPP TS 23.380 [2].

Solution #3 proposes utilizing the network-requested PDU session release mechanism defined in 3GPP TS 24.501 [6] to release the PDU session used by the UE to send SIP signalling (e.g. PDU session to the IMS DNN) and thus preventing repeated SIP registration attempts by the UE. P-CSCF failure is detected e.g., by the SMF according to the features defined in the existing specifications such as P-CSCF Failure Detection at SMF/UPF defined in the clause 5.8.3 of 3GPP TS23.380 [2]. Upon detection, the SMF initiates the PDU session release used for SIP signalling and sets the 5GSM cause IE in the PDU SESSION RELEASE COMMAND to cause value #67. This prevents the UE from attempting SIP registration during the P-CSCF failure period. Additionally, the SMF includes a Back-off Timer IE in the PDU SESSION RELEASE COMMAND message, indicating the duration for which the UE shall refrain from re-establishing a PDU session for SIP signalling.

Solution #4 proposes leveraging UAC with access category 9 (MO IMS registration) as defined in 3GPP TS 24.501 [6]. By applying access control to UEs using this category, the barring rate can be gradually relaxed to prevent traffic surges. Since the access category 9 is limited to IMS registration signaling, this approach enables targeted barring without affecting other communication services.

Solution #z proposes an enhancement to Solution #3. While Solution #3 introduces a back-off timer to suppress immediate retries, its effectiveness is limited by the unpredictable behaviour of the UE when the back-off timer is expired, which leads to implementation-dependent and unpredictable recovery. Solution #z builds upon the congestion control mechanism of Solution #3 and introduces an "Autonomous Re-establishment" parameter in the NAS MO. This enhancement ensures that the UE autonomously initiates a PDU session establishment immediately upon back-off timer expiry for a deterministic and controllable network recovery.

The solutions are presented with pro and con in the Table 7.1-1.

Table 7.1-1 Comparison table of Solutions

|  |  | Solution#1 | Solution#2 | Solution#3 | Solution#4 | Solution#z |
| --- | --- | --- | --- | --- | --- | --- |
| Impact | Pro | No impacts to IMS Network. No impacts to the packet domain. | No impacts to IMS Network | No impact to IMS Network. Works with legacy UEs. | No impact to IMS Network and SIP Client of the UE. Works with legacy UEs. | No impacts to IMS Network |
|  | Con | The SIP Client of the UE needs to be modified, i.e. the solution relies on UE supporting the new functionality. Because the initial timer value is locally preconfigured with fixed value at UE, it does not reflect the severity of the P-CSCF failure, causing the UE to proceed with PDN establishment regardless of the network condition. | The SIP Client of the UE needs to be modified i.e. SIP registration suppression. Minimal functional enhancements to the 5GC (SMF, AMF) such as extending IEs in PCO/ePCO. Relying on a single detection method, such as Rx interface monitoring, might lead to inaccurate determination of P-CSCF failure. A bulk failure notification procedure implies sending individual notifications for all users currently registered in the P-CSCF, | This solution requires EPS readiness to accommodate fallback traffic for voice services. | Retried SIP requests from IMS registered UE cannot be prevented, resulting in remaining load. In case of roaming and network sharing including MVNO scenario, SLA or OAM level agreement may be needed. | The UE needs to support the MO parameters and implementing the logic to trigger autonomous PDU session establishment upon back-off timer expiry. |
| Access dependency |  | Access independent, it can support 5G, 4G and all other accesses. | Covers 5G | Covers 5G | Covers 5G | Covers 5G |

Editor's note:	This clause is subject to revision in response to input from SA2, CT1 and CT3.

* * * End of Changes * * * *