3GPP TSG-CT WG4 Meeting #123	C4-242325

# Hyderabad, India; 27th – 31t May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.500 | CR | 0433 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Exchanging data or analytics between PLMNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Excerpts from TS 23.288 Based on operator's policy and local regulations (e.g. privacy), data or analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN). In a PLMN, an NWDAF is used as exchange point to exchange analytics and to collect input data for analytics with other PLMNs. […] NOTE 4: Interactions between RE-NWDAFs of different PLMNs may be via SEPPs, which are not depicted in the architecture for the sake of clarity. The highlighted quote requires that SEPPs between PLMNs to be able to understand the operator's policy each other and determine to allow the related for exchanging data or analytics traffic to be sent over N32. Manual configuration of operator's policy to SEPP for every partner operator increases operational cost, and therefore requires efficient mechanism. Therefore, explicit exchange of the policy via signaling is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two new values are defined for the custom header, 3gpp-Sbi-Interplmn-Purpose. (NETWORK_OPERATION_DATA_EXCHANGE, NETWORK_OPERATION_DATA_EXCHANGE_TEST). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Manual configuration of operator's policy is not practical, and without adequate configuration in SEPP exchange of information related to data/analytics may be filtered/blocked by the SEPP unintentionally (due to unintended default filtering policy). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.3.11, D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1) Cover page -Reason for change: spelling correction (oprator's -> operator's) -Summary of change: rename the added values(DATA_ANALYTICS_EXCHANGE -> NETWORK_OPERATION_DATA_EXCHANGE) == -5.2.3.3.11: rename the added values -D.2: complemnt header description & rename the added values |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2.3.3.11	3gpp-Sbi-Interplmn-Purpose

The header contains the intended purpose for inter-PLMN signaling. See clauses 6.14.

The encoding of the header follows the ABNF as defined in IETF RFC 9110 [11].

Sbi-Interplmn-Purpose-Header = "3gpp-Sbi-Interplmn-Purpose:" OWS N32Purpose ":" OWS additional-info OWS

N32Purpose = "ROAMING"

/ "INTER_PLMN_MOBILITY"

/ "SMS_INTERCONNECT"

/ "ROAMING_TEST"

/ "INTER_PLMN_MOBILITY_TEST"

/ "SMS_INTERCONNECT_TEST"

/ "SNPN_INTERCONNECT"

/ "SNPN_INTERCONNECT_TEST"

/ "DISASTER_ROAMING"

/ "DISASTER_ROAMING_TEST"

/ "NETWORK_OPERATION_DATA_EXCHANGE"

/ "NETWORK_OPERATION_DATA_EXCHANGE_TEST"

/ token

additional-info = token

-	N32Purpose: The parameter for N32Purpose indicates the intended purpose of inter-PLMN signaling, and values specified in 3GPP TS 29.573 [27] clause 6.1.5.3.9 are used.

EXAMPLE:	3gpp-Sbi-Interplmn-Purpose: ROAMING: usecaseA

* * * Next Change * * * *

D.2	ABNF definitions (Filename: "TS29500_CustomHeaders.abnf")

; ----------------------------------------

;   RFC 5234

; ----------------------------------------

HTAB   = %x09 ; horizontal tab

LF     = %x0A ; linefeed

CR     = %x0D ; carriage return

SP     = %x20

DQUOTE = %x22 ; " (Double Quote)

DIGIT  = %x30-39 ; 0-9

ALPHA  = %x41-5A / %x61-7A ; A-Z / a-z

VCHAR  = %x21-7E ; visible (printing) characters

WSP    = SP / HTAB ; white space

CRLF   = CR LF ; Internet standard newline

HEXDIG = DIGIT / "A" / "B" / "C" / "D" / "E" / "F"

; ----------------------------------------

;   RFC 3986

; ----------------------------------------

unreserved    = ALPHA / DIGIT / "-" / "." / "_" / "~"

pct-encoded   = "%" HEXDIG HEXDIG

sub-delims    = "!" / "$" / "&" / "'" / "(" / ")" / "*" / "+" / "," / ";" / "="

pchar         = unreserved / pct-encoded / sub-delims / ":" / "@"

segment       = *pchar

segment-nz    = 1*pchar

path-abempty  = *( "/" segment )

path-absolute = "/" [ segment-nz *( "/" segment ) ]

path-rootless = segment-nz *( "/" segment )

path-empty    = 0pchar

IPvFuture     = "v" 1*HEXDIG "." 1*( unreserved / sub-delims / ":" )

dec-octet     = "25" %x30-35 / "2" %x30-34 DIGIT / "1" 2DIGIT / %x31-39 DIGIT / DIGIT

h16           = 1*4HEXDIG

ls32          = ( h16 ":" h16 ) / Ipv4address

Ipv4address   = dec-octet "." dec-octet "." dec-octet "." dec-octet

Ipv6address   = 6( h16 ":" ) ls32

/ "::" 5( h16 ":" ) ls32

/ [ h16 ] "::" 4( h16 ":" ) ls32

/ [ *1( h16 ":" ) h16 ] "::" 3( h16 ":" ) ls32

/ [ *2( h16 ":" ) h16 ] "::" 2( h16 ":" ) ls32

/ [ *3( h16 ":" ) h16 ] "::" h16 ":" ls32

/ [ *4( h16 ":" ) h16 ] "::" ls32

/ [ *5( h16 ":" ) h16 ] "::" h16

/ [ *6( h16 ":" ) h16 ] "::"

IP-literal    = "[" ( Ipv6address / IpvFuture  ) "]"

reg-name      = *( unreserved / pct-encoded / sub-delims )

host          = IP-literal / Ipv4address / reg-name

port          = *DIGIT

scheme        = ALPHA *( ALPHA / DIGIT / "+" / "-" / "." )

userinfo      = *( unreserved / pct-encoded / sub-delims / ":" )

authority     = [ userinfo "@" ] host [ ":" port ]

hier-part     = "//" authority path-abempty / path-absolute / path-rootless / path-empty

query         = *( pchar / "/" / "?" )

fragment      = *( pchar / "/" / "?" )

URI           = scheme ":" hier-part [ "?" query ] [ "#" fragment ]

; ----------------------------------------

;   RFC 5322

; ----------------------------------------

obs-FWS         = 1*WSP *( CRLF 1*WSP )

FWS             = ( [ *WSP CRLF ] 1*WSP ) / obs-FWS

obs-NO-WS-CTL   = %d1-8 / %d11 / %d12 / %d14-31 / %d127

obs-ctext       = obs-NO-WS-CTL

ctext           = %d33-39 / %d42-91 / %d93-126 / obs-ctext

obs-qp          = "\" ( %d0 / obs-NO-WS-CTL / LF / CR )

quoted-pair     = ( "\" ( VCHAR / WSP ) ) / obs-qp

ccontent        = ctext / quoted-pair / comment

comment         = "(" *( [ FWS ] ccontent ) [ FWS ] ")"

CFWS            = ( 1*( [ FWS ] comment ) [ FWS ] ) / FWS

day-name        = "Mon" / "Tue" / "Wed" / "Thu" / "Fri" / "Sat" / "Sun"

obs-day-of-week = [ CFWS ] day-name [ CFWS ]

day-of-week     = ( [ FWS ] day-name ) / obs-day-of-week

obs-day         = [ CFWS ] 1*2DIGIT [ CFWS ]

day             = ( [ FWS ] 1*2DIGIT FWS ) / obs-day

month           = "Jan" / "Feb" / "Mar" / "Apr" / "May" / "Jun"

/ "Jul" / "Aug" / "Sep" / "Oct" / "Nov" / "Dec"

obs-year        = [ CFWS ] 2*DIGIT [ CFWS ]

year            = ( FWS 4*DIGIT FWS ) / obs-year

date            = day month year

obs-hour        = [ CFWS ] 2DIGIT [ CFWS ]

hour            = obs-hour / 2DIGIT

obs-minute      = [ CFWS ] 2DIGIT [ CFWS ]

minute          = obs-minute / 2DIGIT

obs-second      = [ CFWS ] 2DIGIT [ CFWS ]

second          = obs-second / 2DIGIT

time-of-day     = hour ":" minute [ ":" second ]

obs-zone        = "UT" / "GMT" / "EST" / "EDT" / "CST" / "CDT" / "MST" / "MDT"

/ "PST" / "PDT" / %d65-73 / %d75-90 / %d97-105 / %d107-122

zone            = ( FWS ( "+" / "-" ) 4DIGIT ) / obs-zone

time            = time-of-day zone

date-time       = [ day-of-week "," ] date time [ CFWS ]

; ----------------------------------------

;   RFC 6749

; ----------------------------------------

NQCHAR = %x21 / %x23-5B / %x5D-7E

; ----------------------------------------

;   RFC 9110

; ----------------------------------------

OWS            = *( SP / HTAB )

RWS            = 1*( SP / HTAB )

tchar          = "!" / "#" / "$" / "%" / "&" / "'" / "*" / "+" / "-"

/ "." / "^" / "_" / "`" / "|" / "~" / DIGIT / ALPHA

token          = 1*tchar

BWS            = OWS

quoted-string  = DQUOTE *( qdtext / quoted-pair ) DQUOTE

qdtext         = HTAB / SP / %x21 / %x23-5B / %x5D-7E / obs-text

obs-text       = %x80-FF

date1          = day-rfc9110 SP month-rfc9110 SP year-rfc9110 ; e.g., 02 Jun 1982

day-rfc9110    = 2DIGIT

month-rfc9110  = %x4A.61.6E ; "Jan", case-sensitive

/ %x46.65.62 ; "Feb", case-sensitive

/ %x4D.61.72 ; "Mar", case-sensitive

/ %x41.70.72 ; "Apr", case-sensitive

/ %x4D.61.79 ; "May", case-sensitive

/ %x4A.75.6E ; "Jun", case-sensitive

/ %x4A.75.6C ; "Jul", case-sensitive

/ %x41.75.67 ; "Aug", case-sensitive

/ %x53.65.70 ; "Sep", case-sensitive

/ %x4F.63.74 ; "Oct", case-sensitive

/ %x4E.6F.76 ; "Nov", case-sensitive

/ %x44.65.63 ; "Dec", case-sensitive

year-rfc9110   = 4DIGIT

codings        = content-coding / "identity" / "*"

content-coding = token

weight         = OWS ";" OWS "q=" qvalue

qvalue         = ( "0" [ "." *3DIGIT ] ) / ( "1" [ "." *3"0" ] )

credentials    = auth-scheme [ 1*SP ( token68

/ [ ( "," / auth-param ) *( OWS "," [ OWS auth-param ] ) ] ) ]

auth-scheme    = token

auth-param     = token BWS "=" BWS ( token / quoted-string )

token68        = 1*( ALPHA / DIGIT / "-" / "." / "_" / "~" / "+" / "/" ) *"="

; ----------------------------------------

;   3GPP TS 29.500

;

;   Version: 18.5.0 (March 2024)

;

;   (c) 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

; ----------------------------------------

;

; Header: 3gpp-Sbi-Message-Priority

;

Sbi-Message-Priority-Header = "3gpp-Sbi-Message-Priority:" OWS

( "3" %x30-31 / %x31-32 DIGIT / DIGIT ) OWS

;

; Header: 3gpp-Sbi-Callback

;

Sbi-Callback-Header = "3gpp-Sbi-Callback:" OWS cbtype *1( ";" OWS "apiversion=" majorversion ) OWS

cbchar              = "-" / "_" / DIGIT / ALPHA

cbtype              = 1*cbchar

majorversion        = *DIGIT

;

; Header: 3gpp-Sbi-Target-apiRoot

;

Sbi-Target-ApiRoot-Header = "3gpp-Sbi-Target-apiRoot:" OWS sbi-scheme "://" sbi-authority

[ prefix ] OWS

sbi-scheme                = "https" / "http"

sbi-authority             = host [ ":" port ]

prefix                    = path-absolute

;

; Header: 3gpp-Sbi-Routing-Binding

;

Sbi-Routing-Binding-Header = "3gpp-Sbi-Routing-Binding:" OWS "bl=" blvalue

1*( ";" OWS parameter )

[ ";" OWS callback-uri-prefix ] OWS

blvalue                    = "nf-instance"

/ "nf-set"

/ "nfservice-instance"

/ "nfservice-set"

parametername              = "nfinst"

/ "nfset"

/ "nfservinst"

/ "nfserviceset"

/ "servname"

/ "backupamfinst"

/ "backupnf"

parameter                  = parametername "=" token

;

; Header: 3gpp-Sbi-Binding

;

Sbi-Binding-Header = "3gpp-Sbi-Binding:" OWS binding-element *( OWS "," OWS binding-element ) OWS

binding-element    = "bl=" blvalue 1*( ";" OWS bh-parameter )

[ ";" OWS recoverytime ]

[ ";" OWS notif-receiver ]

[ ";" OWS "group=" groupvalue ]

[ 1*( ";" OWS groupparameter ) ]

[ ";" OWS "no-redundancy=" no-red-value ]

[ ";" OWS callback-uri-prefix ] OWS

bh-parametername   = parametername / "scope"

bh-parameter       = bh-parametername "=" token

recoverytime       = "recoverytime=" OWS DQUOTE date-time DQUOTE

notif-receiver     = "nr=" URI

groupvalue         = "true" / "false"

groupparametername = "oldgroupid"

/ "groupid"

/ "uribase"

/ "oldnfinst"

/ "oldservset"

/ "oldservinst"

/ "guami"

groupparameter     = groupparametername "=" token

no-red-value       = "true"

;

; Header: 3gpp-Sbi-Producer-Id

;

Sbi-Producer-Id-Header = "3gpp-Sbi-Producer-Id:" OWS "nfinst=" nfinst

[ OWS ";" OWS "nfservinst=" nfservinst ]

[ OWS ";" OWS "nfset=" nfset ]

[ OWS ";" OWS "nfserviceset=" nfserviceset ] OWS

nfinst                 = 8HEXDIG "-" 4HEXDIG "-" 4HEXDIG "-" 4HEXDIG "-" 12HEXDIG

nfservinst             = token

nfset                  = token

nfserviceset           = token

;

; Header: 3gpp-Sbi-Oci

;

Sbi-Oci-Header  = "3gpp-Sbi-Oci:" OWS oci-element *( OWS "," OWS oci-element ) OWS

oci-element     = timestamp ";" RWS validityPeriod ";" RWS olcMetric ";" RWS olcScope

timestamp       = "Timestamp:" RWS DQUOTE date-time DQUOTE

validityPeriod  = "Period-of-Validity:" RWS 1*DIGIT "s"

olcMetric       = "Overload-Reduction-Metric:" RWS ( "100" / %x31-39 DIGIT / DIGIT ) "%"

olcScope        = nfProducerScope / nfConsumerScope / scpScope / seppScope

nfProducerScope = ( ( "NF-Instance:" RWS nfinst )

/ ( "NF-Set:" RWS nfset )

/ ( "NF-Service-Instance:" RWS nfservinst [ ";" RWS "NF-Inst:" RWS nfinst ] )

/ ( "NF-Service-Set:" RWS nfserviceset )

) [ ";" RWS sNssaiList ";" RWS dnnList ]

nfConsumerScope = ( "NFC-Instance:" RWS nfinst [ ";" RWS "Service-Name:" RWS servname ] )

/ ( "NFC-Set:" RWS nfset [ ";" RWS "Service-Name:" RWS servname ] )

/ ( "NFC-Service-Instance:" RWS nfservinst [";" RWS "NF-Inst:" RWS nfinst ] )

/ ( "NFC-Service-Set:" RWS nfserviceset )

/ ( "Callback-Uri:" RWS DQUOTE URI DQUOTE *( RWS "&" RWS DQUOTE URI DQUOTE ) )

servname        = token

scpScope        = "SCP-FQDN:" RWS fqdn

seppScope       = "SEPP-FQDN:" RWS fqdn

fqdn            = token

dnnList         = "DNN:" RWS 1*tchar *( RWS "&" RWS 1*tchar )

sNssaiList      = "S-NSSAI:" RWS snssai *( RWS "&" RWS snssai )

snssai          = 1*tchar

;

; Header: 3gpp-Sbi-Lci

;

Sbi-Lci-Header    = "3gpp-Sbi-Lci:" OWS lc-element *( OWS "," OWS lc-element ) OWS

lc-element        = timestamp ";" RWS lcMetric ";" RWS lcScope

lcMetric          = "Load-Metric:" RWS ( "100" / %x31-39 DIGIT / DIGIT ) "%"

lcScope           = lcNfProducerScope / scpScope / seppScope

lcNfProducerScope = ( ( "NF-Instance:" RWS nfinst )

/ ( "NF-Set:" RWS nfset )

/ ( "NF-Service-Instance:" RWS nfservinst [ ";" RWS "NF-Inst:" RWS nfinst ] )

/ ( "NF-Service-Set:" RWS nfserviceset )

) [ ";" RWS sNssaiList ";" RWS dnnList ";" RWS relativeCapacity ]

relativeCapacity  = "Relative-Capacity:" RWS ( "100" / 1*2DIGIT ) "%"

;

; Header: 3gpp-Sbi-Client-Credentials

;

Sbi-Client-Credentials-Header = "3gpp-Sbi-Client-Credentials:" OWS jwt OWS

jwt                           = 1*b64urlchar "." 1*b64urlchar "." 1*b64urlchar

b64urlchar                    = ALPHA / DIGIT / "-" / "_"

;

; Header: 3gpp-Sbi-Source-NF-Client-Credentials

;

Sbi-Source-NF-Client-Credentials-Header = "3gpp-Sbi-Source-NF-Client-Credentials:" OWS jwt OWS

;

; Header: 3gpp-Sbi-Nrf-Uri

;

Sbi-Nrf-Uri-Header = "3gpp-Sbi-Nrf-Uri:" OWS nrfUriParam *( OWS ";" OWS nrfUriParam ) OWS

nrfUriParam        = nrfUriParamName ":" RWS ( nrfUriParamValue1 / nrfUriParamValue2 )

nrfUriParamName    = "nnrf-disc" / "nnrf-nfm" / "nnrf-oauth2" / "oauth2-requested-services" / token

nrfUriParamValue1  = DQUOTE URI DQUOTE

nrfUriParamValue2  = ( nrfServiceName *( RWS "&" RWS nrfServiceName ) )

nrfServiceName     = "nnrf-disc" / "nnrf-nfm"

;

; Header: 3gpp-Sbi-Target-Nf-Id

;

Sbi-Target-Nf-Id-Header = "3gpp-Sbi-Target-Nf-Id:" OWS "nfinst=" nfinst

[ ";" OWS "nfservinst=" nfservinst ] OWS

;

; Header: 3gpp-Sbi-Max-Forward-Hops

;

Sbi-Max-Forward-Hops-Header = "3gpp-Sbi-Max-Forward-Hops:" OWS ( %x31-39 DIGIT / DIGIT )

";" OWS "nodetype=" nodetypevalue OWS

nodetypevalue               = "scp"

;

; Header: 3gpp-Sbi-Originating-Network-Id

;

Sbi-Originating-Network-Id-Header = "3gpp-Sbi-Originating-Network-Id:" OWS 3DIGIT "-" 2*3DIGIT

[ "-" 11HEXDIG ] [ ";" OWS srcinfo ] OWS

srcinfo                           = "src" ":" RWS srctype "-" srcfqdn

srctype                           = "SCP" / "SEPP"

srcfqdn                           = 4*( ALPHA / DIGIT / "-" / "." )

;

; Header: 3gpp-Sbi-Access-Scope

;

Sbi-Access-Scope-Header = "3gpp-Sbi-Access-Scope:" OWS scope-token *( SP scope-token ) OWS

scope-token             = 1*NQCHAR

;

; Header: 3gpp-Sbi-Other-Access-Scopes

;

Sbi-Other-Access-Scopes-Header = "3gpp-Sbi-Other-Access-Scopes:" OWS scope-token

*( SP scope-token ) OWS

;

; Header: 3gpp-Sbi-Access-Token

;

Sbi-Access-Token-Header = "3gpp-Sbi-Access-Token:" OWS credentials OWS

;

; Header: 3gpp-Sbi-Target-Nf-Group-Id

;

Sbi-Target-Nf-Group-Id-Header = "3gpp-Sbi-Target-Nf-Group-Id:" OWS "nfgid=" nfGroupIdValue OWS

nfGroupIdValue                = DQUOTE token DQUOTE

;

; Header: 3gpp-Sbi-Nrf-Uri-Callback

;

Sbi-Nrf-Uri-Callback-Header = "3gpp-Sbi-Nrf-Uri-Callback:" OWS

nrfUriCallbackParam *( OWS ";" OWS nrfUriCallbackParam ) OWS

nrfUriCallbackParam         = nrfUriCallbackParamName ":" RWS nrfUriCallbackParamValue

nrfUriCallbackParamName     = "nnrf-disc" / "nnrf-nfm" / token

nrfUriCallbackParamValue    = DQUOTE URI DQUOTE

;

; Header: 3gpp-Sbi-NF-Peer-Info

;

Sbi-NF-Peer-Info-Header = "3gpp-Sbi-NF-Peer-Info:" OWS peerinfo *( ";" OWS peerinfo ) OWS

peerinfo                = peertype "=" token

peertype                = "srcinst"

/ "srcservinst"

/ "srcscp"

/ "srcsepp"

/ "dstinst"

/ "dstservinst"

/ "dstscp"

/ "dstsepp"

;

; Header: 3gpp-Sbi-Sender-Timestamp

;

Sbi-Sender-Timestamp-Header = "3gpp-Sbi-Sender-Timestamp:" OWS

day-name "," SP date1 SP time-of-day "." milliseconds SP "GMT" OWS

milliseconds                = 3DIGIT

;

; Header: 3gpp-Sbi-Max-Rsp-Time

;

Sbi-Max-Rsp-Time-Header = "3gpp-Sbi-Max-Rsp-Time:" OWS 1*5DIGIT OWS

;

; Header: 3gpp-Sbi-Correlation-Info

;

Sbi-Correlation-Info-Header = "3gpp-Sbi-Correlation-Info:" OWS

correlationinfo *( ";" OWS correlationinfo ) OWS

correlationinfo             = ctype "-" cvalue

ctype                       = extension-token / "imsi" / "impi" / "suci" / "nai" / "gci" / "gli"

/ "impu" / "msisdn" / "extid" / "imeisv" / "imei" / "mac" / "eui"

extension-token             = 1*( "!" / "#" / "$" / "%" / "&" / "'" / "*" / "+" / "." / "^" / "_"

/ "`" / "|" / "~" / DIGIT / ALPHA )

cvalue                      = 1*( tchar / "@" )

;

; Header: 3gpp-Sbi-Alternate-Chf-Id

;

Sbi-Alternate-Chf-Id-Header = "3gpp-Sbi-Alternate-Chf-Id:" OWS

"nfinst=" nfinst ";" OWS ( "primary" / "secondary" ) OWS

;

; Header: 3gpp-Sbi-Notif-Accepted-Encoding

;

Sbi-Notif-Accepted-Encoding-Header = "3gpp-Sbi-Notif-Accepted-Encoding:" OWS

encoding-element *( OWS "," OWS encoding-element ) OWS

encoding-element                   = codings [ weight ]

;

; Header: 3gpp-Sbi-Consumer-Info

;

Sbi-Consumer-Info-Header = "3gpp-Sbi-Consumer-Info:" OWS

consumer-info-element *( OWS "," OWS consumer-info-element ) OWS

consumer-info-element    = ( supportedService ";" OWS supportedVersions

[ ";" OWS supportedFeatures ]

[ ";" OWS acceptEncoding ]

[ ";" OWS callback-uri-prefix ]

) [ ";" OWS intraPlmnCallbackRoot ";" OWS interPlmnCallbackRoot ]

supportedService         = "service=" servicename

servicename              = 1*( "-" / %x30-39 / %x41-5A / "_" / %x61-7A )

supportedVersions        = "apiversion=" "(" OWS

[ apimajorversion *( RWS apimajorversion ) OWS ] ")"

apimajorversion          = %x31-39 [ *DIGIT ]

supportedFeatures        = "supportedfeatures=" features

features                 = *HEXDIG

acceptEncoding           = "acceptencoding=" %x22 encodingList %x22

encodingList             = [ encoding-element *( OWS "," OWS encoding-element ) ]

intraPlmnCallbackRoot    = "intraPlmnCallbackRoot="

DQUOTE sbi-scheme "://" sbi-authority [ prefix ] DQUOTE

interPlmnCallbackRoot    = "interPlmnCallbackRoot="

DQUOTE sbi-scheme "://" sbi-authority [ prefix ] DQUOTE

callback-uri-prefix      = "callback-uri-prefix=" DQUOTE prefix DQUOTE

;

; Header: 3gpp-Sbi-Response-Info

;

Sbi-Response-Info-Header = "3gpp-Sbi-Response-Info:" OWS

resp-info-param *( OWS ";" OWS resp-info-param ) OWS

resp-info-param          = resp-info-param-name "=" OWS resp-info-param-value

resp-info-param-name     = "request-retransmitted"

/ "nfinst"

/ "nfset"

/ "nfservinst"

/ "nfserviceset"

/ "context-transferred"

/ "no-retry"

/ token

resp-info-param-value    = token

;

; Header: 3gpp-Sbi-Selection-Info

;

Sbi-Selection-Info-Header = "3gpp-Sbi-Selection-Info:" OWS

selection-info-element *( OWS "," OWS selection-info-element ) OWS

selection-info-element    = ( "reselection=" reselectionvalue *( ";" OWS selection-criteria ) )

/ ( selection-criteria *( ";" OWS selection-criteria ) )

reselectionvalue          = "true" / "false"

selection-criteria        = selection-action "=" token

selection-action          = "not-select-nfservinst"

/ "not-select-nfserviceset"

/ "not-select-nfinst"

/ "not-select-nfset"

;

; Header: 3gpp-Sbi-Interplmn-Purpose

;

Sbi-Interplmn-Purpose-Header = "3gpp-Sbi-Interplmn-Purpose:" OWS N32Purpose ":" OWS

additional-info OWS

N32Purpose                   = "ROAMING"

/ "INTER_PLMN_MOBILITY"

/ "SMS_INTERCONNECT"

/ "ROAMING_TEST"

/ "INTER_PLMN_MOBILITY_TEST"

/ "SMS_INTERCONNECT_TEST"

/ "SNPN_INTERCONNECT"

/ "SNPN_INTERCONNECT_TEST"

/ "DISASTER_ROAMING"

/ "DISASTER_ROAMING_TEST"

/ "NETWORK_OPERATION_DATA_EXCHANGE"

/ "NETWORK_OPERATION_DATA_EXCHANGE_TEST"

/ token

additional-info              = token

;

; Header: 3gpp-Sbi-Request-Info

;

Sbi-Request-Info-Header = "3gpp-Sbi-Request-Info:" OWS req-param *( ";" OWS req-param ) OWS

req-param               = req-param-name "=" OWS req-param-value

req-param-name          = "retrans"

/ "redirect"

/ "reason"

/ "idempotency-key"

/ "receivedrejectioncause"

/ "callback-uri-prefix"

/ "nfinst"

/ "nfservinst"

/ "redirection-cause"

/ token

req-param-value         = token

;

; Header: 3gpp-Sbi-Retry-Info

;

Sbi-Retry-Info-Header = "3gpp-Sbi-Retry-Info:" OWS retriesindication OWS

retriesindication     = "no-retries"

* * * End of Changes * * * *