3GPP TSG-CT WG6 Meeting #122	C6-250253

Bratislava, Slovakia, 20 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.121 | CR | 0590 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing subclause 5.7.1.4 in test case 5.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | TSG CT WG6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R18 |  |  |  |  |  | Date: |  |  | 2025-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Following CT6#121 meeting was approved CR0578 (C6-250085) that introduced new TCs for SUCI 5G NSWO calculation by USIM. Unfortunalety, subclause 5.7.1.4 was missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of subclause 5.7.1.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unconsistency on numbering |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 5.7	Handling subscription identifier privacy for 5G NSWO

### 5.7.1	UE identification by SUCI during initial registration – SUCI 5G NSWO calculation by USIM using profile A

#### 5.7.1.1	Definition and applicability

If the operator's decision, indicated by the USIM, is that the USIM shall calculate the SUCI, then the USIM shall not give the ME any parameter for the calculation of the SUCI including the Home Network Public Key Identifier, the Home Network Public Key, and the Protection Scheme Identifier. If the ME determines that the calculation of the SUCI, indicated by the USIM, shall be performed by the USIM, the ME shall delete any previously received or locally cached parameters for the calculation of the SUCI including the Routing Indicator, the Home Network Public Key Identifier, the Home Network Public Key and the Protection Scheme Identifier. If the UE determines to use the NSWO service, the UE shall use the SUCI in NAI format as its identity when SUPI Type configured on the USIM is IMSI. If 5G NSWO capability is enabled via configuration on the USIM, configuration on the USIM shall take precedence over the ME.

#### 5.7.1.2	Conformance requirement

1)	SUCI calculation procedure shall be performed by the USIM if services n°124, n°125 are "available" in EFUST.

2)	5G NSWO procedures shall be performed if service n°142 is "available" in EFUST.

3)	The ME shall use the GET IDENTITY command in SUCI NSWO as per 7.5.1.2 in 3GPP TS 31.102 [4].

4)	The ME shall use the SUCI calculated by the USIM in EAP-ID-Response as defined in TS 33.501 [41] Annex S.

Reference:

-	3GPP TS 31.102 [4], clauses 4.4.11.11, 4.4.11.19, 5.3.48, 5.3.53 and 7.5.1.2;

-	3GPP TS 33.501 [41], clause Annex C and Annex S clause S.3.2;

-	3GPP TS 24.501 [42], clauses 5.5.1.2.2, 5.5.1.2.4;

-	3GPP TS 23.003 [14], clauses 28.7.9.2, 28.7.12.

#### 5.7.1.3	Test purpose

1)	To verify that the GET IDENTITY command in SUCI NSWO is performed correctly by the ME.

2)	To verify that the ME includes the SUCI received from the 5G-NR UICC supporting Rel-17 features within GET IDENTITY response in the EAP-ID-Response message.

#### 5.7.1.4	Method of test

##### 5.7.1.4.1	Initial conditions

The NG-SS WLAN Cell Aw and UE are configured as in clause 4.14.

The NG-SS shall be configured with Home Network Private Key for profile A:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | C5 | 3C | 22 | 20 | 8B | 61 | 86 | 0B |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 06 | C6 | 2E | 54 | 06 | A7 | B3 | 30 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | C2 | B5 | 77 | AA | 55 | 58 | 98 | 15 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 10 | D1 | 28 | 24 | 7D | 38 | BD | 1D |

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF):	Not available to the ME.

The 5G-NR UICC supporting Rel-17 features is used with the following changes:

EFUST (USIM Service Table)

Logically:

| Service n°125 | SUCI calculation by the USIM | available |
| --- | --- | --- |
| Service n°142 | 5G NSWO support | available |

Coding:

| Byte: | B1 |  | B2 |  | B3 |  |  | B4 |  | B5 |  |  | B6 |  | B7 |  | B8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xxxx |  | xxxx xxxx |  | xxxx xxxx |  |  | xxxx xxxx |  | xxxx xxxx |  |  | xxxx xxxx |  | xxxx xxxx |  | xxxx xxxx |  |  |
|  | B9 |  | B10 |  | B11 |  |  |  |  | B16 |  |  | B17 |  | B18 |  | B19 |  |  |
|  | xxxx xxxx |  | xxxx xxxx |  | xxxx xxxx |  |  | ..... |  | xxx1 xxxx |  |  | xxxx xxxx |  | xx1x xxxx |  | xxxx xxxx |  |  |

EF5GNSWO_CONF (5G Non-Seamless WLAN Offload configuration)

Logically:

5G Non-Seamless WLAN Offload usage Indicator:	5G Non-Seamless WLAN Offload usage Indicator is enabled

Coding:

| Byte | B1 |
| --- | --- |
| Hex | 01 |

5G-NR UICC supporting Rel-17 features is configured with:

Protection Scheme Identifier:				ECIES scheme profile A

Key Index:									1

Home Network Public Key Identifier:	30

Home Network Public Key:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 5A | 8D | 38 | 86 | 48 | 20 | 19 | 7C |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 33 | 94 | B9 | 26 | 13 | B2 | 0B | 91 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 63 | 3C | BD | 89 | 71 | 19 | 27 | 3B |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | F8 | E4 | A6 | F4 | EE | C0 | A6 | 50 |

##### 5.7.1.4.2	Procedure

a)	The UE is switched on.

b)	Bring up WLAN AP Cell Aw.

c) The UE establishes a WLAN connection with the WLAN AN, as specified in IEEE 802.11 [62].

d) The WLAN AN sends an EAP Identity/Request to the UE.

e) The UE sends an EAP Response/Identity message to the WLAN AN.

#### 5.7.1.5	Acceptance criteria

1)	After step d) the ME shall send GET IDENTITY command in SUCI NSWO with Identity Context in P2 as SUCI 5G NSWO (0x02) to the USIM.

2)	On step e) the UE shall include the SUCI returned by the USIM in the EAP Response/Identity message.

SUPI format:						0

Home Network Identifier:		246/081

Routing indicator:				17

Protection scheme id:			01

Home network public key Id:	30

Scheme output:					ECC ephemeral public key, encryption of 357935793 and MAC tag value

* * * End of Changes * * * *