pCR for text inclusion in TR 45.820 :

C-UNB Network capacity evaluation

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.4 of the Technical Report 45.820 revision 1.4.0.

The proposed text describes the network capacity evaluation of the C-UNB solution.

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band

### General description

### Downlink physical layer design

### Uplink physical layer design

### Link layer aspects

### Radio resource management

### Concept evaluation

#### Latency evaluation

#### Battery life evaluation

#### Network capacity evaluation

##### Evaluation methodology

For network capacity evaluation, the present document sets assumptions on traffic model and MS density and encourages using simulation tools for capacity evaluation. In the present contribution, network capacity is evaluated with an analytical model of the C-UNB radio access technology. Considering the simplicity of the C-UNB RAT, we propose to consider these results to be realistic enough to get preliminary results on the capacity. More detailed results based on simulation model are FFS.

###### Traffic model

In the present study, the traffic model per MS is defined with two parameters: the number of MAR per day and the distribution of the MAR size.

The number of MAR per day is defined with for values of the inter-arrival time. As there is always a large number of MS in a given cell, we propose to evaluate the network capacity with one "mean type" of MS, implementing a "mean value" of inter-arrival time (i.e. an average number of MAR per day). Table 7.4.6.6 gives the value used in the C-UNB analytical model.

Table 7.4.6.6: Average # of MAR per day used in C-UNB capacity analysis

| ratio | inter-arrival time | equivalent number of MAR per day | average number of MAR per day |
| --- | --- | --- | --- |
| 40% | 1 day | 1 | 11,2 |
| 40% | 2 hours | 12 |  |
| 15% | 1 hour | 24 |  |
| 5% | 30mn | 48 |  |

In the present study, the second parameter for defining the traffic model is the size of application data carried by MARs. This size is defined as a Pareto distribution with min value of 20 bytes, max value of 200 bytes and a coefficient of 2.5. This formula is quite complex to evaluate in an analytical model, such as the one we use in C-UNB. Therefore we propose to split the calculation in four parts and made a weighted average at the end:

use four different values of MAR sizes,

evaluate the network capacity with these four MAR sizes

make a weighted average of rough capacity results with the Pareto ratios.

As there is a full decade between min and max values of the application data size, we propose to define the MAR size with a log scale. The four MAR sizes and corresponding Pareto's ratios are given in table 7.4.6.7.

Table 7.4.6.7: MAR sizes and corresponding Pareto ratios
used in network capacity evaluation

| index | MAR size (in bytes) | Pareto ratio |
| --- | --- | --- |
| 1 | 20 | 0,833 |
| 2 | 40 | 0,147 |
| 5 | 100 | 0,015 |
| 10 | 200 | 0,005 |

###### Coverage

Annex E hereunder defines hexagonal tiles for the cellular coverage and the cell site distance of tri-sectorized BTS (see figure 7.4.6.3). In common cellular system, a MS is attached to a given sector; hence the network capacity is evaluated per sector that is hexagon shape.

In the case of C-UNB, the cooperative reception feature requires an MS to be seen by several base stations. In our capacity evaluation, we have enlarged the coverage of a given sector up to the next BTS. Each sector has a diamond shape (see figure 7.4.6.4) giving three overlaps per sector.

Figure 7.4.6.3: Cell site sector area definition (from [6])

Figure 7.4.6.4: C-UNB sector definition (a: theoritical; b: used in analytical model)

###### MAR segmentation assumption

The present study defines two protocol overheads (65 bytes normal and 29 bytes compressed) for the MARs. As a matter of fact, protocols overheads can be omitted over the C-UNB RAT because they can be generated by MS avatars hosted in C-UNB server (see architecture subclauses above). Nevertheless, capacity evaluation is given with and without headers for the sake of comparison.

As MAC-PDUs have limited payload size in C-UNB, table 7.4.6.8 details the number of segments and MAC-PDU size for three assumptions on the header size: 65, 29 and 0 bytes.

Table 7.4.6.8: segmentation with various headers (all size in bytes)

| Assumption | Data size | Application packet size | Segmentation | Number and size of MAC-PDUs |
| --- | --- | --- | --- | --- |
| full header of 65 bytes | 20 | 85 | 3x29 | 3x 46,5 |
|  | 40 | 105 | 4x27 | 4x 44,5 |
|  | 100 | 165 | 6x28 | 6x 45,5 |
|  | 200 | 265 | 9x30 | 9x 47,5 |
| compressed header of 29 bytes | 20 | 49 | 2x25 | 2x 42,5 |
|  | 40 | 69 | 3x23 | 3x 40,5 |
|  | 100 | 129 | 5x26 | 5x 43,5 |
|  | 200 | 229 | 8x29 | 8x 46,5 |
| header not transmitted over the air | 20 | 20 | 1x20 | 1x 37,5 |
|  | 40 | 40 | 2x20 | 2x 37,5 |
|  | 100 | 100 | 4x25 | 4x 42,5 |
|  | 200 | 200 | 7x29 | 8x 46,5 |

###### Other assumptions

The present study defines other assumptions for the capacity evaluation, such as the ratio of MS transmitting MAR and receiving network command, the protocol overheads and the system level assumptions (annex D).

The specific features of C-UNB radio access technology allow us to simplify some of the assumptions without going against the objectives of the study. The extra assumptions made in the present contribution are:

network capacity is mainly limited by uplink and not by downlink in C-UNB

propagation loss and building penetration loss have no impact on the network capacity because base station can cope with highly dynamic signals.

##### Results

Table 7.4.6.9 gives the maximum capacity of a C-UNB network defined as a number of MSs per sector. These figures are computed with a repetition rate (i.e. blind repetition) of 2 PHY-PDU per MAC-PDU and 10% of retransmission due to BLER.

Table 7.4.6.9: C-UNB capacity (preliminary results from analytical model)

| Header | Max number of MS per sector |
| --- | --- |
| full size, 65 bytes | 21 460 |
| compressed, 29 bytes | 34 640 |
| not over the air, 0 byte | 75 900 |

These preliminary results, given by analytical model of the C-UNB RAT, show that the required capacity (i.e. 52 547 MSs per sector) is exceeded when transmission over the air interface is optimized, that is to say, when protocol overhead is not transmitted over the air, but managed by the MS avatars located in the C-UNB server. Capacity evaluation based on simulation tools is FFS.

End of Changes