pCR for text inclusion in TR 45.820 :

C-UNB Battery Life Evaluation

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.4 of the Technical Report 45.820 revision 1.4.0.

The proposed text describes the battery life evaluation of the C-UNB solution.

This pCR is a revision of pCR GP-150697.

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band

### General description

### Downlink physical layer design

### Uplink physical layer design

### Link layer aspects

### Radio resource management

### Concept evaluation

#### Latency evaluation

#### Battery life evaluation

##### Evaluation methodology

###### Common assumptions

The battery life evaluation for C-UNB MS is based on methodology detailed in [7] and two assumptions, specific to the C-UNB solution:

MAR are transmitted with two blind repetitions at physical layer, which gives a BLER of 10%

ack-nack packet in DL are transmitted right after the loop-back delay (i.e. with no extra delay due to overload condition in DL).

In most cases, the second assumption is valid because the large overlap of base stations allows the C-UNB server to select the overlapping base station, which is available for DL transmission.

###### MAR segmentation

In the present document, the battery life evaluation assumes two lengths for the MAR: 50 and 200 bytes at SNDCP level. In C-UNB, such data packets are segmented and encapsulated as stated in table 7.4.6.2. MAC-PDU size and total number of transmission give the inputs for power consumption.

Table 7.4.6.2: MAR mapping in C-UNB PHY-PDU  (all sizes in bytes)

| MAR size | Number of segments | Segment size | LLC-PDU size | MAC-PDU size | Total # of PHY-PDU | PHY-PDU duration |
| --- | --- | --- | --- | --- | --- | --- |
| 50 | 2 | 25 | 26 | 42,5 | 4 | 1 360ms |
| 200 | 7 | 29 | 30 | 46,5 | 14 | 1 488ms |

###### Energy consumption profile

Table 7.4.6.3 gives electrical parameters for energy consumption analysis. Figures for Tx and Rx power are those from off-the-shelves sub-gigaHertz chipsets or what is expected in the next generation of chipsets where architecture and power consumption will be optimized for ultra narrow band transmission. For example, a power consumption of 450mW at +24dBm is expected to be achievable the following way: 100mW for transceiver and 350mW for Power Amplifier (PA) with 71% efficiency, which is expected to be available with UNB optimization.

Table 7.4.6.3: Key electrical parameters for energy consumption analysis

| (1) Battery capacity (Wh) | (2) Battery power during Tx (mW) | (3) Battery power for Rx (mW) | (4) Battery power in Processing (mW) | (5) Battery power in Power Save (mW) |
| --- | --- | --- | --- | --- |
| 5 | 450 @ +24dBm 100 @ +14dBm 30 @ +4dBm | 40 | 10 | 0,015 |

Table 7.4.6.4: Key timing parameters for energy consumption analysis (in ms)

| (6) trigger event | (7) Processing time from PS to Tx mode | (8) Beacon reception window | (9) Processing time between two Tx (ms) | (10) Transmit time for one PHY-PDU | (11) ack/nack reception window |
| --- | --- | --- | --- | --- | --- |
| 0 | 50 | 70 to 200 | 25 | see table 1 | 875 |

In table 7.4.6.4, the duration of the reception window for beacon checking takes into account the real time clock drift between two consecutive transmissions of periodic MAR.

Figure 7.4.6.2: Energy consumption model for C-UNB devices

##### Results

Table 7.4.6.5 gives the battery life of C-UNB devices with a combination of two packet sizes, two reporting intervals and three coverage classes.

Table 7.4.6.5: Battery life (in years)

End of Changes