# Dependency on number of coverage classes for EC-EGPRS on system capacity (update of GP-151125)

# Introduction

At GERAN#67 a new Work Item called Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things was approved, see [1].

The work item is based on the outcome of the feasibility study [2] where some logical channels used three, five, or six different transmissions modes in the different evaluations.

The intention of this contribution is to investigate the impact on the EC-EGPRS system by aligning all logical channels to use four different transmission modes/Coverage Classes.

The lower the number of coverage classes the smaller the impact on implementation and specifications. However, it should not come at a cost of significant impact to system performance.

Updates to the document from earlier meetings are the inclusion of the proposal to repeat 48 repetitions at most on EC-RACH, and to use either 1 TS or 2 TS.

# Number of coverage classes and impact to system performance

Coverage classes are used to allow users to be reached in coverage exceeding normal GPRS/EGPRS coverage. The finer level of granularity in different coverage levels, the more optimized system design is possible, e.g. minimizing resource usage. Using fewer coverage classes would typically imply an increased usage of resources since more blind repetitions are used than necessary. An alternative design could be to have more aggressive selection of coverage classes (i.e., fewer repetitions are used for a given coupling loss), so that the number of blind repetitions is not significantly impacted, but then the opposite effect is seen, that for some users too few blind repetitions are used, and the resulting effect can be an increase in failed delivery of reports. This trade-off has for example been observed in [3].

# Simulations

## Simulation assumptions

Simulators that have been used and described in the study report [5] are re-used for this contribution with the change that a smaller number of coverage classes are used.

The system simulator used for EC-PDTCH/EC-PACCH simulations use a network wide timeslot alignment with a random timeslot offset between cells.

For simulations with random access and access grant, full synchronization has been assumed on frame and timeslot level to maximize overlap between different coverage classes. On top of this, external interference from legacy users have been added, as described in [5].

To focus the contribution on important aspects of the evaluation, previous details on simulator settings etc. are only referenced to.

### Random access and access grant

For details on EC-CCCH specific settings, please refer to [3]. Simulation settings worth pointing out are:

- Interference from legacy CS users to EC-EGPRS users have been assumed

- The most challenging BPL scenario has been simulated (scenario 2, inter-site correlation 0.75)

- Access burst access type.

In addition to this, different number of repetitions has been used to model four coverage classes, according to Table 1 and Table 2. The full set of six coverage classes with [1, 2, 4, 8, 16, 32] repetitions on DL, and, [1, 2, 4, 8, 16, 48] repetitions on UL respectively, has been simulated as reference.

Also, coverage class adaptation as presented in [7] has been activated with 2 failed attempts before adapting coverage class, and with at most 2 adaptations per system access attempt.

Table 1: Repetitions for different coverage class settings – EC-AGCH

| Configuration | Coverage Class [CC1, CC2, CC3, CC4] |
| --- | --- |
| 1 | [1,2,4,32] |
| 2 | [1,2,8,32] |
| 3 | [1,4,8,32] |
| 4 | [1,8,16,32] |

Table 2: Repetitions for different coverage class settings – EC-RACH

| Configuration | Coverage Class [CC1, CC2, CC3, CC4] |
| --- | --- |
| 1 | [1,2,4,48] |
| 2 | [1,2,8,48] |
| 3 | [1,4,8,48] |
| 4 | [1,4,16,48] |

### MAR periodic reporting

For details on EC-PDTCH/EC-PACCH specific settings, please refer to [4]. Simulation settings worth pointing out are:

- The most challenging BPL scenario has been simulated (scenario 2, inter-site correlation 0.75)

- No IP header compression is assumed

In addition to this, different number of repetitions has been used to model four coverage classes, according to Table 1. The full set of five coverage classes with [1, 2, 4, 8, 16] repetitions, respectively, has been simulated as reference.

Table 3. Repetitions for different coverage class settings

Coverage Class [CC1, CC2, CC3, CC4]

[1,2,4,16]

[1,2,8,16]

[1,4,8,16]

## Simulation output

Of relevance for the investigation on number of coverage classes, is primarily the resource usage and failed reports, as mentioned in Section 2.

On the EC-CCCH a failed report implies that the maximum number of attempts (6) on the EC-RACH has been reached without a corresponding response on the EC-AGCH. The resource usage is presented as number of bursts/user. Ideally, this metric would be 1.0 if every attempt by each user is successful (and hence there will not be any re-attempts), and all users are in coverage class 1.

For the delivery of the MAR reports, a failed attempt/failed report is classified as a report not delivered within 5 seconds for devices with 33 dBm output power and 8 seconds for devices with 23 dBm output power. The resource usage for the MAR periodic reports is presented as average number of TS required to support the targeted number of users (around 52000 per sector).

# Results

## Random access and access grant

Table 4 and Table 5 summarize the simulated scenarios and results.

Table 4. Simulated scenarios and results for random access (1 TS) and access grant

| CIoT device output power [dBm] | # Coverage Classes | Configuration | Average Resource Usage UL [bursts/user] | Average Resource Usage DL [bursts/user] | Failed access attempts [%] |
| --- | --- | --- | --- | --- | --- |
| 33 | 6 | - | 1.3 | 2.4 | 0.04 |
| 33 | 4 | 1 | 1.5 | 2.6 | 0.03 |
| 33 | 4 | 2 | 1.4 | 2.5 | 0.02 |
| 33 | 4 | 3 | 1.4 | 2.5 | 0.02 |
| 33 | 4 | 4 | 1.4 | 2.7 | 0.01 |
| 23 | 6 | - | 3.2 | 2.2 | 0.17 |
| 23 | 4 | 1 | 4.2 | 2.3 | 0.12 |
| 23 | 4 | 2 | 3.8 | 2.3 | 0.11 |
| 23 | 4 | 3 | 3.7 | 2.3 | 0.10 |
| 23 | 4 | 4 | 3.4 | 2.5 | 0.10 |

Table 5. Simulated scenarios and results for random access (2 TS) and access grant

| CIoT device output power [dBm] | # Coverage Classes | Configuration | Average Resource Usage UL [bursts/user] | Average Resource Usage DL [bursts/user] | Failed access attempts [%] |
| --- | --- | --- | --- | --- | --- |
| 33 | 6 | - | 1.3 | 2.4 | 0.05 |
| 33 | 4 | 1 | 1.5 | 2.6 | 0.04 |
| 33 | 4 | 2 | 1.4 | 2.6 | 0.03 |
| 33 | 4 | 3 | 1.4 | 2.6 | 0.02 |
| 33 | 4 | 4 | 1.3 | 2.7 | 0.01 |
| 23 | 6 | - | 2.7 | 2.2 | 0.14 |
| 23 | 4 | 1 | 3.7 | 2.3 | 0.11 |
| 23 | 4 | 2 | 3.2 | 2.3 | 0.08 |
| 23 | 4 | 3 | 3.1 | 2.3 | 0.07 |
| 23 | 4 | 4 | 3.0 | 2.5 | 0.06 |

As can be seen, the resource usage is generally increased with a lower number of coverage classes, although not always the case. If using the recommended CC set from [6], the increase is for 33 dBm devices 3 %  and 4 % on UL for 1 TS and 2 TS EC-RACH respectively. On the DL the increase is roughly 11%. For 23 dBm devices the corresponding impact is 6 % and 9 % on UL for 1 TS and 2 TS EC-RACH respectively, and 12 % on DL. Considering that 23 dBm devices uses significantly more resources than 33 dBm devices, the overall impact is still limited with a good choice of repetitions per coverage class. The more noticeable increase in the DL is due to the choice of repetition factor to align with other logical channels for the same coverage class, see [6]. Other combinations of four coverage classes would reduce the DL resource usage, but instead result in a worse alignment with other logical channels for the system operation.

The failed access attempt rate is impacted quite substantially by the reduction in number of coverage classes. Hence, aiming at the same failed rate for 6 and 4 coverage classes, the increase seen in resource usage with 4 CC would go down.

Table 6 summarizes the distribution of all users in the different uplink coverage classes in the case of four coverage classes in EC-RACH.

Table 6. EC-RACH CC distribution for 33 and 23 dBm device output power

| UL Coverage Class | Distribution of users [%] |  |
| --- | --- | --- |
|  | CIoT device output power [dBm] |  |
|  | 33 | 23 |
| 1 | 98.2 | 90.2 |
| 2 | 1.3 | 6 |
| 3 | 0.3 | 2.2 |
| 4 | 0.2 | 1.6 |

## MAR periodic reporting

Table 7 summarizes the simulated scenarios and results on EC-PDCH.

Table 7. Simulated scenarios and results for EC-PDCH

| CIoT device output power [dBm] | # Coverage Classes | Repetitions per coverage class | Average Resource Usage UL [#TS] | Average Resource Usage DL [#TS] | Failed attempts [%] | Out of coverage [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 33 | 5 | [1,2,4,8,16] (reference case) | 0.9 | 0.7 | 0.1 | 0.05 |
| 33 | 4 | [1,2,4,16] | 0.9 | 0.7 | 0.1 | 0.05 |
| 33 | 4 | [1,2,8,16] | 0.9 | 0.7 | 0.1 | 0.05 |
| 33 | 4 | [1,4,8,16] | 0.9 | 0.7 | 0.1 | 0.05 |
| 23 | 5 | [1,2,4,8,16] (reference case) | 2.0 | 0.5 | 0.4 | 0.8 |
| 23 | 4 | [1,2,4,16] | 2.1 | 0.5 | 0.4 | 0.8 |
| 23 | 4 | [1,2,8,16] | 2.0 | 0.5 | 0.4 | 0.8 |
| 23 | 4 | [1,4,8,16] | 2.0 | 0.5 | 0.4 | 0.8 |

As can be seen, the impact of going from 5 to 4 CCs is basically within the error margin of the simulations, i.e. no significant impact is seen on either average resources used or failed attempts. Comparing to the simulations for random access and access grant only one coverage class is removed compared to earlier evaluations (two were removed for random access/access grant). Also, the EC-PDTCH will be more resilient to errors in coverage class estimation due to the use of chase combining. If for example the delay requirement allows for an extensive number of retransmissions, then it is more efficient from a radio resource perspective to use less blind repetitions and only retransmit what has been lost.

Table 8 summarizes the distribution of all users in the different EC-PDCH uplink coverage classes for the [1,4,8,16] repetitions per coverage class and for the simulated case where all devices have 33 or 23 dBm output power. For the 33 dBm devices 96.9 % of all users are in normal coverage. The corresponding number for 23 dBm devices is 87.4 %.

Table 8. EC-PDCH CC distribution for 33 and 23 dBm device output power

| UL Coverage Class | Distribution of users [%] |  |
| --- | --- | --- |
|  | CIoT device output power [dBm] |  |
|  | 33 | 23 |
| 1 | 96.9 | 87.4 |
| 2 | 2.2 | 7.9 |
| 3 | 0.5 | 2.2 |
| 4 | 0.4 | 2.5 |

# Conclusions

The contribution has investigated the impact on resource usage and failed attempts/reports when reducing the number of coverage classes either from six to four, or five to four:

- On EC-CCCH (EC-RACH and EC-AGCH) when going from six to four coverage classes for EC-EGPRS. It was seen that the resource usage is increased by roughly 3-9 % on UL and 11-12 % on DL with a reduced number of coverage classes. The failed access attempt rate is however reduced, and hence aiming at the same failed rate for 4 CC and 6CC, the increase in resource usage seen would be lowered.

- For dedicated channels, EC-PDTCH and EC-PACCH, there is no visible impact on resource usage and failed attempts when reducing the set of coverage classes from five to four. Also, the choice of which repetitions to be used give no differences. It is hence proposed to use the already used set of repetitions in CRs submitted to GERAN, i.e. 1, 4, 8, 16. Using a multiple of four in the repetitions will also allow overlaid CDMA to operate without the use of a multi-rate code, to ensure orthogonality.

# References

- GP-151039, “New Work Item on Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things”, source Ericsson LM, Intel, Gemalto N.V., MediaTek Inc., TeliaSonera AB, Sierra Wireless S.A., Telit Communications S.p.A., ORANGE, Nokia Networks, Alcatel-Lucent. GERAN#67.

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

- GP-150760, ”EC-GSM, Random access system capacity evaluation (update of GPC150491)”, source Ericsson LM. GERAN#67.

- GP-150756, “EC-GSM, MAR periodic and network command system capacity evaluation (update of GPC150622)”, source Ericsson LM. GERAN#67.

- 3GPP TR 45.820 v.13.0.0

- GP-160030, “Alignment of Coverage Classes for EC-EGPRS”, source Ericsson LM. GERAN Telco#4 on EC-GSM and eDRX

- GP-160035, “Coverage Class Adaptation on EC-RACH “, source Ericsson LM. GERAN Telco#4 on EC-GSM and eDRX

The different transmission modes basically corresponded to the same number of Coverage Classes, although in case of EC-PDTCH/EC-PACCH six coverage classes were defined but only five different transmission modes.