3GPP TSG-GERAN #69 	GP-160052

# Malta, February 15th – 19th 2016	             (Revision of GP-151101)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.005 | CR | 0578 | rev | 3 | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2016-02-09 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | A new type of mobile device is introduced, characterized by low data throughput and extended coverage requirements compared to GPRS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Characteristics and requirements specific for the new device are introduced at applicable parts. Compared to GP-151101, comments received during GERAN #68 have to a large extent been taken into account. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for this type of low-cost devices will be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 1.2, 1.3, 4.1.1, 4.3.2, 4.6.3 (new), 4.6.4 (new), 6.1a, 6.1b, 6.2.2, 6.2.4, 6.3.2, 6.3.3, 6.3.4, 6.3.5, 6.4, 6.5, 6.7 and new performance tables. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.013 0135 TS 43.022 0035 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR 1025,1026 TS 44.060 CR 1609,1610,1611,1612 TS 45.001 CR 0079,0080 TS 45.002 CR 0183,0184,0185,0186 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.010 CR 0065 TS 45.008 CR 0626,0627 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51010 ... CR ... TS 51021 ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First modification *****

# 1	Scope

The present document defines the requirements for the transceiver of the pan-European digital cellular telecommunications systems GSM.

Requirements are defined for two categories of parameters:

-	those that are required to provide compatibility between the radio channels, connected either to separate or common antennas, that are used in the system. This category also includes parameters providing compatibility with existing systems in the same or adjacent frequency bands;

-	those that define the transmission quality of the system.

The present document defines RF characteristics for the Mobile Station (MS) and Base Station System (BSS). The BSS will contain Base Transceiver Stations (BTS), which can be normal BTS,micro-BTS or pico-BTS. The precise measurement methods are specified in 3GPP TS 51.010 and 3GPP TS 51.021.

Unless otherwise stated, the requirements defined in this EN apply to the full range of environmental conditions specified for the equipment (see annex D).

In the present document some relaxation's are introduced for GSM 400 MSs, GSM 900 MSs, GSM 700 MSs and GSM 850 MSs which pertain to power class 4, 5 or 6 (see subclause 4.1.1). In the present document these Mobile Stations are referred to as "small MS".

In the present document some relaxations to receiver requirements are introduced for a MS indicating support for Downlink Multi Carrier (DLMC), see 3GPP TS 24.008, when in DLMC configuration. DLMC configurations are specified for only GSM 850, GSM 900, DCS 1800 and PCS 1900.

MSs may operate on more than one of the frequency bands specified in clause 2. These MSs are referred to as "Multi band MSs" in this EN. Multi band MSs shall meet all requirements for each of the bands supported. The relaxation on GSM 400 MSs, GSM 900 MSs, GSM 700 MSs and GSM 850 MSs for a "small MS" are also valid for a multi band MS if it complies with the definition of a small MS.

The RF characteristics of repeaters are defined in annex E of this EN. Annexes D and E are the only clauses of this EN applicable to repeaters. Annex E does not apply to the MS or BSS. The precise measurement methods for repeaters are specified in 3GPP TS 51.026 [35].

The present document also includes specification information for mixed mode operation at 850 MHz and 1900 MHz (MXM 850 and MXM 1900). 850 MHz and 1900 MHz mixed-mode is defined as a network that deploys both 30 kHz RF carriers and 200 kHz RF carriers in geographic regions where the Federal Communications Commission (FCC) regulations are applied or adopted.

The requirements for a MS in a mixed-mode system, MXM 850 and MXM 1900, correspond to the requirements for GSM 850 MS and PCS 1900 MS respectively.

Annex M defines the minimum performance requirements for A-GPS for MSs that support A-GPS. Annex M does not apply to the BSS.

The present document also includes specific requirements for multicarrier BTS, wherever explicitly stated in the text, that apply for all classes of multicarrier BTS (Wide Area, Medium Range and Local Area) if nothing else is stated. All other requirements designated for BTS and normal BTS apply if not otherwise stated. The multicarrier BTS classes have relaxed requirements in the areas of Tx spurious emissions, intermodulation attenuation and, when multicarrier receiver is included, Rx blocking. Usage of multicarrier BTSs in some geographical regions might be subject to regulatory restrictions to protect other radio systems operating in bands of adjacent frequency assignments, in particular for all safety related applications like railway applications. In areas where such systems coexist with multicarrier BTSs, the received interference power originating from multicarrier BTSs might have to be limited.

The document also includes entry points in some tables for the multicarrier BTS requirements to which TS 37.104 [33] for Multi-Standard Radio Base Stations (MSR BS) is referring to as specific GSM/EDGE single-RAT requirements not covered by the general requirements. These entry points are marked with M) and, as described in a note in each applicable table, identify the relevant column(s) that are applicable as MSR BS requirements. In general the requirements for multicarrier BTS equipped with multicarrier receiver also apply to Multi-Standard Radio Base Stations. The GSM requirements for Multi-Standard Radio Base Stations are defined for GSM 850, GSM 900, DCS 1800 and PCS 1900 only. Requirements for other frequency bands and MXM base stations are excluded. Annex P defines the minimum performance for the receiver in MSR BS.
For equipment not declared as MSR BS the M) indications can be ignored.

The present document defines requirements for the usage of the ER-GSM band. The national implementation might be subject to regulatory coordination agreements to avoid system impacts (RF scenarios for ER-GSM introduction are given in 3GPP TR 45.050).

The present document defines requirements for supporting low-complexity, low data throughput devices in environments experiencing high propagation attenuation as indoors in basements etc. This service, based on EGPRS, with extended coverage is called EC-EGPRS. The requirements for EGPRS apply unless otherwise stated.

***** Next modification *****

## 1.2	Abbreviations

Abbreviations used in the present document are listed in 3GPP TR 21.905.

In addition to abbreviations in 3GPP TR 21.905, the following abbreviations are applied:

CC	Coverage Class

SCPIR	Sub Channel Power Imbalance Ratio (see 3GPP TS 45.004)

SCPIR_DL	Sub Channel Power Imbalance Ratio on DownLink

SCPIR_UL	Sub Channel Power Imbalance Ratio on UpLink

DLMC	Downlink Multi Carrier

## 1.3	Definitions

For the purposes of the present document, the following terms and definitions apply.

Coverage Class: see 3GPP TS 43.064.

DLMC configuration: see 3GPP TS 44.060.

DLMC inter-band reception: Resource assignment in up to two frequency bands to a MS indicating support for DLMC inter-band reception, see 3GPP TS 24.008.The Maximum Bandwidth signalled by the mobile station, see 3GPP TS 24.008, applies to each band.

DLMC non-contiguous intra-band reception: Simultaneous reception of carriers in a given frequency band with greater carrier separation than indicated by the signalled Maximum Bandwidth, see 3GPP TS 24.008. The assigned carriers are grouped into two separate groups during each TDMA frame, where the carrier separation indicated by the Maximum Bandwidth applies in each group.

EC-EGPRS operation: See 3GPP TS 43.064

Maximum supported DLMC carrier frequency spacing: Maximum spacing between the centre frequencies of simultaneously received carriers for the signalled Maximum Bandwidth, see 3GPP TS 24.008.

Minimum carrier frequency spacing: minimum spacing between the centre frequencies of simultaneously transmitted or received GSM carriers of a BTS belonging to a multicarrier BTS class.

Subchannel power imbalance ratio on downlink: in the downlink, the subchannel power imbalance ratio is the SCPIR as defined in 3GPP TS 45.004.

NOTE:	Information bits of VAMOS subchannel 2 and VAMOS subchannel 1 respectively modulate the quadrature component and inphase component of the AQPSK symbols (see 3GPP TS 45.002).

Subchannel power imbalance ratio on uplink: the ratio of the average received uplink power of VAMOS subchannel 2 (Pu2) to the average received uplink power of VAMOS subchannel 1 (Pu1) expressed as 10*log10(Pu2/Pu1) dB.

VAMOS I/II/III MS: Mobile Station supporting VAMOS level I/II/III respectively (see 3GPP TS 24.008).

***** Next modification *****

### 4.1.1	Mobile Station

The MS maximum output power and lowest power control level shall be, according to its class, as defined in the following tables.

i) MS maximum output power

Table 4.1-1 MS maximum power at GMSK modulation

| Power | GSM 400 & GSM 900 & ER-GSM 900 & GSM 850 & GSM 700 | DCS 1 800 | PCS 1 900 | Tolerance (dB) |  |
| --- | --- | --- | --- | --- | --- |
| class | Nominal Maximum output | Nominal Maximum output | Nominal Maximum output | for conditions |  |
|  | power | power | power | normal | extreme |
| 1 | - - - - - - | 1 W (30 dBm) | 1 W (30 dBm) | ±2 | ±2,5 |
| 2 | 8 W (39 dBm) | 0,25 W (24 dBm) | 0,25 W (24 dBm) | ±2 | ±2,5 |
| 3 | 5 W (37 dBm) | 4 W (36 dBm) | 2 W (33 dBm) | ±2 | ±2,5 |
| 4 | 2 W (33 dBm) |  |  | ±2 | ±2,5 |
| 5 | 0,8 W (29 dBm) |  |  | ±2 | ±2,5 |
| 6 | 0,2 W (23 dBm)1) | 0,16 W (22 dBm) | 0,16 W (22 dBm) | +2/-0,5 | +2,5/-1 |
| NOTE 1: Power class 6 applies only to GSM 850 and GSM 900, excluding T-GSM 810 |  |  |  |  |  |

Table 4.1-2 MS maximum power at other modulations

| Power | GSM 400 and GSM 900 & ER-GSM 900 & GSM 850 & GSM 700 | GSM 400 and GSM 900 & ER-GSM 900 & GSM 850 & GSM 700 |  | DCS 1 800 | PCS 1 900 | DCS 1 800 & PCS 1 900 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| class | Nominal Maximum output | Tolerance (dB) for conditions |  | Nominal Maximum output | Nominal Maximum output | Tolerance (dB) for conditions |  |
|  | Power | normal | extreme | power | power |  | Power |
| E1 | 33 dBm | ±2 | ±2,5 | 30 dBm | 30 dBm | ±2 | ±2,5 |
| E2 | 27 dBm | ±3 | ±4 | 26 dBm | 26 dBm | -4/+3 | -4,5/+4 |
| E3 | 23 dBm | 3 | ±4 | 22 dBm | 22 dBm | 3 | 4 |
| E4 | 19 dBm1) | 2 | ±2,5 | 18 dBm | 18 dBm | 2 | ±2,5 |
| NOTE 1: Power class E4 applies only to GSM 850 and GSM 900, excluding T-GSM 810. |  |  |  |  |  |  |  |

The maximum power for power class E1-E3 is corrected for the different modulations according to the table below

Table 4.1-3 Correction factor of maximum power for different modulations in table 4.1-2

| Modulation | Correction factor (dB) |
| --- | --- |
| QPSK | 0 |
| 8-PSK | 0 |
| 16-QAM | -2 |
| 32-QAM | -2 |
| NOTE: In the case and only in the case of EGPRS2-B with the spectrally wide pulse shaping filter and the tight spectrum requirement at 400 kHz offset from the carrier (see section 4.2.1), the actual maximum output power may be up to 2 dB lower than the lower limit of the maximum output power's tolerance range defined by the power class table above and the correction factors of this table. In this case and only in this case, the MS need not use the highest power control level or the two highest power control levels for the respective modulation. |  |

Maximum output power for GMSK in any one band shall always be equal to or higher than maximum output power for all other modulations for the same equipment in the same band.

A multi band MS has a combination of the power class in each band of operation from the table above. Any combination may be used.

For EC-EGPRS capable mobile stations only GMSK modulation is mandatory. 8-PSK modulation is optional. 
For EC-EGPRS mobile stations only power classes 4 and 6 apply for GSM850 MHz and GSM900 MHz, and power classes 1 and 6 apply for DCS1800 and PCS1900.

The PCS 1 900, including its actual antenna gain, shall not exceed a maximum of 2 Watts (+33 dBm) EIRP per the applicable FCC rules for wideband PCS services [FCC Part 24, Subpart E, Section 24.232]. Power Class 3 is restricted to transportable or vehicular mounted units.

For GSM 850 MS, including its actual antenna gain, shall not exceed a maximum of 7 Watts (+38,5 dBm) ERP per the applicable FCC rules for public mobile services. [FCC Part 22, Subpart H, Section 22.913]

For GSM 700 MS, including its actual antenna gain, shall not exceed a maximum of 3 Watts (+35 dBm) ERP for handheld devices and maximum of 30 Watts (+45 dBm) ERP for other mobile devices per the applicable FCC rules. [FCC Part 27, Subpart C, Section 27.50].

ii) The different power control levels needed for adaptive power control (see 3GPP TS 45.008) shall have the nominal output power as defined in the tables below, starting from the power control level for the lowest nominal output power up to the power control level for the maximum nominal output power corresponding to the class of the particular MS as defined in the tables above. Whenever a power control level commands the MS to use a nominal output power equal to or greater than the maximum nominal output power for the power class of the MS, the nominal output power transmitted shall be the maximum nominal output power for the MS class, and the tolerance specified for that class (see tables 4.1-1, 4.1-2 and 4.1-3 above) shall apply.

Table 4.1-4a MS power control levels

| GSM 400, GSM 900, ER-GSM 900, GSM 850 and GSM 700 |  |  |  |
| --- | --- | --- | --- |
| Power control level | Nominal Output power (dBm) | Tolerance (dB) for conditions |  |
|  |  | normal | extreme |
| 0-2 | 39 | ±2 | ±2,5 |
| 3 | 37 | ±3 | ±4 |
| 4 | 35 | ±3 | ±4 |
| 5 | 33 | ±3 | ±4 |
| 6 | 31 | ±3 | ±4 |
| 7 | 29 | ±3 | ±4 |
| 8 | 27 | ±3 | ±4 |
| 9 | 25 | ±3 | ±4 |
| 10 | 23 | ±3 | ±4 |
| 11 | 21 | ±3 | ±4 |
| 12 | 19 | ±3 | ±4 |
| 13 | 17 | ±3 | ±4 |
| 14 | 15 | ±3 | ±4 |
| 15 | 13 | ±3 | ±4 |
| 16 | 11 | ±5 | ±6 |
| 17 | 9 | ±5 | ±6 |
| 18 | 7 | ±5 | ±6 |
| 19-311) | 5 | ±5 | ±6 |

| 192) | 5 | ±5 | ±6 |
| --- | --- | --- | --- |
| 202) | 3 | ±5 | ±6 |
| 212) | 1 | ±5 | ±6 |
| 222) | -1 | ±5 | ±6 |
| 232) | -3 | ±5 | ±6 |
| 242) | -5 | ±5 | ±6 |
| 25-312) | -7 | ±5 | ±6 |

NOTE 1: This requirement applies only to MS that do not support EC-EGPRS NOTE 2: The power levels apply to devices supporting EC-EGPRS as mandatory requirement.

Table 4.1-4b MS power control levels

| DCS 1 800 |  |  |  |
| --- | --- | --- | --- |
| Power control level | Nominal Output power (dBm) | Tolerance (dB) for conditions |  |
|  |  | normal | extreme |
| 29 | 36 | ±2 | ±2,5 |
| 30 | 34 | ±3 | ±4 |
| 31 | 32 | ±3 | ±4 |
| 0 | 30 | ±3 | ±4 |
| 1 | 28 | ±3 | ±4 |
| 2 | 26 | ±3 | ±4 |
| 3 | 24 | ±3 | ±4 |
| 4 | 22 | ±3 | ±4 |
| 5 | 20 | ±3 | ±4 |
| 6 | 18 | ±3 | ±4 |
| 7 | 16 | ±3 | ±4 |
| 8 | 14 | ±3 | ±4 |
| 9 | 12 | ±4 | ±5 |
| 10 | 10 | ±4 | ±5 |
| 11 | 8 | ±4 | ±5 |
| 12 | 6 | ±4 | ±5 |
| 13 | 4 | ±4 | ±5 |
| 14 | 2 | ±5 | ±6 |
| 15-281) | 0 | ±5 | ±6 |
| 152) | 0 | ±5 | ±6 |
| 162) | -2 | ±5 | ±6 |
| 172) | -4 | ±5 | ±6 |
| 182) | -6 | ±5 | ±6 |
| 19-282) | -8 | ±5 | ±6 |
| NOTE 1: This requirement applies only to MS that do not support EC-EGPRS NOTE 2: The power levels apply to devices supporting EC-EGPRS as mandatory requirement. |  |  |  |

NOTE 1:	For DCS 1 800, the power control levels 29, 30 and 31 are not used when transmitting the parameter MS_TXPWR_MAX_CCH on BCCH, for cross phase compatibility reasons. If levels greater than 30 dBm are required from the MS during a random access attempt, then these shall be decoded from parameters broadcast on the BCCH as described in 3GPP TS 45.008.

Furthermore, the difference in output power actually transmitted by the MS between two power control levels where the difference in nominal output power indicates an increase of 2 dB (taking into account the restrictions due to power class), shall be +2 ± 1,5 dB. Similarly, if the difference in output power actually transmitted by the MS between two power control levels where the difference in nominal output power indicates an decrease of 2 dB (taking into account the restrictions due to power class), shall be -2 ± 1,5 dB.

NOTE 2:	A 2 dB nominal difference in output power can exist for non-adjacent power control levels e.g. power control levels 18 and 22 for GSM 400 and GSM 900; power control levels 31 and 0 for class 3 DCS 1 800 and power control levels 3 and 6 for class 4 GSM 400 and GSM 900.

A change from any power control level to any power control level may be required by the base transmitter. The maximum time to execute this change is specified in 3GPP TS 45.008.

# Table 4.1-4c MS power control levels

# PCS 1 900

| Power Control Level | Output Power (dBm) | Tolerance (dB) for conditions |  |
| --- | --- | --- | --- |
|  |  | Normal | Extreme |
| 22-29 | Reserved | Reserved | Reserved |
| 30 | 33 | 2 dB | 2,5 dB |
| 31 | 32 | 2 dB | 2,5 dB |
| 0 | 30 | 3 dB1 | 4 dB1 |
| 1 | 28 | 3 dB | 4 dB |
| 2 | 26 | 3 dB | 4 dB |
| 3 | 24 | 3 dB1 | 4 dB1 |
| 4 | 22 | 3 dB | 4 dB |
| 5 | 20 | 3 dB | 4 dB |
| 6 | 18 | 3 dB | 4 dB |
| 7 | 16 | 3 dB | 4 dB |
| 8 | 14 | 3 dB | 4 dB |
| 9 | 12 | 4 dB | 5 dB |
| 10 | 10 | 4 dB | 5 dB |
| 11 | 8 | 4 dB | 5 dB |
| 12 | 6 | 4 dB | 5 dB |
| 13 | 4 | 4 dB | 5 dB |
| 14 | 2 | 5 dB | 6 dB |
| 15 | 0 | 5 dB | 6 dB |
| 16-21 2) | Reserved | Reserved | Reserved |
| 163) | -2 | 5 dB | 6 dB |
| 173) | -4 | 5 dB | 6 dB |
| 183) | -6 | 5 dB | 6 dB |
| 193) | -8 | 5 dB | 6 dB |
| 20-213) | Reserved | Reserved | Reserved |
| NOTE 1: Tolerance for MS Power Classes 1 and 2 is 2 dB normal and 2,5 dB extreme at Power Control Levels 0 and 3 respectively. NOTE 2: This requirement applies only to MS that do not support EC-EGPRS NOTE 3: The power levels apply to devices supporting EC-EGPRS as mandatory requirement. |  |  |  |

The output power actually transmitted by the MS at each of the power control levels shall form a monotonic sequence, and the interval between power steps shall be 2 dB 1,5 dB except for the step between power control levels 30 and 31 where the interval is 1 dB 1 dB.

The MS transmitter may be commanded by the BTS to change from any power control level to any other power control level. The maximum time to execute this change is specified in 3GPP TS 45.008.

For CTS transmission, the nominal maximum output power of the MS shall be restricted to:

-	11 dBm (0,015 W) in GSM 900 i.e. power control level 16;

-	12 dBm (0,016 W) in DCS 1 800 i.e. power control level 9.

iii) In order to manage mobile terminal heat dissipation resulting from transmission on multiple uplink timeslots, the mobile station may reduce its maximum output power by up to  the values in table 4.1-5. A mobile station in EC-EGPRS operation  shall always transmit at the declared nominal output power without reduction, for any number of timeslots used.

Table 4.1-5 MS power reduction when transmitting on multiple uplink timeslots

| Number of timeslots in uplink assignment | Permissible nominal reduction of maximum output power, (dB) |
| --- | --- |
| 1 | 0 |
| 2 | 3,0 |
| 3 | 4,8 |
| 4 | 6,0 |
| 5 | 7,0 |
| 6 | 7,8 |
| 7 | 8,5 |
| 8 | 9,0 |

The actual supported maximum output power shall be in the range indicated by the parameters XXX_MULTISLOT_POWER_PROFILE (See 3GPP TS 24.008) for n assigned uplink timeslots:

a  MS maximum output power  min(MAX_PWR, a + b)

Where:

a = min (MAX_PWR, MAX_PWR + XXX_MULTISLOT_POWER_PROFILE – 10log(n));

MAX_PWR equals to the MS maximum output power according to the relevant power class;

XXX_MULTISLOT_POWER_PROFILE refers either to GMSK_MULTISLOT_POWER PROFILE or 8-PSK_MULTISLOT_POWER_PROFILE depending on the modulation type concerned, and

XXX_MULTISLOT_POWER_PROFILE 0 = 0 dB;

XXX_MULTISLOT_POWER_PROFILE 1 = 2 dB;

XXX_MULTISLOT_POWER_PROFILE 2 = 4 dB;

XXX_MULTISLOT_POWER_PROFILE 3 = 6 dB.

For DCS 1800 and PCS 1900 frequency bands b = 3 dB, for all other bands b = 2 dB.

For  QPSK, 16-QAM and 32-QAM modulations 8-PSK_MULTISLOT_POWER_PROFILE shall apply, corrected for the difference in MAX_PWR for each modulation.

The supported maximum output power for each number of uplink timeslots shall form a monotonic sequence. The maximum reduction of maximum output power from an assignment of n uplink timeslots to an assignment of n+1 uplink timeslots shall be equal to the difference of maximum permissible nominal reduction of maximum output power for the corresponding number of timeslots, as defined in the table 4.1-5 above.

As an exception, in case of a multislot uplink assignment, the first power control step down from the maximum output power is allowed to be in the range 0…2 dB.

In case the MS transmits on more uplink slots than assigned (e.g. due to a polling response, see 3GPP TS 44.060), the MS may reduce uplink power as above for a multislot uplink configuration but as a function of the number of active uplink slots on a TDMA frame basis.

On a multislot uplink configuration the MS may restrict the interslot output power control range to a 10 dB window, on a TDMA frame basis. On those timeslots where the ordered power level is more than 10 dB lower than the applied power level of the highest power timeslot, the MS shall transmit at a lowest possible power level within 10 dB range from the highest applied power level, if not transmitting at the actual ordered power level.

Multislot power reduction is not allowed in EC-EGPRS operation.

Multislot power reduction is not allowed in EC-EGPRS operation.

### 4.1.2	Base station

a)	Requirements for base stations except multicarrier BTS

For a normal BTS, the maximum output power measured at the input of the BSS Tx combiner, shall be, according to its class, as defined in the following table.

Table 4.1-6 Normal BTS power classes

| GSM 400 & GSM 900 & ER-GSM 900 & GSM 850 & MXM 850 and GSM 700 |  | DCS 1 800 & PCS 1 900 & MXM 1900 |  |
| --- | --- | --- | --- |
| TRX | Maximum | TRX | Maximum |
| power class | output power | power class | output power |
| 1 | 320 - (< 640) W | 1 | 20 - (< 40) W |
| 2 | 160 - (< 320) W | 2 | 10 - (< 20) W |
| 3 | 80 - (< 160) W | 3 | 5 - (< 10) W |
| 4 | 40 - (< 80) W | 4 | 2,5 - (< 5) W |
| 5 | 20 - (< 40) W |  |  |
| 6 | 10 - (< 20) W |  |  |
| 7 | 5 - (< 10) W |  |  |
| 8 | 2,5 - (< 5) W |  |  |

For a micro-BTS or a pico-BTS, the maximum output power per carrier measured at the antenna connector after all stages of combining shall be, according to its class, defined in the following table.

Table 4.1-7 Micro BTS and Pico BTS power classes

| GSM 900 & ER-GSM 900 & GSM 850 & MXM 850 and GSM 700 micro and pico-BTS |  | DCS 1 800 & PCS 1 900 & MXM 1900 micro and pico-BTS |  |
| --- | --- | --- | --- |
| TRX power class | Maximum output power | TRX power class | Maximum output power |
| Micro |  | Micro |  |
| M1 | (> 19) - 24 dBm | M1 | (> 27) - 32 dBm |
| M2 | (> 14) - 19 dBm | M2 | (> 22) - 27 dBm |
| M3 | (> 9) - 14 dBm | M3 | (> 17) - 22 dBm |
| Pico |  | Pico |  |
| P1 | (> 13) - 20 dBm | P1 | (> 16) - 23 dBm |

For BTS supporting QPSK, AQPSK, 8-PSK, 16-QAM and/or 32-QAM the manufacturer shall declare the maximum output power capability for GMSK and for each additionally supported combination of modulation and symbol rate.

The TRX power class is defined by the highest single carrier output power capability for any modulation.

b)	Requirements for multicarrier BTS

For BTS belonging to a multicarrier BTS class, the manufacturer shall declare the maximum output power per carrier in case that all carriers are operated at the same nominal output power. The declaration shall be given for each modulation and for all supported number of carriers up to the maximum number on each antenna port. Additionally, the maximum total power supported shall be declared.

The maximum total output power measured at the antenna connector after all stages of combining shall be as declared by the manufacturer but within the limits of the multicarrier BTS class, specified in table 4.1-8.

Table 4.1-8 Multicarrier BTS classes

| All bands |  |
| --- | --- |
| Multicarrier BTS class | Total ouput power limit per antenna port |
| Wide Area | (NOTE 2) |
| Medium Range | 38 dBm |
| Local Area | 24 dBm |
| NOTE 1: Medium Range and Local Area classes are not applicable for MXM 850 and MXM 1900 NOTE 2: There is no upper power limit for the Wide Area multicarrier BTS |  |

c) Requirements for all types of base stations

The tolerance of the actual maximum output power of the BTS for each supported modulation shall be ±2 dB under normal conditions and ±2,5 dB under extreme conditions. Settings shall be provided to allow the output power to be reduced from the maximum level for the modulation with the highest output power capability in at least six steps of nominally 2 dB with an accuracy of ±1 dB for each modulation to allow a fine adjustment of the coverage by the network operator. In addition, the actual absolute output power for each supported modulation at each static RF power step (N), with the exception below for the highest RF power level for 8-PSK, AQPSK, QPSK, 16-QAM and 32-QAM shall be 2*N dB below the absolute output power at static RF power step 0 for the modulation with the highest output power capability with a tolerance of ±3 dB under normal conditions and ±4 dB under extreme conditions. The static RF power step 0 shall be the actual output power according to the TRX power class.

As an option the BSS can utilize downlink RF power control. In addition to the static RF power steps described above, the BSS may then for each supported modulation utilize up to 15 steps of power control levels with a step size of 2 dB ± 1,5 dB, in addition the actual absolute output power for each supported modulation at each power control level (N), with the exception below for the highest power level for QPSK, AQPSK, 8-PSK, 16-QAM and 32-QAM, shall be 2*N dB below the absolute output power at power control level 0 for the modulation with the highest output power capability with a tolerance of ±3 dB under normal conditions and ±4 dB under extreme conditions. The power control level 0 shall be the set output power according to the TRX power class and the six power settings defined above.

The output power for GMSK, QPSK, AQPSK, 8-PSK, 16-QAM and 32-QAM shall be nominally the same for any supported static RF power step and power control level. An exception is allowed for the maximum output power levels of respectively QPSK, AQPSK, 8-PSK, 16-QAM and 32-QAM which may be lower than the GMSK output power for the same power step or power control level. The nominal size of the first step down from the respective maximum power level of QPSK, AQPSK, 8-PSK, 16-QAM and 32-QAM may be in the range 0…2 dB. The output power for the GMSK, QPSK, AQPSK, 8-PSK, 16-QAM and 32-QAM at this power control level shall still be considered the same when required in 3GPP TS 45.008. The output power of QPSK, AQPSK, 8-PSK, 16-QAM and 32-QAM for the remaining power steps or power control levels shall be the same as the GMSK power for the corresponding power step or power control level within a tolerance of ±1 dB. The number of static RF power steps and the total number of power control steps may be different for GMSK and other modulations.

Network operators or manufacturers may also specify the BTS output power including any Tx combiner, according to their needs.

#### 4.1.2.1	Additional requirements for PCS 1 900 and MXM 1900 Base stations

The BTS transmitter maximum rated output power per carrier, measured at the input of the transmitter combiner, shall be, according to its TRX power class, as defined in the table 4.1-6 above. The base station output power may also be specified by the manufacturer or system operator at a different reference point (e.g. after transmitter combining).

The maximum radiated power from the BTS, including its antenna system, shall not exceed a maximum of 1 640 W EIRP, equivalent to 1 000 W ERP, per the applicable FCC rules for wideband PCS services [FCC part 24, subpart E, section 24.237].

#### 4.1.2.2	Additional requirements for GSM 850 and MXM 850 Base stations

The BTS transmitter maximum rated output power per carrier, measured at the input of the transmitter combiner, shall be, according to its TRX power class, as defined in the table 4.1-6 above. The base station output power may also be specified by the manufacturer or system operator at a different reference point (e.g. after transmitter combining).

The maximum radiated power from the BTS, including its antenna system, shall not exceed a maximum of 500 W ERP, per the applicable FCC rules for public mobile services [FCC part 22, subpart H, section 22.913].

#### 4.1.2.3	Additional requirements for GSM 700 Base stations

The BTS transmitter maximum rated output power per carrier, measured at the input of the transmitter combiner, shall be, according to its TRX power class, as defined in the table 4.1-6 above. The base station output power may also be specified by the manufacturer or system operator at a different reference point (e.g. after transmitter combining).

The maximum radiated power from the BTS, including its antenna system, shall not exceed a maximum 1000 W ERP for GSM 700 BTS per the applicable FCC rules [FCC Part 27, Subpart C, Section 27.50].

#### 4.1.2.4	Additional requirements for ER-GSM 900 Base stations

The BTS transmitter maximum rated output power per carrier, measured at the input of the transmitter combiner, shall be, according to its TRX power class, as defined in the table 4.1-6 above.

The BTS transmitter maximum rated output power per carrier shall be subject to regulatory coordination to avoid uncoordinated system impacts (RF scenarios for ER-GSM introduction and the MCL to be used are given in 3GPP TR 45.050) based on the case of uncoordinated or coordinated deployment in the same geographical area with other systems in the E-GSM band as specified in this clause.

##### 4.1.2.4.1 Uncoordinated deployment

In case of uncoordinated deployment with other systems in the E-GSM band, in order to prevent blocking, the BTS transmitter maximum rated output power per carrier, measured at the input of the transmitter combiner, in the frequency range 918-921 MHz, shall be at most:

i) -40.4 dBm + MCL + (f-918.2)*6 dB in case of coexistence with GSM BTS

ii) -58.7 dBm + MCL + (f-918.2)*11 dB in case of coexistence with UTRA and E-UTRA BS

where f = DL frequency in MHz, 918.2 ≤ f ≤ 921.0 and MCL=67dB.

##### 4.1.2.4.2 Coordinated deployment

In case of coordinated deployment with other systems in the E-GSM band, MCL higher than 67 dB can be taken into account to allow higher output power from an ER-GSM BTS transmitting in 918-921 MHz.

***** Next modification *****

### 4.6.3	Phase coherency

#### 4.6.3.1	Definition

Phase coherency occurs when the phase of the transmitted waveform of the useful part of a burst (including tail bits) is the same between transmitted bursts. Using the same pseudo-random data sequence in all active timeslots, the phase coherency error on the useful part of the burst (including tail bits), shall be measured by computing the difference between the phase of one transmitted waveform of a burst and the phase of the one transmitted in the previous, adjacent time slot. The RMS value of the phase difference over the useful part of a burst and the peak deviation shall not exceed the values below.

#### 4.6.3.2	EC-EGPRS MS

Extended coverage performance in EGPRS operation (EC-EGPRS) is based on blind physical layer transmissions of bursts in multiple timeslots and/or multiple TDMA-frames. To allow the receiver to coherently accumulate the blind physical layer transmissions over multiple timeslots within a TDMA-frame, phase coherency between these transmissions is required. No requirement on phase coherency of blind physical layer transmissions between TDMA frames is required.

The deviation from phase coherency between any two bursts of the blind physical layer transmissions transmitted over consecutive timeslots for EC-channels in CC other than CC1 shall not exceed [tbd] degrees rms and [tbd] degrees peak.

The phase coherency requirement applies to the 2 TS EC-RACH mapping, EC-PDTCH and EC-PACCH, see 3GPP TS 45.002 for how the bursts of a block for a specific logical channel are mapped onto the physical channel(s).

#### 4.6.3.3	BTS

For a BTS using EC-EGPRS channels, extended coverage performance is based on blind physical layer transmissions of blocks in multiple timeslots and/or multiple TDMA-frames. To allow the receiver to coherently accumulate the blind physical layer transmissions over multiple timeslots withing a TDMA, phase coherency between these transmissions is required. No requirement on phase coherency of blind physical layer transmissions between TDMA frames is required.

The deviation from phase coherency between any two bursts of the blind physical layer transmissions transmitted over consecutive timeslots for EC-channels in any supported CC other than CC1 shall not exceed [tbd] degrees rms and [tbd] degrees peak.

The phase coherency requirement applies to EC-PDTCH and EC-PACCH, see 3GPP TS 45.002 for how the bursts of a block for a specific logical channel are mapped onto the physical channel(s).

***** Next modification *****

## 6.1a	MS conditions

In order to assess the error rate performance that is described in this clause it is required for a mobile equipment to have a "loop back" facility by which the equipment transmits back the same information that it decoded, in the same mode. This facility is specified in 3GPP TS 44.014. In this clause power levels are given also in terms of field strength, assuming a 0 dBi gain antenna, to apply for the test of MS with integral antennas.

The requirements specified in this clause shall be met by a MS in CTS mode. In particular the requirement of subclause 6.6 on frequency hopping performance shall be met by a MS performing CTS frequency hopping (as specified in 3GPP TS 45.002 subclause 6.2).

The requirements for the receiver performance in non-static channels with 16QAM, 32QAM and QPSK modulations are specified for training sequence 6 (TSC-6, as defined in 3GPP TS 45.002).

In case of  VAMOS mode, one of the VAMOS subchannels (see 3GPP TS 45.001) shall use training sequence 5 from TSC set 1 (and is referred to as VAMOS subchannel 1) and the other VAMOS subchannel shall use training sequence 5 from TSC set 2 (and is referred to as VAMOS subchannel 2). The requirements for the receiver performance for speech and control channels in VAMOS mode shall be met on VAMOS subchannel 1 and VAMOS subchannel 2 when the values of SCPIR_DL specified in tables 1s, 1t, 1u, 1x, 1y, 2aa, 2ab and 2ag are used for the AQPSK signal.

An EC-EGPRS capable MS only needs to support packet-switched service. Support of modulation- and coding scheme MCS-1 for all Coverage Classes (CC1, CC2, CC3 and CC4) is mandatory. Other GMSK modulation- and coding schemes shall be supported only in CC1. The support of 8-PSK modulation- and coding schemes is optional and in this case only in CC1. 
A MS in EC-EGPRS operation shall apply overlaid cdma coding (see 3GPP TS 45.004) in CC2, CC3 and CC4, when requested by BTS. In this case the signals with different cdma codes are referred to as “EC-EGPRS subchannels 1 to 4”.
For a MS supporting EC-EGPRS conformance to requirements in 5.1.2, 5.2.2 and 5.3.2 shall be fulfilled assuming coding scheme MCS-1 and associated reference sensitivity performance.

For a MS indicating support for DLMC the following applies for the requirements specified in this subclause and in clause 5:

In case of a DLMC configuration, DLMC specific requirements apply, except

-	whenever the maximum supported carrier frequency spacing only allows for a single carrier to be received in a given radio block period, see 3GPP TS 45.002.

-	when fallback to single carrier reception is performed, see 3GPP TS 45.002. In case of inter-band reception, the requirements only apply for the carrier in the frequency band on which the fallback is performed

-	when PTCCH/D is being received,

in which case the requirements not specific to DLMC apply.

In case of MS in DLMC configuration, the stated performance at each receive antenna port shall be achieved for any of the wanted input signals at the specified power level when receiving up to the supported maximum number of downlink carriers at equal power within the maximum supported DLMC carrier frequency spacing. The required receiver resources for the supported maximum number of downlink carriers (see 3GPP TS 24.008) shall be allocated and activated simultaneously, distributed over maximum supported DLMC frequency spacing during the complete test.

NOTE:	When testing for compliance of DLMC specific performance requirements, the applied input signals may, when appropriate, be limited to two useful signals with equal power for requirements in subclause 5.1 and 5.2, and one useful signal for requirements in clause 6, unless otherwise stated.

For a MS supporting DLMC the signalled Maximum Bandwidth, see 3GPP TS 24.008, corresponds to a maximum supported DLMC carrier frequency spacing as shown in Table 6a-1.

Table 6a-1. Nominal maximum supported DLMC carrier frequency spacing.

| Maximum DLMC Bandwidth (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| Maximum supported DLMC carrier frequency spacing / Number of GSM channels | 4.2 MHz/22 | 8.8 MHz/45 | 13.2 MHz/67 | 18 MHz /91 |

For a MS indicating support for DLMC, no separate requirements apply for inter-band reception unless otherwise stated.

For a MS indicating support for non-contiguous intra-band reception, requirements additionally apply at carrier spacings larger than the maximum supported DLMC carrier frequency spacing, with carrier frequency spacing in each of the two carrier groups separated by at most the maximum supported DLMC carrier frequency spacing.

NOTE:	 There are no requirements defined for EGPRS2-B for DLMC configuration.

## 6.1b	BTS conditions

In the case of base transceiver stations the values apply for measurement at the connection with the antenna of the BTS, including any external multicoupler.

The Rx performance requirements of BTS for modulation schemes using higher symbol rate are based on input signals using wide pulse shaping filter unless otherwise stated. When the wanted input signal is such a signal, it is called Wanted signal Wide.

When the wanted input signal for BTS is using the higher symbol rate with narrow pulse shaping filter, it is called Wanted signal Narrow.

For channels with higher symbol rate the requirements for BTS for non-static propagation conditions are specified with RX diversity with two antennas applied and without RX diversity in the cases where the BTS has only one antenna port. The RX diversity requirements are specified for no correlation or gain imbalance between the two receive branches.

The requirements for the receiver performance in non-static channels with 16QAM, 32QAM and QPSK modulations are specified for training sequence 6 (TSC-6, as defined in 3GPP TS 45.002).

For speech and control channels in VAMOS mode, the requirements for BTS are specified only for non-static propagation conditions. These requirements are with RX diversity with two antennas with no correlation or gain imbalance between the two receive branches.

Unless explicitly stated, the requirements specified for GMSK modulated channels are applicable only for channels that are not in VAMOS mode.

In case of VAMOS mode, one of the VAMOS subchannels shall use training sequence 5 from TSC set 1 (and is referred to as VAMOS subchannel 1) and the other VAMOS subchannel shall use training sequence 5 from TSC set 2 (and is referred to as VAMOS subchannel 2).  For the cases where SCPIR is not equal to 0 dB, the VAMOS subchannel 2 shall have lower power than VAMOS subchannel 1.The requirements for the receiver performance for speech and control channels in VAMOS mode in tables 1v and 2ac are applicable for both VAMOS subchannels (see 3GPP TS 45.001).

A BTS supporting EC-EGPRS shall, in addition to fulfilling EGPRS requirements, fulfil the requirements for EC-channels in at least coverage classes [CC1 and CC4]. The set of coverage classes supported on EC-CCCH is broadcasted in System Information (EC-SI) and included in the manufacturer’s declaration.The BTS shall support EC-RACH (at least one of 1 TS or 2 TS mapping, see 3GPP TS 45.002), EC-PACCH, EC-AGCH and EC-PCH, and at least support MCS-1 for EC-PDTCH.
In case of EC-EGPRS channel in higher coverage class than 1 is used, the BTS shall be able to distinguish between signals from up to 4 different assigned MSs on the same timeslot when overlaid cdma (see 3GPP TS 45.002 and 3GPP TS 45.004) is applied, i.e. when EC-EGPRS subchannels are applied. For the cases where SCPIR is not equal to 0 dB, the subchannel n+1 shall have lower power than EC-EGPRS subchannel n, where n is equal to 1 to N-1 for subchannel N.The requirements for the receiver performance for packet data and control channels in table 1ad is applicable for each EC-EGPRS subchannel (see 3GPP TS 45.001) when in specified coverage class and the other subchannels always are in CC2.

In case of multicarrier BTS equipped with multicarrier receiver, the stated performance at each receiver antenna port connected to a multicarrier receiver shall be achieved for any of the wanted input signals at the specified power level when receiving up to the maximum supported number of wanted signals at equal power within the declared maximum Base Station RF bandwidth at the receiver antenna connector. The stated performance shall be achieved for requirements specifying an input level of each wanted signal up to and including -40 dBm. For requirements specifying a higher input level of each wanted signal, the stated performance applies for the total peak input level to the multicarrier BTS equipped with multicarrier receiver being not greater than the peak level when receiving a single wanted carrier for the same requirement. The stated performance shall be achieved provided that the frequency spacing between each wanted signal and each other signal that is not a dedicated interferer for that wanted signal is at least 600 kHz.

NOTE:	Minimum receiver performance requirements for multicarrier base station declared as multistandard radio base station are defined in Annex P.

***** Next modification *****

### 6.2.2	Packet-switched channels

i) For packet switched channels, the minimum input signal level for which the reference performance shall be met in case of normal symbol-rate, Basic Transmit-Time-Interval (BTTI) and no Piggy-backed ACK/NACK reporting (PAN) used, is specified in table 1a for GMSK modulated input signals, tables 1b and 1c for 8-PSK modulated input signals (convolutional coding), table 1l for input signals modulated by 8-PSK (turbo coding), tables 1k and 1l for input signals modulated by 16-QAM and 32-QAM respectively, according to the type of channel and the propagation condition. In case higher symbol rate, Basic Transmit-Time-Interval (BTTI) and no Piggy-backed ACK/NACK reporting (PAN) are used, the minimum input signal level for which the reference performance shall be met, is specified in table 1m and 1n for input signals modulated by QPSK or 16-QAM or 32-QAM respectively, according to the type of channel and the propagation condition. For Reduced Transmit-Time-Interval (RTTI) the minimum performance requirements are the same as for Basic Transmit-Time-Interval (BTTI) on a static channel. When Piggy-backed ACK/NACK reporting (PAN) is used, the minimum performance requirements of tables 1o and 1p apply. The minimum input signal level for which the reference performance shall be met for PAN is specified in table 1q and 1r. Tables 1p and 1q apply to BTS for input signals with wide pulse shaping filter in the case of higher symbol rate. The performance requirements for GSM 400 and GSM 700 systems are as for GSM 900, except that the GSM 400 MS speed is doubled from that of GSM 900, e.g. TU50 becomes TU100, and the GSM 700 MS speed is increased by a factor of 1.2, e.g. TU50 becomes TU60. The levels are given for normal BTS for GMSK modulated signals. For QPSK, 8-PSK, 16-QAM and 32-QAM modulated signals, the required levels are given for normal BTS and MS separately. When in EC-EGPRS operation the requirements in table 1z and 1aa (for BTS and MS respectively) for GMSK modulation and 1aa and 1ab (for BTS and EC-EGPRS MS respectively) for 8-PSK modulation shall be met only in case of BTTI, no PAN, and in the downlink randomly altered USF in each of the blind physical layer transmissions, assuming one user per timeslot. In case EC-EGPRS subchannels are muliplexed on the same time slot in coverage classes CC2, CC3 and CC4, the requirements in table 1ad shall be met, when the model of time and frequency offsets defined in Annex R is applied.
The levels shall be corrected by the following values:

Table 6.2-4 Correction factor of performance requirements for different equipments

|  | MS, GMSK modulated signals |  |
| --- | --- | --- |
| - | for DCS 1 800 class 1 or class 2 MS | +2/+4 dB** |
| - | for DCS 1 800 class 3 MS | +2 dB |
| - | for GSM 400 small MS, GSM 900 small MS, ER-GSM 900 small MS, GSM 850 small MS and GSM 700 small MS | +2 dB |
| - | for other GSM 400, GSM 900 MS, ER-GSM 900 and GSM 850 MS and GSM 700 MS | 0 dB |
|  | for PCS 1900 class 1 or class 2 MS | +2 dB |
|  | for other PCS 1900 MS | 0 dB |
|  | MS, QPSK, 8-PSK, 16-QAM and 32-QAM modulated signals |  |
| - | for GSM 400, GSM 900, ER-GSM 900, GSM 850 and GSM 700 small MS | 0 dB |
| - | for other GSM 400, GSM 900, ER-GSM 900, GSM 850 and GSM 700 MS | -2 dB |
| - | for DCS 1 800 and PCS 1900 class 1 or class 2 MS | 0 dB |
| - | for other DCS 1 800 and PCS 1900 MS | -2 dB |
|  | BTS |  |
| - | for normal BTS | 0 dB |
| - | for GSM 900, ER-GSM 900, GSM 850, MXM 850 and GSM 700 micro BTS M1 | +7 dB |
| - | for GSM 900, ER-GSM 900, GSM 850, MXM 850 and GSM 700 micro BTS M2 | +12 dB |
| - | for GSM 900, ER-GSM 900, GSM 850, MXM 850 and GSM 700 micro BTS M3 | +17 dB |
| - | for GSM 900, ER-GSM 900, GSM 850, MXM 850 and GSM 700 pico BTS P1 | +16 dB |
| - | for DCS 1 800, PCS 1900 and MXM 1900 micro BTS M1 | +2 dB |
| - | for DCS 1 800, PCS 1900 and MXM 1900 micro BTS M2 | +7 dB |
| - | for DCS 1 800, PCS 1900 and MXM 1900 micro BTS M3 | +12 dB |
| - | for DCS 1 800, PCS 1900 and MXM 1900 pico BTS P1 | +9 dB |
| - | for Wide Area multicarrier BTS | 0 dB |
| - | for Medium Range multicarrier BTS | +6 dB |
| - | for Local Area multicarrier BTS | +14 dB |

**	For DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

The reference performance shall be according to Table 6.2-5:

Table 6.2-5 Reference performance for Packet channels

| Logical channel | Requirement |
| --- | --- |
| Packet data channels (PDCH) | BLER £ 10% unless otherwise stated |
| Uplink State Flags (USF) | BLER £ 1% |
| Packet Random Access Channels (PRACH) | BLER £ 15% |
| Piggy-backed ACK/NACK report (PAN) | PAN error rate £ 5% |
| Extended Coverage packet data channels (EC-PDTCH/U) | BLER £ 50% |
| Extended Coverage packet data channels (EC-PDTCH/D) | BLER £ 20% |
| Extended Coverage random access channels (EC-RACH) | BLER £ 20% |
| Extended Coverage control channels (EC-CCCH, EC-PACCH and EC-BCCH) | BLER £ 10% |
| Extended Coverage synchronization channel (EC-SCH) | BLER £ 10% |

| - |  |  |
| --- | --- | --- |
| - |  |  |
| - |  |  |

where BLER is the Block Error Rate, referring to all erroneously decoded data blocks including any headers, stealing flags, parity bits as well as any implicit information in the training sequence, or any downlink block discrimination error between EC-PDTCH and EC-PACCH.

For PDCH the BLER refers to RLC blocks, and hence there can be up to two block errors per 20ms radio block for EGPRS MCS7, MCS8, MCS9, UAS-7, UAS-8, UAS-9, UBS-7, UBS-8, DAS-8, DAS-9, DAS-10, DBS-7 and DBS-8, up to three block errors per radio block for UAS-10, UAS-11, UBS-9, UBS-10, DAS-11, DAS-12, DBS-9 and DBS-10, and up to four block errors per radio block for UBS-11, UBS-12, DBS-11 and DBS-12. The BLER refers to the initial transmission of RLC blocks, i.e. the channel decoding without incremental redundancy. For USF, the BLER only refers to the USF value. 
For EC-PDTCH and EC-PACCH channels in downlink, the BLER shall be evaluated with random USF in each of the blind physical layer transmissions.

ii) If BTTI USF mode is used when sending downlink data blocks in RTTI configuration and different modulations are used in the two data blocks sent in a 20 ms block period, the USF will be sent with mixed modulation. In this case, the performance of the mixed modulation USF shall meet the less stringent requirement of the two modulations for static propagation conditions according to:

Table 6.2-6 USF performance in mixed modulation

| GMSK | USF/MCS-1 to 4 | (table 1a) |
| --- | --- | --- |
| 8PSK | USF/MCS5 to 9 | (table 1c) |
| 16QAM (EGPRS2-A) | USF/DAS-8 to 9 | (table 1l) |
| 32QAM (EGPRS2-A) | USF/DAS-10 to 12 | (table 1l) |
| QPSK (EGPRS2-B) | USF/DBS-5 to 6 | (table 1n) |
| 16QAM (EGPRS2-B) | USF/DBS-7 to 9 | (table 1n) |
| 32QAM (EGPRS2-B) | USF/DBS-10 to 12 | (table 1n) |

For PDCH channels, the performance requirements for some modulation and coding schemes and propagation conditions are specified at higher BLER. Where applicable, the BLER value noted in the respective performance requirement table applies.

***** Next modification *****

### 6.2.4	Repeated associated control channel performance

For Repeated Downlink FACCH and Repeated SACCH (see 3GPP TS 44.006), the minimum input signal level for which the reference performance shall be met is specified in table 1i, 1s, 1t, 1v and 1x, according to the propagation condition and type of equipment. The performance requirements for GSM 400 and GSM 700 systems are as for GSM 900 in table 1i, 1s, 1t, 1v and 1x, except that the GSM 400 MS speed is doubled from that of GSM 900, e.g. TU50 becomes TU100, and the GSM 700 MS speed is increased by a factor of 1.2, e.g. TU50 becomes TU60.

The reference performance for Repeated Downlink FACCH and Repeated SACCH shall be FER £ 5%. When calculating FER, a FACCH frame and its repetition or a SACCH frame and its repetition respectively, shall be counted as one frame and a frame erasure shall be counted when neither the FACCH frame nor its repetition or neither the SACCH frame nor its repetition respectively, could be successfully decoded.

The reference performance levels for Repeated Downlink FACCH and Repeated SACCH in table 1i and 1v shall be corrected according to the values in the table 6.2-4.

### 6.2.4a	Repeated control channel and data channel performance for EC-EGPRS

For EC-channels in Coverage Classes higher than CC1 (i.e. CC2, CC3 and CC4) blind physical layer transmissions are used (EC-PDTCH, EC-RACH, EC-PACCH, EC-BCCH, EC-AGCH, EC-PCH). The minimum input signal level for which the reference performance shall be met is specified in table 1z, 1aa, 1ab, 1ac and 1ad, according to the propagation condition and type of equipment, assuming the input signal fulfils the phase coherency requirements in subclause 4.6.4, where applicable.

***** Next modification *****

### 6.3.2	Co-channel reference interference performance

#### 6.3.2.1	MS requirements

For GMSK modulated channels, packet switched and ECSD and speech channels (AMR-WB), and for 8-PSK modulated channels, packet switched and ECSD and speech channels (AMR and AMR-WB), and for 16-QAM, 32-QAM and QPSK modulated packet switched channels, the minimum interference ratio for which the reference performance for cochannel interference (C/Ic) shall be met is specified in table 2a, 2e and 2j (GMSK), 2c, 2e, 2k, and 2s (8-PSK), 2s, and 2u (16-QAM), 2s, and 2u (32-QAM), respectively, according to the type of channel and the propagation condition, when BTTI and no PAN are used.

For FLO, the minimum interference ratio for which the reference performance for cochannel interference (C/Ic) shall be met is specified in table 2m according to the type of reference TFC and the propagation condition.

For Repeated Downlink FACCH and Repeated SACCH (see 3GPP TS 44.006), the minimum interference ratio for which the reference performance for cochannel interference (C/Ic) shall be met is specified in table 2p according to the propagation condition.

The performance requirements for GSM 400 and GSM 700 systems are as for GSM 900 in table 2a, 2c, 2e, 2j, 2k, 2m, 2o, 2p, 2q, 2s, 2u, except that the GSM 400 MS speed is doubled from that of GSM 900, e.g. TU50 becomes TU100, and the GSM 700 MS speed is increased by a factor of 1.2, e.g. TU50 becomes TU60. The reference performance is the same as defined in subclause 6.2.

For equipment supporting 8-PSK, and for MS indicating support for Downlink Advanced Receiver Performance – phase I (see 3GPP TS 24.008), the applicable requirements in table 2a, 2c, 2e, 2j, 2k, 2m, 2p, 2s, and 2u apply for both GMSK and 8-PSK modulated interfering signals.

For AQPSK modulated speech channels ( TCH/HS, TCH/AFSx, TCH/AHSx, TCH/EFS, TCH/WFSx – in downlink), and their associated control channels, the applicable requirements are in tables 2aa for VAMOS I MS, 2ab for VAMOS II MS and 2ag for VAMOS III MS.

A VAMOS III MS shall fulfil the requirements in table 2ag for the antenna correlation and antenna gain imbalance setup defined in annex Q.7.

In case of DLMC configuration, the reference interference performance specified above for TU50 no FH applies. Co-channel interference requirements for other propagation conditions are not specified.

For a mobile station in EC-EGPRS operation, the requirements on EC-channels for co-channel performance in the table 2ai apply for GMSK modulated channels and table 2ak for 8-PSK modulated channels.

#### 6.3.2.2	BTS requirements

For GMSK modulated channels, packet switched and ECSD and speech channels (AMR-WB), and for 8-PSK modulated channels, packet switched and ECSD and speech channels (AMR and AMR-WB), and for 16-QAM, 32-QAM and QPSK modulated packet switched channels, the minimum interference ratio for which the reference performance for cochannel interference (C/Ic) shall be met is specified in table 2a, 2d and 2j (GMSK), 2b, 2d and 2k, (8-PSK), 2r and 2t (16-QAM), 2t (32-QAM), respectively, according to the type of channel and the propagation condition, when BTTI and no PAN are used.

For FLO, the minimum interference ratio for which the reference performance for cochannel interference (C/Ic) shall be met is specified in table 2m according to the type of reference TFC and the propagation condition.

For Repeated SACCH (see 3GPP TS 44.006), the minimum interference ratio for which the reference performance for cochannel interference (C/Ic) shall be met is specified in table 2p according to the propagation condition.

The performance requirements for GSM 400 and GSM 700 systems are as for GSM 900 in table 2a, 2b, 2d, 2j, 2k, 2m, 2p, 2r, 2t, except that the GSM 400 MS speed is doubled from that of GSM 900, e.g. TU50 becomes TU100, and the GSM 700 MS speed is increased by a factor of 1.2, e.g. TU50 becomes TU60. The reference performance is the same as defined in subclause 6.2.

For equipment supporting 8-PSK, the applicable requirements in table 2a, 2b, 2d, 2j, 2k, 2m, 2p, 2r and 2t apply for both GMSK and 8-PSK modulated interfering signals.

For GMSK modulated speech channels in VAMOS mode (TCH/HS, TCH/AFSx, TCH/AHSx, TCH/EFS, TCH/WFSx – in uplink), and their associated control channels, the applicable requirements are in table 2ac.

In case BTS is using EC-EGPRS channels, the requirements on EC-channels for co-channel performance in the table 2ah apply for GMSK modulated channels and table 2aj for 8-PSK modulated channels.

### 6.3.3	Adjacent channel reference interference performance

The corresponding interference ratio for adjacent channel interference shall be:

#### 6.3.3.1	Normal symbol rate used

For equipment supporting 8-PSK, the requirements apply for both GMSK and 8-PSK modulated interfering signals.

##### 6.3.3.1.1	MS requirements

Table 6.3-1a Reference interference ratio requirements in adjacent channels for Packet-switched (Normal symbol-rate), Enhanced circuit-switched data, Wideband AMR and 8-PSK modulated AMR channels, speech and associated control channels in VAMOS mode, TIGHTER - MS

|  |  | Modulation of wanted signal |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GMSK | 8-PSK | 16-QAM | 32-QAM | AQPSK |
| for adjacent (200 kHz) interference | C/Ia1 = | C/Ic - 18 dB, see table 2af for TIGHTER MS | See table 2l for speech, see tables 2g, 2i, 2n and 2w for other channels, see table 2af for TIGHTER MS | See table 2w, see table 2af for TIGHTER MS | See table 2w, see table 2af for TIGHTER MS | See table 2aa, 2ab and 2aq |
| for adjacent (400 kHz) interference | C/Ia2 = | C/Ic - 50 dB | C/Ic - 50 dB | C/Ic - 48 dB | C/Ic - 48 dB | Note 1 |
| for adjacent (400 kHz) interference in DLMC configuration | C/Ia2 = | C/Ic –41 dB | C/Ic –41 dB | C/Ic –39 dB | C/Ic –39 dB |  |
| for adjacent (600 kHz) interference | C/Ia3 = | C/Ic - 58 dB | C/Ic - 58 dB |  |  |  |
| NOTE 1: The adjacent channel interference @ 400 kHz requirement (C/Ia2) does not apply to channels in VAMOS mode and for EC-channels. NOTE 2: The adjacent channel interference @ 600 kHz requirement (C/Ia3) does not apply in DLMC configuration. |  |  |  |  |  |  |

NOTE:	The C/Ia3 figure is given for information purposes and will not require testing. It was calculated for the case of an equipment with an antenna connector, operating at output power levels of +33 dBm and below. Rejection of signals at 600 kHz is specified in subclause 5.1.

For packet switched channels, the requirements for adjacent channel performance in the tables above apply to channels with BTTI and no PAN used.

The values in tables 2g, 2i, 2l, 2n, 2w and 2af, are also valid for GSM 400 with the exception that MS speed is doubled, e.g. TU50 becomes TU100. For GSM 700 the values in tables 2g, 2i, 2l, 2n, 2v, 2w and 2af, are valid with the exception that GSM 700 MS speed is increased by a factor of 1.2, e.g. TU50 becomes TU60.

In case of DLMC configuration the reference interference performance specified above for TU50 no FH applies. Adjacent channel interference requirements for other propagation conditions are not specified.

For a mobile station in EC-EGPRS operation, the requirements on EC-channels for adjacent channel performance in the table 2an apply for GMSK modulated channels and table 2ap for 8-PSK modulated channels.

##### 6.3.3.1.2	BTS requirements

Table 6.3-1b Reference interference ratio requirements in adjacent channels for Packet-switched (Normal symbol-rate), Enhanced circuit-switched data, Wideband AMR and 8-PSK modulated AMR channels, speech and associated control channels in VAMOS mode - BTS

| Modulation of wanted signal |  |  |  |  |  | GMSK | 8-PSK | 16-QAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - | for adjacent (200 kHz) interference |  | C/Ia1 |  | = | C/Ic - 18 dB See table 2ac for channels in VAMOS mode | See table 2l for speech, see tables 2f, 2h, and 2n for other channels | See tables 2v |
| - | for adjacent (400 kHz) interference |  | C/Ia2 |  | = | C/Ic - 50 dB | C/Ic –50 dB | C/Ic –48 dB |
| - | for adjacent (600 kHz) interference |  | C/Ia3 |  | = | C/Ic - 58 dB | C/Ic –58 dB |  |

NOTE 1:	The adjacent channel interference @ 400 kHz requirement (C/Ia2) does not apply to channels in VAMOS mode and for EC-channels.

NOTE 2:	The C/Ia3 figure is given for information purposes and will not require testing. It was calculated for the case of an equipment with an antenna connector, operating at output power levels of +33 dBm and below. Rejection of signals at 600 kHz is specified in subclause 5.1.

For packet switched channels, the requirements for adjacent channel performance in the tables above apply to channels with BTTI and no PAN used.

The values in tables 2f, 2h, 2l, 2n and 2v, are also valid for GSM 400 with the exception that MS speed is doubled, e.g. TU50 becomes TU100. For GSM 700 the values in tables 2f, 2h, 2l, 2n and 2v, are valid with the exception that GSM 700 MS speed is increased by a factor of 1.2, e.g. TU50 becomes TU60.

In case BTS is using EC-EGPRS channels, the requirements for adjacent channel performance in the table 2am apply for GMSK modulated channels and table 2ao for 8-PSK modulated channels.

***** Next modification *****

### 6.3.4	Reference interference performance – signal levels

For speech and CS data services, unless stated otherwise below, the wanted signal input level shall be 20 dB above the reference sensitivity level listed in table 6.2-1a and table 6.2-1b.

For packet switched and AMR-WB speech GMSK modulated channels the wanted input signal level shall be: -93 dBm + Ir + Corr, where:

Ir = the interference ratio according to tables 2a, table 2j , 2ah and 2ai for the packet switched, and AMR-WB speech channels respectively 
Corr = the correction factor for reference performance according to table 6.2-4.

For 8-PSK modulated speech channels (AMR and AMR-WB), ECSD channels and 8-PSK modulated packet-switched channels, the wanted input signal level shall be: - 93 dBm + Ir + Corr, where:

Ir = the interference ratio according to tables 2b, 2c, 2s. 2aj and 2ak for packet switched channels, tables 2d and 2e for ECSD, table 2k for speech (AMR and AMR-WB) and associated control channels, 
Corr = the correction factor for reference performance according to table 6.2-4.

For QPSK, 16-QAM and 32-QAM modulated packet-switched channels, the wanted input signal level shall be: - 93 dBm + Ir + Corr, where:

Ir = the interference ratio according to tables 2t and 2u for QPSK modulated packet switched channels, tables 2r, 2s, 2t and 2u for 16-QAM modulated packet switched and tables 2s, 2t and 2u for 32-QAM modulated packet switched channels.
Corr = the correction factor for reference performance according to table 6.2-4.

For AQPSK modulated speech channels and control channels in downlink, the wanted input signal level shall be: -93 dBm + Ir, where:

Ir = the interference ratio according to tables 2aa for VAMOS I MS, 2ab for VAMOS II MS and 2ag for VAMOS III MS for VDTS-1, VDTS-2 and VDTS-3 (see subclause Q.1) for speech and associated control channels in VAMOS mode in downlink.

For GMSK modulated speech channels and control channels in VAMOS mode in uplink, the input signal level of VAMOS subchannel 2 shall be: -93 dBm + Ir + Corr, where:

Ir = the interference ratio according to tables 2ac for VUTS-1,  VUTS-3 and VUTS-4 (see subclause Q.2).
Corr = the correction factor for reference performance according to table 6.2-4.

For FLO, the wanted input signal level shall be: -93 dBm + Ir + Corr, where:

Ir = the interference ratio according to table 2m.
Corr = the correction factor for reference performance according to table 6.2-4.

For Repeated Downlink FACCH and Repeated SACCH (see 3GPP TS 44.006), the wanted input signal level shall be: -93 dBm + Ir + Corr, where:

Ir = the interference ratio according to table 2p.
Corr = the correction factor for reference performance according to table 6.2-4.

For all channels specified in table 2ad for TIGHTER MS, the wanted signal level shall be: -93 dBm + Ir + Corr, where:

Ir = the interference ratio according to table 2ad
Corr = the correction factor for reference performance according to table 6.2-4.

For adjacent channel performance for packet-switched channels except for the adjacent channel (200 kHz) requirements of EGPRS2 specific channels or EC-EGPRS channels, the wanted input signal level shall be set to the value calculated using the formulas above for cochannel performance.

For the adjacent channel (200 kHz) requirements of EGPRS2-A packet-switched channels, UAS-7 to 11 and DAS-5 to 12, and EGPRS2-B packet-switched channels, UBS-5 to 12 and DBS-5 to 12, the wanted input signal level shall be: -75 dBm + Iar + Corr, where:

Iar = the adjacent channel (200 kHz) interference ratio according to table 2v for UAS-7 to 11, table 2w for DAS-5 to 12, table 2y for DBS-5 to 12 and table 2x and 2z for UBS-5 to 12 respectively.
Corr = the correction factor for reference performance according to table 6.2-4.

For the adjacent channel (200 kHz) requirements for EC-EGPRS packet-switched channels, the wanted input signal level shall be: -75 dBm + Iar + Corr, where:

Iar = the adjacent channel (200 kHz) interference ratio according to table 2am, 2an, 2ao and 2ap.
Corr = the correction factor for reference performance according to table 6.2-4.

For the adjacent channel (200 kHz) requirements of speech and control channels in VAMOS mode in downlink, the wanted input signal level of the AQPSK modulated signal shall be: -75 dBm + Iar, where:

Iar = the adjacent channel (200 kHz) interference ratio according to tables 2aa, 2ab and 2ag for VAMOS I MS, VAMOS II MS and VAMOS III MS respectively for VDTS-4 (see subclause Q.1).

For the adjacent channel (200 kHz) requirements of speech and control channels in VAMOS mode in uplink, the input signal level of the GMSK modulated VAMOS subchannel 2 signal shall be: -75 dBm + Iar + Corr, where:

Iar = the adjacent channel (200 kHz) interference ratio according to table 2ac for VUTS-2 (see subclause Q.2).
Corr = the correction factor for reference performance according to table 6.2-4.

For all adjacent channel (200 kHz) requirements specified in table 2af for TIGHTER MS, the wanted signal level shall be: -75 dBm + Iar + Corr, where:

Iar = the adjacent channel (200 kHz) interference ratio according to table 2af
Corr = the correction factor for reference performance according to table 6.2-4.

The signal levels above, where DLMC configuration is applicable, apply to each wanted signal for a MS in DLMC configuration. All adjacent channel interferers need to be located within the maximum DLMC carrier separation per contiguous frequency allocation.

### 6.3.5	Additional reference interference performance requirements and conditions

If not stated differently, all reference interference specifications in section 6.3 apply only for a random, continuous, GMSK modulated interfering signal.

In case of frequency hopping, the interference and the wanted signals shall have the same frequency hopping sequence. In any case the wanted and interfering signals shall be subject to the same propagation profiles (see annex C), independent on the two channels. In case of EC- EGPRS channels the interferer shall be subject to independent propagation in each burst.

For a GSM 400 MS, a GSM 900 MS, a ER-GSM 900, a GSM 850 MS, a GSM 700 MS, a DCS 1 800 MS and a PCS 1 900 MS the reference interference performance according to table 2 and 2j for co-channel interference (C/Ic) shall be maintained for RA500/250/130 propagation conditions if the time of arrival of the wanted signal is periodically alternated by steps of 8µs in either direction. The period shall be 32 seconds (16 seconds with the early and 16 seconds with the late time of arrival alternately).

For pico-BTS and Local Area multicarrier BTS, propagation conditions other than static and T15 are not specified and only the no FH case need be tested.  The performance requirement for GSM 900, ER-GSM 900, GSM 850, GSM 700, DCS 1 800, PCS 1900, MXM 850 and MXM 1900 pico-BTS and Local Area multicarrier BTS with TI5 propagation condition is the same as theTU50 no FH (900MHz) performance requirement.  The interference ratio at which this requirement shall be met is, for GMSK modulated wanted signals, 4dB above the interference ratio specified above in this sub-clause (in combination with Table 2a for packet service). For 8-PSK modulated wanted signals, the interference ratio for this requirement is 4 dB above the interference ratio specified above in this sub-clause (in combination with Table 2b and 2d for packet service). For 16-QAM, 32-QAM and QPSK modulated wanted signals, the interference ratio for this requirement is 4 dB above the interference ratio specified above in this sub-clause (in combination with Table 2r and 2t (16-QAM), and Table 2t (32-QAM and QPSK) for packet service). For Local Area multicarrier BTS and speech channels in VAMOS Mode and their associated control channels, the interference ratio for this requirement is 6 dB above the interference ratio specified above in this sub-clause 6.3 (in combination with Table 2ac).  For adjacent channel interference propagation conditions other than TU50 need not be tested. There is an exception in the case of the pico-BTS and Local Area multicarrier BTS in that the specified propagation condition is TI5 instead of TU50; the respective test for pico-BTS and Local Area multicarrier BTS is described in the paragraph following table 6.3-3 below.

If, in order to ease measurement, a TU50 (no FH) faded wanted signal, and a static adjacent channel interferer are used, the reference interference performance shall be:

Table 6.3-3 Reference interference ratio measurements for GMSK-modulated speech channels - simplified

|  | GSM 850 & GSM 900 & ER-GSM 900 & GSM 700 | DCS 1 800 & PCS 1 900 |
| --- | --- | --- |
| TCH/FS (FER): | 10,2 % | 5,1 % |
| Class Ib (RBER): | 0,72/ % | 0,45/ % |
| Class II (RBER): | 8,8 % | 8,9 % |
| FACCH (FER): | 17,1 % | 6,1 % |

For pico-BTS and Local Area multicarrier BTS, adjacent channel and cochannel interference propagation conditions other than TI5 need not be tested.    If, in order to ease adjacent channel measurements, a TI5 (no FH) faded wanted signal, and a static adjacent channel interferer are used, the interference performance shall be the same as that specified above for a TU50 no FH channel (900MHz). The interference ratio at which this performance shall be met is 4dB above the reference interference ratio specified above in this sub-clause.

In addition, MS indicating support for Downlink Advanced Receiver Performance – phase I (see 3GPP TS 24.008) shall fulfil the requirements in table 2o for wanted signals on GMSK modulated channels under TU50 no FH propagation conditions and GMSK modulated interferers for the test scenarios defined in annex L, unless when the MS is assigned a Downlink Dual Carrier configuration (see 3GPP TS 44.060) comprising  a total of more than 8 timeslots, in which case, the MS shall only fulfil the requirements at least on all the timeslots that are assigned on the carrier 1 (see 3GPP TS 44.060). The reference performance shall be:

Table 6.3-4 Reference performance for DARP – phase I

| For speech channels (TCH/FS, TCH/AFSx, TCH/AHSx) | FER: | 1 % |
| --- | --- | --- |
| For signalling channels (FACCH/F, SDCCH) | FER: | 5 % |
| For packet switched channels (PDTCH) | BLER: | 10 % |

The values in table 2o are given as the  C/I1 ratio, where C is the power level of the wanted signal and I1 is the power level of the dominant co-channel interferer  (Co-channel 1, see annex L).
In addition for speech channels the residual class Ib BER and residual class II BER performance shall not exceed the specified values in table 2o at the corresponding C/I1.

A MS indicating support for Downlink Advanced Receiver Performance – phase II Capability (see 3GPP TS 24.008) shall fulfil the requirements in table 2q for the test scenarios defined in annex N. The reference performance shall be:

Table 6.3-5 Reference performance for DARP – phase II

| For speech channels (TCH/FS, TCH/AFSx, TCH/AHSx) | FER: | 1 % |
| --- | --- | --- |
| For packet switched channels (PDTCH) | BLER: | 10 % |

The values in table 2q are given as the  C/I1 ratio, where C is the power level of the wanted signal and I1 is the power level of the dominant co-channel interferer  (Co-channel 1, see annex N).
In addition for speech channels the residual class Ib BER and residual class II BER performance shall not exceed the specified values in table 2q at the corresponding C/I1.

The reference interference performance specified above for Downlink Advanced Receiver Performance – phase II need not be met by an MS when it is in a Downlink Dual Carrier configuration (see 3GPP TS 43.064).

A MS indicating support for TIGHTER Capability (see 3GPP TS 24.008) shall also indicate support for DARP – phase I (see 3GPP TS 24.008), and shall fulfil the requirements in table 2ad for cochannel interference (C/Ic), table 2af for adjacent channel (200 kHz) interference (C/Ia1), and the additional requirements in table 2ae for wanted signals on GMSK modulated channels for the test scenarios defined in annex L. The reference performance shall be:

Table 6.3-6 Reference performance for TIGHTER

| For speech channels (TCH/FS, TCH/HS, TCH/EFS, TCH/AFSx, TCH/AHSx, TCH/WFSx ) | FER: | 1 % |
| --- | --- | --- |
| For signalling channels (FACCH/F, FACCH/H, SDCCH) | FER: | 5 % |
| For packet switched channels (PDTCH) | BLER: | 10 % unless otherwise stated |

For equipment supporting 8-PSK, and for MS indicating support for Downlink Advanced Receiver Performance – phase I (see 3GPP TS 24.008), the applicable TIGHTER requirements in table 2ad for TCH/WFSx and PDTCH, and table 2af for TCH/WFSx, PDTCH/CSx, PDTCH/MCSx and PDTCH/DASx apply for both GMSK and 8-PSK modulated interfering signals.

The values in table 2ae are given as the  C/I1 ratio, where C is the power level of the wanted signal and I1 is the power level of the dominant co-channel interferer  (Co-channel 1, see annex L).

In addition to table 6.3-6, for speech channels the residual class Ib BER and residual class II BER performance shall not exceed the specified values in table 2ad, 2ae and 2af  at the corresponding interference ratio C/Ic, C/I1, and C/Ia1, respectively.

## 6.4	Erroneous frame indication performance

a)	On a speech TCH (TCH/FS, TCH/EFS, TCH/HS, TCH/AFS, TCH/AHS, TCH/WFS, O-TCH/AHS, O-TCH/WFS or O-TCH/WHS) or a SDCCH with a random RF input, of the frames believed to be FACCH, O-FACCH, SACCH, or SDCCH frames, the overall reception performance shall be such that no more than 0,002 % of the frames are assessed to be error free.

b)	On a speech TCH (TCH/FS, TCH/EFS, TCH/HS, TCH/AFS, TCH/AHS, TCH/WFS, O-TCH/AHS, O-TCH/WFS or O-TCH/WHS) with a random RF input, the overall reception performance shall be such that, on average, less than one undetected bad speech frame (false bad frame indication BFI) shall be measured in one minute.

c)	On a speech TCH (TCH/FS, TCH/EFS, TCH/HS, TCH/AFS, TCH/AHS, TCH/WFS, O-TCH/AHS, O-TCH/WFS or O-TCH/WHS), when DTX is activated with frequency hopping through C0 where bursts comprising SID frames, SACCH frames and Dummy bursts are received at a level 20 dB above the reference sensitivity level and with no transmission at the other bursts of the TCH, the overall reception performance shall be such that, on average less than one undetected bad speech frame (false bad frame indication BFI) shall be measured in one minute for MS. This performance shall also be met in networks with one of the configurations described in 3GPP TS 45.002 - annex A, excepted combinations #1 and #6 of table A.2.5.1 for which there is no performance requirement.

d)	On a speech TCH (TCH/FS, TCH/EFS, TCH/HS, TCH/AFS, TCH/AHS, TCH/WFS, O-TCH/AHS, O-TCH/WFS or O-TCH/WHS), when DTX is activated with SID frames and SACCH frames received 20 dB above the reference sensitivity level and with no transmission at the other bursts of the TCH, the overall reception shall be such that, on average, less than one undetected bad speech frame (false bad frame indication BFI) shall be measured in one minute for BTS.

e)	For a BTS on a RACH, EC-RACH or PRACH with a random RF input, the overall reception performance shall be such that less than 0,02 % of frames are assessed to be error free.

f)	For a BTS on a PRACH with a random RF input, the overall reception performance shall be such that less than 0,02 % of frames are assessed to be error free.

- For an MS assigned a USF on a PDCH with a random RF input or a valid PDCH signal with a random USF not equal to the assigned USF, the overall reception shall be such that the MS shall detect the assigned USF in less than 1% of the radio blocks for GMSK modulated signals, and 1 % for 8-PSK modulated signals, and 1 % for QPSK, 16-QAM and 32-QAM modulated signals. This requirement shall be met for all input levels up to –40 dBm for GMSK, QPSK, 8PSK, 16QAM and 32QAM modulated signal. In case of DLMC configuration the performance shall, at the multiple input signals arbitrarily distributed over the maximum supported DLMC carrier frequency spacing, be met for all input levels up to -40 dBm per carrier but with the total input power limited to -37 dBm, irrespective of the number of carriers.

h)	The FER on an SACCH associated to an adaptive speech traffic channel (TCH/AFS, TCH/AHS, TCH/WFS, O-TCH/AHS, O-TCH/WFS or O-TCH/WHS) received at 3 dB below the reference co-channel interference level shall be less than [40%] tested under TU 3 / TU 1.5 propagation conditions.

i)	On a speech TCH (TCH/AFS, TCH/AHS, TCH/WFS, O-TCH/AHS, O-TCH/WFS or O-TCH/WHS), a RATSCCH message, respectively a RATSCCH marker, shall be detected if more than 72% of the bits of the RATSCCH identification field (defined in 3GPP TS 45.003) are matched by the corresponding gross bits of the received frame.

## 6.5	Random access and paging performance at high input levels

a)	MS requirements

Under static propagation conditions with a received input level from 20 dB above the reference sensitivity level

-	up to -15 dBm for GSM 400, GSM 700, GSM 850 and GSM900 and

-	up to -23 dBm for DCS1800 and PCS 1 900,

the MS FER shall be less than 0,1% for PCH  and MS BLER when in EC-EGPRS operation,less than [0,1%] for EC-PCH.

b)	BTS requirements

Under static propagation conditions with a received input level from 20 dB above the reference sensitivity level

-	up to -15 dBm for GSM 400, GSM 700, GSM 850 and GSM900 normal BTS and multicarrier BTS excluding multicarrier BTS equipped with multicarrier receiver,

-	up to -16 dBm for GSM400, GSM 850 and GSM900 multicarrier BTS equipped with multicarrier receiver and

-	up to -23 dBm for DCS1800 and PCS 1 900, normal BTS and multicarrier BTS

and a single MS sending an access burst or access bursts in case of EC-EGPRS operation using blind physical layer transmissions, the BTS FER shall be less than 0,5% for RACH and BTS BLER less than [0,5%] for EC-RACH.

For multicarrier BTS equipped with multicarrier receiver, input levels shall be adjusted by 0 dB, +4 dB or +9 dB for the Wide Area, Medium Range and Local Area classes, respectively.

***** Next modification *****

## 6.7	Incremental Redundancy Performance for EGPRS and EGPRS2 MS

Support for Incremental Redundancy reception is mandatory for all EGPRS capable MSs. In Incremental Redundancy RLC mode soft information from multiple, differently punctured, versions of an RLC data block may be used when decoding the RLC data block. This significantly increases the link performance.

An EGPRS capable MS shall under the conditions stated in the below table 6.7-1 achieve a long-term throughput of 20 kbps per time slot (see note), measured between LLC and RLC/MAC layer.

An EGPRS2 capable MS shall under the conditions stated in the below table 6.7-1 achieve a long-term throughput of 33 kbps per time slot, measured between LLC and RLC/MAC layer -

An EC-EGPRS capable MS, supporting only MCS 1-4, shall under the conditions stated in the below table 6.7-1 achieve a long-term throughput per time slot according to table 6.7-1, measured between LLC and RLC/MAC layer. In case the MS supports MCS 1-9 the requirements for EGPRS apply.

Table 6.7-1 Incremental Redundancy performance

|  | EGPRS | EGPRS2 | EC-EGPRS, 8PSK supported | EC-EGPRS, GMSK supported | EC-EGPRS, MCS-1/16 |
| --- | --- | --- | --- | --- | --- |
| Required throughput [kbps / timeslot] | 20,0 | 33,0 | 20,0 | [tbd] | [tbd] |
| Propagation conditions | Static | Static | Static | Static | Static |
| Input level [dBm] | -97,0 | -94,0 | -97,0 | [tbd] | [tbd] |
| Modulation and Coding Scheme | MCS-9 | DAS-12 | MCS-9 | MCS-4 | MCS-1/16 |
| Acknowledgements polling period [RLC data blocks] | 32 | 32 | 8 | 8 | 8 |
| Roundtrip time [ms] | 120 | 120 | 120 | 120 | [200] |
| Number of timeslots | Maximum capability of the MS | Maximum capability of the MS | Maximum capability of the MS | Maximum capability of the MS | 4 |
| Transmit window size | Maximum for the MS timeslot capability | Maximum for the MS timeslot capability | 16 | 16 | 16 |

NOTE:	The requirement for EGPRS corresponds to an equivalent block error rate of approximately 0.66 using the prescribed MCS-9.

***** Next modification *****

# Table 1z: Input signal level (for normal BTS) at reference performance for GMSK modulated signals for different Coverage Classes (CC)

| GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) |
| EC-PACCH/U | CC1 | dBm | [tbd] | [tbd] | [tbd] |  |
| EC-PACCH/U/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-2 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-3 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-4 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH2) | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) |
| EC-PACCH/U | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-2 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-3 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-4 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH2) | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test.. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. |  |  |  |  |  |  |

# Table 1aa: Input signal level (for EC-EGPRS MS) at reference performance for GMSK modulated signals for different Coverage Classes (CC)

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |
| Channel |  |  | Static |  | TU1.2 (no FH) |  | TU1.21) (ideal FH) |  | TU30 (no FH) |  |
| EC-SCH | - | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-BCCH | - | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/4 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/8 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/16 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D2) | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D/8 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D/16 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D/32 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/4 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/8 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/16 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-2 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-3 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-4 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |
| channel |  |  | Static |  | TU1.2 (no FH) |  | TU1.21) (ideal FH) |  | TU30 (no FH) |  |
| EC-SCH | - | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-BCCH | - | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/4 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/8 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/D/16 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D2) | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D/8 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D/16 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-CCCH/D/32 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/4 | CC2 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/8 | CC3 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/16 | CC4 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-2 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-3 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-4 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test.. NOTE 2: The performance requirements for EC-CCCH apply for EC-PCH and EC-AGCH. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. |  |  |  |  |  |  |  |  |  |  |  |  |

# Table 1ab: Input signal level (for normal BTS) at reference performance for 8-PSK modulated signals for Coverage Classe 1 (CC1)

| GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) |
| EC-PDTCH/MCS-5 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) |
| EC-PDTCH/MCS-5 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dBm | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. |  |  |  |  |  |  |

# Table 1ac: Input signal level (for EC-EGPRS MS) at reference performance for 8-PSK modulated signals for Coverage Classe 1 (CC1)

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |
| Channel |  |  | Static |  | TU1.2 (no FH) |  | TU1.21) (ideal FH) |  | TU30 (no FH) |  |  |
| EC-PDTCH/MCS-5 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| EC-PDTCH/MCS-6 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| EC-PDTCH/MCS-7 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| EC-PDTCH/MCS-8 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| EC-PDTCH/MCS-9 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |
| channel |  |  | Static |  | TU1.2 (no FH) |  | TU1.21) (ideal FH) |  | TU30 (no FH) |  |  |
| EC-PDTCH/MCS-5 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| EC-PDTCH/MCS-6 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| EC-PDTCH/MCS-7 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| EC-PDTCH/MCS-8 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| EC-PDTCH/MCS-9 | CC1 | dBm |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |  |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. |  |  |  |  |  |  |  |  |  |  |  |

# Table 1ad: Input signal level (for BTS) at reference performance for GMSK modulated signals in Coverage Classes 2, 3 and 4 (CC2-CC4) with multi-users in time slot

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of channel |  |  |  |  | Propagation conditions: TU1,2 no FH |  |  |  |  |  |  |
|  |  |  |  |  | 2 users per time slot |  |  |  | 4 users per time slot |  |  |
| SCPIR_UL |  |  |  |  | 0 dB |  | [tbd] dB |  | 0 dB |  | 0,[tbd1], [tbd2], [tbd3] dB |
| EC-PACCH/U/4 | CC2 |  | dBm | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/U/8 | CC3 |  | dBm | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PACCH/U/16 | CC4 |  | dBm | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/4 | CC2 |  | dBm | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/8 | CC3 |  | dBm | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| EC-PDTCH/MCS-1/16 | CC4 |  | dBm | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |  |
| DCS 1800 & PCS 1900 |  |  |  |  |  |  |  |  |  |  |  |
| Type of channel |  |  |  |  | Propagation conditions: TU1,2 no FH |  |  |  |  |  |  |
|  |  |  |  |  | 2 users per time slot |  |  |  | 4 users per time slot |  |  |
| SCPIR_UL |  |  |  |  | 0 dB |  | 0, [tbd1], [tbd2], [tbd3] dB |  | 0 dB |  | 0,[tbd1], [tbd2], [tbd3] dB |
| EC-PACCH/U/4 | CC2 | dBm |  |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |
| EC-PACCH/U/8 | CC3 | dBm |  |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |
| EC-PACCH/U/16 | CC4 | dBm |  |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dBm |  |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dBm |  |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dBm |  |  | [tbd] |  | [tbd] |  | [tbd] |  | [tbd] |

***** Next modification *****

Table 2ah: Cochannel interference ratio at reference performance (for normal BTS) for GMSK modulated signals

| GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PACCH/U | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-2 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-3 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-4 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH2) | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PACCH/U | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-2 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-3 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-4 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH2 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. |  |  |  |  |  |  |  |

Table 2ai: Cochannel interference ratio at reference performance (for EC-EGPRS MS) for GMSK modulated signals

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  |  |  |  |  | Propagation conditions |  |  |  |  |
| Channel |  |  |  |  |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |  |
| EC-SCH | - |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-BCCH | - |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PACCH/D | CC1 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PACCH/D/4 | CC2 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PACCH/D/8 | CC3 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PACCH/D/16 | CC4 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-CCCH/D2) | CC1 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-CCCH/D/8 | CC2 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-CCCH/D/16 | CC3 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-CCCH/D/32 | CC4 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PDTCH/MCS-1 | CC1 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PDTCH/MCS-1/4 | CC2 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PDTCH/MCS-1/8 | CC3 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PDTCH/MCS-1/16 | CC4 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PDTCH/MCS-2 | CC1 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PDTCH/MCS-3 | CC1 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PDTCH/MCS-4 | CC1 |  |  | dB |  |  | [tbd] | [tbd] | [tbd] | [tbd] |  |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |  |  |  |  |
| Type of |  |  |  |  |  |  | Propagation conditions |  |  |  |
| Channel |  |  |  |  |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-SCH |  | - |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-BCCH |  | - |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D |  | CC1 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/4 |  | CC2 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/8 |  | CC3 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/16 |  | CC4 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D2) |  |  | CC1 |  |  | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D/8 |  |  | CC2 |  |  | dB | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-CCCH/D/16 |  |  | CC3 |  |  | dB | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-CCCH/D/32 |  |  | CC4 |  |  | dB | [tbd] | [tbd] | [tbd] | [tbd] |  |
| EC-PDTCH/MCS-1/8 |  | CC2 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 |  | CC3 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/32 |  | CC4 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1 |  | CC1 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 |  | CC2 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 |  | CC3 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 |  | CC4 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-2 |  | CC1 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-3 |  | CC1 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-4 |  | CC1 |  |  | dB |  | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: The performance requirements for EC-CCCH apply for EC-PCH and EC-AGCH. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. |  |  |  |  |  |  |  |  |  |  |  |

Table 2aj: Cochannel interference ratio (for normal BTS) at reference performance for 8-PSK modulated input signals

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PDTCH/MCS-5 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PDTCH/MCS-5 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU30 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. |  |  |  |  |  |  |  |

Table 2ak: Cochannel interference ratio (for EC-EGPRS MS) at reference performance for 8-PSK modulated input signals

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PDTCH/MCS-5 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PDTCH/MCS-5 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU30 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. |  |  |  |  |  |  |  |

Table 2am: Adjacent channel interference ratio at reference performance (for normal BTS) for GMSK modulated signals

| GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PACCH/U | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-2 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-3 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-4 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH2) | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PACCH/U | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-2 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-3 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-4 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH2) | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. |  |  |  |  |  |  |  |

Table 2an: Adjacent channel interference ratio at reference performance (for EC-EGPRS MS) for GMSK modulated signals

| GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-SCH | - | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-BCCH | - | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D2) | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D/8 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D/32 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-2 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-3 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-4 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-SCH | - | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-BCCH | - | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PACCH/D/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D2) | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D/8 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D/16 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-CCCH/D/32 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-2 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-3 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-4 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: The performance requirements for EC-CCCH apply also for EC-PCH and EC-AGCH. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. |  |  |  |  |  |  |

Table 2ao: Adjacent channel interference ratio (for normal BTS) at reference performance for 8-PSK modulated input signals

| GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PDTCH/MCS-5 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PDTCH/MCS-5 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU30 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. |  |  |  |  |  |  |

Table 2ap: Adjacent channel interference ratio (for EC-EGPRS MS) at reference performance for 8-PSK modulated input signals

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PDTCH/MCS-5 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU30 (no FH) | TU301) (ideal FH) |
| EC-PDTCH/MCS-5 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-6 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-7 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-8 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-9 | CC1 | dB | [tbd] | [tbd] | [tbd] | [tbd] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU30 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. |  |  |  |  |  |  |  |

***** End of modifications *****

Annex R (normative):
Reference Test Scenarios for EC-EGPRS Multi-user channels on One Slot

# R.1	Time and frequency offset in uplink

[TBD]