pCR 45.871 Downlink MIMO – Performance Summary

INTRODUCTION

As part of the study item on downlink MIMO, opened in GERAN#57 [1], performance evaluation in different scenarios as agreed in [2] was carried out through simulations and the results were presented in section 8.4 and 8.5 in the latest version of the Draft TR on MIMO for Downlink [3]. This paper presents the overall summary of performance evaluation based on the discussion in the Downlink MIMO Telco#4 held prior to GERAN#64 [4]. The paper is written as a direct input to section 8.6 of the Draft TR on MIMO for Downlink [3] using the present section numbering.

CONCLUSION

This document provides the DL MIMO performance evaluation summary. We propose to include these items into the Draft TR on MIMO for Downlink [3].

REFERENCES

- GP-130288 “New Study Item on Downlink MIMO”, source Nokia Siemens Networks, NOKIA Corporation, Com-Research GmbH, TELECOM ITALIA S.p.A, GERAN#57

- GP-140335, “Downlink MIMO Simulation Assumptions for Final Performance Evaluation”, source NSN, GERAN#62.

- GP-140991, “Draft TR 45.871 MIMO for Downlink v0.4.0”, source SI Rapporteur, GERAN#64

- GP-140918 – “Meeting Minutes of Downlink MIMO Telco#4”, source SI Rapporteur, GERAN#64

For Information

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	GP-130288 “New Study Item on Downlink MIMO”, source Nokia Siemens Networks, NOKIA Corporation, Com-Research GmbH, TELECOM ITALIA S.p.A.

[3]	3GPP TS 45.005 V7.25.0 Radio transmission and reception (Release 7).

[4]	3GPP TR 45.860 V11.5.0 Signal Precoding Enhancements for EGPRS2 DL.

[5]	3GPP TS 45.002 V11.0.0 Multiplexing and multiple access on the radio path

[6]	3GPP TS 44.060 V11.6.0 Radio Link Control / Medium Access Control (RLC/MAC) protocol

[7]	3GPP TS 36.101 V11.0.0 Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 11).

[8]	3GPP TR 25.814 V7.1.0 Physical layer aspects for evolved Universal Terrestrial Radio Access (UTRA) (Release 7).

[9]	3GPP TR 25.996 V10.0.0 Spatial channel model for Multiple Input Multiple Output (MIMO) simulations (Release 10)

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].)

BMD	Blind Modulation Detection

DAS	Downlink level A modulation and coding scheme

MCS	Modulation and coding scheme

MSRD	Mobile Station Receive Diversity

NSR	Normal Symbol Rate

SCPIR	Sub Channel Power Imbalance Ratio

SINR	Signal to Interference and Noise Ratio

TSC	Training Sequence Code

USF	Uplink State Flag

First modified section

# 8	Performance Evaluation

## 8.1	Transmitter assumption

Transmitter impairment values for a typical BTS as given in Table 8.1.1 are used for performance evaluations [8-1].

Table 8.1.1: Transmitter Impairments

| Impairment | Legacy single carrier BTS (per TRX) |
| --- | --- |
| Phase noise [degrees (RMS)] | 1.2 |
| I/Q gain imbalance [dB] | 0.1 |
| I/Q phase imbalance [degrees] | 0.1 |
| DC offset [dBc] | -45 |
| Frequency error [Hz] | 15 (900 MHz) 30 (1800 MHz) |
| Tx path time misalignment [normal symbol periods] | 0.25 |

## 8.2	Receiver assumptions

The MIMO receiver being used in the analysis in section 7.2 is also used for the performance evaluation. The block diagram of the receiver is shown in Figure 8.2.1. This receiver employs channel estimation, followed by separate interference cancellation (IRC) and bit-detection for each stream. Before bit detection, a frequency domain prefiltering is used to convert the system to minimum phase. The frequency domain prefilter (SC-FDE), shown in Figure 8.2.2, multiplies FFT output of signal, {Rn}, by the complex-valued forward equalizer coefficients {Wn} (which compensate for the frequencyselective channel’s variations of amplitude and phase with frequency). An IFFT operation brings the weight-equalized complex-valued samples in time-domain sequences. Finally, the time domain sequence goes through the reduced state bit-detection process for soft bit detection in each stream.

This is neither a successive interference cancellation (SIC) type of receiver, nor a joint detection (JD) receiver and it is seen in a number of simulations that the performance is similar to SIC or JD receivers but with much less complexity.

Blind modulation detection (BMD) is enabled in the receiver. BMD is performed after the channel estimation assuming all possible modulations schemes including GMSK. In addition, blind mode detection is enabled. The mode detection in the receiver is based on SNR estimation after the interference cancellation. If SNR of the second stream is found to be below the chosen threshold of 2 dB, the receiver assumes that single stream is transmitted [8-2]. The blocks with hatched lines in Figure 8.2.1 are used if the mode detection process detects the presence of second stream.

The blocks with solid white filling constitute the reference MSRD receiver for legacy non-MIMO operation. Therefore, the complexity of the MIMO receiver is approximately twice the complexity of the reference non-MIMO receiver.

Receiver impairment values are given in Table 8.2.1.

Figure 8.2.1: MIMO receiver structure used for performance evaluation

Figure 8.2.2: Single Carrier Frequency Domain Equaliser (SC-FDE)

Table 8.2.1: Receiver Impairments

| Impairment | Rx diversity capable device (per Rx path) (Taken from 3GPP TR 45.860 [4]) |
| --- | --- |
| Phase noise [degrees (RMS)] | 1.2 |
| I/Q gain imbalance [dB] | 0.2 |
| I/Q phase imbalance [degrees] | 2.0 |
| DC offset [dBc] | -40 |
| Frequency error [Hz] | 25 (900 MHz) 50 (1800 MHz) |
| Rx path time misalignment [symbols] | negligible |

## 8.3	Simulation scenario

The link level performance evaluation is first grouped according to the correlation models used, i.e. SCM and variable correlation channel model. In each group, the results are divided into sensitivity and interference scenarios. In each scenario, the results are further grouped according to propagation conditions. The simulation parameters relevant to simulation scenarios are listed in Table 8.3.1 [8-1].

Table 8.3.1: Simulation scenarios and parameters

| Parameter | Value |  |
| --- | --- | --- |
| Transmission Mode | Single stream MSRD (reference) Single stream (in MIMO mode, blind mode detection enabled) - with MSRD - with Tx diversity and MSRD Dual stream (2x2 MIMO) |  |
| Training Sequence | Single stream mode: TSC 5 from TSC Set 1 Dual stream mode: TSC 5 from TSC Set 1 and TSC Set 2. In case of mixed modulation, other combinations can be used if benefit is seen. |  |
| Channel model | A modified SCM as described in section 6.2 for dual stream mode Variable correlation model as described in section 6.3 for both dual stream mode and single stream mode |  |
| Modulations | Single stream mode: all supported modulations in EGPRS and EGPRS2-A Dual stream mode: all supported modulations in EGPRS and EGPRS2-A except GMSK. Both streams can have the same or different modulations. |  |
| Frequency bands | - 900 MHz (antenna correlation model) -1800 MHz (SCM) |  |
| Propagation conditions | 900 MHz - Sensitivity - SCM-A3iFH - TU3iFH - Interference - SCM-A3iFH - TU3iFH 1800 MHz - Sensitivity - SCM-A3iFH - TU3iFH - RA130nFH - Interference - SCM-A3iFH - TU3iFH - RA130nFH |  |
| Interference | Multiple interference sources as defined for DTS-2 interference profile with 8PSK modulation. |  |
| Channel Correlation | Wanted signal: For SCM model, it is SCM specific. For variable correlation model: α, β = {0.0, 0.0}, {0.0, 0.7} and {0.3, 0.7} Interference: 0.7 (if SCM model is used for wanted signal) β (if variable correlation model is used for wanted signal) |  |
| SCPIR [dB] | 0 dB and 10 dB [SCPIR = 10log10(Power of stream 1/power of stream 2)] |  |
| Back-off [dB] | Sensitivity: Theoretical PAR taken from TR 45.860, section 8: 3.2 dB for 8PSK, 4.7 dB for 16-QAM and 5.1 dB for 32-QAM DTS-2: 0 dB for all modulations Additional back-off based on SCPIR (as given in Table 7.2.1.1.2) in the transmit power in dual stream mode and single stream Tx diversity mode, for both sensitivity and DTS-2, to compare with single antenna transmission power. |  |
| MCSs | EGPRS2-A | GMSK (MCS-1…MCS-4) 8-PSK (DAS-5…DAS-7) 16-QAM (DAS-8…DAS-9) 32-QAM (DAS-10…DAS-12) |
|  | EGPRS | GMSK (MCS-1…MCS-4) 8-PSK (MCS-5…MCS-9) |
| Blind modulation detection | Enabled |  |
| Blind MIMO mode detection | Enabled |  |
| MCS link adaptation | Ideal |  |

## 8.4	Simulation results using SCM profiles

In these simulations the channel correlation values are taken from the SCM-A profile as described in section 6.2 and in [8]. The tap-delay settings are taken from the following three propagation conditions:

  - SCM-A 3 km/hr with ideal frequency hopping ([8]),

  - TU 3 km/hr with ideal frequency hopping ([3]) and

  - RA 130 km/hr with no frequency hopping ([3]).

As mentioned in Table 8.3.1, only the 1800 MHz frequency band is used. In case of dual stream transmission, modulations in both streams are the same if SCPIR is 0 dB and mixed otherwise.

The plots below show the envelope throughput with ideal link adaptation in different scenarios mentioned above (i.e. sensitivity and DTS-2 interference profile).

a) SCM-A 3 km/hr ideal FH 		b) TU3 km/hr ideal FH

c) RA 130 km/hr no FH

Figure 8.4.1: Sensitivity performance with SCM-A correlation, 1800 MHz, SCPIR=0 dB

From Figures 8.4.1 (a) and (b), it can be seen that the dual stream MIMO mode offers significant increase in throughput compared with single stream mode at moderate and high Es/No. While this is true for the low speed scenario, at high speed (RA 130 km/hr) (see Figure 8.4.1c), we do not see benefit of the dual stream MIMO mode.

a) SCM-A 3 km/hr ideal FH 		b) TU3 km/hr ideal FH

c) RA 130 km/hr no FH

Figure 8.4.2: Sensitivity performance with SCM-A correlation, 1800 MHz, SCPIR=10 dB

From Figures 8.4.2 (a) and (b), it can be seen that, if the SCPIR is high, the dual stream MIMO mode offers increased throughput compared with the single stream mode only at high Es/No. If high SCPIR is combined with high speed (e.g. RA 130 km/hr in Figure 8.4.2c), single stream MSRD mode actually outperforms dual stream mode.

a) SCM-A 3 km/hr ideal FH 		b) TU3 km/hr ideal FH

c) RA 130 km/hr no FH

Figure 8.4.3: DTS-2 interference performance SCM-A correlation, 1800 MHz, SCPIR=0 dB

a) SCM-A 3 km/hr ideal FH 		b) TU3 km/hr ideal FH

c) RA 130 km/hr no FH

Figure 8.4.4: DTS-2 interference performance in RA 130 km/hr no FH with SCM-A correlation, 1800 MHz, SCPIR=10 dB

MIMO performance in case of DTS-2 interference scenario is similar to the performance in sensitivity scenario as can be seen in Figures 8.4.3 and 8.4.4. Dual stream MIMO mode offers significant increase in throughput compared with the single stream mode at moderate and high C/I if SCPIR is low and only at high C/I if SCPIR is high. If high SCPIR is combined with high speed (e.g. RA 130 km/hr), single stream MSRD mode actually outperforms dual stream mode.

## 8.5	Simulation results with variable channel/antenna correlation

In these simulations, the variable correlation channel model as described in section 6.3 and in [7] is used. The tap-delaysettings are taken from the following two propagation conditions:

- SCM-A 3 km/hr with ideal frequency hopping ([8]) and

- TU 3 km/hr with ideal frequency hopping ([3])

As mentioned in Table 8.3.1, only the 900 MHz frequency band is used. Three different correlation settings as mentioned in Table 8.3.1 are simulated. In order to limit the simulation effort only SCPIR of 0 dB is considered. In case of dual stream transmission, modulations in both streams are considered the same.

The plots below show the envelope throughput with ideal link adaptation in different scenarios mentioned above (i.e. sensitivity and DTS-2 interference profile).

a) SCM-A 3 km/hr ideal FH 			b) TU3 km/hr ideal FH

Figure 8.5.1: Sensitivity performance in 900 MHz, SCPIR=0 dB, correlation α=0, β=0

a) SCM-A 3 km/hr ideal FH 			b) TU3 km/hr ideal FH

Figure 8.5.2: Sensitivity performance in 900 MHz, SCPIR=0 dB, correlation α=0, β=0.7

a) SCM-A 3 km/hr ideal FH 			b) TU3 km/hr ideal FH

Figure 8.5.3: Sensitivity performance 900 MHz, SCPIR=0 dB, correlation α=0.3, β=0.7

a) SCM-A 3 km/hr ideal FH 			b) TU3 km/hr ideal FH

Figure 8.5.4: DTS-2 interference performance 900 MHz, SCPIR=0 dB, correlation α=0, β=0

a) SCM-A 3 km/hr ideal FH 			b) TU3 km/hr ideal FH

Figure 8.5.5: DTS-2 interference performance in 900 MHz, SCPIR=0 dB, correlation α=0, β=0.7

a) SCM-A 3 km/hr ideal FH 			b) TU3 km/hr ideal FH

Figure 8.5.6: DTS-2 interference performance in 900 MHz, SCPIR=0 dB, correlation α=0.3, β=0.7

From the figures in this section, we can see that the MIMO performance in case of variable correlation channel model is similar to the performance in SCM-A model. Dual stream MIMO mode offers significant increase in throughput compared with the single stream mode at moderate and high Es/No and C/I. In case of higher correlation settings i.e. α=0.3, β=0.7, there is a slight degradation in performance.

## 8.6	Performance Summary

Based on the detailed performance evaluation results provided in section 8.4 and 8.5, this section provides a summary of the throughput comparison between dual stream MIMO mode and single stream MSRD mode. In order to make the comparison over a range of realistic SINR values, a SINR probability distribution, shown in Table 8.6.1, is taken from Table A.1 in Annex A of TR 45.860 [4] for a 3/9 frequency reuse network.

Table 8.6.1: SINR distribution used in Sensitivity and DTS-2 scenario.

| SINR | Probability |
| --- | --- |
| 10 | 0.04 |
| 15 | 0.18 |
| 20 | 0.29 |
| 25 | 0.23 |
| 30 | 0.13 |
| 35 | 0.07 |
| 40 | 0.06 |

In a particular propagation profile, the throughput gains at each SINR value given in Table 8.6.1 is multiplied by the probability of the respective SINR and the products for all the SINR values are added to compute the overall throughput gain in that profile.

In case of DTS-2 simulation, the C/I1 levels are converted into appropriate SINR value using the relative levels of multiple interference sources and the thermal noise compared to the dominant interference level as specified for DTS-2 setup in [3].

The overall summary of 2x2 MIMO throughput gain as percentage improvement over single stream MSRD mode is given in Table 8.6.2 to Table 8.6.5.

It is already explained in section 8.4 and 8.5 that at high SCPIR and high speed, single stream MSRD outperforms dual stream MIMO mode. Therefore, we see negative gain in case of RA130 profile in the summary tables.

The overall throughput of dual stream MIMO, however, can be about 33% higher than that of single stream MSRD mode at low MS speed. The gain is reduced in case very high SCPIR or high spatial correlation.

It is also evident that the overall gain of the dual stream MIMO mode over the single stream MSRD mode is seen more in EGPRS than in EGPRS2-A. It is because, in case of EGPRS2-A operation, the gain is observed only at a very high SINR range. The probability of experiencing such high SINR is much less in the real network and hence the overall gain is less compared to the gain seen in case of EGPRS operation.

Table 8.6.2: 2x2 MIMO Throughput Gain over Single Stream MSRD: EGPRS, SCM Profiles

|  | SCPIR 0 dB |  |  | SCPIR 10 dB |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH | RA130 km/hr no FH | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH | RA130 km/hr no FH |
| Sensitivity | 32% | 33% | 2% | 12% | 11% | -20% |
| DTS-2 | 27% | 37% | 7% | 15% | 18% | -18% |

Table 8.6.3: 2x2 MIMO Throughput Gain over Single Stream MSRD: EGPRS2-A, SCM Profiles

|  | SCPIR 0 dB |  |  | SCPIR 10 dB |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH | RA130 km/hr no FH | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH | RA130 km/hr no FH |
| Sensitivity | 17% | 12% | -16% | -2% | -7% | -20% |
| DTS-2 | 2% | 1% | -24% | -7% | -12% | -22% |

Table 8.6.4: 2x2 MIMO Throughput Gain over Single Stream MSRD: EGPRS, Variable Correlation Profiles (SCPIR 0 dB)

|  | α=0.0, β=0.0 |  | α=0.0, β=0.7 |  | α=0.3, β=0.7 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH |
| Sensitivity | 33% | 27% | 32% | 27% | 31% | 26% |
| DTS-2 | 35% | 28% | 31% | 26% | 30% | 25% |

Table 8.6.5: 2x2 MIMO Throughput Gain over Single Stream MSRD: EGPRS2-A, Variable Correlation Profiles (SCPIR 0 dB)

|  | α=0.0, β=0.0 |  | α=0.0, β=0.7 |  | α=0.3, β=0.7 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH | SCM-A 3 km/hr ideal FH | TU 3 km/hr ideal FH |
| Sensitivity | 22% | 12% | 20% | 13% | 19% | 11% |
| DTS-2 | 11% | -3% | 7% | -5% | 5% | -6% |

The legacy single stream transmission mode with single receive antenna is out of scope in the comparison analysis, but it is expected to see further gains from dual stream MIMO if we use single stream mode with single antenna receiver as the baseline.

## 8.7	References

- GP-140335, “Downlink MIMO Simulation Assumptions for Final Performance Evaluation”, source NSN, GERAN#62.

- GP-140581, “Mode Adaptation in Downlink MIMO”, source Nokia Networks, GERAN#63.

- David Falconer, "Frequency domain Equalisation for Single-Carrier Broadband Wireless Systems," IEEE Communications Magazine, Apr. 2002, pp. 58-66.

- GP-130686 – “MIMO Channel Model with Variable Spatial Correlation”, source NSN, GERAN#59.

End of modifications