3GPP TSG RAN Meeting #100			RP-230864

Taipei, 12th – 14th June, 2023

Agenda item:		9.3.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Uplink coverage enhancements

Document for:		Discussion and Decision

# Introduction

This contribution relates to a work item in RAN#94-e, namely “Further NR coverage enhancements” [1]. We consider power domain enhancements and the following objective captured in the WID:

- Study and if necessary specify following power domain enhancements

  - […]

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1).

W.r.t. “Study and if necessary specify” there has been considerable progress in both RAN4 and RAN1:

- RAN1 #110bis-e made the following agreements for the work split:

  - RAN1 performs link level simulations of candidate solutions for power domain enhancements […]

  - RAN1 is not expected to perform RF simulations of candidate solutions for power domain enhancements

  - RAN1 understands that RAN4 is responsible for selecting the Rel-18 MPR/PAR reduction solution, if any.

- RAN4 #104bis-e made the following agreements:

  - RAN4 follows [below] RAN1 agreements and focus on prepare for RF simulations

  - Actual conclusion of the MPR/PAR reduction methods should be based on net coverage gain results combining transmitter and receiver performance

- RAN4 #106bis-e

  - RAN4 agrees to analyze provided simulation results […] and provide conclusions based on these at RAN4#107.

  - It is expected a decision on selecting the Rel-18 MPR/PAR reduction solution(s) according to the WID can be taken at RAN4#107.

# Discussion

Despite the agreement made in RAN4 #106bis-e, as well as substantial net gain results showing significant coverage gains, RAN4 #107 was not able to make a decision on selecting the Rel-18 MPR/PAR reduction solution(s) according to the WID. Hence, given all the analysis that has been carried out in RAN4 already, RAN needs to decide the way forward to allow completion of the related Rel-18 WI / MPR/PAR reduction objective in time.

Prior to RAN4 #107, different companies had provided transmitter performance and receiver performance results in [2] for the set of PRB/MCS configurations agreed in RAN1 #111 (see Appendix). The focus has been on the scenario with the most significant gain potential, namely DFT-s-OFDM waveform and QPSK modulation. The considered MPR/PAR reduction schemes according to WID include:

- Transparent scheme: FDSS without spectrum extension (FDSS)

- Non-transparent scheme: FDSS with spectrum extension (FDSS-SE)

- Non-transparent scheme: Tone reservation (with very limited number of results, and smaller net gains).

It is worth observing that link-level performance of a number of transparent schemes other than FDSS without spectrum extension have been evaluated by some companies, according to existing RAN1 agreements, and completed by corresponding companies with transmitter performance evaluation, despite the absence of suitable RAN4 agreements.

Review of net gain results from different companies [3]:

Efforts to aggregate and analyze quantitatively the net-gain results from different company inputs have been made by at least one company prior to RAN4 #107 [3]. This was used as a starting point to discuss net gain results during RAN4 #107.

- Figures 1 and 2 illustrate average net gains of FDSS and FDSS-SE for inner and outer allocations, taken from “Average”-columns of Tables 1-8 in [3].

- The net gain results consider both the increased Tx power (due to reduced OBO) and potential Rx loss.

Figure 1 Average Net Gains of FDSS and FDSS-SE for Inner Allocations

Figure 2 Average Net Gains of FDSS and FDSS-SE for Outer Allocations

The way forward document [3] summarizes the findings as:

FDSS:

- “For 700MHz companies report net-gain improvements versus baseline with an average of +0.12dB for inner and +0.39dB for outer allocations

- For 4GHz companies report net-gain improvements versus baseline with an average of +0.06dB for inner and +0.43dB for outer allocations”

FDSS-SE:

- “For 700MHz companies report net-gain improvements versus baseline with an average of +0.63dB for inner and +1.04dB for outer allocations

- For 4GHz companies report net-gain improvements versus baseline with an average of +0.96dB for inner and +1.19dB for outer allocations”

The average gains from Figure 1 and Figure 2 can be summarized in the following way:

Observation 1: Based on reported net gain, FDSS-SE provides considerable average gain (vs. baseline = Rel-17):

- For inner allocations [700 MHz, 4GHz]: [0.63 0.96] dB (average 0.80 dB)

- For outer allocations [700 MHz, 4GHz]: [1.04 1.19] dB (average 1.12 dB)

Observation 2: Based on reported net gain, FDSS provides only small average gain (vs. baseline = Rel-17):

- For inner allocations [700 MHz, 4GHz]: [0.12 0.06] dB (average 0.09 dB)

- For outer allocations [700 MHz, 4GHz]: [0.39 0.43] dB (average 0.41 dB)

Additional results submitted by Nokia are summarized in Appendix B. They show that the net gain from transparent FDSS is typically 1-1.5 dB smaller compared to that of FDSS with spectral extension. For the largest allocations, the difference is up to 2 dB. In many cases (esp. with FR2) the net gain from transparent FDSS is close to zero.

Conclusions:

Based on the substantial number of evaluated net gain results during the WI by different companies (including Nokia) showing significant coverage gains, we make the following proposal:

Proposal 1: RAN#100 should confirm that at least FDSS with spectrum extension (FDSS-SE) shall be specified as a solution for MPR/PAR reduction in Rel-18.

# Conclusions

Based on the discussion above, we make the following observations and a proposal:

Observation 1: Based on reported net gain, FDSS-SE provides considerable average gain (vs. baseline = Rel-17):

- For inner allocations [700 MHz, 4GHz]: [0.63 0.96] dB (average 0.80 dB)

- For outer allocations [700 MHz, 4GHz]: [1.04 1.19] dB (average 1.12 dB)

Observation 2: Based on reported net gain, FDSS provides only small average gain (vs. baseline = Rel-17):

- For inner allocations [700 MHz, 4GHz]: [0.12 0.06] dB (average 0.09 dB)

- For outer allocations [700 MHz, 4GHz]: [0.39 0.43] dB (average 0.41 dB)

Proposal 1: RAN#100 should confirm that at least FDSS with spectrum extension shall be specified as a solution for MPR/PAR reduction in Rel-18.

# References

- RP-213579, “New WI: Further NR coverage enhancements”, China Telecom

- R4-2308110, “Collected Simulation Results for MPR/PAR evaluations”, Nokia, Nokia Shanghai Bell

- R4-2310303, “WF on coverage enhancement part2”, Apple

- R4-2308108, “RF simulation results for non-transparent schemes”, Nokia, Nokia Shanghai Bell

- R4-2308105, Net gain analysis and next steps for MPR/PAR - objective”, Nokia, Nokia Shanghai Bell

# Appendix A

The set of configurations defining MCS/PRB allocations for simulations was made in RAN1 #111 and the configurations are shown in Table 3.

Table 3 Set of configurations for simulations

|  |  | No spectrum extension |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TBS value | Tput estimation for DDDSU @4GHz | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| 2408 | 963.2 kbps | 16 | 7 | 14 | 16 | 8 | 1/8 |
| 5376 | ~2.15 Mbps | 32 | 8 | 28 | 32 | 9 | 1/8 |
| 272 | 108.8 kbps | 8 | 0 | 6 | 8 | 1 | ¼ |
| 1032 | 412.8 kbps | 8 | 6 | 6 | 8 | 8 | ¼ |
| 2152 | ~0.9 Mbps | 40 | 2 | 30 | 40 | 3 | ¼ |
| 4992 | ~2.0 Mbps | 40 | 6 | 30 | 40 | 8 | ¼ |
| 552 | 220.8 kbps | 16 | 0 | 10 | 16 | 2 | 3/8 |
| 1736 | 694.6 kbps | 32 | 2 | 20 | 32 | 4 | 3/8 |
| 432 | 172.8 kbps | 8 | 2 | 6 | 8 | 3 | ¼ |
| 808 | 323.2 kbps | 24 | 0 | 18 | 24 | 1 | ¼ |

# Appendix B

Discussion on Nokia’s results in [2]:

Table 1 and Table 2 (copied from [5]) summarize the net gain for Nokia results provided in the Results Excel [2]. They illustrate the maximum net gain among the four tested filters (#1-4). Table 1 represents 700 MHz rural scenario, and Table 2 4GHz urban scenario, respectively. In addition to allocation size addressed in Figures 1 and 2, these results also highlight the impact of MCS and Extension factor (not visible in Figure 1 and Figure 2).

Table 1 Net gain results for FDSS (transparent) and FDSS-SE (non-transparent) for scenarios shown in APPENDIX A, 700MHz rural, QPSK modulation, DFT-s-OFDM.

Table 2 Net gain results for FDSS (transparent) and FDSS-SE (non-transparent) for scenarios shown in APPENDIX, 4GHz urban, QPSK modulation, DFT-s-OFDM. Net gain simulated also for the additional case with extension factor 1/9 (as per discussion in RAN4 #106bis-e).

The following observations can be made from Table 1 and Table 2:

- FDSS-SE provides considerable net gain (compared to baseline “no filter”)

  - The gain is up-to 1.3 dB for outer allocations (extension factor 0.25)

  - The gain is up-to 1.0 dB for inner allocations (extension factor 0.25)

  - The gain is largest with low MCS (MCS0 and MCS2)

  - The gain for outer allocations is more than 1 dB for each evaluated scenario with low MCS

    - Gain varies between 1.03 and 1.33 dB (extension factor 0.25).

  - The net gain performance for carrier frequency set at 700MHz and 4GHz is quite similar

  - The gain with extension factor 0.375 is between 0.6 and 1.3 dB

  - The gain with extension factor 0.125 is up-to 0.4 dB.

  - The net gain performance between 700MHz and 4GHz is quite similar.

- Net gain for transparent scheme (FDSS) is smaller compared to FDSS-SE. The largest difference happens for inner allocations, where net gain for FDSS is close to zero.

Discussion on Nokia’s additional results in [4]:

Results reported in Excel [2] cover 10 different scenarios with different MCS/RB allocations (see APPENDIX). In [4], we provide a broader view for the net gain covering all possible RB start values in the following scenarios:

- 700 MHz rural, 20 MHz, FR1:

    - Subcarrier spacing: 15 kHz

    - 4 allocation sizes: [16, 32, 64, 96] RBs

    - 6 code rates (1/16, 1/8, 1/6, ¼, 1/3, ½)

    - 3 extension sizes: [0.125, 0.25, 0.375]

      - 72 scenarios

- 4GHz urban, 100 MHz, FR1:

    - Subcarrier spacing: 30 kHz

    - 5 allocation sizes: [16, 32, 64, 128, 256] RBs

    - 6 code rates (1/16, 1/8, 1/6, ¼, 1/3, ½)

    - 3 extension sizes: [0.125, 0.25, 0.375]

      - 90 scenarios

- 28 GHz, 400 MHz, FR2:

    - Subcarrier spacing: 120 kHz

    - 5 allocation sizes: [16, 32, 64, 128, 256] RBs

    - 6 code rates (1/16, 1/8, 1/6, ¼, 1/3, ½)

    - 3 extension sizes: [0.125, 0.25, 0.375]

      - 90 scenarios

Thus, the total number of scenarios studied in [4] is 252 (on top of 10 scenarios included in Excel [2]). The additional net gain results in [4] are well inline with net gain results from different companies [2, 3], and can be summarized in the following way:

- Net gain from non-transparent scheme varies according to Tx filter, extension size, code rate, and RB allocation.

  - Considerable net gains are observed for all RB allocations. For inner allocations, the gains can be more than 1.5 dB. For outer allocations, the gains can be more than 2 dB.

  - Net gain is the largest for the largest RB allocations. However, net gain around 1 dB is available also for the smallest RB allocation (16RBs) considered in simulation scenarios.

  - Net gain reduces when the code rate increases. However, positive net gain is available even for the highest code rates considered in the simulation scenarios.

  - 25% extension maximizes the net gain in most of the simulation scenarios.

  - Trend: Less aggressive filters (such as TRRC) maximize the net gain for small/inner allocations, while more aggressive filters (such as [0.335 1 0.335]) maximize the net gain for large/outer allocations.

  - 20 MHz CBW and 100 MHz CBW provide similar net gains.

  - Similar net gains are available for both FR1 and FR2.

- Net gain from transparent scheme is typically 1-1.5 dB smaller compared to that of non-transparent schemes. For the largest allocations, the difference is up-to 2 dB. In many cases (esp. with FR2) the net gain from transparent schemes is close to zero.

2-tap filter [1 0.28], 3-tap filter [0.335 1 0.335], 3-tap filter [0.28 1 0.28], TRRX [0.5 0.1667]

Without extension.