3GPP TSG-RAN5 Meeting #99	R5-232520

# Incheon, Korea, 22nd May – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2766 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR NTN test frequencies for n255 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR NTN test frequencies for band n255 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new section, 4.3.1.9.1.2, to capture NR NTN test frequencies for NR NTN operating band n255 using 15 kHz SSB SCS. Add test frequencies for n255 in three tables with SCS=15, 30 and 60 kHz respectively. Test frequencies for n255 using 30 kHz SSB SCS is for further study. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RF tests of NR NTN cannot be fully specified without test frequencies for band n255. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Change ***

###### 4.3.1.9.1.2	Reference test frequencies for NR NTN operating band n255

Table 4.3.1.9.1.2-1: Test frequencies for NR operating band n255 with SCS 15 kHz and SSB SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1527.5 | 305500 | 1525.25 | 305050 | 0 | 15 | 3818 | 305530 | 4 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1521.39 | 304278 | 102 |  | 3855 | 308430 | 4 | 1 | 1(2) | 105 |
|  |  |  | High | 1556.5 | 311300 | 1463.53 | 292706 | 504 |  | 3892 | 311330 | 4 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1629 | 325800 | 1626.75 | 325350 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1550.53 | 310106 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1658 | 331600 | 1654.67 | 330934 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1530 | 306000 | 1525.32 | 305064 | 0 | 15 | 3819 | 305550 | 6 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1518.96 | 303792 | 102 |  | 3849 | 307950 | 6 | 1 | 1(2) | 105 |
|  |  |  | High | 1554 | 310800 | 1458.6 | 291720 | 504 |  | 3879 | 310350 | 6 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1631.5 | 326300 | 1626.82 | 325364 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1548.1 | 309620 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1655.5 | 331100 | 1649.74 | 329948 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1532.5 | 306500 | 1525.39 | 305078 | 0 | 15 | 3820 | 305570 | 8 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1516.53 | 303306 | 102 |  | 3843 | 307470 | 8 | 1 | 1(2) | 105 |
|  |  |  | High | 1551.5 | 310300 | 1453.67 | 290734 | 504 |  | 3866 | 309370 | 8 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1634 | 326800 | 1626.89 | 325378 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1545.67 | 309134 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1653 | 330600 | 1644.81 | 328962 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1535 | 307000 | 1525.46 | 305092 | 0 | 15 | 3818 | 305530 | 2 | 0 | 1(2) | 2 |
|  |  |  | Mid | 1542 | 308400 | 1514.1 | 302820 | 102 |  | 3837 | 306990 | 10 | 1 | 1(2) | 105 |
|  |  |  | High | 1549 | 309800 | 1448.74 | 289748 | 504 |  | 3856 | 308450 | 6 | 1 | 2(4) | 509 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1626.96 | 325392 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1543.24 | 308648 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1650.5 | 330100 | 1639.88 | 327976 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.2-2: Test frequencies for NR operating band n255 with SCS 30 kHz and SSB SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 1530 | 306000 | 1520.64 | 304128 | 0 | 15 | 3813 | 305070 | 2 | 0 | 3(8) | 16 |
|  |  |  | Mid | 1542 | 308400 | 1495.92 | 299184 | 102 |  | 3843 | 307470 | 2 | 0 | 3(8) | 220 |
|  |  |  | High | 1554 | 310800 | 1363.2 | 272640 | 504 |  | 3873 | 309870 | 2 | 0 | 3(8) | 1024 |
|  |  | Uplink | Low | 1631.5 | 326300 | 1622.14 | 324428 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1452.7 | 290540 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1655.5 | 331100 | 1643.98 | 328796 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1532.5 | 306500 | 1518.28 | 303656 | 0 | 15 | 3808 | 304610 | 6 | 0 | 3(8) | 16 |
|  |  |  | Mid | 1542 | 308400 | 1491.06 | 298212 | 102 |  | 3831 | 306510 | 6 | 0 | 3(8) | 220 |
|  |  |  | High | 1551.5 | 310300 | 1355.84 | 271168 | 504 |  | 3854 | 308410 | 6 | 0 | 3(8) | 1024 |
|  |  | Uplink | Low | 1634 | 326800 | 1619.78 | 323956 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1447.84 | 289568 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1653 | 330600 | 1636.62 | 327324 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1535 | 307000 | 1515.92 | 303184 | 0 | 15 | 3800 | 304090 | 14 | 0 | 2(7) | 14 |
|  |  |  | Mid | 1542 | 308400 | 1486.2 | 297240 | 102 |  | 3816 | 305310 | 2 | 0 | 0(5) | 214 |
|  |  |  | High | 1549 | 309800 | 1348.48 | 269696 | 504 |  | 3835 | 306770 | 22 | 0 | 0(5) | 1018 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1617.42 | 323484 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1442.98 | 288596 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1650.5 | 330100 | 1629.26 | 325852 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.2-3: Test frequencies for NR operating band n255 with SCS 60 kHz and SSB SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1530 | 306000 | 1526.04 | 305208 | 0 | 15 | - | 305568 |
|  |  |  | Mid | 1542 | 308400 | 1464.6 | 292920 | 102 |  | - | 307968 |
|  |  |  | High | 1554 | 310800 | 1187.16 | 237432 | 504 |  | - | 310368 |
|  |  | Uplink | Low | 1631.5 | 326300 | 1627.54 | 325508 | 0 | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1276.66 | 255332 | 504 |  | - | - |
|  |  |  | High | 1655.5 | 331100 | 1647.22 | 329444 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1532.5 | 306500 | 1526.02 | 305204 | 0 | 15 | - | 305564 |
|  |  |  | Mid | 1542 | 308400 | 1462.08 | 292416 | 102 |  | - | 307464 |
|  |  |  | High | 1551.5 | 310300 | 1182.14 | 236428 | 504 |  | - | 309364 |
|  |  | Uplink | Low | 1634 | 326800 | 1627.52 | 325504 | 0 | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1274.14 | 254828 | 504 |  | - | - |
|  |  |  | High | 1653 | 330600 | 1642.2 | 328440 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1535 | 307000 | 1526.36 | 305272 | 0 | 15 | - | 305632 |
|  |  |  | Mid | 1542 | 308400 | 1459.92 | 291984 | 102 |  | - | 307032 |
|  |  |  | High | 1549 | 309800 | 1177.48 | 235496 | 504 |  | - | 308432 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1627.86 | 325572 | 0 | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1271.98 | 254396 | 504 |  | - | - |
|  |  |  | High | 1650.5 | 330100 | 1637.54 | 327508 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

*** End of Change ***