3GPP TSG RAN meeting #100										RP-231022

Taipei, June 12 - 14, 2023

Status Report to TSG

Agenda item:			9.3.2.3

| WI / SI Name | Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: - |
| Acronym | NR_DualTxRx_MUSIM |  |  |  |  |
| Unique ID | 941001 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230751 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: - | Core part: 12/2023 | Performance part: 06/2024 | Testing part: - |  |
| Overall Completion level | Study Item: - | Core part: Overall: 40% | Performance Part: - Overall: 0% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Xiaodong Yang |
|  | Company | vivo |
|  | Email | Yangxiaodong5g@vivo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? |  |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2 121 bis

Temporary capability restriction

- Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

- Support “early indication” from UE to network during RRC connection setup/resume procedure.

- FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted.

- FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

- No consensus to support UE-initiated SCell deactivation for MUSIM in Rel-18.

- 1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B.

- 3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B.

- 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI.  FFS if this is only for NW A or also NW B.

- 8: The maximum UL power may be changed due to Rel-18 MUSIM dual active operation, but there is no need to introduce any new UE behavior for reporting this change.

- 6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

- 9: RAN2 should avoid duplicating all the capabilities that UE reports via the UECapabilityInformation in the UAI for R18 MUSIM purpose.

- RAN2 can discuss P2, P5 and P7 from R2-2304397 during RAN2#123.

R17 MUSIM Gap coordination

- RAN2 will aim to address the RAN4 LS in Rel-18 signalling. Should discuss how to handle Rel-17 gaps without priority (e.g. lowest, highest, network-decided somehow, etc.). Handled in email [231]

- 1: Introduce 1 optional per-UE capability bit (without xDD/FRx differentiation) to indicate MUSIM gap priority configuration and preference. A UE supporting this feature shall also support musim-GapPreference-r17.

- 2: Introduce a new indication in the OtherConfig to indicate whether UE is allowed to report MUSIM gap priority preference via UAI.

- 4: The existing IE GapPriority-r17 is re-used to configure the priority for periodic MUSIM gap.

Other

- 9: RAN2 assumes no RAN4 impact is expected on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2.

2.2.2	Remaining Open issues

1 Continue to discuss the mechanism to indicate preference on temporary UE capability restriction in NW A for MUSIM purpose.

2 MUSIM Gap coordination and priority handling

3 UE Capability definition and reporting for R18 MUSIM

2.3	RAN3

2.3.1	Agreements

    - Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4 106bis

Issue 1-1-3: Total number of gaps when MUSIM gaps are configured

Agreement

P2 with wording update as the following:

- When MUSIM gaps are configured and Rel-17 Con-MGs is not configured or supported, the number of legacy MGs can be

  - Up to 1 per-UE MG, or

  - Up to 1 per-FR MG in each FR

- When MUSIM gaps are configured, when Rel-17 con-MG is configured, the number of legacy MGs can be one of the following cases:

  - Up to 2 per-UE MGs

  - Up to 2 per-FR MGs in each FR and up to 3 per-FR MGs across FRs

  - Up to 1 per-UE MG and up to 1 per-FR MG in each FR

P3: Allocation of MUSIM gaps does not impact the non-MUSIM gap allocation capability.

Issue 2-1-2: Priority/usage indication on MUSIM gaps from UE side

Agreements:

Agree P2 - Network A assigns priority levels to all configured periodic MUSIM gaps even if UE does not indicate preferred priority for one or some periodic MUSIM gaps

Issue 2-1-2-1: On how to delivery priority/usage indication on MUSIM gaps from UE side

GTW agreement:

  - It is RAN4 understanding that the signalling design of priority levels indication/configuration for MUSIM gaps is up to RAN2 decision.

Issue 2-1-5: Priority setting for aperiodic MUSIM gaps

Agreements from GTW

  - Option 1

    - The priority level of aperiodic MUSIM gap can be configured by NW A

    - If the priority level is not configured by NW A then the aperiodic MUSIM gap by default has the highest priority level

    - The aperiodic MUSIM gap priority level can be optionally requested by UE from NW A

  - Option 2

    - Aperiodic MUSIM gap by default has the highest priority level.

    - The gap priority level is not explicitly configured by the NW

Issue 2-3-1:  Solutions for collision between MUSIM gap and Type-2 MG

Agreements:

Update previous agreement “Priority-based gap collision handling introduced in concurrent gaps design can be used as a base for collisions between MUSIM gap and Type -2 MG” in R4-2220443 as the following:

Priority-based gap collision handling rule introduced in Rel-17 MG_enh WI is reused to solve collisions between MUSIM gap and Type -2 MG.

Issue 2-4-1: Definition of the collision between MUSIM gaps and L1/L3 measurement resources

Agreements:

- A L1/L3 measurement resource is considered to be fully overlapped with a periodic MUSIM gap if all of the resource instances overlap with MUSIM gap occasions in the time domain

- A L1/L3 measurement resource is considered to be partially overlapped with a periodic MUSIM gap if some but not all of the resource instances overlap with MUSIM gap occasions in the time domain

- A L1/L3 measurement resource is considered to be overlapped with an aperiodic MUSIM gap if it at least one of its resource instances overlaps with the aperiodic MUSIM gap occasion in the time domain

Issue 2-4-2: Priority of MUSIM against SMTC for L3/ L1 measurement

Agreements

P1 - MUSIM gaps have higher priority when colliding with SMTC/SSB for L3/L1 measurement.

Issue 3-1-3: On the time window W for aperiodic gap

Agreements:

P1 - Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer

Issue 4-1-2: Network B requirements framework

Agreements: P2: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline

RAN4 107

Issue 1-1-2: MUSIM overhead

Agreement: Option 1.

Option 1: Do not define overhead cap for MUSIM gaps

Issue 2-2-2: Solutions for collision between different MUSIM gaps

Agreements

  - Define two solutions for collision handling between different MUSIM gaps

    - 1) Priority based solution (i.e., network controls the MUSIM gaps priority)

    - 2) “Keep” solution (i.e., keep all collided MUSIM gaps)

  - FFS on the mechanism to select and/or switch between the solutions

Issue 2-4-3: Collision between SMTC and MUSIM gaps for handover and Scell activation

Agreements

- When MUSIM gaps are configured, UE is still required to meet Scell activation RRM requirements for NW-A. FFS whether to capture this conclusion in the specifications.

    - No test case will be defined to verify this case

- FFS whether the agreement applies for handover

Issue 4-1-6: Others NW B requirements

Agreement: P1

P1: NW B requirements for measurements of intra-frequency/ inter-frequency NR cells and inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion are not considered in Rel-18

2.4.2	Remaining Open issues

1. Collisions between gaps and priority rules

2. On network A requirements

3. On network B requirements

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	RAN Plenary Intervention

5.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2 #121 bis

R2-2302430	LS on priority for MUSIM gaps (R4-2303249; contact: vivo)	RAN4	LS in	Rel-18	NR_DualTxRx_MUSIM-Core	To:RAN2

R2-2303266	MUSIM Stage 2 running CR	vivo	discussion	Rel-18

R2-2302550	Procedures for MUSIM temporary capability restriction	OPPO	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2302721	UE Capability restrictions for Dual-Active MUSIM	Qualcomm Incorporated	discussion

R2-2302725	Consideration on capability restriction for dual Rx/Tx MUSIM	DENSO CORPORATION	discussion	NR_DualTxRx_MUSIM-Core

R2-2302781	Further considerations on the capability restriction request for Rel-18 MUSIM	Intel Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303188	Baseline signalling procedure options  for temporary capability restrictions.	Nokia, Nokia Shanghai Bell	discussion

R2-2303225	Procedure of dual Tx/Rx Multi-SIM	Lenovo	discussion	Rel-18

R2-2303267	Procedures for MUSIM temporary capability restriction	vivo	discussion	Rel-18

R2-2303409	Procedures for MUSIM temporary capability restriction	Apple	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303639	Overall Dual-RX/TX MUSIM procedure	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303669	Procedures for MUSIM temporary capability restriction	Samsung R&D Institute India	discussion

R2-2303774	Procedure of UE Capability Restriction for eMUSIM	Sharp	discussion

R2-2303874	Temporary Capability Restriction for Idle/Inactive State Transfer	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2304026	Procedures for MUSIM Temporary Capa Restriction	LG Electronics	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2302551	Allowed MUSIM temporary capability restrictions	OPPO	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2302782	Signalling to indicate temporary capability reduction for Rel-18 MUSIM	Intel Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2302966	Allowed MUSIM temporary capability restrictions	Samsung R&D Institute India	discussion	Rel-18

R2-2303189	Adidtional aspects related to capability restriction signalling	Nokia, Nokia Shanghai Bell	discussion

R2-2303268	Discussion on temporary capability restriction for Rel-18 Multi-SIM	vivo	discussion	Rel-18

R2-2303350	Capability sharing issue for SRS Tx switching capability	Xiaomi	discussion	Rel-18	NR_DualTxRx_MUSIM-Core	R2-2301116

R2-2303351	Remaining issues on band combination restrictions due to band conflict	Xiaomi	discussion	Rel-18	NR_DualTxRx_MUSIM-Core	R2-2301117

R2-2303410	Parameters for MUSIM temporary capability restriction	Apple	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303455	Further discussion on the UE-initiated SCell/SCG deactivation and activation for MUSIM	Huawei, HiSilicon, Vodafone, Vivo	discussion	Rel-18

R2-2303470	Further discussion on MUSIM temporary capability restrictions	Huawei, HiSilicon	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303623	Discussion on temporary UE capability restriction for MUSIM	MediaTek Inc.	discussion	R2-2300816

R2-2303624	Disucssion on UE capability restriction signaling	China Telecommunications	discussion

R2-2303640	Discussion on restricted UE capabilities	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303779	Support of UE requesting SCell/SCG Deactivation for eMUSIM	Sharp	discussion

R2-2303873	Consideration on the Temporary Capability Restriction	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303938	Discussion on temporary capability restriction for Dual Tx/Rx Multi-SIM	ASUSTeK	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2304027	Simple Methods for MUSIM Temporary Capa Restriction	LG Electronics	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2302724	Remaining issues for MUSIM gaps	Qualcomm Incorporated	discussion

R2-2302783	Gap collision handling for Rel-17 gaps	Intel Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303190	On MUSIM gap priority and uplink power sharing aspects of MUSIM operation	Nokia, Nokia Shanghai Bell	discussion

R2-2303269	Discussion on MUSIM gap priorities	vivo	discussion	Rel-18

R2-2303352	Discussion on MUSIM gap priorities	Xiaomi	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303411	Views on RAN4 LS for MUSIM gap priorities	Apple	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2303471	Discussion on MUSIM gaps and other RAN4 topics	Huawei, HiSilicon	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303641	MUSIM gap priorities	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303828	Discussion on MUSIM gap priorities and maximum UL power change	Samsung Electronics Austria	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303875	Consideration on the Scheduling Gap Priority	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2303937	Discussion on maximum UL power change for Dual Tx/Rx Multi-SIM	ASUSTeK	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2304028	MUSIM Gap Priority	LG Electronics	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

RAN4 #106bis

R4-2304079	Further considerations on general aspects for MUSIM gaps	vivo

R4-2304080	Further considerations on collisions between gaps and priority rules for MUSIM gaps	vivo

R4-2304081	Further considerations on network A requirements for RRM requirements of MUSIM gaps	vivo

R4-2304082	Further considerations on network B requirements for RRM requirements of MUSIM gaps	vivo

R4-2304297	On R18 MUSIM - General aspects	Apple

R4-2304298	On R18 MUSIM - collisions between gaps and priority rules	Apple

R4-2304299	On R18 MUSIM - network A requirements	Apple

R4-2304300	On R18 MUSIM - network B requirements	Apple

R4-2304516	LS on priority for MUSIM gaps	vivo

R4-2304616	Discussion on the general aspects of MUSIM gaps	MediaTek inc.

R4-2304617	Discussion on RRM requirements for MUSIM gaps collision handling	MediaTek inc.

R4-2304618	Discussion on NW A RRM requirements for MUSIM	MediaTek inc.

R4-2304619	Discussion on NW B RRM requirements for MUSIM	MediaTek inc.

R4-2304777	Discussion on general issues for Rel-17 MUSIM gaps	Xiaomi

R4-2304778	Discussion on collisions between gaps and priority rules for Rel-17 MUSIM gaps	Xiaomi

R4-2304779	Discussion on network A requirements for Rel-17 MUSIM gaps	Xiaomi

R4-2304780	Discussion on network B requirements for Rel-17 MUSIM gaps	Xiaomi

R4-2304849	Discussion on collisions between gaps and priority rules for MUSIM gaps	CMCC

R4-2304850	Discussion on network A requirements for MUSIM gaps	CMCC

R4-2304851	Discussion on network B requirements for MUSIM gaps	CMCC

R4-2304852	Discussion on open issues for MUSIM gaps	CMCC

R4-2304884	Discussions on general issues in MUSIM gaps	Ericsson

R4-2304885	Discussions on collision between MUSIM gaps	Ericsson

R4-2304886	Discussions on NW-A’s requirement in MUSIM gaps	Ericsson

R4-2304887	Discussions on NW-B’s requirement in MUSIM gaps	Ericsson

R4-2305032	Discussion on collisions between gaps and priority rules	ZTE Corporation

R4-2305033	Discussion on Network A requirements	ZTE Corporation

R4-2305034	Discussion on Network B requirements	ZTE Corporation

R4-2305223	Discussion on general RRM requirements for Rel-17 MUSIM gaps	OPPO

R4-2305224	Discussion on collision between gap and priority rules	OPPO

R4-2305225	Discussion on network A requirements	OPPO

R4-2305330	Discussion on collision handling for MUSIM gaps	Huawei, HiSilicon

R4-2305331	Discussion on NW-A requirements with MUSIM gaps	Huawei, HiSilicon

R4-2305332	Discussion on NW-B requirements with MUSIM gaps	Huawei, HiSilicon

R4-2305333	Discussion on general issues related to MUSIM gaps	Huawei, HiSilicon

R4-2305550	Discussion on General aspects	Nokia, Nokia Shanghai Bell

R4-2305551	Collisions between gaps and priority rules	Nokia, Nokia Shanghai Bell

R4-2305552	On network A requirements	Nokia, Nokia Shanghai Bell

R4-2305553	On network B requirements	Nokia, Nokia Shanghai Bell

R4-2305668	On requirements for Rel-17 MUSIM gaps - Gap collisions	Qualcomm Incorporated

R4-2305669	On requirements for Rel-17 MUSIM gaps - Network A requirements	Qualcomm Incorporated

R4-2305670	On requirements for Rel-17 MUSIM gaps - Network B requirements	Qualcomm Incorporated

R4-2305697	Discussion on collisions between gaps and priority rules of MUSIM	Charter Communications, Inc

R4-2306168	Topic summary for [106-bis-e][220] NR_DualTxRx_MUSIM	Moderator (Vivo)

R4-2306251	Topic summary for [106-bis-e][220] NR_DualTxRx_MUSIM	Moderator (Vivo)

R4-2306357	WF on NR Dual TxRx Multi-SIM	Vivo

RAN4 #107

R4-2307447	Considerations on issues for general aspects for MUSIM gaps	vivo

R4-2307448	Considerations on issues for collisions between gaps and priority rules for MUSIM gaps	vivo

R4-2307449	Considerations on issues for network A RRM requirements of MUSIM gaps	vivo

R4-2307450	Considerations on issues for network B RRM requirements of MUSIM gaps	vivo

R4-2307568	Discussion on collisions between gaps and priority rules for MUSIM gaps	CMCC

R4-2307569	Discussion on network A requirements for MUSIM gaps	CMCC

R4-2307570	Discussion on network B requirements for MUSIM gaps	CMCC

R4-2307571	Discussion on open issues for MUSIM gaps	CMCC

R4-2307653	Discussion on general aspects of R18 MUSIM	Apple

R4-2307654	Discussion on collisions between gaps and priority rules of R18 MUSIM	Apple

R4-2307655	Discussion on network A requirements of R18 MUSIM	Apple

R4-2307656	Discussion on network B requirements of R18 MUSIM	Apple

R4-2307960	Discussion on general issues for Rel-17 MUSIM gaps	Xiaomi

R4-2307961	Discussion on collisions between gaps and priority rules for Rel-17 MUSIM gaps	Xiaomi

R4-2307962	Discussion on network A requirements for Rel-17 MUSIM gaps	Xiaomi

R4-2307963	Discussion on network B requirements for Rel-17 MUSIM gaps	Xiaomi

R4-2308047	Discussion on collisions between gaps and priority rules	ZTE Corporation

R4-2308048	Discussion on Network A requirements	ZTE Corporation

R4-2308049	Discussion on Network B requirements	ZTE Corporation

R4-2308437	Discussions on general issues in MUSIM gaps	Ericsson

R4-2308438	Discussions on collision between MUSIM gaps	Ericsson

R4-2308439	Discussions on NW-A’s requirement in MUSIM gaps	Ericsson

R4-2308440	Discussions on NW-B’s requirement in MUSIM gaps	Ericsson

R4-2308470	Discussion on general RRM requirements for Rel-17 MUSIM gaps	OPPO

R4-2308471	Discussion on collision between gap and priority rules	OPPO

R4-2308472	Discussion on network A requirements	OPPO

R4-2308662	Discussion on general issues related to MUSIM gaps	Huawei, HiSilicon

R4-2308663	Discussion on collision handling for MUSIM gaps	Huawei, HiSilicon

R4-2308664	Discussion on NW A requirements with MUSIM gaps	Huawei, HiSilicon

R4-2308665	Discussion on NW B requirements with MUSIM gaps	Huawei, HiSilicon

R4-2308772	Discussion on General aspects	Nokia, Nokia Shanghai Bell

R4-2308773	Collisions between gaps and priority rules	Nokia, Nokia Shanghai Bell

R4-2308774	Network A requirements	Nokia, Nokia Shanghai Bell

R4-2308775	Network B requirements	Nokia, Nokia Shanghai Bell

R4-2309126	On requirements for Rel-17 MUSIM gaps - Gap collisions	Qualcomm Incorporated

R4-2309127	On requirements for Rel-17 MUSIM gaps - Network A requirements	Qualcomm Incorporated

R4-2309128	On requirements for Rel-17 MUSIM gaps - Network B requirements	Qualcomm Incorporated

R4-2309551	Discussion on the general aspects of MUSIM gaps	MediaTek inc.

R4-2309552	Discussion on RRM requirements for MUSIM gaps collision handling	MediaTek inc.

R4-2309553	Discussion on NW A RRM requirements for MUSIM	MediaTek inc.

R4-2309554	Discussion on NW B RRM requirements for MUSIM	MediaTek inc.

R4-2309970	Topic summary for [107][225] NR_DualTxRx_MUSIM	Moderator (Vivo)

R4-2310069	WF on MUSIM RRM requirements  vivo

R4-2310165	WF on MUSIM RRM requirements   vivo