3GPP TSG RAN Meeting #100	                                            			    RP- 231044

Taipei, June 12-14, 2023

Agenda Item:	9.2.2

Source:	Apple

Title:	On remaining aspects for ambient IoT

Document for:	Discussion/Decision

# Introduction

In RAN#97-e, a RAN SID for ambient power enabled IoT has been approved and further revised in RAN#98-e with following objectives in [1]:

This study targets at a new 3GPP IoT technology, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power.

In terms of energy storage, the study will consider the following device characteristics:

•	Pure batteryless devices with no energy storage capability at all, and completely dependent on the availability of an external source of energy

•	Devices with limited energy storage capability that do not need to be replaced or recharged manually.

Device categorization based on corresponding characteristics (e.g. energy source, energy storage capability, passive/active transmission, etc.) may be discussed during the study, in relation with the relevant use cases.

The device’s peak power consumption shall be limited by its practical form factor for the intended use cases, and shall consider its energy source.

  - Identify the suitable deployment scenarios and their characteristics, at least for the use cases/services agreed in SA1’s “Study on Ambient power-enabled internet of Things”, comprising among at least the following aspects

    - Indoor/outdoor environment

    - Basestation characteristics, e.g. macro/micro/pico cells-based deployments

    - Connectivity topologies, including which node(s) , e.g. basestation, UE, relay, repeater, etc. can communicate with target devices

    - TDD/FDD, and frequency bands in licensed or unlicensed spectrum

    - Coexistence with UEs and infrastructure in frequency bands for existing 3GPP technologies

    - Device originated and/or device terminated traffic assumption

NOTE: There can be more than one deployment scenario identified for a use case, and a deployment scenario may be common to more than one use case.

NOTE: Where more than one deployment scenario is identified for a use case, the trade-offs between them should also be studied.

NOTE: The study shall not prioritize deployment aspects that should be coordinated with SA, e.g. public or private network, with or without CN connection.

NOTE: A representative use case can be studied for a group of use cases that have similar requirements.

  - Formulate a set of RAN design targets based on the identified deployment scenarios and their characteristics for the relevant use cases, at least including

      - Power consumption

      - Complexity

      - Coverage

      - Data rate

      - Positioning accuracy

NOTE: The requirements from SA1 on the relevant use cases shall be taken into consideration.

NOTE: The study shall aim to provide better coverage compared to existing non-3GPP technologies for the relevant use cases.

NOTE: Other RAN design targets in relation to connection density, mobility, security, latency, reliability etc. may be discussed, if necessary for the relevant use cases.

NOTE: Detailed definitions of the RAN design targets should be discussed during the study.

  - Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported.

NOTE: This is not to require a detailed WG-level of analysis.

Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

In this contribution, we provide our views on remaining aspects for ambient IoT study in RAN plenary including:

- Representative use-cases, grouping and deployment scenarios

- Design targets

# Discussion

## Representative use-cases, grouping, and deployment scenarios

In RAN#99-e [2], following agreement related to representative use-case groups and corresponding mapping have been made:

The two groupings are used to form representative use cases (rUCs) as follows:

- rUC1: Indoor inventory

- rUC2: Indoor sensors

- rUC3: Indoor positioning

- rUC4: Indoor command

- rUC5: Outdoor inventory

- rUC6: Outdoor sensors

- rUC7: Outdoor positioning

- rUC8: Outdoor command

This resulted in the following mapping from SA1 use cases and traffic scenarios onto RAN rUCs:

| rUC | Applicable SA1 UCs / traffic scenarios |
| --- | --- |
| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label |
| rUC2: Indoor sensor | 5.6 Smart homes 5.13 Base station machine room environmental supervision 5.15 Smart laundry 5.20 Smart agriculture 5.23 Smart pig farm 6.2 Cow stable |
| rUC3: Indoor positioning | 5.8 Finding Remote Lost Item 5.9 Location service 5.10 Ranging in a home 5.12 Personal belongings finding 5.14 Positioning in shopping centre 5.21 Museum Guide |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |
| rUC5: Outdoor inventory | 5.2 Medical instruments inventory management and positioning 5.4 Non-public network for logistics 5.7 Airport terminal / shipping port 5.16 Automated supply chain distribution |
| rUC6: Outdoor sensor | 5.3 Smart grids 5.19 Forest Fire Monitoring 5.22 Dairy farming 5.24 Smart manhole cover safety monitoring 5.25 Smart bridge health monitoring |
| rUC7: Outdoor positioning | 5.8 Finding remote lost item 5.9 Location service 5.12 Personal belongings finding |
| rUC8: Outdoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.30 Controller in smart agriculture |

Furthermore, the representative use-cases and corresponding deployment scenarios with description have been agreed as follows in RAN#99 [2]:

Agreement:

The following 5 deployment scenarios are studied and captured in the TR:

- Note1: all the detailed values (with or without square brackets) in the tables are to be further discussed

- Note2: the location of assisting or intermediate node, where applicable for the corresponding topologies, would need to be defined additionally.

Deployment 1: Device indoors, base station indoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico- cell BS |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or C |

Deployment 2: Device indoors, base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2) |
|  | Spectrum | Licensed FDD, or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 3: Device indoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory [indoor sensor] indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Deployment 4: Device outdoors, Outdoor base station

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 5: Device outdoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory [outdoor sensor] outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

For the agreed deployment scenarios, one key aspect that needs to be discussed is addition of the environment of intermediate and assisting node. This is relevant for topology (2) and (3) only. In our view, additional row on the environment for assisting or intermediate node and corresponding description can be added for the deployment scenarios 1, 2 and 4. In our view, the location of the assisting or intermediate node could ideally be at the boundary of indoor-outdoor environment in case when the device is indoor. However, this could vary depending up on specific coverage requirements. Therefore, from study perspective, the environment of the assisting or intermediate node could either be indoor or outdoor in case of deployment scenario 2. For deployment scenarios 1 and 4, since both the device and BS are either indoors or outdoors, the assisting or intermediate node could be indoor or outdoor, respectively.

Proposal 1: For deployment scenario 1, add additional row on the environment for intermediate or assisting node with following description:

Deployment 1: Device indoors, base station indoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Environment of intermediate or assisting node (if any) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico- cell BS |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or C |

For deployment scenario 2, another aspect under discussion is the applicable device characteristic. In this case, the base station is outdoors, and the device is indoor. Therefore, the coverage might be an issue. However, in terms of connectivity topology, both topology (1) and (2) are considered. In case of topology (1), due to absence of any assisting or intermediate node, device A or device B might be challenging to communicate with. With topology (2), due to the presence of assisting or intermediate node, device A or B might still be reasonable to support. Therefore, the device characteristic for deployment scenario 2 can be topology dependent.

Proposal 2: For deployment scenario 2, add additional row on the environment for intermediate or assisting node and also include device characteristic based on topology with following description:

Deployment 2: Device indoors, base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Environment of intermediate or assisting node (if any) | Indoor or Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2) |
|  | Spectrum | Licensed FDD, or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C Device C only with Topology (1) Device A or B or C with Topology (2) |

For deployment scenario 3, indoor sensor use-case should be included. Also, co-existence with only new UE can be considered for study. Regarding device characteristic, all device types can be considered

Proposal 3: For deployment scenario 3, add additional row on the environment for intermediate or assisting node and also include following updates:

Deployment 3: Device indoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory [indoor sensor] indoor positioning indoor command | Environment (of device) | Indoor |
|  | Environment of intermediate or assisting node (if any) | None |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or nNew UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

For deployment scenario 4, similar to deployment scenario 2, another aspect under discussion is the applicable device characteristic. In this case, the base station is outdoors, and the device is outdoors as well. Depending up on BS deployment, the coverage might be an issue in certain scenarios. However, in terms of connectivity topology, topology (1), (2)and (3) are considered. In case of topology (1), due to absence of any assisting or intermediate node, device A or device B might be challenging to communicate with. With topology (2) or (3), due to the presence of assisting or intermediate node, device A or B might still be reasonable to support. Therefore, the device characteristic for deployment scenario 4 can be topology dependent.

Proposal 4: For deployment scenario 4, add additional row on the environment for intermediate or assisting node and also include following updates:

Deployment 4: Device outdoors, Outdoor base station

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Environment (of intermediate or assisting node) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C Device C only with Topology (1) Device A or B or C with Topology (2) or (3) |

For deployment scenario 5, outdoor sensor use-case should be included. Also, co-existence with only new UE can be considered for study. Regarding device characteristic, all device types can be considered

Proposal 5: For deployment scenario 5, add additional row on the environment for intermediate or assisting node and also include following updates:

Deployment 5: Device outdoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory [outdoor sensor] outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or nNew UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

## Design target consideration for ambient IoT

In RAN#99, following agreements related to design targets have been made in [2] and captured in [3]:

Agreement:

Agree to set at least the design targets below in Ambient IoT in the RAN SI.

- Device power consumption

- Device complexity

- Coverage

- Data rate

- Maximum message size (or maximum ‘TB’ size)

- Latency

- Positioning accuracy

- Connection/device density

- Device speed (FFS absolute or relative or both)

Agreement

Device design target for power consumption during transmitting/receiving is:

- [Device A ≤ 10 μW] or [Device A ≤ 1 μW]

- Device A ≪ Device B < Device C, or Device A ≤ Device B < Device C

- Device C ≤ 1 mW to ≤ 10 mW

Agreement:

User experienced data rate design target is, at least for the uplink:

- Maximum  not less than: 5 kbps

- Minimum  not less than 0.1 kbps

Working assumption

Device complexity design target is:

- Device A: Comparable to UHF RFID ISO18000-6C (EPC C1G2)

- Device A < Device B < Device C

- Device C: Orders-of-magnitude lower than NB-IoT

For the power consumption target, currently 2 options are considered for device A including  ≤ 10 μW or ≤ 1 μW. In our view, to be comparable with RFID devices, ≤ 1 μW should be considered. However, achieving such low power consumption may require quite detailed analysis and discussion. This would be more suited to study in a potential new study in WGs. For this RAN study, we can keep both the options. Similarly, for device B, there are 2 options under consideration including device A ≪ device B or device A ≤ device B. In our view, this will depend on the target power consumption value for device A. If ≤ 10 μW is selected for device A, then Device A ≤ Device B should be selected, otherwise if  ≤ 1 μW is selected for device A, then Device A ≪ Device B should be selected.

Proposal 6: For device design target for power consumption, both options can be considered to be included in the TR for device A, i.e. device A ≤ 10 μW and device A ≤ 1 μW

Proposal 7: For device design target for power consumption, depending up on what option can be is selected for device A, corresponding selection of the power consumption comparison between device A and device B can be made:

- If device A ≤ 1 μW is selected, then Device A ≪ Device B < Device C

- If device A ≤ 10 μW is selected, then Device A ≤ Device B < Device C

Regarding device complexity design target, the working assumption can be confirmed. Further details regarding the orders of magnitude compared to NB-IoT device type can be considered in potential study item in the WGs.

Proposal 8: For device complexity design target, confirm the following working assumption:

Device complexity design target is:

- Device A: Comparable to UHF RFID ISO18000-6C (EPC C1G2)

- Device A < Device B < Device C

- Device C: Orders-of-magnitude lower than NB-IoT

For other design targets including coverage, data rate, maximum TB size, latency, positioning accuracy, connection/device density and device speed, the baseline requirements should be based on the representative use-cases. Furthermore, mapping of specific design target requirements to device types might be needed. However, considering the limiting time and scope of RAN study, we think that more detailed design requirements values and corresponding mapping to device types should be handled in potential study item in the WGs.

Proposal 9: For design targets including coverage, data rate, maximum TB size, latency, positioning accuracy, connection/device density and device speed, the baseline requirements should be based on the representative use-cases

Proposal 10: For specific device types, corresponding design target values is needed

- this could be more appropriate for WGs level study

On the device speed, there is an FFS whether absolute or relative speed is considered. In our view, this could depend on which topology is applied. For connectivity topology (4), since UE bases reader is used and it could be mobile, therefore, design target speed could be relative. For other topologies where BS based reader is used, absolute speed should be considered.

Proposal 11: For device speed design target, consider relative speed at least for topology (4)

- for other topologies including (1), (2) and (3), only absolute speed can be considered

# Conclusion

In this contribution, we have provided following proposals on the remaining aspects for ambient IoT:

Proposal 1: For deployment scenario 1, add additional row on the environment for intermediate or assisting node with following description:

Deployment 1: Device indoors, base station indoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Environment of intermediate or assisting node (if any) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico- cell BS |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or C |

Proposal 2: For deployment scenario 2, add additional row on the environment for intermediate or assisting node and also include device characteristic based on topology with following description:

Deployment 2: Device indoors, base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Environment of intermediate or assisting node (if any) | Indoor or Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2) |
|  | Spectrum | Licensed FDD, or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C Device C only with Topology (1) Device A or B or C with Topology (2) |

Proposal 3: For deployment scenario 3, add additional row on the environment for intermediate or assisting node and also include following updates:

Deployment 3: Device indoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory [indoor sensor] indoor positioning indoor command | Environment (of device) | Indoor |
|  | Environment of intermediate or assisting node (if any) | None |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or nNew UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Proposal 4: For deployment scenario 4, add additional row on the environment for intermediate or assisting node and also include following updates:

Deployment 4: Device outdoors, Outdoor base station

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Environment (of intermediate or assisting node) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C Device C only with Topology (1) Device A or B or C with Topology (2) or (3) |

Proposal 5: For deployment scenario 5, add additional row on the environment for intermediate or assisting node and also include following updates:

Deployment 5: Device outdoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory [outdoor sensor] outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or nNew UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Proposal 6: For device design target for power consumption, both options can be considered to be included in the TR for device A, i.e. device A ≤ 10 μW and device A ≤ 1 μW

Proposal 7: For device design target for power consumption, depending up on what option can be is selected for device A, corresponding selection of the power consumption comparison between device A and device B can be made:

- If device A ≤ 1 μW is selected, then Device A ≪ Device B < Device C

- If device A ≤ 10 μW is selected, then Device A ≤ Device B < Device C

Proposal 8: For device complexity design target, confirm the following working assumption:

Device complexity design target is:

- Device A: Comparable to UHF RFID ISO18000-6C (EPC C1G2)

- Device A < Device B < Device C

- Device C: Orders-of-magnitude lower than NB-IoT

Proposal 9: For design targets including coverage, data rate, maximum TB size, latency, positioning accuracy, connection/device density and device speed, the baseline requirements should be based on the representative use-cases

Proposal 10: For specific device types, corresponding design target values is needed

- this could be more appropriate for WGs level study

Proposal 11: For device speed design target, consider relative speed at least for topology (4)

- for other topologies including (1), (2) and (3), only absolute speed can be considered

# Reference

- 3GPP RP-223396, “SID revised: Study on Ambient IoT”, Huawei, HiSilicon

- 3GPP RAN#99-e, “Chairman Notes”

- 3GPP TR 22.840 V0.1.0, “Study on Ambient power-enabled Internet of Things (Release 19)”