3GPP TSG RAN Meeting #100									  RP-231112

Taipei, June 12-14, 2023

## Status Report to TSG

Agenda item:			9.3.4.6

| WI / SI Name | Even Further RRM enhancement for NR and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_enh3 |  |  |  |  |
| Unique ID | 950078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221696 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:80% RAN4: 80% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Roy Hu |
|  | Company | OPPO |
|  | Email | hurongyi@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#122 meeting

LS R2-2306863 was agreed by RAN2.

Based on the discussion in RAN2, RAN2 thinks that the existing L3 mesaurement framework can be reused for the unkown SCell measurement reporting based on SCell activaton command. Based on RAN2 understanding, RAN2 would like to provide the following answers to RAN4 questions: Question 1: How to configure the report of L3 measurement result for unknown FR2 SCell activation enhancement [RAN2 Answer to Q1] - The unkown SCell measurement report is configured via RRC measurement configuration (i.e. measConfig). Question 2: How to trigger the report of L3 measurement result for unknown FR2 SCell activation enhancement [RAN2 Answer to Q2] - The unkown SCell measurement report is triggered by SCell activation command MAC CE. Question 3: Contents of the report of L3 measurement result for unknown FR2 SCell activation enhancement and how to send the report [RAN2 Answer to Q3] - Legacy RRC MeasurementReport message is used for the unkown SCell measurement report, which can already carry the beam level L3 measurement result and beam index.

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#106-bis-e meeting

The following tdocs have been endorsed and approved in RAN4#106-bis-e meeting:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2304380 | DraftCR applicability Rule for FR1+FR1 | Qualcomm Incorporated |
| R4-2305057 | Draft CR on PSCell addition/release delay for FR1+FR1 NR-DC | vivo |
| R4-2305235 | Discussion on remaining issues of RRM requirements for FR1+FR1 NR-DC | OPPO |
| R4-2305236 | Draft CR on CSSF requirements for FR1+FR1 NR-DC | OPPO |
| R4-2306315 | WF on R18 RRM enhancement (part 1) | Apple |
| R4-2306321 | LS on FR2 unknown SCell activation enhancement | Apple |
| R4-2306322 | WF on R18 RRM enhancement - FR1+FR1 NR-DC | OPPO |
| R4-2306323 | DraftCR Scheduling availability for FR1+FR1 | Qualcomm Incorporated |
| R4-2306324 | Draft CR for FR1-FR1 NR-DC on scheduling availability of UE during candidate beam detection | MediaTek inc. |
| R4-2306325 | DraftCR on scheduling availability during L1-RSRP measurement for FR1-FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2306326 | Draft CR on HO with PSCell requirements for FR1-FR1 NR-DC | Xiaomi |
| R4-2306327 | draft CR for Rel-18 eFeRRM NR-DC FR1-FR1 SCG activation | Ericsson |
| R4-2306328 | Draft CR on scheduling availability of UE during intra-frequency measurements for FR1+FR1 NR-DC | OPPO |
| R4-2306329 | Draft Big CR for R18 RRM enhancement - FR1+FR1 NR-DC | OPPO |

RRM core requirements for FR2 SCell activation delay reduction

Issue 1-1-1: when to report L3 measurement results for unknown FR2 SCell activation enhancement

- Agreements (GTW, Monday Apr 17, 2023)

  - UE needs to report the L3 measurement result after SCell activation command

  - FFS if additional solutions should be considered. Decision on additional solutions need to be made no later that in RAN4 #107.

Issue 1-1-2: FFS on how to trigger L3 report for unknown FR2 SCell activation enhancement

- Agreements (GTW, Monday Apr 17, 2023)

  - The L3 measurement report for unknown FR2 Scell activation after Scell activation command is triggered by Scell activation command.

  - FFS whether to have additional events for report triggering

    - Send LS (R4-2306321) to let RAN2 to decide.

Issue 1-1-3: FFS on how to configure L3 report for unknown FR2 SCell activation enhancement

- Agreement:

  - The one-time report after SCell activation command is assumed in this enhancement

  - Send LS (R4-2306321) to let RAN2 to decide.

Issue 1-1-4: FFS on how to report L3 measurement result for unknown FR2 SCell activation enhancement

- Agreement:

  - Send LS (R4-2306321) to let RAN2 to decide.

- Technical enhancement goal: RAN4 had following agreements in RAN4 #106-bis-e meeting, - Agreements (GTW, Monday Apr 17, 2023) - UE needs to report the L3 measurement result after SCell activation command - FFS if additional solutions should be considered. Decision on additional solutions need to be made no later that in RAN4 #107.. - Action Request to RAN2: RAN4 requests RAN2 to design corresponding signaling for “report of L3 measurement result” after SCell activation command for unknown FR2 SCell activation enhancement, and it’s up to RAN2’s decision on which layer’s signaling shall be used.

Issue 1-1-5: FFS on content in valid L3 measurement report for unknown FR2 SCell activation enhancement

Agreement:

- L3 measurement result with SSB index

Issue 1-2-1: Beam sweeping factor enhancement in L3 and L1 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

Agreement:

- For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement.

  - if UE has full set (N=8) of beam sweeping during AGC settling part during FR2 unknown SCell activation procedure.

    - Introduce beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement

    - Beam sweeping factor capability X1/X2 are two independent capabilities

- Note: above enhancement only applies for FR2 unknown SCell activation enhancement

- Note: How to capture in spec can be discussed in CR stage

- The candidate values for X1/X2 are FFS

Issue 1-4-1: For FR1 Scell activation enhancement, FFS the feasibility of configuring QCL type C information from FR1 inter-band active serving cell

Agreement:

    - pause discussion in R18 eFeRRM WI for Scell activation enhancement utilizing inter-band information

    - whether or how to reuse conclusions from NES WI for FR1 Scell activation delay reduction is FFS

    - the completion of R18 eFeRRM shall not be delayed/impacted by the progress of feature of utilizing inter-band active serving cell information to the target FR1 Scell activation

Issue 1-4-2: RS related enhancement of FR1 unknown SCell activation

Agreement: Up to the conclusion of issue 1-4-1.

Issue 1-5-1: whether to use SSB periodicity instead of SMTC periodicity for FR2 unknown Scell activation

Agreement (GTW, Tuesday, Apr 25, 2023)

- FFS whether to use SSB periodicity instead of SMTC periodicity for FR2 unknown Scell activation

  - Option 1:

    - Do not change the definition/usage of SMTC periodicity for FR2 unknown Scell activation requirements.

  - Option 2:

    - For enhanced unknown FR2 Scell activation requirement, RAN4 to use SSB periodicity instead of SMTC periodicity.

    - Such enhancement is only applied for FR2 unknown SCell activation.

Issue 2-4-1: Other enhancement for L1 part

Agreement:

- The L1-RSRP reporting based on a smaller beam sweeping factor shall fulfil the performance requirements as specified in TS38.133 clause 10.1.20.1.

- Whether and how to capture above bullet into the spec can be discussed during CR stage.

RRM core requirements for FR1-FR1 NR-DC

#### Issue 1-1-2: Requirements for RACH-less SCG activation (Tsearch)

GTW Agreements:

Requirements for RACH-less SCG activation (Tsearch)

  - Define requirements for cases when

    - 4) Target PSCell is known and TCI is known

  - Do not define requirements for other cases

    - 1) Target PSCell is unknown but TCI is known

    - 2) Target PSCell is unknown and TCI is unknown

    - 3) Target PSCell is known but TCI is unknown

Agreement:

For the case 4) when Target PSCell is known and TCI is known, requirements for RACH-less SCG activation for FR1+FR1 NR-DC can be:

- If RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell.

#### Issue 1-1-3: Requirements for RACH based PSCell activation

Agreement:

- For RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms.

RAN4#107 meeting

The following tdocs have been agreed in RAN4#107 meeting:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2310080 | Big CR for R18 RRM enhancement- FR1+FR1 NR-DC | OPPO |
| R4-2310062 | Ad-hoc minutes for NR RRM enhancements | Apple |
| R4-2310081 | WF on NR RRM enhancements – FR2 SCell activation | Apple |
| R4-2310083 | LS on FR2 SCell activation enhancements | Apple |
| R4-2310167 | WF on NR RRM enhancements – FR1-FR1 NR DC | OPPO |

RRM core requirements for FR2 SCell activation delay reduction

Issue 0-1: 	Whether FR2 SCell activation enhancement in topic #1/2/3 is appliable for direct SCell activation and PUCCH SCell activation

- Agreement:

  - Descope the L3 reporting based FR2 unknown SCell activation enhancement for direct SCell activation in R18.

  - The enhancement under discussion except the L3 reporting based enhancement can also apply  to direct SCell activation.

  - The enhancement under discussion can also apply to other single PUCCH SCell activation

Issue 0-3: 	whether to consider SCell activation requirements for FR2-2

- Agreement:

Descope the SCell activation enhancement in FR2-2 in R18.

Issue 1-1-1: Additional solutions of report L3 measurement results for unknown FR2 SCell activation enhancement (previous issue 1-1-1 in R4-2306315)

- Agreement:

  - Do not consider additional solution for report L3 measurement results for unknown FR2 SCell activation enhancement in R18 efeRRM WI.

Issue 1-1-2: waiting RAN2 conclusions for when/how/what to report L3 measurement results for unknown FR2 SCell activation enhancement (previous issue 1-1-1, 1-1-2, 1-1-3, 1-1-4, 1-1-5 in R4-2306315), except the FFS for additional solution in issue 1-1-1

- Agreement:

  - RAN4 to wait for RAN2 conclusion on triggering/configuration/reporting, and there is no need to have further discussion in RAN4.

Issue 1-1-3: If measurement results are available, the UE will report them to the NW. How to determine the measurement result is available?

- Agreement:

  - The report of L3 measurement result after SCell activation command needs to be valid.

  - The report of L3 measurement result is considered as valid only if it fulfils the measurement requirement for a deactivated SCell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2) and accuracy requirements in TS 38.133 Clause 10.

Issue 1-1-4: FFS on necessity of L3 measurement reporting if UE has no valid measurement results?

- Agreement:

  - UE does not need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results.

Issue 1-1-6: FFS: When the valid L3 measurement result with SSB index is reported after SCell activation command, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported.

- Agreement:

  - It is up to NW implementation to choose whether to configure TCI based on L3 report after SCell activation command or based on L1-RSRP as legacy cases.

  - When the valid L3 measurement result with SSB index is reported after SCell activation command and NW configures TCI based on L3 report (i.e., before UE reported L1 measurements), the activation delay requirements is adjusted as follows

    - L3 measurement delay component (TFirstSSB_MAX + 15*TSMTC_MAX+8*Trs) is removed

    - L1 measurement delay component is removed

  - When the valid L3 measurement result with SSB index is reported after SCell activation command, and UE reported L1 measurement result before NW configures TCI to this UE, the activation delay requirements is adjusted as follows

    - Legacy AGC settling delay component (TFirstSSB_MAX + 15*TSMTC_MAX) is removed

    - FFS: whether L3 cell search/measurement component(e.g., 8*Trs or X1*Trs) is removed

Issue 1-2-1: can X1(Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation) be zero? (not related with WI of FR2 multi-Rx chain DL reception)

- Agreement:

  - X1=1 is one of the candidate values for beam sweeping factor in L3 part for FR2 unknown SCell activation enhancement.

  - X1 is greater than 0 and less than 8.

Issue 1-2-2: Beam sweeping factor enhancement in L3 and L1 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

- Agreement:

  - X1= {1,2,4,6}

  - X2= {0,1,2,3,4,5,6,7}

  - If X1 is absent, beam sweeping factor for cell detection part is 8

  - If X2 is absent, beam sweeping factor for SSB-based L1 measurement is 8

Issue 2-1-1: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation (for the case when no valid L3 measurement result is reported after SCell activation command)?

- Agreement:

  - Up to the conclusion in issue 1-2-2

Issue 2-1-2: if L1-RSRP measurement can be skipped in issue 2-1-1, whether to skip L1-RSRP reporting or TCI indication of FR2 unknown SCell activation (for the case when no valid L3 measurement result is reported after SCell activation command)?

- Agreement:

  - L1-RSRP reporting cannot be skipped. It is needed to inform network the beam information of the SCell.

Issue 2-3-1: Aperiodic RS for TFineTiming during FR2 unknown SCell activation (this AP-RS is not QCLed to a RS on inter-band serving cell)

- Agreement:

  - Do not consider above enhancements in R18 unknown SCell activation enhancement.

Issue 3-1-1: Scheduling enhancement for FR2 unknown SCell activation

- Agreement:

  - The early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed.

  - UE is allowed to be scheduled with data transmission after UE completes the TCI activation.

  - Above clarification will not be captured in the spec.

RRM core requirements for FR1-FR1 NR-DC

Issue 1-1-1: UE behaviour when UE detected either BFD or RLF on the deactivated PSCell for RACH-less SCG activation

- Agreement:

No requirement shall be defined for this case when UE detected either BFD or RLF on the deactivated PSCell during RACH-less SCG activation.

Issue 1-1-2: UE report on the beam failure if Beam failure has been declared or TCI become unknown during SCG activation procedure

- Agreement in AdHoc (May 23rd):

  - FFS

    - Option 1: RAN4 assumes whether to report beam failure to the network can be up to UE implementations if beam failure has been detected during SCG activation procedure.

    - Option 2: RAN4 assumes UE should not indicate beam failure to the network if beam failure has been detected during SCG activation procedure.

  - FFS whether LS is needed to check with RAN2 on the indication for above case.

- Agreement in RRM session (May 24th):

  - Update ad-hoc agreement as follows:

    - Do not specify UE behavior in RAN4 specifications

#### 2.4.2	Remaining Open issues

As summarized in approved way forward R4-2310081 and R4-2310167 (open options in the way forward).

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #106-bis-e meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2304147 | On L3 part enhancement for FR2 SCell activation | Apple |
| R4-2304148 | On L1 part enhancement for FR2 SCell activation | Apple |
| R4-2304149 | On other potential enhancement for FR2 SCell activation | Apple |
| R4-2304150 | On RRM requirements for FR1-FR1 NR-DC | Apple |
| R4-2304177 | L3 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2304178 | L1 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2304179 | Other enhancements on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2304240 | Discussion on L3 part enhancement for FR2 SCell activation | Intel Corporation |
| R4-2304241 | Discussion on L1 part enhancement for FR2 SCell activation | Intel Corporation |
| R4-2304377 | Discussion on L3 part enhancement for FR2 SCell activation | Qualcomm Incorporated |
| R4-2304378 | Discussion on L1 part enhancement for FR2 SCell activation | Qualcomm Incorporated |
| R4-2304379 | Discussion on other enhancement for FR2 SCell activation | Qualcomm Incorporated |
| R4-2304380 | DraftCR applicability Rule for FR1+FR1 | Qualcomm Incorporated |
| R4-2304381 | DraftCR Scheduling availability for FR1+FR1 | Qualcomm Incorporated |
| R4-2304620 | Discussion on L3 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2304621 | Discussion on L1 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2304622 | Draft CR for FR1-FR1 NR-DC on scheduling availability of UE during candidate beam detection | MediaTek inc. |
| R4-2304696 | DraftCR on scheduling availability during L1-RSRP measurement for FR1-FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2304697 | discussion on FR1-FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2304704 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | LG Electronics Inc. |
| R4-2304705 | Discussion on L1 enhancement for FR2 SCell activation delay reduction | LG Electronics Inc. |
| R4-2304761 | Discussion on L3 part enhancement for FR2 SCell activation | Xiaomi |
| R4-2304762 | Discussion on L1 part enhancement for FR2 SCell activation | Xiaomi |
| R4-2304773 | Discussion on RRM core requirements for FR1-FR1 NR-DC | Xiaomi |
| R4-2304774 | Draft CR on HO with PSCell requirements for FR1-FR1 NR-DC | Xiaomi |
| R4-2304806 | Discussion on L3 aspects of FR2 SCell activation enhancements | vivo |
| R4-2304807 | Discussion on L1 aspects of FR2 SCell activation enhancements | vivo |
| R4-2304808 | LS on the measurement and report mechanism for SCell activation delay reduction | vivo |
| R4-2304842 | Discussion on L3 part enhancement for FR2 SCell activation | CMCC |
| R4-2304843 | Discussion on L1 part enhancement for FR2 SCell activation | CMCC |
| R4-2304897 | Discussion on FR1-FR1 NR-DC RRM requirements | Ericsson |
| R4-2304898 | draft CR for Rel-18 eFeRRM NR-DC FR1-FR1 SCG activation | Ericsson |
| R4-2304987 | Discussion on other aspects of RRM requirements enhancement for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2304988 | Discussion on the L1 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2304989 | Discussion on the L3 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2305021 | Discussion on L3 part enhancement for FR2 SCell activation | China Telecom |
| R4-2305022 | Discussion on L1 part enhancement for FR2 SCell activation | China Telecom |
| R4-2305056 | Discussion on RRM core requirements for FR1-FR1 NR-DC | vivo |
| R4-2305057 | Draft CR on PSCell addition/release delay for FR1+FR1 NR-DC | vivo |
| R4-2305168 | Discussion on L1 part enhancement for FR2 SCell activation | NTT DOCOMO, INC. |
| R4-2305232 | Discussion on L3 enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2305233 | Discussion on L1 enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2305234 | Discussion on other enhancements for FR2 Scell activation delay reduction | OPPO |
| R4-2305235 | Discussion on remaining issues of RRM requirements for FR1+FR1 NR-DC | OPPO |
| R4-2305236 | Draft CR on CSSF requirements for FR1+FR1 NR-DC | OPPO |
| R4-2305237 | Draft CR on scheduling availability of UE during beam failure detection for FR1+FR1 NR-DC | OPPO |
| R4-2305238 | Draft CR on scheduling availability of UE during intra-frequency measurements for FR1+FR1 NR-DC | OPPO |
| R4-2305264 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2305265 | Discussion on L1 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2305266 | Discussion on FR2 SCell activation enhancement | Huawei, HiSilicon |
| R4-2305267 | Discussion on NR-DC requirements in Rel-18 | Huawei, HiSilicon |
| R4-2305756 | On FR2 SCell activation delay reduction: L3 aspects | Ericsson |
| R4-2305757 | On FR2 SCell activation delay reduction: L1 RSRP | Ericsson |
| R4-2305758 | On FR2 SCell activation delay reduction: Others | Ericsson |
| R4-2306152 | Topic summary for [106-bis-e][204] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2306153 | Topic summary for [106-bis-e][205] NR_RRM_enh3_part2 | Moderator (OPPO) |
| R4-2306235 | Topic summary for [106-bis-e][204] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2306236 | Topic summary for [106-bis-e][205] NR_RRM_enh3_part2 | Moderator (OPPO) |
| R4-2306315 | WF on R18 RRM enhancement (part 1) | Apple |
| R4-2306316 | Topic summary for [106-bis-e][204] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2306321 | LS on FR2 unknown SCell activation enhancement | Apple |
| R4-2306322 | WF on R18 RRM enhancement - FR1+FR1 NR-DC | OPPO |
| R4-2306323 | DraftCR Scheduling availability for FR1+FR1 | Qualcomm Incorporated |
| R4-2306324 | Draft CR for FR1-FR1 NR-DC on scheduling availability of UE during candidate beam detection | MediaTek inc. |
| R4-2306325 | DraftCR on scheduling availability during L1-RSRP measurement for FR1-FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2306326 | Draft CR on HO with PSCell requirements for FR1-FR1 NR-DC | Xiaomi |
| R4-2306327 | draft CR for Rel-18 eFeRRM NR-DC FR1-FR1 SCG activation | Ericsson |
| R4-2306328 | Draft CR on scheduling availability of UE during intra-frequency measurements for FR1+FR1 NR-DC | OPPO |
| R4-2306329 | Draft Big CR for R18 RRM enhancement - FR1+FR1 NR-DC | OPPO |

RAN4 #107 meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2307320 | On L3 part enhancement for FR2 SCell activation | Apple |
| R4-2307321 | On L1 part enhancement for FR2 SCell activation | Apple |
| R4-2307322 | On other potential enhancement for FR2 SCell activation | Apple |
| R4-2307323 | On RRM requirements for FR1-FR1 NR-DC | Apple |
| R4-2307356 | L3 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2307357 | L1 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2307358 | Other enhancements on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2307464 | Discussion on L3 part enhancement for FR2 SCell activation | NTT DOCOMO, INC. |
| R4-2307465 | Discussion on L1 part enhancement for FR2 SCell activation | NTT DOCOMO, INC. |
| R4-2307560 | Discussion on L3 part enhancement for FR2 SCell activation | CMCC |
| R4-2307561 | Discussion on L1 part enhancement for FR2 SCell activation | CMCC |
| R4-2307710 | Discussion on RRM core requirements for FR1-FR1 NR-DC | vivo |
| R4-2307809 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | LG Electronics Inc. |
| R4-2307944 | Discussion on L3 part enhancement for FR2 SCell activation | Xiaomi |
| R4-2307945 | Discussion on L1 part enhancement for FR2 SCell activation | Xiaomi |
| R4-2307957 | Discussion on RRM core requirements for FR1-FR1 NR-DC | Xiaomi |
| R4-2308016 | Discussion on L3 part enhancement for FR2 SCell activation | China Telecom |
| R4-2308017 | Discussion on L1 part enhancement for FR2 SCell activation | China Telecom |
| R4-2308212 | Discussion on L3 aspects of FR2 SCell activation delay reduction | vivo |
| R4-2308213 | Discussion on L1 aspects of FR2 SCell activation delay reduction | vivo |
| R4-2308214 | Discussion on aspects related to multiple SCell activation for FR2 SCell activation delay reduction | vivo |
| R4-2308316 | Discussion on general aspects for SCell activation delay reduction | Huawei, HiSilicon |
| R4-2308317 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2308318 | Discussion on L1 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2308319 | Discussion on other FR2 SCell activation enhancement | Huawei, HiSilicon |
| R4-2308320 | Discussion on NR-DC requirements in Rel-18 | Huawei, HiSilicon |
| R4-2308480 | On L3 enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2308481 | On L1 enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2308482 | On other enhancements for FR2 Scell activation delay reduction | OPPO |
| R4-2308483 | On remaining issues of RRM requirements for FR1+FR1 NR-DC | OPPO |
| R4-2308484 | Big CR for R18 RRM enhancement- FR1+FR1 NR-DC | OPPO |
| R4-2308717 | Discussion on other aspects of RRM requirements enhancement for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2308718 | Discussion on the L1 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2308719 | Discussion on the L3 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2308819 | Discussion on FR1-FR1 NR-DC RRM requirements | Ericsson |
| R4-2308820 | draft CR to 38.133 for rel-18 NRDC remaining issues | Ericsson |
| R4-2309122 | Discussion on FR1-FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2309430 | Discussion on remaining issues on L3 part enhancements for FR2 Unknown SCell activation | Qualcomm Incorporated |
| R4-2309431 | Discussion on remaining issues on L1 part enhancements for FR2 Unknown SCell activation | Qualcomm Incorporated |
| R4-2309432 | Discussion on remaining issues on other enhancements for FR2 Unknown SCell activation | Qualcomm Incorporated |
| R4-2309555 | Discussion on L3 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2309556 | Discussion on L1 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2309557 | Discussion on other potential enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2309587 | On FR2 SCell activation delay reduction: L3 aspects | Ericsson |
| R4-2309588 | On FR2 SCell activation delay reduction: L1 RSRP | Ericsson |
| R4-2309589 | On FR2 SCell activation delay reduction: Others | Ericsson |
| R4-2309954 | Topic summary for [107][209] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2309955 | Topic summary for [107][210] NR_RRM_enh3_part2 | Moderator (OPPO) |
| R4-2310062 | Ad-hoc minutes for NR RRM enhancements | Apple |
| R4-2310079 | draft CR to 38.133 for rel-18 NRDC remaining issues | Ericsson |
| R4-2310080 | Big CR for R18 RRM enhancement- FR1+FR1 NR-DC | OPPO |
| R4-2310081 | WF on NR RRM enhancements – FR2 SCell activation | Apple |
| R4-2310082 | WF on NR RRM enhancements – FR1-FR1 NR DC | OPPO |
| R4-2310083 | LS on FR2 SCell activation enhancements | Apple |

RAN2#122 meeting

| Tdoc | Title | Source |
| --- | --- | --- |
| R2-2304636 | LS on FR2 unknown SCell activation enhancement (R4-2306321; contact: Apple) | RAN4 |
| R2-2305406 | Discussion on FR2 unknown SCell activation enhancement | ZTE Corporation, Sanechips |
| R2-2305428 | Discussion on FR2 unknown SCell activation enhancement | vivo |
| R2-2305476 | Measurement reporting for FR2 unknown SCell | LG Electronics Inc. |
| R2-2306017 | Discussion on FR2 unknown SCell activation enhancement | Ericsson |
| R2-2306164 | FR2 SCell Enhancement | Apple |
| R2-2306165 | Draft LS reply on on FR2 unknown SCell activation enhancement | Apple |
| R2-2306190 | Discussion on RAN4 LS on FR2 unknown SCell activation enhancement | Huawei, HiSilicon |
| R2-2306368 | Scell activation and L3 reporting | Nokia, Nokia Shanghai Bell |
| R2-2306823 | Draft LS reply on FR2 unknown Scell activation enhancement | Apple |
| R2-2306863 | LS reply on FR2 unknown Scell activation enhancement | RAN2 |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template