3GPP TSG RAN Meeting #100									              RP-231156

Taipei, Jun 12-14, 2023

Status Report to TSG

Agenda item:			9.3.3.1

| WI / SI Name | # Further enhancement of data collection for SONMDT in NR standalone and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |
| Unique ID | 941007 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221825 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 12/2023 | Performance part: mm/yyyy | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx% | Core part: 60% | Performance Part: xx% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Liang LIU |
|  | Company | CMCC |
|  | Email | liuliang@chinamobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The change of the target completion date is due to postponement of Stage-3 functional completion to June 2022.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2 #121bis-e meeting

- MRO for inter-system handover for voice fallback

Agreements:

1	RAN2 to support the scenario of “after RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell, which is agreed in RAN3”.

2	FFS: Introduce an indication for the scenario of RLF after successful voice fallback HO in the LTE RLF report regarding voice fallback.

3	UE logs the agreed indication regarding voice fallback in the NR RLF report.

4	FFS: RAN2 agree to differentiate an acceptable E-UTRA cell from a suitable E-UTRA cell in the RLF report in case of voiceFallback HOF. FFS explicit or implicit indications.

- RACH enhancement

=>	1	FFS: Include the actual number of msg3 repetitions in RA report.

2	FFS: Include NSAG priority in RA report.

3b	FFS: UE reports NSAG IDs which are associated with the S-NSSAI(s) that triggered the random access attempt or NSAG IDs which associated with the S-NSSAI(s) triggering the access attempt and that are included in SIB1.

3c	FFS: Include S-NSSAI(s) in RA report.

RAN2 #122 meeting

- Running CR

R2-2305986	Running CR for Rel-18 SON MRO

=>	use as the baseline for further running CR construction.

R2-2306753: Running 38.331 CR for logged MDT enhancements and NPN

=>	use as the baseline for further running CR construction.

R2-2306531	Running 38.331 CR for RACH report

=>	use as the baseline for further running CR construction.

R2-2306754: Running 36.331 CR for logged MDT enhancements

=>	use as the baseline for further running CR construction.

R2-2306530	Running 36.331 CR for SN RACH report

=>	use as the baseline for further running CR construction.

- SON for NR-U

Agreements:

1	Only the preamble transmission attempts for which LBT was successful are represented in the “per RA attempt info list” for a given beam.

2	On how to represent the preamble transmission attempts blocked by LBT,

Introduce a field (or reusing the existing field) that counts the number of preamble transmissions blocked by LBT per RA procedure, and a flag indicating transmission failures experienced right before beam switching. Details can FFS.

3	For the RA-Report, the enhancements on the handling of the “per RA attempt info list” (i.e. as per Proposal 1) apply only to the last RA procedure in the last BWP prior to the random access success.

4	For the other BWPs in which the UE experienced the consistent LBT failure, the UE logs in the RA-InformationCommon:

a.	The locationAndBandwidth information of the BWP

b.	The subcarrierSpacing information of the BWP

c.	The absoluteFrequencyPointA information of the BWP ( How to log once for all the BWPs of the cell is FFS)

5	As baseline, RAN2 assumes the following:

a.	Enhancements discussed for the RA-InformationCommon for the RA-Report are applicable also to the RLF-Report

b.	The detailed “per RA attempt info” are only reported in the RLF-Report for the last RA procedure before RLF/HOF, FFS whereas limited information are reported for the other BWPs in which consistent LBT failure is detected

c.	The above bullets may be revisited case by case depending on future agreements.

6	The UE logs RA-InformationCommon including LBT info in the RLF-Report, in case of HOF and when the RLF cause is randomAccessProblem or beamFailureRecoveryFailure (as in legacy).

7	The UE logs the available RSSI measurement in the RLF-Report. FFS in which case.

8	The UE should log the following RSSI values in the RLF-Report:

a.	For RLF, the latest measured RSSI of the NR-U channel of the last serving cell if measRSSI-ReportConfig is configured for the corresponding frequency.

b.	FFS: For HOF, the latest measured RSSI of the NR-U channel of the source cell, and  the latest measured RSSI of the NR-U channel of the target cell, if measRSSI-ReportConfig is configured for the corresponding frequency.

- MRO for inter-system handover for voice fallback

Agreements:

1	Introduce a new indication in the LTE RLF report for the case an RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback.

2	UE to log the time until reconnection during RRC connection establishment to the acceptable cell and reconnection cell ID in is absent, which will reuse the legacy field.

- MDT override

Agreements:

1	In NR, considering UE capability, UE reports availability of signalling based logged MDT configuration without checking the RAT information.

2	RAN2 confirms that sigLogMeasConfigAvailable can be re-used for to indicate the availability of the LTE signalling based logged MDT in NR.

- RACH enhancement

Agreements:

RACH Partitioning

1	RAN2 confirms agreed “used feature combination” is all the features configured in the FeatureCombination applied for the RACH procedure.

2	Feature specific RACH information is included in RA-InformationCommon and is also included for RLF report and CEF report.

Msg3 repetition

3	Not include the number of Msg3 repetition applied in RACH procedure in RA report.

SN RACH Report

4	When reporting SN NR RA-report to LTE BS, the unique PSCell identities (i.e. if a PSCell occurs more than once in NR RA-ReportList, it is recorded only once in the list of PSCell identities) are included outside the NR RA report container.

5	Revert the agreement that UE does not support reporting NR RA report to LTE when it is in standalone LTE mode i.e., eNB may fetch the NR RA report irrespective to whether the UE is in single connectivity or dual connectivity.

6	No need to introduce availability bit to notify LTE BS there are available NR RA report for fetching.

7	Enhance the LTE UE information Request procedure with NR RA-Report request flag to fetch the NR RA-Report in LTE.

8	For NR RACH report, UE performs RPLMN checking before sending the NR RACH report to LTE BS.

9	A new UE capability is introduced to indicate whether UE supports NR RACH Report in LTE.

- SON/MDT enhancements for Non-Public Networks

Agreements:

1	Include CAG ID(s) in the logged MDT area configuration.

- Other

Agreements:

1	RAN2 confirms scenario of near failure fast MCG recovery.

2	RAN2 confirms scenario f1, i.e., SCG fails or is deactivated before the UE sends the MCGFailureInformation. FFS RAN2 impact.

- SHR and SPCR

=>	intra-NR SHR and Inter-RAT SHR from LTE to NR will be deprioritized in RAN2 for R18.

=>	SPR except the critical issues will not be further enhanced from this meeting until the end of R18.

Agreements:

SPR

1	 For values of triggering conditions of SPR, Percentage based threshold variables for SHR (T310/T312/T304) can be reused for SPR is applied.

- Approved LS

R2-2306896	Reply LS on SHR and SPR

=>	Approved

R2-2306848 Reply LS on RACH enhancement

=>	Approved

- Post-meeting email discussions

There are the following post-meeting email discussions:

Note: Deadline Thursday August 10th, 2023, 1000 UTC (unless otherwise stated)

- [Post122][584][R18 SON/MDT]  Open issues on fast MCG recovery (CMCC)

Scope: Discussion should focus on the proposals raised and not concluded in R2-2305779.

Intended outcome: Report

Deadline: Long

- [Post122][555][R18 SON/MDT] Running CR for Rel-18 SON MRO (Ericsson)

Scope: Use R2-2305986 as baseline to continue the running 38.331CR for R18 SON MRO. If impact on 36.331 is identified, also provide corresponding running 36.331 CR.

Intended outcome: Running CR baselines for R18 SON MRO

Deadline: Long

- [Post122][556][R18 SON/MDT] Running CR for Rel-18 for logged MDT enhancements and NPN (Huawei)

Scope: Use R2-2306753 and R2-2306754 as baselines to continue the running 38.331CR and 36.331 CR for R18 logged MDT enhancements and NPN.

Intended outcome: Running CRs baseline for R18 logged MDT enhancements and NPN

Deadline: Long

- [Post122][557][R18 SON/MDT] Running CR for Rel-18 SON on RACH report (ZTE)

Scope: Use R2-2306531 and R2-2306530 as baselines to continue the running 38.331CR and 36.331 CR for R18 SON on RACH report

Intended outcome: Running CRs baseline for R18 SON on RACH report

Deadline: Long

- [Post122][590][R18 SON/MDT] Open issues of SON NR-U (Ericsson)

Scope: The above issues which marked as FFS.

Intended outcome: Report which is assumed to have the consensus on how to handle these issues.

Deadline: Long

2.2.2	Remaining Open issues

- Support of data collection for SON features, including, MRO for MR-DC SCG failure scenario, and MRO enhancement for inter-system handover voice fallback,

- Specification of the UE reporting necessary to enhance the mobility parameter tuning [RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces    
[RAN3]

- Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PScell change report

- Successful Handover Report (e.g. inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

- Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

2.3	RAN3

2.3.1	Agreements

RAN3 #119bis-e meeting：

- BL CR

R3-231125, Endorsed as BL CR

R3-231126, Endorsed as BL CR

R3-231127, Endorsed as BL CR

R3-231906, Endorsed as BL CR

R3-231129, Endorsed as BL CR

- SHR and SPR

2.1 	Inter-RAT SHR

If a different NR node (different from source NR node) retrieves the SHR collected during an inter-RAT HO (NR to LTE), reuse ACCESS AND MOBILITY INDICATION message (over XnAP and F1AP) and Uplink/Downlink RAN configuration transfer procedures (over NGAP) to forward the SHR to the source NR node

No further discussion in RAN3 on above RACH related information.

Support collection of SHR during successful inter-RAT HO (LTE to NR) for T304 trigger without any LTE impacts in R18, if the following principles are used. Send LS to RAN2 to confirm the first 4 bullets:

- Target gNB can send SHR configuration (T304 trigger) to UE via NR container (targetRAT-MessageContainer) in MobilityFromEUTRACommand

- UE stores this SHR configuration in NR format

- If T304 trigger is met, UE records SHR in NR format

- UE reports this SHR to only an gNB (either the target gNB or another gNB)

- gNB retrieving this SHR can forward this SHR to the target gNB for SHR optimizations

The SHR collected during inter-RAT HO (LTE to NR) should include at least Source LTE cell and Target NR cell (assuming RAN2 confirms no LTE impacts based on the principles in Proposal 4)

In case the SHR collected during an intra-NR HO is retrieved in a NR node different from source/target NR node, the receiving node performs initial analysis (identifies the node(s) to which the SHR is to be forwarded) and forwards the SHR to the corresponding node(s) which generates the SHR trigger condition that triggered the SHR (i.e., Option 3 is agreed)

TP to 38.473: R3-232137 Agreed

TP to 38.423: R3-232002 Agreed

TP to 38.413: R3-232021 Agreed

LS to RAN2 on intra-system inter-RAT SHR and on SPR in R3-232140 Agreed unseen

2.2 	SPR

WA: The triggers for SPR should be represented in terms of percentage values (similar to SHR)

In case the SPR is retrieved in a “new node” (different from the node that sent the SPR configuration to the UE i.e., “old MN”), the SPR is always sent from the “new node” to the “old MN” which then forwards to the respective node(s) which should perform the SPR optimization.

To assist in the forwarding of SPR, UE may include the following in SPR

- CGI of the PCell which sent the SPR configuration (presence of this IE is to be discussed)

- WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism)

Reuse ACCESS AND MOBILTY INDICATION to forward SPR over XnAP and F1AP and use Uplink/Downlink RAN Configuration Transfer for forwarding SPR over NGAP

To identify the UE context in the old source SN/old target SN when SPR is forwarded by old MN for SPR optimization, old MN identifies the UE context and sends the stored respective SN Mobility Information together with SPR to the old source SN/old target SN

If the trigger is T312/T310, the objective of SPR is to

- optimize PSCell change configuration and associated mobility thresholds

- optimize lower layer issues of source PSCell (e.g., optimize T310/T312 timer values)

Further, T310/T312 related SPR triggers can also be optimized to ensure UE doesn’t unnecessarily collect SPR or only rarely collects SPR

Irrespective of option 1/2/3, in case SPR is collected during MN-initiated PSCell change, SPR optimizations are done in both MN and source SN

- MN is responsible to optimize PSCell change configuration and associated mobility thresholds

- Source SN is responsible to optimize lower layer issues (e.g., optimize T310/T312 timer values)

In case of SN initiated PSCell change,

- Source SN may send the SN Mobility Information to MN via SN Change Required message  (already supported by existing specifications)

- Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message

- If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP

In case of MN initiated PSCell change,

- Source SN may send the SN Mobility Information to MN via SN Release Request Acknowledge

- Target SN may send the SN Mobility Information to MN in SN Addition Request Acknowledge message

- If received, MN stores the SN Mobility Information of both source SN and target SN and sends it to the “node performing SPR optimization” along with SPR in ACCESS AND MOBILITY INDICATION over XnAP

- MRO

3.1	MRO for CPC

Definitions of MRO events for CPAC will be introduced in TS 37.340 in a new chapter (based on [1226], CPA needs to be added).

During CPAC configuration, the value of the Time Stay IE for the source PSCell UHI, sent in S-NODE ADDITION REQUEST message, does not reflect the exact time the UE stayed in the source PSCell.

TP to 37.340: R3-232062 Agreed

3.2	MRO for the fast MCG recovery

Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed.

Scenario ‘a’ is redefined: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running).

3.3	MRO for inter-system handover for voice fallback

Stage-2 description of the detection mechanism will be introduced in TS 38.300 (based on R3-231585).

TP to 38.300: R3-232151 Agreed

- RACH enhancements

Only gNB-CU UE F1AP ID is included in the RACH INDICATION message.

The Random Access Indication IE in the RACH INDICATION message is not needed.

The criticality of the RACH indication list IE in the RACH INDICATION message is “reject”.

To align the naming of RA report with RAN2 spec in TS 38.300, TS 38.401, TS 38.423 and TS 38.473.

To use “RA report” in TS 38.300, TS 38.401, TS 38.423 and TS 38.473.

TP to 38.423: R3-232142 Agreed unseen

TP to 38.300: R3-232099  Agreed

TP to 38.473: R3-232141 Agreed unseen

TP to TS 38.401: R3-232101 Agreed

TP to TS 38.423: R3-232143 Agreed unseen

TP to TS 38.473: R3-232103 Agreed

TP to TS 36.423: R3-232104 Agreed

Rely LS on RACH enhancement for R18 SONMDT to RAN2:  R3-232144  Agreed unseen

- SON/MDT Enhancements for Non-Public Networks

How to introduce NID or PNI-NPN ID into SON/MDT reports depends on RAN2’s decision.

It is up to configuration and operator’s policies whether PN and NPN information can be included in the UE History Information.

TP to 38.423: R3-232091 Agreed

TP to 38.413: R3-232027 Agreed

LS to RAN2 on support of separate MDT reports for public and non-public networks in R3-232118 Agreed

- SON for NR-U

Enhancements of RLF reports and RA reports are beneficial to separate mobility related errors from the LBT-related ones.

RLF Report and RA report can be enhanced to include information concerning the LBT failures in RA procedures, the granularity and implementation details needs to be further discussed based on progress in RAN2.

WA: a gNB sends in resource status reporting via Xn an EDT UL that reflects at least the maximum EDT UL configured for the UEs.

The presence of COT percentage UL in F1 is optional.

TP to 38.423: R3-232067 Agreed

TP to 38.473: R3-232068 Agreed

- MDT Enhancements

RAN3 confirms that the scenarios for inter-RAT signalling based logged MDT protection includes the following:

Scenario 1: Inter-system inter-RAT: EPC –> 5GC

Scenario 2: Intra-system Inter-RAT and intra-5GC: LTE –> NR

LS to SA5 on MDT measurements collection in MR-DC in R3-232070 Agreed

RAN3 #120 meeting：

- BL CR

R3-232524, Endorsed as BL CR

R3-232525, Endorsed as BL CR

R3-232526, Endorsed as BL CR

R3-232527, Endorsed as BL CR

R3-232528, Endorsed as BL CR

R3-232554, Endorsed as BL CR

R3-232555, Endorsed as BL CR

R3-232556, Endorsed as BL CR

R3-232568, Endorsed as BL CR

- SHR and SPR

2.1 	Inter-RAT SHR

TPs to 38.300: R3-233399 Agreed

LS to RAN2 on inter-RAT SHR and SPR in R3-233380 Agreed

2.2 	SPR

If SPR available indication via SN RRCReconfigComplete is received by SN, SN should inform MN that an SPR is available at the UE e.g.., a new IE can be added in S-NODE MODIFICATION REQUIRED message.

Turn the following WA to agreement:

WA: The triggers for SPR should be represented in terms of percentage values (similar to SHR).

TPs to 37.340: R3-233379 Agreed

TPs to 38.300: R3-233408 Agreed

- MRO

3.1	MRO for the fast MCG recovery

Agree to define case c in R18. Solution can be further discussed.

3.2	MRO for inter-system handover for voice fallback

WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.

TP to 36.300: R3-233396 Agreed

TP to 38.300: R3-233397 Agreed

- RACH enhancements

Not consider the NG and S1 forwarding of RA report.

Include the “PSCell list” optionally in addition to the “RACH Report Container” in ACCESS AND MOBILITY INDICATION over Xn.

New BL CR assignment:

- Assign BL CR for 38.420 to Qualcomm Incorporated

- Assign BL CR for 38.470 to CMCC

TP to 37.340: R3-233456  Agreed

TP to 38.470: R3-233498 Agreed unseen

TP to 36.300: R3-233450 Agreed

TP to 36.423: R3-233499 Agreed unseen

TP to 38.423: R3-233500 Agreed unseen

- SON/MDT Enhancements for Non-Public Networks

Set the range of maxnoofCAGforMDT to 256

Add separate SNPN Cell Based MDT and SNPN TAI Based MDT area scope

No need to introduce user consent of SNPN over interfaces.

Add multi-SNPN MDT area scope.

TP to 38.413: R3-233470 Agreed

TP to 38.423: R3-233452  Agreed

- SON for NR-U

There is no need for the UE to report the average of the applied EDT UL in RLF report.

The EDT in UL included in the XnAP RESOURCE STATUS UPDATE message reflects the average of the configured maximum EDT UL. TP for XnAP is in R3-233491.

WA: introduce an optional load metric on Radio Resource Status per NR-U Channel in XnAP RESOURCE STATUS UPDATE message and in F1AP RESOURCE STATUS UPDATE message.

TP to 38.423: R3-233491 Agreed

- MDT Enhancements

There is no requirement that NR signalling based logged MDT has a lower priority than E-UTRAN signalling based logged MDT.

2.3.2	Remaining Open issues

- Support of data collection for SON features, including, MRO for MR-DC SCG failure scenario, and MRO enhancement for inter-system handover voice fallback,

- Specification of the UE reporting necessary to enhance the mobility parameter tuning [RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces    
[RAN3]

- Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PScell change report

- Successful Handover Report (e.g. inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

- Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

RAN2 #121bis-e:

R2-2302423	LS on MRO for CPC and CPA and fast MCG recovery (R3-230992; contact: Huawei)	RAN3

R2-2302452	Reply LS on user consent of Non-public Network (S3-231399; contact: Vodafone)	SA3

R2-2302613	Consideration on Inter-system Handover for Voice Fallback	CATT

R2-2302614	RACH enhancement for SON	CATT

R2-2302856	RA report retrieval	Nokia, Nokia Shanghai Bell

R2-2302857	Discussion on SON for NR-U	Nokia, Nokia Shanghai Bell

R2-2302858	Discussion on storing LBT-FailureRecoveryConfig (Reply LS to R2-2300031)	Nokia, Nokia Shanghai Bell

R2-2303113	SON Enhancement for NR-U	CATT

R2-2303143	Consideration on MRO for inter-system handover for voice fallback	ZTE Corporation, Sanechips

R2-2303144	Consideration on NR-U related SON	ZTE Corporation, Sanechips

R2-2303145	Consideration on RACH enhancements	ZTE Corporation, Sanechips

R2-2303182	SON on fast MCG recovery	OPPO

R2-2303183	Further discussion on MRO of inter-system HO voice fallback	OPPO

R2-2303244	MRO for inter-system handover for voice fallback	Lenovo

R2-2303245	Discussion on MRO for NR-U	Lenovo

R2-2303246	Discussion on MRO for CPAC and fast MCG link recovery	Lenovo

R2-2303368	Remaining issues of SON enhancements for RACH	Apple

R2-2303453	MRO for inter-system handover for voice fallback	Ericsson

R2-2303454	RA report enhancement	Ericsson

R2-2303670	SON/MDT enhancements for RACH	Samsung R&D Institute India

R2-2303673	SON/MDT enhancements for NR-U	Samsung R&D Institute India

R2-2303683	MRO for inter-system handover for voice fallback	Samsung R&D Institute India

R2-2303694	Data collection for MRO for inter-system handover for voice fallback	Qualcomm Incorporated

R2-2303695	Discussion on NR-U Related Enhancements	Qualcomm Incorporated

R2-2303783	Discussion on RACH enhancement for SON	China Telecom

R2-2303787	Discussion on CPAC failure report	NTT DOCOMO, INC.

R2-2303798	Further considerations on RACH Enhancement	CMCC

R2-2303799	Further considerations on fast MCG recovery	CMCC

R2-2303803	SONMDT enhancement for NR-U	CMCC

R2-2303804	Discussion on SPR	NTT DOCOMO, INC.

R2-2303806	Consideration on the SON enhancements for RACH report	Beijing Xiaomi Software Tech

R2-2303829	SON enhancement for RA report	Sharp

R2-2303956	Discussion on MRO for inter-system handover for voice fallback	Huawei, HiSilicon

R2-2303957	Discussion on RACH enhancement	Huawei, HiSilicon

R2-2303958	Discussion on SON MDT enhancements for NPN and NR-U	Huawei, HiSilicon

R2-2304031	Discussion on SON for NR-U	Xiaomi

R2-2304111	Enhancements of SON reports for NR-U	Ericsson

R2-2304196	Summary of 7.13.6 RACH enhancement	Apple (moderator)

R2-2304198	[Pre121bis-e][822][SONMDT] Summary of agenda item 7.13.2 on MRO for inter-system handover for voice fallback	CATT

R2-2304200	Summary of AI 7.13.5 SON for NR-U (Ericsson)	Ericsson

RAN2 #122:

R2-2304622	LS on MRO for CPC and CPA and fast MCG recovery (R3-230992; contact: Huawei)	RAN3

R2-2304628	LS on potential override of logged MDT reports upon moving from SNPN to PLMN (R3-232118; contact: Ericsson)	RAN3

R2-2304630	LS on intra-system inter-RAT SHR and SPR (R3-232140; contact: Huawei)	RAN3

R2-2304631	Reply LS on RACH enhancement for R18 SONMDT (R3-232144; contact: Huawei)	RAN3

R2-2304656	Reply LS on user consent of Non-public Network (S3-231399; contact: Vodafone)	SA3

R2-2304930	Consideration on the SON enhancements for RACH report	Beijing Xiaomi Software Tech

R2-2304931	Discussion on the SONMDT enhancement for NPN	Beijing Xiaomi Software Tech

R2-2304932	Considerations on MDT override enhancement for E-UTRAN	Beijing Xiaomi Software Tech

R2-2305070	SON enhancements for RACH	Apple

R2-2305273	Discussion on MDT override protection	LG Electronics

R2-2305324	Remaining issues on SPR	vivo

R2-2305325	Discussion on SON enhancements for NPN	vivo

R2-2305326	Discussion on MRO for CPAC	vivo

R2-2305340	SON on fast MCG recovery	OPPO

R2-2305421	Signalling based logged MDT override protection	Nokia, Nokia Shanghai Bell

R2-2305422	Discussion on SON for inter-RAT SHR	Nokia, Nokia Shanghai Bell

R2-2305423	SPR and SHR related enhancements	Nokia, Nokia Shanghai Bell

R2-2305424	Discussion on SON for NR-U	Nokia, Nokia Shanghai Bell

R2-2305425	Discussion on RACH enhancement for SON	Nokia, Nokia Shanghai Bell

R2-2305426	Discussion on NP related issues in SON/MDT	Nokia, Nokia Shanghai Bell

R2-2305483	Further Consideration on Inter-system Handover for Voice Fallback	CATT

R2-2305484	Further discussion on inter-RAT SHR and SPR	CATT

R2-2305485	SON Enhancement for NR-U	CATT

R2-2305486	RACH enhancement for SON	CATT

R2-2305487	SON and MDT Enhancement for NPN	CATT

R2-2305488	Discussion on Fast MCG recovery and MHI Enhancement	CATT

R2-2305616	Further considerations on RACH Enhancement	CMCC

R2-2305617	SON enhancement for SPR	CMCC

R2-2305647	SON/MDT enhancements for NPN	Samsung R&D Institute India

R2-2305658	SON/MDT enhancements for NR-U	Samsung R&D Institute India

R2-2305660	SON/MDT enhancements for RACH	Samsung R&D Institute India

R2-2305661	SON/MDT enhancements for RACH	Samsung R&D Institute India

R2-2305667	SON/MDT enhancements for SHR and SPR	Samsung R&D Institute India

R2-2305678	MRO for inter-system handover for voice fallback	Samsung R&D Institute India

R2-2305703	MRO for inter-system handover for voice fallback	Lenovo

R2-2305704	Discussion on Successful Handover Report	Lenovo

R2-2305705	SON enhancements for SPR	Lenovo

R2-2305706	Discussion on MRO for NR-U	Lenovo

R2-2305707	SON enhancements for CPAC	Lenovo

R2-2305708	MRO for fast MCG link recovery	Lenovo

R2-2305722	MRO for inter-system handover for voice fallback	Samsung R&D Institute India

R2-2305728	Discussion on SON for NR-U	Xiaomi

R2-2305777	SONMDT enhancement for NR-U	CMCC

R2-2305778	Further consideration on voice fallback	CMCC

R2-2305779	Further considerations on fast MCG recovery	CMCC

R2-2305780	SON MDT enhancement for MR-DC CPAC	CMCC

R2-2305781	MHI Enhancement for SCG Activation Deactivation	CMCC, Ericsson, CATT

R2-2305986	Running CR for Rel-18 SON MRO	Ericsson

R2-2305987	Mobility Robustness Optimization – all topics	Ericsson

R2-2305988	MDT enhancements	Ericsson

R2-2305989	RA report enhancement	Ericsson

R2-2305990	SON support for NPN	Ericsson

R2-2305991	RAN observability issues for DRBs with stringent QoS requirements	Ericsson

R2-2306042	Data collection for MRO for inter-system handover for voice fallback 	Qualcomm Incorporated

R2-2306043	Discussion on NR-U Related Enhancements	Qualcomm Incorporated

R2-2306100	Discussion on RAN2 impacts due to the LS R3-232144	Huawei, HiSilicon

R2-2306101	Discussion on SON for NR-U	Huawei, HiSilicon

R2-2306102	Discussion on RACH enhancement	Huawei, HiSilicon

R2-2306103	Discussion on Fast MCG recovery, CPAC and MDT overide	Huawei, HiSilicon

R2-2306204	SON enhancement for SPR	SHARP Corporation

R2-2306207	SON enhancement for RA report	SHARP Corporation

R2-2306209	Discussion on failure information for CPAC	SHARP Corporation

R2-2306219	MRO for fast MCG recovery	SHARP Corporation

R2-2306245	Consideration on MRO for inter-system handover for voice fallback	ZTE Corporation, Sanechips

R2-2306246	Remaining issues on SHR and SPCR	ZTE Corporation, Sanechips

R2-2306247	Consideration on NR-U related SON	ZTE Corporation, Sanechips

R2-2306248	Consideration on RACH enhancements	ZTE Corporation, Sanechips

R2-2306249	Consideration on SON-MDT support for NPN	ZTE Corporation, Sanechips

R2-2306250	Remaining issues on fast MCG recovery enhancement	ZTE Corporation, Sanechips

R2-2306290	Discussion on RAN2 impacts due to the LS R3-232140	Huawei, HiSilicon

R2-2306291	Discussion on MRO for inter-system handover for voice fallback	Huawei, HiSilicon

R2-2306292	Discussion on SHR and SPR	Huawei, HiSilicon

R2-2306293	Discussion on SONMDT enhancements for NPN	Huawei, HiSilicon

R2-2306339	Further Discussion on RACH enhancements for SON	China Telecom Corporation Ltd.

R2-2306358	Discussion on the “LS on potential override of logged MDT reports upon moving from SNPN to PLMN” from RAN3 (R3-232118)	Beijing Xiaomi Software Tech

R2-2306390	MRO for Fast MCG Recovery, CPAC and SCGFailureInformation	Samsung R&D Institute India

R2-2306391	MRO for Fast MCG Recovery, CPAC and SCGFailureInformation	Samsung R&D Institute India

R2-2306450	Enhancements of SON reports for NR-U	Ericsson

R2-2306452	Summary of AI 7.13.5 SON for NR-U (Ericsson)	Ericsson

R2-2306455	Discussion on inter-system HO for voice fallback	NTT DOCOMO, INC.

R2-2306456	Discussion on CPAC failure report	NTT DOCOMO, INC.

R2-2306462	Discussion on SPR	NTT DOCOMO, INC.

R2-2306530	Running 36.331 CR for SN RACH report	ZTE Corporation, Sanechips

R2-2306531	Running 38.331 CR for RACH report	ZTE Corporation, Sanechips

R2-2306557	Summary of AI 7.13.5 SON for NR-U (Ericsson)	Ericsson

R2-2306558	Open issues and proposals on AI 7.13.5 SON for NR-U (Ericsson)	Ericsson

R2-2306752	Pre-meeting summary of 7.13.4	Huawei

R2-2306753	Running 38.331 CR for logged MDT enhancements and NPN	Huawei, HiSilicon

R2-2306754	Running 36.331 CR for logged MDT enhancements	Huawei, HiSilicon

RAN3 #119bis-e:

R3-231112	Reply LS on RACH enhancement for R18 SONMDT	RAN2, Huawei

R3-231125	(BLCR to 38.423) Addition of SON features enhancement	Samsung

R3-231126	(BLCR to 38.473) Addition of SON features enhancement	Huawei

R3-231127	BLCR to 38.300:enhancement of SON	CMCC

R3-231128	(BLCR to 38.413) for SON	Ericsson

R3-231129	(BLCR to 38.401) Addition of SON features enhancement	ZTE

R3-231184	(TP for MDT BL CR for TS 38.423) Addition of PNI-NPN in MDT Area Scope	Huawei, China Telecom, CMCC

R3-231185	(TP for SON BLCR for TS 37.320): MDT support in SNPN	Huawei, Orange, China Telecom, CMCC

R3-231189	[TP to 38.423, SON] Configuration coordination for the successful PSCell change report	Nokia Netherlands

R3-231200	(TP for SON BLCR for 38.423) SON enhancement for SHR and SPR	Samsung

R3-231201	(TP for SON BLCR for 37.340 and 38.423) SON enhancements for CPAC and MCG failure recovery	Samsung

R3-231202	(TP for SON BLCR for 38.413 and 38.300) MRO for inter-system handover for voice fallback	Samsung

R3-231203	Discussion on SON for RACH	Samsung

R3-231204	Discussion on SON for NR-U	Samsung

R3-231226	[TP to 37.340, SON] Further consideration on the CPC Execution to wrong PSCell	Nokia, Nokia Shanghai Bell

R3-231227	[TP to TS38423, SON] Support for MRO for fast MCG recovery in case of SCG deactivation and for pre-Rel.18 UEs	Nokia, Nokia Shanghai Bell

R3-231228	The LBT waiting time for correct NR-U MRO analysis	Nokia, Nokia Shanghai Bell

R3-231269	(TPs for SON BLCRs for TS 38.300) SHR	Huawei

R3-231270	(TPs for SON BLCRs for TS 38.300) SPR	Huawei

R3-231271	(TPs for SON BLCRs for TS 38.300): MRO related objectives	Huawei

R3-231299	Inter-RAT SHR and SPR	Intel Corporation

R3-231300	(TP for SON BL CR for TS 38.473) RACH enhancements	Intel Corporation

R3-231301	(TP for SON BL CR for TS 38.413) MDT enhancements for SNPN	Intel Corporation

R3-231302	NR-U enhancements for MRO	Intel Corporation

R3-231337	Discussion on SON/MDT for NPN	BEIJING SAMSUNG TELECOM R&D

R3-231339	Successful Handover Report and Successful PSCell Change Report	Qualcomm Incorporated

R3-231340	MRO enhancements for CPAC, voice fallback and fast MCG recovery	Qualcomm Incorporated

R3-231341	RACH optimization enhancements	Qualcomm Incorporated

R3-231342	SON MDT enhancements for Non-Public networks	Qualcomm Incorporated

R3-231343	SON enhancements for NR-U	Qualcomm Incorporated

R3-231372	Discussion on related issue of inter-RAT SHR	NEC

R3-231373	Discussion on remaining issue of MRO for fast MCG recovery	NEC

R3-231423	SON enhancements for SPR	Lenovo

R3-231424	SON enhancements for SHR	Lenovo

R3-231425	(TP for SON BLCR for 37.340) SON enhancements for CPAC	Lenovo

R3-231426	(TP for SON BLCR for 38.300) MRO for inter-system handover for voice fallback	Lenovo, CMCC

R3-231427	MRO for fast MCG link recovery	Lenovo

R3-231428	Discussion on MRO for NR-U	Lenovo

R3-231552	Discussion on SON enhancement for SHR and SPR	CATT

R3-231553	Discussion on MRO for CPAC	CATT

R3-231554	Discussion on MRO for fast MCG recovery and voice fallback	CATT

R3-231555	Discussion on RACH enhancement	CATT

R3-231556	Discussion on SONMDT enhancement for NPN	CATT

R3-231557	Discussion on SON enhancement for NR-U	CATT

R3-231584	Inter-RAT SHR and SPR discussion	Ericsson

R3-231585	(TP for SON BL CR for TS 36.300, TS 38.300, TS 38.423) SON enhancements for MRO	Ericsson

R3-231586	RACH Optimization enhancement	Ericsson

R3-231587	Reply LS on RACH enhancement for R18 SONMDT	Ericsson

R3-231588	SON enhancements for Non-public networks	Ericsson

R3-231589	Radio Resource Status per NR-U Channel	Ericsson, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Qualcomm Incorporated

R3-231590	(TP for SON BL CR for TS 38.423) Radio Resource Status per NR-U Channel	Ericsson, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Qualcomm Incorporated

R3-231591	(TP for SON BL CR for TS 38.473) Radio Resource Status per NR-U Channel	Ericsson, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Qualcomm Incorporated

R3-231592	(TP for SON BL CR for TS 38.423) NR-U metrics	Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung

R3-231593	(TP for SON BL CR for TS 38.473) NR-U metrics	Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung

R3-231594	NR-U enhancements for MRO and MLB	Ericsson

R3-231628	(TP for SON BL CR to TS 38.473) Further discussion on RACH optimisation	Nokia, Nokia Shanghai Bell

R3-231629	(TP for MDT BL CR to TS 38.413) Support of NPN in MDT area scope	Nokia, Nokia Shanghai Bell

R3-231689	(TPs for SON BL CR TS37.340 and TS38.413) MRO enhancemetns in Rel-18	ZTE

R3-231690	Further discussion on NR-U optimizations	ZTE

R3-231708	(TPs for SON BLCRs for TS 38.300 TS 38.413 TS 38.473 and TS 38.423)Inter-RAT SHR and SPR	ZTE

R3-231709	(TPs for SON BLCRs for TS 38.473 and TS 38.423)RACH enhancements	ZTE

R3-231710	(TPs for SON BLCRs for TS 38.413 and TS 38.423)MDT support in NPN	ZTE

R3-231711	(TPs for MDT BLCRs for TS 38.413)MDT Enhancements	ZTE

R3-231740	(TPs for SON BLCRs for TS 38.300, TS 38.401, TS 38.423, TS 36.423,TS 38.473): Remaining issues for RACH optimisation	Huawei

R3-231741	[draft] Rely LS on RACH enhancement for R18 SONMDT	Huawei

R3-231742	(TPs for MDT BLCRs for TS 38.413, TS 38.423): MDT support in SNPN	Huawei

R3-231743	(TPs for SON BLCR for TS 38.423, TS 38.473): SON for NR-U	Huawei

R3-231744	(TP for MDT BL CR for TS 38.413): Signalling based immediate MDT in NR-DC	Huawei, Deutsche Telekom, Orange, China Telecom, CMCC

R3-231789	Update of work Plan for Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC WI	CMCC

R3-231790	MRO for CPC and CPA	CMCC

R3-231791	SON enhancement for Inter-RAT SHR	CMCC

R3-231792	SON enhancement for SPR	CMCC

R3-231800	Further considerations on fast MCG recovery	CMCC

R3-231868	CB: # SONMDT1_SHRSPR- Summary of email discussion	Qualcomm - moderator

R3-231869	CB: # SONMDT2_MRO- Summary of email discussion	Nokia - moderator

R3-231870	CB: # SONMDT3_RACH- Summary of email discussion	Intel - moderator

R3-231871	CB: # SONMDT4_NPN- Summary of email discussion	ZTE - moderator

R3-231872	CB: # SONMDT5_NRU- Summary of email discussion	Ericsson - moderator

R3-231873	CB: # SONMDT6_MDT- Summary of email discussion	Huawei - moderator

R3-231906	(BLCR to 38.413) for SON	Ericsson

R3-231995	LS on MDT measurements collection in MR-DC	Huawei

R3-231996	(TP for SON BL CR for TS 38.423) NR-U metrics	Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung

R3-231997	(TP for SON BL CR for TS 38.473) NR-U metrics	Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung

R3-231999	(TP for SON BL CR to TS 38.473) Further discussion on RACH optimisation	Nokia, Nokia Shanghai Bell

R3-232001	(TP for SON BLCR for 37.340) UHI for CPAC	Samsung

R3-232002	(TP for SON BLCR for 38.423) SON enhancement for SHR and SPR	Samsung

R3-232004	CB: # SONMDT1_SHRSPR- Summary of email discussion	Qualcomm - moderator

R3-232011	CB: # SONMDT2_MRO- Summary of email discussion	Nokia - moderator

R3-232020	(TPs for SON BLCRs for TS 38.423)RACH enhancements	ZTE, Intel

R3-232021	(TPs for SON BLCRs for TS 38.413 )Inter-RAT SHR and SPR	ZTE, Qualcomm

R3-232022	(TPs for SON BLCRs for TS 38.473) Inter-RAT SHR and SPR	ZTE, Qualcomm

R3-232025	CB: # SONMDT3_RACH- Summary of email discussion	Intel Corporation (moderator)

R3-232027	(TPs for MDT BLCRs for TS 38.413)MDT support in NPN	ZTE

R3-232060	(TPs for SON BLCRs for TS 38.300) SHR	Huawei

R3-232061	LS on intra-system inter-RAT SHR and on SPR	Huawei

R3-232062	[TP to 37.340, SON] Definitions for MRO failure events for CPAC	Nokia, Nokia Shanghai Bell, Lenovo, ZTE, Ericsson, Samsung

R3-232067	(TP for SON BL CR for TS 38.423) NR-U metrics	Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung

R3-232068	(TP for SON BL CR for TS 38.473) NR-U metrics	Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung

R3-232069	CB: # SONMDT5_NRU- Summary of email discussion	Ericsson - moderator

R3-232070	LS on MDT measurements collection in MR-DC	Huawei

R3-232091	(TP for MDT BL CR for TS 38.423) Addition of PNI-NPN in MDT Area Scope	Huawei, China Telecom, CMCC

R3-232099	(TPs for SON BLCR for TS 38.300): Naming update for RA report	Huawei

R3-232100	[draft] Rely LS on RACH enhancement for R18 SONMDT	Huawei

R3-232101	(TPs for SON BLCR for TS 38.401): Naming update for RA report	Huawei

R3-232102	(TPs for SON BLCR for TS 38.423): Naming update for RA report	Huawei

R3-232103	(TPs for SON BLCR for TS 38.473): Naming update for RA report	Huawei

R3-232104	(TPs for SON BLCR for TS 36.423): Naming update for RA report	Huawei

R3-232112	CB: # SONMDT4_NPN- Summary of email discussion	ZTE - moderator

R3-232118	LS on potential override of logged MDT reports upon moving from SNPN to PLMN	RAN3(Ericsson)

R3-232137	(TPs for SON BLCRs for TS 38.473) Inter-RAT SHR and SPR	ZTE, Qualcomm, Samsung

R3-232140	LS on intra-system inter-RAT SHR and on SPR	Huawei

R3-232141	(TP for SON BL CR to TS 38.473) TP for RACH optimisation	Nokia, Nokia Shanghai Bell, Huawei, ZTE, Intel, Ericsson

R3-232142	(TPs for SON BLCRs for TS 38.423)RACH enhancements	ZTE, Intel, Ericsson

R3-232143	(TPs for SON BLCR for TS 38.423): Naming update for RA report	Huawei

R3-232144	[draft] Rely LS on RACH enhancement for R18 SONMDT	Huawei

R3-232147	Update of WF on the MRO scenarios	Lenovo

R3-232151	(TP for SON BL CR for TS 38.300) SON enhancements for MRO	Ericsson

RAN3 #120:

R3-232524	(BLCR to 38.300) enhancement of SON	CMCC

R3-232525	(BLCR to 38.401) Addition of SON features enhancement	ZTE

R3-232526	(BLCR to 38.413) for SON	Ericsson

R3-232527	(BLCR to 38.423) Addition of SON features enhancement	Samsung

R3-232528	(BLCR to 38.473) Addition of SON features enhancement	Huawei

R3-232554	(BLCR to 36.423)  Addition of SON features enhancement	CATT

R3-232555	(BLCR to 37.340) Addition of SON Rel.18 features	Nokia, Nokia Shanghai Bell

R3-232556	(BLCR to 38.423) for MDT	Huawei

R3-232568	(BLCR to 38.413) for MDT	Ericsson

R3-232587	(TP for SON BL CR to TS 38.473) Further discussion on RACH optimisation	Nokia, Nokia Shanghai Bell

R3-232588	(TP for MDT BL CR to TS 38.413) Support of NPN in MDT area scope	Nokia, Nokia Shanghai Bell

R3-232674	Discussion on SONMDT enhancement for NPN	CATT

R3-232677	Signalling based logged MDT override protection	Nokia, Nokia Shanghai Bell

R3-232690	[TP to 38.423, SON] SHR and Configuration coordination for the successful PSCell change report	Nokia Austria

R3-232693	Further discussion on the MRO for the fast MCG recovery and for CPAC	Nokia, Nokia Shanghai Bell

R3-232694	LBT waiting time for correct classification of MRO events	Nokia, Nokia Shanghai Bell

R3-232736	Discussion on remaining issue of inter-RAT SHR	NEC

R3-232737	Discussion on MRO for fast MCG recovery	NEC

R3-232752	(TPs for SON BLCRs for TS 38.300) SHR	Huawei

R3-232753	(TPs for SON BLCRs for TS 38.300, 38.423, 38.413) SPR	Huawei

R3-232754	(TPs to TS 38.413 and 36.300 BL CRs) MRO	Huawei

R3-232805	Discussion on SON for RACH	Samsung

R3-232806	Discussion on SON for NR-U	Samsung

R3-232817	(TP for SON BLCR for 38.423, 38.300 and 37.340) SON enhancement for SHR and SPR	Samsung

R3-232818	(TP for SON BLCR for 38.423 and 37.340) SON enhancements for CPAC and MCG failure recovery	Samsung

R3-232819	(TP for SON BLCR for 38.413, 38.300 and 36.300) MRO for inter-system handover for voice fallback	Samsung

R3-232820	(TPs for SON BLCR for TS 38.470, TS 38.420, TS 36.423 and TS 36.000) Remaining issues for RACH optimisation	Huawei

R3-232821	(TP for MDT BL CRs for TS 38.413 and TS 38.423): Remaining issues for MDT in NPN	Huawei

R3-232822	(TP for MDT BLCR for TS 37.320): MDT support in NPN	Huawei, Orange, China Telecom, CMCC

R3-232823	(TPs for SON BLCR for TS 38.423 and TS 38.473): SON for NR-U	Huawei

R3-232844	Successful Handover Report and Successful PSCell Change Report	Qualcomm Incorporated

R3-232845	MRO enhancements for CPAC, voice fallback and fast MCG recovery	Qualcomm Incorporated

R3-232846	RACH optimization enhancements	Qualcomm Incorporated

R3-232847	SON MDT enhancements for Non-Public networks	Qualcomm Incorporated

R3-232848	SON enhancements for NR-U	Qualcomm Incorporated

R3-232891	Discussion on SON enhancement for SHR and SPR	CATT

R3-232892	Discussion on MRO for CPAC	CATT

R3-232893	Discussion on MRO for fast MCG recovery and voice fallback	CATT

R3-232894	Discussion on RACH enhancement	CATT

R3-232895	Discussion on SON enhancement for NR-U	CATT

R3-232917	(TPs for MDT BLCRs for TS 38.413 and TS 38.423) SON/MDT for NPN	BEIJING SAMSUNG TELECOM R&D

R3-232946	[TP to 38.423, SON] SHR and Configuration coordination for the successful PSCell change report	Nokia, Nokia Shanghai Bell

R3-233022	SON enhancements for SHR	Lenovo

R3-233023	SON enhancements for SPR	Lenovo

R3-233024	(TP for SON BLCR to TS36.300) MRO for inter-system handover for voice fallback	Lenovo

R3-233025	SON enhancements for CPAC	Lenovo

R3-233026	Update of the MRO scenarios	Lenovo, Qualcomm, Huawei

R3-233027	Discussion on MRO for NR-U	Lenovo

R3-233086	(TPs for SON BL CR 36.300, 38.300 and 38.413) MRO enhancemetns in Rel-18	ZTE

R3-233087	Further discussion on NR-U optimizations	ZTE

R3-233091	(TP for SON BL CR for TS 38.423) Inter-RAT SHR and SPR discussion	Ericsson

R3-233092	(TP for SON BL CR for TS 36.300, TS 37.340) SON enhancements for MRO	Ericsson

R3-233093	RACH Optimization enhancement	Ericsson

R3-233094	(TP for SON BL CR for TS 38.413) SON enhancements for Non-public networks	Ericsson

R3-233095	Reply LS on user consent of Non-public Network	Ericsson

R3-233096	NR-U enhancements for MRO and MLB	Ericsson

R3-233097	CAC per NR-U Channel	Ericsson, Deutsche Telekom, Qualcomm Incorporated

R3-233098	(TP for SON BL CR for TS 38.423) CAC per NR-U Channel	Ericsson, Deutsche Telekom, Qualcomm Incorporated

R3-233099	Radio Resource Status per NR-U Channel	Ericsson, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Qualcomm Incorporated

R3-233100	(TP for SON BL CR for TS 38.423) Radio Resource Status per NR-U Channel	Ericsson, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Qualcomm Incorporated

R3-233101	(TP for SON BL CR for TS 38.473) Radio Resource Status per NR-U Channel	Ericsson, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Qualcomm Incorporated

R3-233188	(TPs for SON BLCRs for TS 38.300 TS 38.413 TS 38.473 and TS 38.423)Inter-RAT SHR and SPR	ZTE

R3-233189	(TPs for SON BLCRs for TS 38.423 TS 38.473 TS 37.340)RACH enhancements	ZTE

R3-233190	MDT support in NPN	ZTE

R3-233191	(TPs for MDT BLCRs for TS 38.413)MDT Enhancements	ZTE

R3-233209	Update of work Plan for Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC WI	CMCC

R3-233210	Discussion on SON enhancement for Inter-RAT SHR	CMCC

R3-233211	Discussion on SON enhancement for SPR	CMCC

R3-233222	Further considerations on fast MCG recovery	CMCC, CATT

R3-233335	CB:#17_SHRSPR	Samsung

R3-233336	CB:#18_MRO	ZTE

R3-233337	CB:#19_SONMDT3_RACH	Qualcomm

R3-233338	CB:#20_SONMDT4_SNPN	Nokia

R3-233339	CB:#21_SONMDT5_NRU	Ericsson

R3-233340	CB:#22_SONMDT6_MDT	Nokia

R3-233379	(TP for SON BLCR for 37.340) SON enhancement for SPR	Samsung

R3-233380	LS on SHR and SPR	RAN3(Samsung)

R3-233396	(TP for SON BL CR for TS36.300) MRO for inter-system handover for voice fallback	ZTE

R3-233397	(TP for SON BL CR for TS38.300) MRO for inter-system handover for voice fallback	Samsung

R3-233399	(TPs for SON BLCRs for TS 38.300) Forwarding mechanism for Inter-RAT SHR and SPR	ZTE, Samsung

R3-233408	(TPs for SON BLCRs for TS 38.300) SPR	Huawei

R3-233448	(TPs for SON BLCR for TS 38.470): Introduction of RACH Indication	Huawei

R3-233449	(TPs for SON BLCR for TS 38.420): Introduction of RACH Indication	Huawei

R3-233450	(TPs for SON BLCR for TS 36.300): Naming update for RA report	Huawei

R3-233451	(TPs for SON BLCR for TS 36.423): Introduction of RACH Indication	Huawei

R3-233452	(TP for MDT BL CR to TS 38.423) Support of SNPN in MDT area scope	Huawei

R3-233456	(TP for SON BLCRs for 37.340)RACH enhancements	ZTE, Qualcomm

R3-233470	(TP for MDT BL CR to TS 38.413) Support of NPN in MDT area scope	Nokia, Nokia Shanghai Bell

R3-233483	(TP for SON BLCR to TS36.423) Addition of PSCell List	Ericsson

R3-233484	(TP for SON BLCR to TS38.423) Addition of PSCell List	Ericsson

R3-233491	(TP for SON BL CR for TS 38.423) EDT in UL	Ericsson

R3-233497	CB:#19_SONMDT3_RACH	Qualcomm

R3-233498	(TPs for SON BLCR for TS 38.470): Introduction of RACH Indication	Huawei, Ericsson, Nokia, ZTE

R3-233499	(TPs for SON BLCR for TS 36.423): Introduction of RACH Indication	Huawei, Ericsson, Nokia

R3-233500	(TP for SON BLCR to TS38.423) Addition of PSCell List	Ericsson