3GPP TSG RAN Meeting #100	RP-231164

Taipei, June 12-14, 2023

Agenda item:	9.2.2

Source:	CMCC

Title:	Discussion on Ambient IoT

Document for:	Discussion

- Introduction

In SA #101, TR 22.840 has updated to V1.1.0 [1]. In RAN #99, Agreements on Design targets, Deployment scenarios tables, connection topologies and rUC are captured in RP-230808/230807/230802, respectively. The remaining issues are as follows [2]:

- To conclude on device characteristics and categorization - To conclude on entries and descriptions of deployment scenario characteristics - Complete the set of RAN design targets based on the identified deployment scenarios and their characteristics for the relevant use cases - Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported.

In this contribution, we discuss on the RAN SI relating issues and recommended conclusions for the SI.

2	Discussion

2.1 RAN design target

According to our previous analysis of the use cases proposed by SA1, and the two use cases recently adopted by SA1, we have summarized the requirements of all use cases of SA1, as shown in the following table:

| Use case category | Use Cases identified in TR 22.840 by SA1 | Max E2E latency | Communication Range | Positioning Accuracy | Data Rate | Message Size | Device density | Device speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inventory | 1; 2; 5; 7; 16; 21; 24; 27 | ~1s | 30 m (indoor) | 3 m, 90% | <1000 bits/s | <256 bit | 1.5 million per km2 | < 6 km/h |
| sensor | 3; 6; 11; 13; 15; 18; 19; 20; 22; 23; 24; 25; 26; 30 | 1~30s | 50 m (indoor), 200 m (outdoor) 500 m (outdoor) | NA | <1000 bits/s | <100 byte | 1000 per km2 | Static |
| Positioning | 2; 4; 7; 8; 9; 10; 12; 14; 16; 24; 27 | 1~10s | 50 m (indoor), 200 m (outdoor) | 1~3 m, 90% | <1000 bits/s | 256 bits | 1000 per km2 | 5 km/h (indoor) 20 km/h (outdoor) |
| Activator / Command | 17; 28; 29 | NA/FFS | NA/FFS | NA/FFS | NA/FFS | NA/FFS | NA/FFS | NA/FFS |

Based on the above analysis and summary, we can make a brief discussion on RAN design target for Ambient IoT.

- Device power consumption

Device power consumption is the main difference among the 3 categories of Ambient IoT devices. It is necessary to emphasize or highlight the power consumption gap between Device B and Device C, because they use different communication method, e.g., amplified backscatter vs active signal generation.

If it is declared that the power consumption gap between the two devices is not obvious, it may cause ambiguity in the subsequent RAN design. For example, although the ambient IoT tag is Device C, it still uses amplified backscatter for communication, and vice versa. Therefore, we believe that it is necessary to widen the gap between Device B and Device C.

Another advantage of enlarge the power consumption gap between Device B and C is that it can make Device C have more capabilities, especially where periodically reporting or monitoring and controlling other devices, or to communicated for 500m is required to guarantee device C can be deployed in the outdoor wide area. A clear gap is also helpful in further discussions.

Proposal 1: Considering device C is targeted for outdoor wide area deployment, RAN is kindly asked to agree to enlarge power consumption gaps between Device B and Device C to guarantee device C can have higher capability and larger coverage that device A and B.

Please note that authentication and security enhancement is for use case 29 and other similar use cases where business confidential must be guaranteed.

- Coverage

In last SA1 meeting, a new use case 30 requires Ambient IoT device to be able to communicate for 500 m. And in RAN #99, companies agree to determine coverage design target based on topology. Therefore, based on all SA1 requirements and RAN agreements. We propose RAN to adopt the following coverage design target.

Proposal 2: RAN is kindly asked to adopt the follow coverage design target table and discuss the FFS.

|  | Topology 1: BS ↔ Ambient IoT device | Topology 2: BS ↔ intermediate node ↔ Ambient IoT device | Topology 3: BS ↔ assisting node ↔ Ambient IoT device ↔ BS | Topology 4: UE ↔ tag |
| --- | --- | --- | --- | --- |
| Device A | Indoor: Typical indoor 3GPP inter-site distances Outdoor: NA | Indoor: Typical indoor 3GPP inter-site distances Outdoor: NA | Indoor: Typical indoor 3GPP inter-site distances Outdoor: NA | Indoor: Typical indoor 3GPP inter-site distances Outdoor: NA |
| Device B | Indoor: Typical indoor 3GPP inter-site distances Outdoor: FFS 1 | Indoor: Typical indoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: Typical indoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: Typical indoor 3GPP inter-site distances Outdoor: FFS 2 |
| Device C | Indoor: Typical outdoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: Typical outdoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: Typical outdoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: FFS 3 Outdoor: FFS 4 |

FFS 1: depending on capability of Device B

FFS 2/3/4: depending on UE capability

For Device A, it is quite difficult to fulfil the requirement of long-range communication in outdoor, so Device A is mainly targeted for indoor scenario. Only if coverage hole is tolerable for the use case, it can be applied to outdoor scenario.

For Device B, it’s slightly different with Device A, the communication range of Device B relates to its amplifier gain and energy storage capability and it can at least communication at 200m outdoor. But for Topologies 1, we are not sure about whether Device C can achieve 500m communication without assistance/intermediate node. Therefore, we marked it as FFS 1. We hope that network vendor or UE vendor can provide some inputs here. For Topologies 2 and 3. with assistance/intermediate node, it’s possible for 500m communication, so we marked it as typical outdoor 3GPP inter-site distances. For Topologies 4, we are not sure the capability of “New UE” or “Ambient IoT compatible UE”, so we left an FFS 2. The FFS can be further discussed during RAN WG level SI.

For Device C, it only has issue in Topologies 4, since the capability of “New UE” or “Ambient IoT compatible UE” is still uncertain., and Device C has higher capability compared to Device B, so we marked it as FFS 3 and 4, If UE capability is confirmed, it should be replaced.

If UE capability is the same as legacy eMBB UE, FFS 2/3/4 can be replaced as typical indoor 3GPP inter-site distances.

- Data rate

We are generally fine with current agreement, but since the data rate has relation with communication range, i.e., the larger the communication range, the slower the data rage. Supposing a certain Device C works far from the typical outdoor 3GPP inter-site distances, the communication range can be slower than 0.1kbps. therefore we suggest to add a note on the data rate stating that the data rate only applies for Ambient IoT device within typical outdoor 3GPP inter-site distances.

Proposal 3: RAN is kindly asked to add a note on data rate design target stating that it only applies for Ambient IoT device within typical outdoor 3GPP inter-site distances.

- Latency

Obviously, latency is directly related to device density and communication range. The communication range will physically affect the speed of message delivery and data rate, and the high density of devices will cause messages between devices to collide with each other, resulting in higher latency.

According to the use case of SA1, the device density can be mainly divided into the 3 categories, spare (<1 per 100m2), moderate (1~100 per 100 m2), and dense (>100 per 100 m2).

The minimum latency can be hard to decide since the dense device density in 3D warehouse, therefore we think RAN can have an overall maximum latency. The detail latency can be discussed for different use cases.

As for DO and DT, we think it can be further studied in SI or WI. Currently, not many solid concludes can be drawn.

We would like to highlight that, even thought the E2E delay defined by SA1 for each use case is around 1s~10s, that is only from view of worst E2E delay for the specific use cases. The Ambient IoT defined by 3GPP should be compatible with the existing RFID. Considering the latest RFID can inventory 500~1000 tags per second, our RAN design targets need to be competitive with that.

Proposal 4: RAN is kindly asked to only have the maximum E2E latency, and discussed maximum E2E latency based on spare (<1 per 100m2), moderate (1~100 per 100 m2), and dense (>100 per 100 m2) device density. Maximum latency can be set as <100ms for spare, <1s for moderate and <10s for dense device density.

Proposal 5: Add a further requirement for RAN design target that the inventory rate of ambient IoT should be competitive with RFID, considering the latest RFID can inventory 500~1000 tags per second.

2.2 For the conclusions of RAN SI on Ambient IoT

A follow-up WG SI is expected to start in Rel-19. It can be crucial that we address the urgency associated with the timeline. If the WG intends to undertake a comprehensive study encompassing all four use cases, three device categories, and four topologies, the evaluation and study alone would necessitate a minimum of 18 months. Nevertheless, it remains a matter of concern whether it is necessary to tackle such an extensive scope right from the initial release, or if our focus should primarily be on the fundamental and common aspects of the scope.

By directing our efforts towards the basic subset of the aforementioned scope, although still challenging, it would be conceivable to complete the SI and WI within Rel-19. The advantage of this approach would be the ability to meet the demands of vertical customers more expeditiously, by the end of 2025 Q4.

Observation 1: There is a need to prioritize and streamline the scope of the Ambient IoT study in order to meet the urgent timeline and cater to the requirements of vertical customers more quickly.

Given the urgency market demands, it is imperative to prioritize certain related scenarios in the context of Ambient IoT. A crucial question arises: should all four use cases be included in the R19 WG SI? The comprehensive study outlined in TR 38.848 highlights representative use cases, namely Inventory, sensors, positioning, and activator. Notably, the Inventory and sensor use cases share similar requirements, except for variations in message size, while positioning entails specific considerations regarding L1/L2 signaling and architecture.

To establish a design framework for Ambient IoT, it is proposed that the use case of inventory be treated as the fundamental scenario, similar to the approach of utilizing eMBB as the baseline for 5G design. By prioritizing the inventory use case, a design solution can be achieved that is compatible with sensor and activator use cases. Use case of sensor may require two more signalling and larger message size to 800bits, comparing with inventory use case. Positioning use cases, on the other hand, would be regarded as technical enhancements built upon the baseline design, addressing the specific requirements associated with L1/L2 signaling and architecture. In the first version, only the basic cell level positioning can be considered. And further high accuracy positioning method can be considered in Rel-20.

Proposal 6: Similar to using eMBB as the NR baseline, the inventory use case serves as a fundamental scenario for Ambient IoT. The physical and L2 design can be compatible with sensor and activator, while positioning use cases are regarded as enhancements above the baseline.

When it comes to the Ambient IoT device categories, we firmly believe in the importance of delving deeper into the intricacies of their communication technologies and logical procedures. By doing so, we can enhance the design process to better accommodate the unique specifications and requirements of these devices. Considering this, we would like to present our preliminary understanding as follows.

|  | Device A/B Passive/Semi-passive device | Device C Active device |
| --- | --- | --- |
| Waveform/frame structure/line code | Yes | Yes |
| Modulation | ASK/FSK/BPSK | ASK/FSK/BPSK/QPSK |
| Channel coding | No or simple coding | Yes |
| Duplexing mode | Full duplex at reader, half duplex at device | FDD at both reader and device |
| Periodic synchronization signaling | No | Yes |
| Broadcast channel | No | Yes |
| PHY DL/UL shared channel | Yes | Yes |
| L2/L3 procedures | Yes | Yes |

Due to variations in capabilities and characteristics among Ambient IoT devices, their utilization of technological pathways in standardization also differs. Therefore, design targets and options for passive and active Ambient IoT devices should be explored to ensure optimal performance and compatibility. Two key options have been identified here:

- Option 1: Consider the passive device as the core functionality, with semi-passive and active devices serving as enhancements to the passive device. The aim is to maximize design reusability across the different device types.

- Option 2: Pursue a parallel and independent design approach for passive and active devices, focusing on supporting higher power consumption and capabilities specifically for active devices.

Upon careful analysis, it is evident that Option 1 has the potential to be accommodated within the scope of the R19 SI+WI, given a singular emphasis on passive device design. Conversely, Option 2 would require a comprehensive 18-month study to effectively address the complexities involved.

If the WG decides to study active devices in Rel-19, it is recommended that the basic design of the passive device be reused to the greatest extent possible, particularly concerning PHY UL/DL shared channel and L2/3 procedures. This approach would minimize the extent of study and standardization work required, allowing the first release to concentrate on the fundamental and common aspects of both passive and active devices.

Observation 2: Option 1 is possible to fit in Rel-19 SI+WI, while Option 2 requires an 18-month study.

Observation 3: If active devices are included, reuse the basic design of passive devices can minimize the study and standardization work.

In line with these considerations, we suggest adopting the passive device as the baseline for device design in Rel-19. If the inclusion of active devices in the SI and WI is desired, the air interface design of the passive device should be maximally leveraged, ensuring compatibility with the active device by reusing elements such as PHY UL/DL shared channels and L2/3 procedures. Specific design enhancements can still be incorporated to cater to the unique requirements of passive devices (e.g., extreme low power) and active devices (e.g., coverage enhancement).

Proposal 7: Passive device can be taken as baseline for Rel-19 device design.

Proposal 8: For active devices, the air interface design of the passive device should be leveraged as much as possible, while allowing for further specific design aspects to address the passive device's low power requirements and the active device's coverage enhancement.

In RAN level SI, another important consideration is which bands should be given priority or considered. Specifically, the focus lies on licensed FDD, licensed TDD, and unlicensed bands.

It can evident that unlicensed operation requires a more comprehensive study and normative work due to its unique characteristics for global development. However, when evaluating the bands from a coverage standpoint, licensed FDD bands hold an advantage over licensed TDD bands as they operate at lower frequencies. And compared to unlicensed, it has less interface which can be helpful for coverage enhancement.

Proposal 9: Considering coverage and interference, it is recommended to prioritize low-frequency licensed bands, specifically licensed FDD. And unlicensed band is not recommended for the first release of Ambient IoT.

It is essential to consider factors such as coverage and interference while making informed decisions regarding the allocation of resources and attention to specific bands.

The identification of candidate positioning methods becomes crucial. Several potential techniques have been considered, including RSRP, phase difference, AOA (Angle of Arrival), and TOA/TDOA (Time of Arrival/Time Difference of Arrival). These methods offer diverse approaches for achieving accurate positioning in Ambient IoT scenarios.

Rel-19 SI+WI prioritizes the development of foundational features essential for Ambient IoT, with a specific emphasis on inventory management. This prioritization ensures that the basic functionality and core requirements of Ambient IoT, such as inventory tracking and monitoring, are effectively addressed. By focusing on these fundamental aspects, the WG can establish a robust foundation for the subsequent stages of development.

In the context of SI evaluation for Rel-19, candidate positioning methods such as RSRP, phase difference, AOA, and TOA/TDOA are being considered. However, the primary focus of Rel-19 SI is on developing the foundational functionality of Ambient IoT, with a specific emphasis on inventory management. This prioritization allows for the establishment of a solid foundation before incorporating enhancements, such as positioning and other advanced features, in subsequent releases.

Proposal 10: Rel-19 SI focus firstly on the basic function of Ambient IoT, while positioning can be taken as enhancement feature for further release.

In a similar vein, the study of RAN topologies can also consider focusing on representative scenarios rather than encompassing all possible topologies. TR 38.848 has identified five topologies, but the inclusion of all these topologies in R19 WG SI is a subject of consideration. A proposed approach involves designing a unified ambient-link interface that caters to topology 1 and 2, ensuring seamless access for ambient IoT devices to either the gNB or intermediate node. However, for other topologies, their prioritization is reduced in Rel-19, allowing for a more targeted and efficient allocation of resources and efforts.

Proposal 11: Design a unified ambient-link interface for topologies 1 and 2, ensuring seamless access for ambient IoT devices to both gNB and intermediate nodes. Other topologies are down prioritized for the first release of Ambient IoT.

3	Conclusion

This contribution discusses on design targets and Rel-19 Ambient IoT, and provides the following proposal:

RAN design targets

Proposal 1: Considering device C is targeted for outdoor wide area deployment, RAN is kindly asked to agree to enlarge power consumption gaps between Device B and Device C to guarantee device C can have higher capability and larger coverage that device A and B.

Proposal 2: RAN is kindly asked to adopt the follow coverage design target table and discuss the FFS.

|  | Topology 1: BS ↔ Ambient IoT device | Topology 2: BS ↔ intermediate node ↔ Ambient IoT device | Topology 3: BS ↔ assisting node ↔ Ambient IoT device ↔ BS | Topology 4: UE ↔ tag |
| --- | --- | --- | --- | --- |
| Device A | Indoor: Typical indoor 3GPP inter-site distances Outdoor: NA | Indoor: Typical indoor 3GPP inter-site distances Outdoor: NA | Indoor: Typical indoor 3GPP inter-site distances Outdoor: NA | Indoor: Typical indoor 3GPP inter-site distances Outdoor: NA |
| Device B | Indoor: Typical indoor 3GPP inter-site distances Outdoor: FFS 1 | Indoor: Typical indoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: Typical indoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: Typical indoor 3GPP inter-site distances Outdoor: FFS 2 |
| Device C | Indoor: Typical outdoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: Typical outdoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: Typical outdoor 3GPP inter-site distances Outdoor: Typical outdoor 3GPP inter-site distances | Indoor: FFS 3 Outdoor: FFS 4 |

FFS 1: depending on capability of Device B

FFS 2/3/4: depending on UE capability

Proposal 3: RAN is kindly asked to add a note on data rate design target stating that it only applies for Ambient IoT device within typical outdoor 3GPP inter-site distances.

Proposal 4: RAN is kindly asked to only have the maximum E2E latency, and discussed maximum E2E latency based on spare (<1 per 100m2), moderate (1~100 per 100 m2), and dense (>100 per 100 m2) device density. Maximum latency can be set as <100ms for spare, <1s for moderate and <10s for dense device density.

Proposal 5: Add a further requirement for RAN design target that the inventory rate of ambient IoT should be competitive with RFID, considering the latest RFID can inventory 500~1000 tags per second.

Proposals on the conclusion of RAN SI and the plan for R19 WG SI

Observation 1: There is a need to prioritize and streamline the scope of the Ambient IoT study in order to meet the urgent timeline and cater to the requirements of vertical customers more quickly.

Proposal 6: Similar to using eMBB as the NR baseline, the inventory use case serves as a fundamental scenario for Ambient IoT. The physical and L2 design can be compatible with sensor and activator, while positioning use cases are regarded as enhancements above the baseline.

Observation 2: Option 1 is possible to fit in Rel-19 SI+WI, while Option 2 requires an 18-month study.

Observation 3: If active devices are included, reuse the basic design of passive devices can minimize the study and standardization work.

Proposal 7: Passive device can be taken as baseline for Rel-19 device design.

Proposal 8: For active devices, the air interface design of the passive device should be leveraged as much as possible, while allowing for further specific design aspects to address the passive device's low power requirements and the active device's coverage enhancement.

Proposal 9: Considering coverage and interference, it is recommended to prioritize low-frequency licensed bands, specifically licensed FDD. And unlicensed band is not recommended for the first release of Ambient IoT.

Proposal 10: Rel-19 SI focus firstly on the basic function of Ambient IoT, while positioning can be taken as enhancement feature for further release.

Proposal 11: Design a unified ambient-link interface for topologies 1 and 2, ensuring seamless access for ambient IoT devices to both gNB and intermediate nodes. Other topologies are down prioritized for the first release of Ambient IoT.

References

- TR 22.840 Study on Ambient power-enabled Internet of Things (Release 19) V1.1.0, 2023-03; 3GPP SA1

- RP-230669 Status report for SI: Study on Ambient IoT (Internet of Things) in RAN, Huawei