3GPP TSG RAN Meeting #100		                                               RP-231167

Taipei, June 12th – 14th, 2023

Agenda Item:	9.3.1.7

Source: 	OPPO

Title:	Discussion on peak data rates of Rel-18 RedCap UEs

Document for:	Discussion & Decision

Introduction

In the previous RAN 1 meeting, the framework of PR1(Peak Data Rate 1) is used as the basis of the discussion. This is basically restricting the minimal boundary of peak data rate for UE to support.

It was intended to be captured in TR38.306:

For the relaxed time constraint of 1/0.5 ms for 15kHz/30kHz SCS in the earlier RAN1 agreement, it will be fitted with X = 3.2 for narrow band Rel-18 RedCap UE as:

- UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X

For the 20MHz bandwidth UE, X=0.75 will be used.

However, it was discussed during the previous working group meetings that the expression of “vLayers·Qm·f ≥” will allow too many options of supported bit rates for Rel-18. This contribution will try to clarify the issue which is suspending further progress in working group meetings.

Multiple data rates vs. single data rate

It was assumed in long time in RAN1 that the PR1 is defined for restricting the vLayers·Qm·f ≥ X. However, RAN plenary agreements in the RAN plenary #99 brings different understanding. It agreed that the supported eRedCap capabilities are as in the following:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

The main bullet about “BW3/PR3 + PR1 designed/targeted to same peak data rate, i.e., 10Mbps” would be interpreted that Rel-18 can only support 10 Mbps.

With the peak data rate determination in 38.306, DL/UL peak data rates was illustrated in the table 1. And, it is clearly shown that the exact 10Mbps cannot be reached. Thus, the RAN#99 agreement should not be looked as only single data rate will be supported.

Table1. The data rates for vLayers·Qm·f = 3.2, 5MHz BW RedCap UE

| Date rates | 15 kHz SCS (25 PRBs) | 30 kHz SCS (12 PRBs) |
| --- | --- | --- |
| DL | 10.07 Mbps | 10.2725 Mbps |
| UL | 11.4469 Mbps | 10.9894 Mbps |

Additionally, “PR1” was defined during the previous agreements in RAN1 that it use the restriction of “vLayers·Qm·f ≥ X”, which did not restrict to only one data rates. In RAN discussion, the framework is not challenged and would be assumed as from RAN1.

Proposal 1: RAN clarify the previous agreements still keep the current framework of definition of UE Peak Data Rates determination.

Further restriction of Rel-18 RedCap UEs

The motivation of have less supportable Data Rates for Rel-18 RedCap UEs would be, less options of implementation RedCap UE chipset. We would consider this issue as a separated discussion than the above clarification of RAN agreements. From our point of views, we would like to have no further pruning on the options, as this framework does not bring any device fragmentation issues even in Rel-17.

In the end, if the further restriction can be agreed in RAN, we should also reflect the change into WID.

Proposal 2: RAN discuss if the further restriction can be introduced to achieve that Rel-18 RedCap UEs only support single peak data rate.

Conclusion

In this contribution, we had discussed and analyzed the pending issues of Rel-18 RedCap UE. To have some more RAN guidance, we have following proposals:

Proposal 1: RAN clarify the previous agreements still keep the current framework of definition of UE Peak Data Rates determination.

Proposal 2: RAN discuss if the further restriction can be introduced to achieve that Rel-18 RedCap UEs only support single peak data rate.

Reference

[1] RP-223544, Revised WID: Enhanced support of reduced capability NR devices, Ericsson