3GPP TSG RAN Meeting #100			RP-231170

Taipei, June 12th – 14th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on CSI prediction for Rel-18 AI/ML

Agenda Item:	9.2.5

Document for: 	Discussion and Decision

- Introduction

RAN1 has been discussing whether to study the potential specification impacts of CSI prediction in Rel-18 for several meetings. At the RAN1#112bis-e meeting, it was agreed that the final conclusion would be made in RAN#100 as below [1].

Agreement

In CSI prediction using UE-side model use case, whether to address the potential spec impact of CSI prediction depends on RAN#100 final conclusion, focusing on the following

- data collection procedure, mainly including RS configuration, measurement and report configuration, resusing as much as possible what is defined for UE side use cases

- monitoring procedure and metric for AI-based CSI prediction.

- Model/functionality selection/switching and finetuning procedure.

- Note: Discussion on potential specification impact is limited to aspects which would NOT duplicate the work in Rel-18 MIMO WI.

- Note: Minimize LCM related potential specification impact discussion that follow the high-level principle of other one-sided model sub-cases.

In this contribution, we provide our view on this issue.

- Discussion

Evaluation of CSI prediction has been conducted for the entire Rel-18, and the potential gain from the existing feature was already observed. In addition, the feasibility of CSI prediction is higher than CSI compression as one-sided model does not require as much NW-UE collaboration as two-sided model. Hence, it is useful to study the potential specification impacts of CSI prediction for the upcoming releases.

The biggest concern for the CSI prediction study is the workload of Rel-18 AI/ML SI. In the above agreement, three aspects were captured as the study points of CSI prediction. Although most aspects are essential and studied for each sub use case, the potential specification of fine-tuning procedure has not been studied in any sub use cases yet. Given that the end of Rel-18 SI is around the corner and the fine-tuning is not specific feature to CSI prediction, it is not reasonable to unnecessarily bring up a new aspect for new sub use cases. Based on above, RAN should make a conclusion on studying CSI prediction regarding the aspects listed by RAN1 other than fine-tuning.

Proposal 1: RAN tasks RAN WGs to study CSI prediction regarding the following aspects (aspects listed in the RAN1 agreement other than fine-tuning).

•	data collection procedure, mainly including RS configuration, measurement and report configuration

•	monitoring procedure and metric for AI-based CSI prediction.

•	model/functionality selection/switching.

- Conclusion

In this contribution, we provided our views regarding CSI prediction for Rel-18 AI/ML, and made the following proposal.

Proposal 1: RAN tasks RAN WGs to study CSI prediction regarding the following aspects (aspects listed in the RAN1 agreement other than fine-tuning).

•	data collection procedure, mainly including RS configuration, measurement and report configuration

•	monitoring procedure and metric for AI-based CSI prediction.

•	model/functionality selection/switching.

References

- Moderator (Apple), R1-2208077 “Summary #4 on other aspects of AI/ML for CSI enhancement”, Aug. 2022.