3GPP TSG RAN meeting #100										RP-231171

Taipei, June 12-14, 2023

Status Report to TSG

Agenda item:			9.3.1.2

| WI / SI Name | Multi-carrier enhancements for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MC_enh |  |  |  |  |
| Unique ID | 940094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222251 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: 06/2024 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 95% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Hiroki Harada |
|  | Company | NTT DOCOMO, INC. |
|  | Email | hiroki.harada@docomo-lab.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#112bis-e

Agreement

R1-2304238 captures the higher layers parameters for the following Rel-18 work items and TEI that are considered stable from RAN1 perspective:

- NR network-controlled repeaters (NR_netcon_repeater-Core)

- Enhancement of NR Dynamic Spectrum Sharing (NR_DSS_enh)

- Multi-carrier enhancements for NR (NR_NC_enh-Core)

- BandWidth Part operation without restriction in NR (BWP_wor)

- 1-symbol PRS (TEI18)

Note: The updates in the list as compared to the already communicated higher layer parameters to RAN2/RAN3 are highlighted in blue.

Note: R1-2304221 captures all the discussed higher layers parameters including the stable and unstable ones.

[112bis-e-R18-38.211-NR_MC_enh] Review of draft CR by April 26 – Stefan (Ericsson)

R1-2304278	Introduction of multi-carrier enhancements	Ericsson

Decision: As per email decision posted on April 27th, the draft CR in R1-2304278 for Rel-18 TS38.211 MC-Enh is endorsed in principle.

[112bis-e-R18-38.212-NR_MC_enh] Review of draft CR by April 26 – Chengyan (Huawei)

R1-2304265	Summary of email discussion [112bis-e-R18-38.212-NR_MC_enh]	Moderator (Huawei)

R1-2304263	Introduction of Rel-18 Multi-carrier enhancements	Huawei	(rev of R1-2303803)

Decision: As per email decision posted on April 27th, the draft CR in R1-2304263 for Rel-18 TS38.212 MC-Enh is endorsed in principle.

[112bis-e-R18-38.213-MC_enh] Review of draft CR by April 26 – Aris (Samsung)

R1-2304193	Summary of email discussions [112bis-e-R18-38.213-NR_MC_enh]	Moderator (Samsung)

R1-2304196	Introduction of multi-carrier enhancements	Samsung	(rev of R1-2303161)

Decision: As per email decision posted on April 25th, the draft CR in R1-2304196 for Rel-18 TS38.213 MC-Enh is endorsed in principle.

[112bis-e-R18-38.214-MC_Enh-Scheduling] Review of draft CR by April 26 – Mihai (Nokia)

R1-2304204	Summary of email discussion on Multi-Cell Scheduling in [112bis-e-R18-38.214-MC_Enh]	Moderator (Nokia)

R1-2304201	Introduction of multi-carrier enhancements	Nokia	(rev of R1-2303012)

Decision: As per email decision, the draft CR in R1-2304201 for Rel-18 TS38.214 MC-Enh CR including only the scheduling/DCI part is endorsed in principle.

Agreement

- Introduce separate FGs for the support of monitoring DCI formats 1_3 and 0_3 as FGs 49-1 and 49-2

  - Note: Some capabilities can be reported separately for DCI formats 1_3 and 0_3, details FFS

  - FFS whether/which capabilities can be commonly applied for DCI formats 1_3 and 0_3, FFS how to report

Agreement

- Introduce FG 49-X as follows

| 49. NR_MC_enh | 49-X | Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands | Indicate supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands - Candidate value set is {switchedUL, dualUL, both} |  | Yes |  | [UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination] | [Per band pair per band combination, details up to RAN2] | [N/A] | [N/A] | [N/A] | This FG is based on the following agreements. RAN1 will not discuss the detail of this FG and the detail is up to RAN2 [Agreement Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination] [Agreement in RAN2#121 For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

From April 20th GTW session

Agreement

- Introduce separate FGs for the support of same and different SCSs between scheduling cell and cells in the set

  - Note: Some capabilities can be reported separately for same and different SCSs, details FFS

  - FFS whether/which capabilities can be commonly applied for same and different SCSs, FFS how to report

  - FFS whether the FG for the support of different SCS is separate or common for DCI format 0_3 and 1_3

Agreement

- Following is reported separately for DCI formats 1_3 and 0_3 as a component of FGs 49-1/1a/1b and 49-2/2a/2b

  - Max number of co-scheduled cells supported by a DCI format for the UE: Candidate value set of {2, 3, 4}

From April 25th GTW session

Agreement:

Introduce FG 49-Y as follows

| 49. NR_MC_enh | 49-Y | Minimum separation time for two uplink switching on more than 2 bands within any two consecutive reference slots | If two uplink switchings are triggered and UL transmissions involved in the two uplink switchings are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching, and X us is reported with a candidate value set of {[0us], 500us} - The reported value X is applied to both one TAG case and two-TAG case (if UE supports two-TAG case) FFS: Note: If the UE reports [0us], the minimum separation time is not applied | 49-X | Yes |  | [two uplink switching cannot be triggered in two consecutive reference slots for UL transmissions on more than 2 bands] | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Introduce following FGs

| 49. NR_MC_enh | 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set [2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.] 3) Scheduling cell and co-scheduled cells have same SCS/carrier type[: candidate value set {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}] 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] FFS whether to report max number of sets of cells supported by UE across PUCCH groups 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [9) Supported types for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}] [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-1a | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) HARQ feedback based on Type 1 HARQ codebook 7) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-1b | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - [Bitmap] indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations FFS: relation between 3a and 3b FFS: whether/how to indicate support of scheduling on unlicensed band(s) 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}. FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [9) Supported types for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}] FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set [2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.] 3) Scheduling cell and co-scheduled cells have same SCS/carrier type[: candidate value set {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}.] 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [8) Supported types for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}] [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2a | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2b | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - [Bitmap] indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations FFS: relation between 3a and 3b FFS: whether/how to indicate support of scheduling on unlicensed band(s) 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} [8) Supported types for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}] FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-3 | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | At least one of {49-1, 49-1a, 49-1b, 49-2, 49-2a, 49-2b} | Yes |  | UE does not support monitoring both legacy DCI format(s) (0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | [Per UE] | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 | [At least one of {49-1, 49-1b, 49-2, 49-2b}] | Yes |  |  | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4b | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | [At least one of {49-1, 49-1b, 49-2, 49-2b}] | Yes |  |  | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5 | Type 2 HARQ CB support for DCI format 1_3 | HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | [Per UE] | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | 10-16 (Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | 25-6 (Enhanced Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on enhanced Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |

RAN1#113

Agreement

- Component 2 in FG 49-1/49-2 is confirmed as: Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.

Agreement

- Candidate value set for component 3 in FG 49-1/49-2 is confirmed as: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}

Agreement

- Component 3 is revised as: Scheduling cell and co-scheduled cells have same SCS/carrier type: value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set

Agreement

- Reporting type of FG 49-1/49-1b/49-2/49-2b is per BC

Agreement

- Component 9 in FG 49-1/49-1b is confirmed as: Supported types Type-2 for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}

- Component 8 in FG 49-2/49-2b is confirmed as: Supported types Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}

- Introduce following FG

| 49. NR_MC_enh | 49-4c | Configurable Type-1A fields for DCI format 0_3/1_3 | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Following component is introduced in FG 49-1

  - The number of unicast DL DCI to process [for a set of cells] configured for multi-cell PDSCH scheduling by a DCI format 1_3

    - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD/TDD scheduling cell

  - FFS whether to count DCI format 1_3 only or both legacy DCI formats and DCI format 1_3

- Introduce advanced UE capability for larger number of unicast DL DCI, details FFS

- Following component is introduced in FG 49-2

  - The number of unicast UL DCI to process [for a set of cells] configured for multi-cell PUSCH scheduling by a DCI format 0_3

    - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD scheduling cell

    - Two unicast DCI per slot of scheduling cell [for the set of cells] for TDD scheduling cell

  - FFS whether to count DCI format 0_3 only or both legacy DCI formats and DCI format 0_3

- Introduce advanced UE capability for larger number of unicast UL DCI, details FFS

Agreement

- Candidate value set for component 3b in FG 49-1b/49-2b is updated as: [Bitmap] Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations

- “FFS: relation between 3a and 3b” is removed from component 3a/3b in FG 49-1b/49-2b

- “FFS: whether/how to indicate support of scheduling on unlicensed band(s)” is removed from component 3a/3b in FG 49-1b/49-2b

Agreement

- Introduce following FG

| 49. NR_MC_enh | 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | At least one of {49-1, 49-1b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-4b | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | At least one of {49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |

Agreement

- Introduce following FG

| 49. NR_MC_enh | 49-4d | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 0_3 2) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 49-X is updated as follows

| 49. NR_MC_enh | 49-X | Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands | Indicate supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands Candidate value set is {switchedUL, dualUL, both} |  | Yes |  | [UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination] | [Per band pair per band combination, details up to RAN2] | [N/A] | [N/A] | [N/A] | This FG is based on the following agreements. RAN1 will not discuss the detail of this FG and the detail is up to RAN2 [Agreement Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination] [Agreement in RAN2#121 For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- If the UE reports 0us in FG 49-Y, the minimum separation time is not applied

  - FFS the consequence if UE does not report FG 49-Y

Ad-hoc chair’s note

- RAN1 does not send reply LS in R1-2304333 from RAN2 in RAN1#113

Agreement

The following is agreed as response to RAN4 LS in R1-2304313.

- When gNB does not provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, and the switching period is located at the switch-to carrier according to the RRC signalling uplinkTxSwitchingPeriodLocation, UE may omit uplink transmission on certain symbol(s) on the switch-to carrier. From RAN1 perspective, T0 is the starting time of uplink transmission from network scheduling perspective.

Final LS is in R1-2306211.

Proposal 3.1 (MCE)

With respect to higher layer parameters for MCE, the value range of nCI-Value is “INTEGER(0…7)”.

- Note: An n_CI value configured for a set of cells can be re-used as a carrier indicator field n_CI for single-cell scheduling of a cell associated with the same scheduling cell

Proposal 3.2 (MCE)

With respect to higher layer parameters for MCE:

- Two FFSs for ScheduledCell-ListDCI-1-3 and ScheduledCell-ListDCI-0-3 are removed.

- Add following notes for ScheduledCell-ListDCI-1-3 and ScheduledCell-ListDCI-0-3

  - When a cell is included in either or both of ScheduledCell-ListDCI-1-3 or ScheduledCell-ListDCI-0-3 for one set of cells MC-DCI-SetofCells, the cell cannot be included in any of ScheduledCell-ListDCI-1-3 or ScheduledCell-ListDCI-0-3 for any other set of cells.

Proposal 3.3a (MCE)

With respect to higher layer parameters for MCE, the minimum size of ScheduledCellCombo-ListDCI-1-3 and ScheduledCellCombo-ListDCI-0-3 is 1, and the FFS for ScheduledCellCombo-ListDCI-1-3 and ScheduledCellCombo-ListDCI-0-3 is removed.

Proposal 3.6 (MCE)

With respect to higher layer parameters for MCE, the minimum size of joint configuration table entries for type-1B fields is 2.

Proposal 3.7-1 (MCE)

With respect to higher layer parameters for MCE, the value range of TDRA-FieldIndexDCI-1-3 is “INTEGER(0…maxNrofDL-Allocations – 1)”.

Proposal 3.7-2 (MCE)

With respect to higher layer parameters for MCE, the value range of TDRA-FieldIndexDCI-0-3 is “INTEGER(0…maxNrofUL-Allocations-r16 – 1)”.

Proposal 3.7-4a (MCE)

With respect to higher layer parameters for MCE, the value range of TCI-DCI-1-3 is “[INTEGER(0…7)]”.

Proposal 3.8 (MCE)

With respect to higher layer parameters for MCE, the value range of numberOfBitsForRV-DCI-1-3 and numberOfBitsForRV-DCI-0-3 is “INTEGER(0…2)”.

Proposal 3.10 (MCE)

With respect to higher layer parameters for MCE, the typo for BandPriority is corrected as “periority”

Proposal 3.10-1 (MCE)

With respect to higher layer parameters for MCE, Column E is moved to column M according to the guidance in R1-2305769.

2.1.2	Remaining Open issues

- Finalize RAN1 CRs

- to introduce support for Multi-cell PUSCH/PDSCH scheduling with a single DCI

- to introduce support for Multi-carrier UL Tx switching scheme

- Finalize RAN1 RRC parameters

- Finalize RAN1 UE capabilities

2.2	RAN2

2.2.1	Agreements

RAN2#121bis-e

P2: RAN2 reuse uplinkTxSwitching-DualUL-TxState-r17 to indicate the state of Tx chains for dualUL mode.

P4: RAN2 introduce an optional list of bands in CellGroupConfig, in which the priority is configured by the order (or similar equivalent change, TBD CR disc).

P5: RAN2 introduce a per-band-pair report of bands that can be transmitted while the other Tx chain is switching across that band pair. Absence of this field means there is interruption in all bands during the switching.

P3-1: R2 assumes that the network ensures the UE supports dualUL for a band and its associated band (config restriction)

we send an LS to RAN1, ask to confirm RAN2 understanding (below)

Baseline R2 understanding:

- When the UE is indicated to switch from two bands to one different band (e.g. A+B => C), follow below logic when determine the switched Tx:

- - If network indicates 1port transmission on band C,

- and uplinkTxSwitching-DualUL-TxState is set to oneT, and the associated band is configured to band C:

- ---- Switch 1Tx chain to band C and switch another Tx chain to associated band;

- - Else if network indicates 1port transmission on band C, but uplinkTxSwitching-DualUL-TxState is not configured or is set to twoT, or associated band is not configured to band C:

- ---- Switching 2Tx chains to band C.

In support of RAN4 agreement, RAN2 intend to introduce support for two per-band-pair UE capabilities, a length of a switching period, for 1Tx-2Tx switching (like Rel-16) and that for 2Tx-2Tx switching (like Rel-17).

FFS if the UE supports 1T-2T, whether the UE need to report this capability for every case (or whether it could/should be inferred from R1617 reporting).

FFS if the absence of 2Tx-2Tx per-band-pair UE capability (switching period) means the UE does not support 2Tx-2Tx UL Tx switching.

For Rel-18 Tx switching, wait for more input from RAN4 on whether to introduce a separate capability for UE transmitting on the Tx chain switched first during the time gap of different switching periods.

RAN2#122

In Rel-18 UL Tx switching, the 3/4 FeatureSetUplink corresponding to the 3/4 UL bands are reported in one row for a given BC including 3/4 UL bands, and fallback and backward compatibility should be supported in the following way:

- - The UE needs to guarantee the FeatureSetUplinks reported for Rel-18 UL Tx switching are applicable to Rel-16/Rel-17 Tx switching (as the Rel-16/Rel-17 switching period is reported for that band pair and the same switching option of the band pair is supported for Rel-16/Rel-17 switching).

- - The UE can report FSC row for Rel-16/Rel-17 UL Tx switching explicitly if the Rel-16/Rel-17 switching period is reported for that band pair in case of different fallback.

RAN2 confirm the intention that Rel-16 band pair list is reused to indicate Rel-18 per-band pair capability which is the same as in Rel-17.

RAN2 to introduce a per-BC capability of Minimum separation time. The exact values of the capability is pending to RAN1.

For Rel-18 UL Tx switching (1T-1T and/or 1T-2T and/or 2T-2T) across 3 or 4 bands the UE shall indicate the support of UL Tx switching (e.g. at least switchedUL) for ALL possible band pairs.

Allow the UE to report switching period for a band pair in which the two bands do not support 2-layers UL MIMO.

For MCE, RAN2 completed functionality parts, but is waiting for RAN1 for configuration parameters for multi-cell-scheduling

2.2.2	Remaining Open issues

- Finalize RAN2 CRs to introduce support for Multi-cell PUSCH/PDSCH scheduling with a single DCI.

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#106bis-e

The following T-docs were approved:

R4-2306621	WF on Rel-18 Tx switching across 3/4 bands

R4-2306660	WF on ambiguity issue when two Tx chains are switched between different band pairs

R4-2306623	LS on Rel-18 Tx switching across 3/4 bands

R4-2306354	WF on RRM requirements for MC enhancements for NR

=============================================================================

Content of R4-2306621 WF on Rel-18 Tx switching across 3/4 bands:

- Way forward on Tx switching with single TAG

Sub-topic 1-1: Exact location of switching period

Way forward after round 2 discussion:

- For Rel-18 Tx switching, further discuss the following options taking into account RAN1 response:

  - Option 1: the switching period immediately precedes the time T0 at which a transmission starts on a carrier following a Tx switch from a preceding transmission

    - Option 1a: the switching period ends at time T0

  - Option 2: define the time relation of the switching period location and T0 for two cases with the switching period located on the switch-from carrier and switch-to carrier respectively.

    - The switching period location is determined based on the following RAN1 agreement:

The gNB configures priority for each band. The UE determines the switching period location on either switching-from band(s) or switching-to band(s) that is involved in the UL Tx switching and is not with the highest priority band.

  - Option 3: No need to define the time relationship between the location of the switching period and T_0 in RAN4 specification

Sub-topic 1-2: Switching case across four bands, i.e., {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T}

Issue 1-2-1: Feasibility of this switching case

WF:

- The switching scenario of {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} is feasible from RAN4 perspective.

Issue 1-2-2:  Location of switching period

- Issue: Is the switching period location in the following figure possible?

WF:

- It is observed that the location of switching period in the figure above is not valid since it is not aligned with the RAN1 agreement.

Issue 1-2-3: Advanced optional UE ability

Way forward after round 2 discussion:

- Further discuss in the next meeting:

  - For the switching scenario of {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}, discuss and decide whether or not to introduce the advanced optional UE ability to allow one Tx chain to start transmission earlier if the switching time for this Tx chain is shorter.

Sub-topic 1-3: Switching case across three bands, i.e., switch between {1T, 1T, 0T} and {0T, 0T, 2T}

Issue 1-3-1: Feasibility of this switching case

Way forward:

- From RAN4 perspective, the switching between {1T, 1T, 0T} and {0T, 0T, 2T} is possible.

Issue 1-3-2: Advanced optional UE ability

- Further discuss the following options in the next meeting

  - Option 1: For the switching scenario without ambiguity, i.e., the two Tx chains are on the same band before or after switching (switch between {1T, 1T, 0T} and {0T, 0T, 2T}), introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1).

    - Tswitch_1 and Tswitch_2 are the switching periods of Tx chain #1 and Tx chain #2 respectively, and Tswitch_1 < Tswitch_2.

    - Option 1a: apply the same UE capability for this scenario and for the following scenario agreed in the previous meetings. (MTK, China Telecom, ZTE)

RAN4 LS approved in R4-2303507:

When one of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), the other Tx chain is maintained on a different band (named “band C” or “band D” in the case of 4-band) and the number of Tx chain on band C or band D is unchanged due to the switching, RAN4 agreed the granularity of the optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged  during UL switching as follows:

- Per band (only for the band(s) in the band combination but not included in the pair of bands before and after switching) for each pair of bands before and after switching in each band combination.

  - Option 2: Not introduce advanced optional UE ability

Sub-topic 1-4: Length of switching time for certain scenarios

Issue 1-4-1: Switching case across four bands, i.e., {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T}

Way forward after round 2 discussion:

- Further discuss in the next meeting:

  - Option A: As optional UE behaviour, total switching period can be extended if UE is not capable for concurrent TX switching on the two TX chains.

    - Option A1: add new values {70, 175} usec in addition to the agreed set of {35 us, 140 us, 210 us}

    - Option A2: Sum of two switching periods

  - When UE is scheduled to switch two TX chains in such way that switching periods may overlap, the switching period is extended for both band pairs and total switching time is the sum of possible switching periods for the band pairs involved.

  - When UE is scheduled for transmissions so that the switching is from two bands with one TX each to another two bands one TX each, denoted for example A+B to C+D, and it cannot be determined if UE switches TX chains from A to C or D or from B to C or D, the switching time is sum of max{Tswitch_A-C, Tswitch_A-D,} and max{Tswitch_B-C, Tswitch_B-D}.

  - Include clearly the aspect that when two TX chains are switched with different lengths of the switching periods, none of the TX chains are expected to be used for transmissions.

  - Option B: Keep the previous agreements

Issue 1-4-2: Switching case across three bands, i.e., {1T, 1T, 0T} to {0T, 0T, 2T}

Way forward after round 2 discussion:

- Further discuss in the next meeting:

  - Option A: As optional UE behaviour, total switching period can be extended if UE is not capable for concurrent TX switching on the two TX chains.

    - Option A1: add new values {70, 175} usec in addition to the agreed set of {35 us, 140 us, 210 us}

    - Option A2: Sum of two switching periods

  - When UE is scheduled to switch two TX chains in such way that switching periods may overlap, the switching period is extended for both band pairs and total switching time is the sum of possible switching periods for the band pairs involved.

  - Option B: Keep the previous agreements

Issue 1-4-3: The unaffected band case

Way forward after round 2 discussion:

- Further discuss in the next meeting:

  - Option 1: When UE is scheduled for UL transmissions on any band other than the bands involved in switching, the switching time is the sum of switching periods indicated for the band pair involved in switching and the band pair including the unaffected band and the switch-to band

  - Option 2: For the two band pairs switching with one common band, e.g., band pair A+B and band pair B+C, the band B is involved in switching event and it is not the unaffected band.

  - Option 3: The previous agreement should be reused

Sub-topic 1-5: 2-layer UL-MIMO support for carrier(s) capable of 2Tx

Way forward after round 2 discussion:

- Further discuss in the next meeting:

  - Option 1: Mandate 2-layer UL-MIMO support for carrier(s) capable of 2Tx

    - DCM: RAN2 agreed that UL MIMO capability is used for UE to indicate 2Tx support. It means that UE need to indicate UL MIMO capability if UE wants to indicate 2Tx capability.

  - For UE capability of 2-port UL transmission, RAN2 reuse the per-FS UL-MIMO UE capability (no spec change).

  - Option 2: Not mandate 2-layer UL-MIMO support for carrier(s) capable of 2Tx

Sub-topic 1-6: Fallback of Rel-18 Tx switching to Rel-16/17 Tx switching

- Captured in the LS

Sub-topic 1-7: Band configurations for Rel-18 Tx switching

- Further discuss the necessity of introducing the band configurations for UL Tx switching across 3 and 4 bands in the specification in the next meeting.

Sub-topic 1-8: Applicability of DL interruption

Way forward:

- Clarify that “no DL interruption applies to Tx switching across 2 UL bands” in the DL_intrpt_combos_TxSW_R18 WI in the next meeting.

- Further discuss for Rel-18 Tx switching across 3 or 4 UL bands.

Sub-topic 1-9: General aspects of the CR

Way forward after round 2 discussion:

- For the time mask figures in the CR, it is proposed to change “NR slot/subslot” to “NR symbol(s)” in the next meeting.

Way forward on Tx switching with dual TAGs

Sub-topic 2-1: RF specification impact

Further discuss the following options in the next meeting:

- Option 1: time masks are also included also for the dual-TAG case to make clear the following

- Option 2: Do not modify the time mask for Tx switching for multiple TAGs. The impact of Tx switching with multiple TAGs can be considered as scheduling restriction.

Option 3: The duration for UE omitting uplink transmission is switching period+1 symbol.

=============================================================================

Content of R4-2306660 WF on ambiguity issue when two Tx chains are switched between different band pairs

    - RAN4 maintains the baseline assumption in Issue 1-2-3 agreed in R4-2220546 during RAN4#105.

    - Neither of the two Tx chains is expected to be used for transmission during the switching periods in Rel-18

    - Encourage companies to study the benefit to resolve the switching pattern ambiguity issue when 2 Tx chains are switched between two different band pairs for one Tx switching instance

      - At least for single TAG

      - FFS whether it applies to dual TAG

=============================================================================

Content of R4-2306354 WF on RRM requirements for MC enhancements for NR:

< Agreement >Issue 1-1: If one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously

Wait for RF agreement on the length of switching time for simultaneous switching of two band pairs.

< Agreement >Issue 1-2: Additional signalling for Tx switching with 2 TAGs

RTD value reporting is not preferred.

< Agreement >Issue 1-3: Minimum Separation Time between two consecutive UL Tx Switching

The following is agreed in RAN1#112: Confirm the working assumption with following updates (working assumption) If two uplink switching are triggered and UL transmissions involved in the two uplink switching are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching. - X us is subject to UE capability with a value set of {0us, 500us}

No RRM impact is observed regarding minimum separation time between two consecutive UL Tx Switching.

=============================================================================

RAN4#107

The following T-docs were approved/agreed:

R4-2310270	CR for 38.101-1: Time mask for switching across three or four uplink bands

R4-2310263	Ad hoc miniutes on multi-carrier operations

R4-2310271	Reply LS on report of switching periods in Rel-18 uplink Tx switching

R4-2310495	LS on multi-carrier enhancement

R4-2310496	WF on ambiguity issues for Tx switching accorss four bands

R4-2310142	DL interruption for Tx switching across 3/4 bands

The following T-doc was endorsed:

R4-2307159	draftCR for 38.101-1 to clarify the time mask for switching with multiple TAGs

=============================================================================

The content of  R4-2310496 WF on ambiguity issues for Tx switching accorss four bands

Agreement

- Views on the benefit:

If the UE is able to perform Tx switching with the band pairs that are corresponded to shorter switching period, the UE could utilize more resources to transmit uplink signal/channels compared to baseline.

  - For example, if UE reports

max{Tswitch_A-C, Tswitch_B-D}=SW1

max{Tswitch_A-D, Tswitch_B-C}=SW2, where SW2 > SW1

It can earn ∆2-1 for uplink transmission utilization improvement.

The switching period can possible be improved as min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}

- Proposals on the solutions to resolve the switching pattern ambiguity issue:

- RAN4 to agree on the band ordering based approach to resolve the ambiguity issue for the parallel switching of two Tx chain in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}:

    - Associating the ordering of bands for defining switch-from and switch-to pairs in switching configuration commands may resolve the ambiguity issue without additional cost.

    - Introduce new per Tx chain-based Tx switching capability

    - Introduce optional UE capability on supporting the band ordering based approach to resolve ambiguity issue

- Supporting the capability means the switching period is improved to min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}

=============================================================================

2.4.2	Remaining Open issues

Finalize RAN4 CRs to accommodate potential corrections/clarifications for Multi-carrier UL Tx switching scheme.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#112bis-e

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2302266 | LS on Rel-18 Multi-carrier enhancement for NR | RAN4, China Telecom |
| R1-2302386 | Discussion on UL Tx switching across 3 or 4 bands in Rel-18 | Huawei, HiSilicon |
| R1-2302446 | Draft LS reply on Rel-18 Multi-carrier enhancement for NR | vivo |
| R1-2302515 | Discussion on UE features for Multi-carrier enhancements | vivo |
| R1-2302518 | Discussion on Rel-18 Multi-carrier enhancements, eDSS, NCR and BWP without restriction | vivo |
| R1-2302577 | Discussion on UE features for multi-carrier enhancement | OPPO |
| R1-2302579 | Discussion on remaining issues in Rel-18 eDSS and MC-enh | OPPO |
| R1-2302639 | Discussion on RAN4 LS on Rel-18 Multi-carrier enhancement for NR | CATT |
| R1-2302741 | Introduction of multi-carrier enhancements | Ericsson LM |
| R1-2302754 | [Draft] Reply LS on Rel-18 Multi-carrier enhancement for NR | ZTE |
| R1-2302763 | Discussion on UE feature for MC enhancements | ZTE |
| R1-2302777 | Discussions on reply LS on Rel-18 multi-carrier enhancement | Intel Corporation |
| R1-2302820 | On remaining issues for R18 NCR and MC-Enh | Intel Corporation |
| R1-2302897 | Initial views on UE features for Multi-carrier Enhancements | Nokia, Nokia Shanghai Bell |
| R1-2302955 | [Draft] Reply LS on Rel-18 Multi-carrier enhancement for NR | xiaomi |
| R1-2303012 | Introduction of multi-carrier enhancements | Nokia, Nokia Shanghai Bell |
| R1-2303013 | Introduction of UL Tx switching across up to 4 bands | Nokia, Nokia Shanghai Bell |
| R1-2303159 | Discussion on UE features for multi-carrier enhancements | Samsung |
| R1-2303161 | Introduction of multi-carrier enhancements for NR | Samsung |
| R1-2303165 | Discussion of RAN4 LS on Rel-18 Multi-carrier enhancement for NR | Spreadtrum Communications |
| R1-2303166 | Discussion on remaining issues for eDSS and MC | Spreadtrum Communications |
| R1-2303327 | On maintenance for MC-Enhancement | MediaTek Inc. |
| R1-2303343 | On UE feature discussion for Rel-18 MC enhancements | MediaTek Inc. |
| R1-2303404 | Draft CRs and higher layer signalling for others | ZTE |
| R1-2303431 | Remaining aspects on Rel-18 NCR and Multi-carrier enhancements | LG Electronics |
| R1-2303462 | Draft reply LS to RAN4 on Rel-18 multi-carrier enhancements for NR | Apple |
| R1-2303512 | Initial views on UE features for Rel-18 multi-carrier enhancements | Apple |
| R1-2303514 | On remaining issues for Rel-18 multi-carrier enhancements | Apple |
| R1-2303562 | Draft Reply to LS on RAN4 LS on Multi-Carrier enhancement for NR | Qualcomm Incorporated |
| R1-2303621 | UE features for MC enhancements | Qualcomm Incorporated |
| R1-2303624 | Views on MC-Enh, BWP without restriction, and eDSS | Qualcomm Incorporated |
| R1-2303629 | Discussion on RAN4 LS for multi-carrier enhancement | OPPO |
| R1-2303689 | Discussion on reply LS on Multi-carrier enhancement for NR | NTT DOCOMO, INC. |
| R1-2303735 | Draft RAN1 UE features list for Rel-18 Multi-carrier enhancements for NR | NTT DOCOMO, INC. |
| R1-2303736 | Discussion on UE features for MC enhancements | NTT DOCOMO, INC. |
| R1-2303739 | Draft RAN1 higher layer parameters list for Rel-18 Multi-carrier enhancements for NR | NTT DOCOMO, INC. |
| R1-2303740 | Discussion on remaining issues on MC enhancements, eDSS and BWP without restriction | NTT DOCOMO, INC. |
| R1-2303762 | UE features for MCE | Ericsson |
| R1-2303766 | Discussion on MCE, eDSS and NCR for Rel18 | Ericsson |
| R1-2303803 | Introduction of Rel-18 Multi-carrier enhancements | Huawei, HiSilicon |
| R1-2303856 | Draft reply LS on UL Tx switching across 3 or 4 bands in Rel-18 | Huawei, HiSilicon |
| R1-2303863 | UE features for MC enhancements | Huawei, HiSilicon |
| R1-2304137 | [112bis-e-LS-03] FL summary of email discussion on reply LS for Rel-18 Tx switching | Moderator (China Telecom) |
| R1-2304193 | Summary of email discussions [112bis-e-R18-38.213-NR_MC_enh] | Moderator (Samsung) |
| R1-2304196 | Introduction of multi-carrier enhancements | Samsung |
| R1-2304201 | Introduction of multi-carrier enhancements | Nokia |
| R1-2304204 | Summary of email discussion on Multi-Cell Scheduling in [112bis-e-R18-38.214-MC_Enh] | Moderator (Nokia) |
| R1-2304205 | Summary of email discussion on the introduction of UL Tx switching across up to 4 bands in [112bis-e-R18-38.214-MC_Enh] | Moderator (Nokia) |
| R1-2304213 | Summary on UE features for MC enhancements | Moderator (NTT DOCOMO, INC.) |
| R1-2304219 | Reply LS on Rel-18 Multi-carrier enhancement for NR | RAN1, China Telecom |
| R1-2304220 | Summary of Email discussion on Rel-18 RRC parameters for LS to RAN2 and RAN3 | Moderator (Ericsson) |
| R1-2304221 | Collection of updated higher layers parameter list for Rel-18 NR | Moderator (Ericsson) |
| R1-2304224 | Session Notes for 9.17.10 | Ad-Hoc Chair (NTT DOCOMO, INC.) |
| R1-2304263 | Introduction of Rel-18 Multi-carrier enhancements | Huawei |
| R1-2304265 | Summary of email discussion [112bis-e-R18-38.212-NR_MC_enh] | Moderator (Huawei) |
| R1-2304278 | Introduction of multi-carrier enhancements | Ericsson |

RAN1#113

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2304313 | LS on Rel-18 Tx switching across 3/4 bands | RAN4, China Telecom |
| R1-2304327 | LS on RRC configuration of Tx state in Rel-18 UL Tx switching | RAN2, NTT DOCOMO |
| R1-2304333 | LS on report of switching periods in Rel-18 UL Tx switching | RAN2, NTT DOCOMO |
| R1-2304454 | Draft LS reply on report of switching periods in Rel-18 UL Tx switching | vivo |
| R1-2304455 | Discussion on report of switching periods in Rel-18 UL Tx switching | vivo |
| R1-2304456 | Draft LS reply on Rel-18 Tx switching across 3/4 bands | vivo |
| R1-2304457 | Discussion on Rel-18 Tx switching across 3/4 bands | vivo |
| R1-2304513 | Discussion on UE features for Multi-carrier enhancements | vivo |
| R1-2304517 | Discussion on Rel-18 Multi-carrier enhancements, eDSS and NCR | vivo |
| R1-2304584 | Discussion on UE features for multi-carrier enhancement | Spreadtrum Communications |
| R1-2304586 | Discussion on remaining issues in RRC signaling for Rel-18 multi-carrier enhancement and eDSS | Spreadtrum Communications |
| R1-2304591 | [Draft] Reply LS on Rel-18 Tx switching across 3/4 bands | ZTE |
| R1-2304592 | [Draft] Reply LS on RRC configuration of Tx state in Rel-18 UL Tx switching | ZTE |
| R1-2304593 | [Draft] Reply LS on report of switching periods in Rel-18 UL Tx switching | ZTE |
| R1-2304603 | Discussion on UE feature for MC enhancements | ZTE |
| R1-2304607 | Draft CRs and higher layer signalling for others | ZTE |
| R1-2304608 | On MC-enhancements and BWP without restrictions | Nokia, Nokia Shanghai Bell |
| R1-2304671 | UE features for MC enhancements | Huawei, HiSilicon |
| R1-2304689 | Discussion on RAN4 LS on Rel-18 Tx switching across 3/4 bands | CATT |
| R1-2304690 | Discussion on RAN2 LS on RRC configuration of Tx state in Rel-18 UL Tx switching | CATT |
| R1-2304691 | Discussion on RAN2 LS on report of switching periods in Rel-18 UL Tx switching | CATT |
| R1-2304801 | On remaining issues for R18 NCR and MCE | Intel Corporation |
| R1-2304853 | [Draft] Reply LS on Rel-18 Multi-carrier enhancement | China Telecom |
| R1-2304868 | On CR for the introduction of UL Tx switching across up to 4 bands | China Telecom |
| R1-2304872 | [Draft] Reply to LS on Rel-18 Tx switching | xiaomi |
| R1-2304891 | Disucssion on UE features for MC enhancements | xiaomi |
| R1-2304961 | Draft reply LS on Rel-18 Tx switching across 3/4 bands | Spreadtrum Communications |
| R1-2305140 | Discussion on reply LS to RAN4 on UL Tx switching across 3 or 4 bands in Rel-18 | LG Electronics |
| R1-2305149 | Discussion on UE features for MC enhancements | LG Electronics |
| R1-2305150 | Remaining aspects on Rel-18 NCR and MC enhancements | LG Electronics |
| R1-2305193 | Discussion on RAN4 LS on Rel-18 Tx switching across 3/4 bands | MediaTek Inc. |
| R1-2305217 | Draft reply LS on report of switching periods in Rel-18 UL Tx switching | Apple |
| R1-2305218 | Draft reply LS on Rel-18 Tx switching across 3/4 bands | Apple |
| R1-2305280 | On UE features for Rel-18 multi-carrier enhancements | Apple |
| R1-2305284 | Views on draft CR for Rel-18 multi-carrier enhancements | Apple |
| R1-2305311 | Discussion on RAN4 LS of Rel-18 UL Tx switching timeline | Qualcomm Incorporated |
| R1-2305312 | Discussion on RAN2 LS on switching periods in Rel-18 | Qualcomm Incorporated |
| R1-2305373 | UE features for MC enhancements | Qualcomm Incorporated |
| R1-2305378 | Views on draft CRs and higher layer signalling for R18 MC enhancements | Qualcomm Incorporated |
| R1-2305471 | Discussion on UE features for multi-carrier enhancement | OPPO |
| R1-2305473 | Discussion on RAN4 LS for Rel-18 Tx switching | OPPO |
| R1-2305493 | Discussion on RAN2 and RAN4 LS on Rel-18 UL Tx switching | Samsung |
| R1-2305545 | UE features for multi-carrier enhancements | Samsung |
| R1-2305548 | Discussions on higher layer signalling for multi-carrier enhancements | Samsung |
| R1-2305578 | UE features for MCE | Ericsson |
| R1-2305580 | Discussion on LSs from RAN2/4 on Rel-18 UL Tx switching | NTT DOCOMO, INC. |
| R1-2305624 | Discussion on UE features for Multi-carrier enhancements | NTT DOCOMO, INC. |
| R1-2305627 | Discussion on higher layer parameters and remaining issues on Rel-18 WIs | NTT DOCOMO, INC. |
| R1-2305661 | On UE feature discussion for Rel-18 MC enhancements | MediaTek Inc. |
| R1-2305675 | On maintenance for MC-Enhancement of multi-cell scheduling | MediaTek Inc. |
| R1-2305716 | Higher layer signaling for NR Rel-18 | Ericsson |
| R1-2305724 | On UE features for Multi-carrier Enhancements | Nokia, Nokia Shanghai Bell |
| R1-2305861 | Reply LS on Rel-18 Tx switching across 3/4 bands | Nokia, Nokia Shanghai Bell |
| R1-2305862 | Reply LS on report of switching periods in Rel-18 UL Tx switching | Nokia, Nokia Shanghai Bell |
| R1-2305866 | [Draft] Reply LS on Rel-18 Tx switching across 3/4 bands | Ericsson |
| R1-2305908 | Discussion on remaining issues of Rel-18 multi-carrier enhancement | Lenovo |
| R1-2305930 | Discussion on RRC configuration of Tx state in Rel-18 UL Tx switching | Huawei, HiSilicon |
| R1-2305931 | Discussion on UL Tx switching across 3 or 4 bands in Rel-18 | Huawei, HiSilicon |
| R1-2305944 | Discussion on report of switching periods in Rel-18 UL Tx switching | Huawei, HiSilicon |
| R1-2306121 | Summary#1 of discussion on RAN2 LS in R1-2304327 | Moderator (NTT DOCOMO, INC.) |
| R1-2306122 | Summary#1 of discussion on higher layer signalling for MC-Enh | Moderator (NTT DOCOMO, INC.) |
| R1-2306175 | [Draft] Reply LS on Rel-18 Multi-carrier enhancement for NR | Moderator (China Telecom) |
| R1-2306176 | FL summary of discussion on reply LS for Rel-18 Tx switching | Moderator (China Telecom) |
| R1-2306185 | Session Notes for 9.16.9 | Ad-Hoc Chair (NTT DOCOMO) |
| R1-2306191 | Summary on UE features for MC enhancements | Moderator (NTT DOCOMO, INC.) |
| R1-2306196 | [Draft] Reply LS on RRC configuration of Tx state in Rel-18 UL Tx switching | Moderator (NTT DOCOMO) |
| R1-2306211 | Reply LS on Rel-18 Multi-carrier enhancement for NR | RAN1, China Telecom |
| R1-2306220 | Initial RAN1 UE features list for Rel-18 LTE after RAN1#113 | Moderators (AT&T, NTT DOCOMO, INC.) |
| R1-2306223 | Updated RAN1 UE features list for Rel-18 NR after RAN1#113 | Moderators (AT&T, NTT DOCOMO, INC.) |
| R1-2306256 | Summary on the [113-R18-Others-02] Email discussion on MC-Enh draft CR for TS38.214 | Nokia |
| R1-2306267 | Summary of Email discussion on LS for Rel-18 higher layer parameters | Moderator (Ericsson) |
| R1-2306268 | Collection of higher layers parameter list for Rel-18 | Moderator (Ericsson) |
| R1-2306269 | DRAFT LS on Rel-18 higher-layers parameter list | Moderator (Ericsson) |
| R1-2306270 | LS on Rel-18 higher-layers parameter list | RAN1, Ericsson |
| R1-2306271 | Consolidated higher layers parameter list for Rel-18 | Moderator (Ericsson) |

RAN2#121bis-e

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2302433 | LS on Rel-18 Multi-carrier enhancement for NR (R4-2303507; contact: China Telecom) | RAN4 |
| R2-2302578 | Discussion on R18 UL Tx Switching | OPPO |
| R2-2302714 | Discussion on Rel-18 UL Tx Switching | CATT |
| R2-2302730 | Summary of [Post121][045][MCE] UL TX Switching (Docomo) | NTT DOCOMO INC. |
| R2-2303063 | Current status of issues on Rel-18 UL Tx switching | NTT DOCOMO INC. |
| R2-2303293 | Discussion on Rel-18 UL Tx switching capability | ZTE Corporation, Sanechips |
| R2-2303399 | UL Tx switching scenarios and 1T-1T band pairs | Apple |
| R2-2303484 | Introduction of Rel-18 UL Tx switching enhancements | Huawei, HiSilicon, NTT DOCOMO INC. |
| R2-2303485 | Introduction of Rel-18 UL Tx switching enhancements | Huawei, HiSilicon, NTT DOCOMO INC. |
| R2-2303664 | On RAN2 aspects for UL TX switching Rel-18 | Ericsson |
| R2-2303825 | discussion on UE capability and RRC configuration for UL tx switching | vivo |
| R2-2304472 | LS on RRC configuration of Tx state in Rel-18 UL Tx switching | RAN2 |
| R2-2304473 | Summary of [AT121bis-e][021][MCE] UL TX Switching (NTT Docomo) | NTT DOCOMO, INC. |
| R2-2304567 | LS on report of switching periods in Rel-18 UL Tx switching | RAN2 |

RAN2#122

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2304645 | LS on Rel-18 Tx switching across 3/4 bands (R4-2306623; contact: China Telecom) | RAN4 |
| R2-2304671 | Discussion on R18 UL Tx Switching | OPPO |
| R2-2305242 | Discussion on Rel-18 UL Tx Switching | CATT |
| R2-2305339 | discussion on UL tx switching | vivo |
| R2-2305398 | Discussion on Rel-18 UL Tx switching capability | ZTE Corporation, Sanechips |
| R2-2305844 | Fallback compatibility for UL Tx switching Rel-18 | Ericsson |
| R2-2306172 | Leftover issues in UL Tx switching | Apple |
| R2-2306186 | RRC configuration for Rel-18 UL Tx switching enhancements | Huawei, HiSilicon, NTT DOCOMO INC. |
| R2-2306187 | UE capability reporting for Rel-18 UL Tx switching enhancements | Huawei, HiSilicon, NTT DOCOMO INC. |
| R2-2306188 | UE capabilities of Rel-18 UL Tx switching enhancements | Huawei, HiSilicon, NTT DOCOMO INC. |
| R2-2306189 | Remaining issues on RAN2 signalling design for Rel-18 UL Tx switching enhancements | Huawei, HiSilicon |
| R2-2306432 | Remaining issues on Rel-18 UL Tx switching | NTT DOCOMO INC. |

RAN4#106bis-e

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2304339 | On UL MIMO and Tx switching capabilities | Apple |
| R4-2304162 | Simultaneous switching | Qualcomm Incorporated |
| R4-2304338 | On switching period in dualUL cases | Apple |
| R4-2304340 | Rel-18 band configurations and fallbacks for TX Switching across 3 or 4 bands | Apple |
| R4-2304401 | Remaining issues for NR Multi-carrier enhancements | China Telecom |
| R4-2304402 | CR for 38.101-1: Time mask for switching across three or four uplink bands | China Telecom, [NTT DOCOMO, Huawei, Hisilicon, CMCC, OPPO, ZTE] |
| R4-2304465 | On the time masks for 3-4 band switching, location of T_0 and support of fallbacks | Ericsson |
| R4-2304873 | Discussion on remaining issues on Tx switching | MediaTek Inc. |
| R4-2304874 | Draft LS on Rel-18 UL Tx switching | MediaTek Inc. |
| R4-2305088 | Further discussion on RF aspects of UL Tx switching with single TAG | vivo |
| R4-2305129 | On Tx switching across 3 or 4 bands for single TAG | ZTE Corporation |
| R4-2305185 | Views on Rel-18 Tx switching | NTT DOCOMO INC. |
| R4-2305252 | Discussion on UL Tx switching | Xiaomi |
| R4-2305447 | Discussion on Multi-carrier enhancements with single TAG | Huawei, HiSilicon |
| R4-2305789 | 3 and 4 band TX switching open issues and requirements | Qualcomm Incorporated |
| R4-2305808 | draft CR on TX Switching when UE is configured for 3 or 4 bands UL | Qualcomm Incorporated |
| R4-2304466 | Time masks for Tx switching with dual TAG | Ericsson |
| R4-2305448 | Discussion on Multi-carrier enhancements with multiple TAG | Huawei, HiSilicon |
| R4-2305449 | draftCR for 38.101-1 to clarify the time mask for switching with multiple TAGs | Huawei, HiSilicon |
| R4-2304124 | DL interruption for Tx switching across 3/4 bands | Nokia, Nokia Shanghai Bell |
| R4-2304125 | DL interruption for Tx switching across 3/4 bands | Nokia, Nokia Shanghai Bell |
| R4-2304170 | DL interruption for Tx switching across 3/4 bands | ZTE Corporation |
| R4-2304290 | On DL interruption for Tx switching across 3/4 bands | Apple |
| R4-2304358 | Multi-carrier enhancement RRM | Qualcomm, Inc. |
| R4-2304623 | Discussion on DL interruption for Tx switching across 3&4 bands | MediaTek inc. |
| R4-2305049 | Remaining issues on DL interruption for Tx switching across 3 to 4 bands | vivo |
| R4-2305282 | Discussion on DL interruption for multi-carrier enhancements | Huawei, HiSilicon |
| R4-2305283 | DL interruption for Tx switching across 3/4 bands | Huawei, HiSilicon |
| R4-2306165 | Topic summary for [106-bis-e][217] NR_MC_enh | Moderator (Huawei) |
| R4-2306218 | Topic summary for [106-bis-e][139] NR_MC_enh_UERF | Moderator (China Telecom) |
| R4-2306248 | Topic summary for [106-bis-e][217] NR_MC_enh | Moderator (Huawei) |
| R4-2306301 | Topic summary for [106-bis-e][139] NR_MC_enh_UERF | Moderator (China Telecom) |
| R4-2306354 | WF on RRM requirements for MC enhancements for NR | Huawei, HiSilicon |
| R4-2306621 | WF on Rel-18 Tx switching across 3/4 bands | China Telecom |
| R4-2306622 | WF on ambiguity issue when two Tx chains are switched between different band pairs | MediaTek |
| R4-2306623 | LS on Rel-18 Tx switching across 3/4 bands | China Telecom |
| R4-2306660 | WF on Tx switching ambiguity issue | MediaTek Inc. |

RAN4#107

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2308161 | Discussion on draft reply LS on report of switching periods in Rel-18 UL Tx switching | ZTE Corporation |
| R4-2308245 | [Draft] Reply LS on report of switching periods in Rel-18 UL Tx switching | vivo |
| R4-2309480 | Rel-18 band configurations for TX Switching across 3 or 4 bands | Apple, Ericsson, Qualcomm |
| R4-2307157 | On ambiguity issue of the exact switching band pairs | Huawei, HiSilicon |
| R4-2307156 | Discussion on Multi-carrier enhancements with single TAG | Huawei, HiSilicon |
| R4-2307254 | Remaining issues for NR Multi-carrier enhancements | China Telecom |
| R4-2307255 | CR for 38.101-1: Time mask for switching across three or four uplink bands | China Telecom, [NTT DOCOMO, Huawei, Hisilicon, CMCC, OPPO, ZTE] |
| R4-2307256 | Draft reply LS on the report of switching periods in Rel-18 UL Tx switching | China Telecom |
| R4-2307304 | Views on Rel-18 Tx switching | NTT DOCOMO, INC. |
| R4-2307742 | On the time masks for Tx switching between 3-4 bands | Ericsson |
| R4-2308162 | On Tx switching across 3 or 4 bands for single TAG | ZTE Corporation |
| R4-2308244 | Further discussion on RF aspects of UL Tx switching with single TAG | vivo |
| R4-2308929 | Discussion on remaining issues on Tx switching | MediaTek Inc. |
| R4-2308930 | Draft LS on Rel-18 UL Tx switching | MediaTek Inc. |
| R4-2308971 | R18 Tx switching across 3 and 4 bands | OPPO |
| R4-2309003 | Discussion on UL Tx switching | Xiaomi |
| R4-2309482 | On UL MIMO and Tx switching | Apple |
| R4-2309683 | UE Switching time in more complicated cases | Qualcomm Incorporated |
| R4-2307158 | Discussion on Multi-carrier enhancements with multiple TAG | Huawei, HiSilicon |
| R4-2307159 | draftCR for 38.101-1 to clarify the time mask for switching with multiple TAGs | Huawei, HiSilicon, Xiaomi |
| R4-2307743 | The dual-TAG case | Ericsson |
| R4-2307744 | Time masks for uplink Tx switching with dual TAG | Ericsson |
| R4-2309479 | On Time T0, switching period location, time masks for TX switching with multi-TAG | Apple |
| R4-2310503 | CR for 38.101-1 to clarify the time mask for switching with multiple TAGs | Huawei, HiSilicon, Xiaomi |
| R4-2308334 | DL interruption for Tx switching across 3/4 bands | Huawei, HiSilicon |
| R4-2308601 | DL interruption for Tx switching across 3/4 bands | Nokia, Nokia Shanghai Bell |
| R4-2309478 | DL interruption for 4-band UL TX switching | Apple |
| R4-2309967 | Topic summary for [107][222] NR_MC_enh | Moderator (Huawei) |
| R4-2310025 | Topic summary for [107][142] NR_MC_enh_UERF | Moderator (China telecom) |

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template