3GPP TSG RAN Meeting #100																			               RP-231260

Taipei, June 12 - 14, 2023

Source:	CATT, ZTE, Ericsson

Title:	Clarification of the scope of (e)DRX in Rel-18 Expanded and Improved NR Positioning

Agenda item:	9.3.1.9

Document for:		Decision

Introduction

The following objective of LPHAP on eDRX can be found in RP-223549[1]:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - Specify solutions for alignment between eDRX and PRS configurations [RAN2].

In the last RAN2#122 meeting, companies have different views on the above objective of “Specify solutions for alignment between eDRX and PRS configurations [RAN2]”. Some companies believe that the “eDRX” in the objective means “(e)DRX”, i.e., including both “DRX” and eDRX”. Actually, RAN2 has recommended the alignment between DRX and PRS configurations in TR 38.859[2]. Thus, both eDRX and DRX should be supported in LPHAP. It is unclear why WID only includes alignment between eDRX and PRS configurations. However, some other companies think the “DRX” is not included and prefer to discuss the scope of the  eDRX issue in RAN plenary. This contribution aims to address the issue and propose a clarification.

Discussion

Considering not all devices can support eDRX in Rel-18, both eDRX and DRX can be used in Rel-18 LPHAP. During the study item, RAN1 proposed to support extending eDRX and RAN2 proposed to support the aligment between DRX and DL-PRS configuration in TR 38.859[2].

However, according to the WID[1], only eDRX is aligned with PRS configuration.

Specify solutions for alignment between eDRX and PRS configurations [RAN2].

We believe the aligment between paging cycles and PRS configurations can be applicable  to both eDRX paging cycles and DRX paging cycles in LPHAP, based on the following reasons:

- R18 devices supporting LPHAP may support either eDRX paging cycles in RRC_INACTIVE mode, or DRX paging cycles in  RRC_INACTIVE mode following three potential configurations as below:

- Rel-18 INACTIVE eDRX (beyond 10.24s) within an optional UE capability, according to the agreement in Rel-18 eRedcap below.

- Rel-17 INACTIVE eDRX (256rf, 512rf, 1024rf) named as extendedPagingCycle, according to TS 38.331[3]

- Rel-17 paging cycle (rf32, rf64, rf128, rf256), according to TS 38.331[3].

The following are the agreements on eDRX paging cycles in eRedcap so far:

RAN2 confirms the enhanced INACTIVE eDRX can be applied to all R18 UEs. FFS if it can only be supported by UEs which support R17 eDRX.

Introduce an optional UE capability with signalling for Rel-18 enhanced eDRX in RRC_INACTIVE.

Observation 1: R18 devices supporting LPHAP may support eDRX paging cycles in RRC_INACTIVE mode as optional.

- The alignment between DRX (i.e. paging cycles) and PRS configurations were studied and recommended by RAN2 as shown in TR 38.859 [2].

Observation 2: The alignment between DRX (i.e. paging cycles) and PRS configurations are studied and recommended by RAN2 in TR 38.859.

- The revision of WID which is clarification won’t bring additional effort to RAN2.

Hence we propose to clarify in the WID that the alignment between paging cycles in RRC_INACTIVE mode and PRS configurations can be applicable  to both DRX and eDRX paging cycles in RRC_INACTIVE. The clarify can be done by changing “Specify solutions for alignment between eDRX and PRS configurations [RAN2]” to “Specify solutions for alignment between (e)DRX in RRC_INACTIVE state and PRS configurations [RAN2]”.

Proposal: Adopt the following changes to the WID:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including:

  - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4]

    - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning.

      - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap.

    - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs

  - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3].

    - SRS for positioning configurations in multiple cells [RAN2, RAN1].

      - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work.

    - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3].

    - SRS for positioning activation/request procedure(s) [RAN2, RAN1].

  - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2].

  - Specify solutions for alignment between (e)DRX in RRC_INACTIVE state and PRS configurations [RAN2].

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

Summary

In this contribution, we discussed the issue of the alignment between DRX/eDRX in RRC_INACTIVE state and PRS configurations and propose a clarification  on the WID. Base on the discussion, we have the following observations and proposal.

Observation 1: R18 devices supporting LPHAP may support eDRX paging cycles in RRC_INACTIVE mode as optional.

Observation 2: The alignment between DRX (i.e. paging cycles) and PRS configurations were studied and recommended by RAN2 in TR 38.859.

Proposal: Adopt the following changes to the WID:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including:

  - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4]

    - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning.

      - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap.

    - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs

  - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3].

    - SRS for positioning configurations in multiple cells [RAN2, RAN1].

      - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work.

    - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3].

    - SRS for positioning activation/request procedure(s) [RAN2, RAN1].

  - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2].

  - Specify solutions for alignment between (e)DRX in RRC_INACTIVE state and PRS configurations [RAN2].

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, Intel Corporation, CATT, Ericsson

- 3GPP TR 38.859 “Study on expanded and improved NR positioning”, v18.0.0

- 3GPP TS 38.331 “Radio Resource Control (RRC) protocol specification”, v17.4.0