3GPP TSG RAN Meeting #100	RP-231284

Taipei, June 12-24, 2023

Source:	Huawei, HiSilicon

Title:	pCR for TR 38.848 on Deployment scenarios and their characteristics

Agenda item:	9.2.2

Document for:	Agreement

- Introduction

This pCR proposes text to add Deployment scenarios and their characteristics on Ambient IoT into TR 38.848 v0.1.0.

Background: Rapporteur has tried to follow an approach of using the ‘Description’ column to capture what most companies think are reasonable ways to use the deployment scenario, and proposes to add the ‘Recommendation for first WG study’ column as a first prioritization within the ‘Description’ based on companies’ views in their papers, where they are provided.

Note that (as mentioned in our company paper) this recommendation does not preclude later studies on more uses of a deployment scenario, and nor does it indicate any particular Release for the “first WG study”, nor for the deployment scenario as a whole.

- Proposed changes

# <Unchanged text is omitted>

4.2	Deployment scenarios and connectivity topologies

Deployment scenarios for Ambient IoT have been studied on the basis of a list of characteristics, and the representative use case(s) applicable to a scenario. The possible descriptions of the characteristics are as follows:

Table 4.2-1: Characteristics of deployment scenarios

| Characteristic | Possible description entries |
| --- | --- |
| Environment (of the device) | Indoor Outdoor Indoor or outdoor |
| Basestation characteristic (if any) | Macro-cell-based deployment Micro-cell-based deployment Pico-cell-based deployment None |
| Connectivity topology | See section 4.2.1 |
| Spectrum | Licensed FDD Licensed TDD Unlicensed Note: In each connectivity topology of the study, if a BS is present, it is assumed to use licensed spectrum |
| Coexistence with existing 3GPP technologies | Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type. Deployed on new sites without an assumption of an existing 3GPP deployment. |
| Traffic assumption | Device-terminated (DT) Device-originated (DO) DO traffic includes DO autonomous (DO-A), and DO device-terminated triggered (DO-DTT) See NOTE 1. |
| Device characteristic | See Section 4.1: Device A Device B Device C |

NOTE 1: For Devices A and B, only DO-DTT is assumed.

# The study has considered Ambient IoT deployments in-band to NR, in guard-band of NR, and in standalone band from NR.

# <Unchanged text is omitted>

4.2.2	Deployment scenarios

4.2.2.1	Deployment scenario 1: Device indoors, basestation indoors

With Ambient IoT device indoors and basestation indoors, this deployment scenario is characterized according to Table 4.2.2.1-1.

Table 4.2.2.1-1: Characteristics of deployment scenario 1

| Applicable representative use cases | Characteristics | Description | Recommendation for first WG study |
| --- | --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico-cell | Micro-cell |
|  | Connectivity topology | Topology (1), (2), (3) | Topology (1) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site | Co-site or new site |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device A or Device B or Device C | Device A or Device B or Device C |

4.2.2.2	Deployment scenario 2: Device indoors, basestation outdoors

With Ambient IoT device indoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.1-2.

Table 4.2.2.1-2: Characteristics of deployment scenario 2

| Applicable representative use cases | Characteristics | Description | Recommendation for first WG study |
| --- | --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS | Macro-cell |
|  | Connectivity topology | Topology (1), (2), (3) Note: The location of intermediate or assisting node (if any) is indoor or outdoor | Topology (1), (2) Note: The location of intermediate node (if any) is indoor |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site | Co-site |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device C for Topology (1), (2), (3), Device A or Device B for Topology (2) | Topology (1): Device C Topology (2): Device A or Device B or Device C |

4.2.2.3	Deployment scenario 3: Device indoors, UE-based reader

With Ambient IoT device indoors and UE-based reader, this deployment scenario is characterized according to Table 4.2.2.1-3.

Table 4.2.2.1-3: Characteristics of deployment scenario 3

| Applicable representative use cases | Characteristics | Description | Recommendation for first WG study |
| --- | --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor | Indoor |
|  | Basestation characteristic (if any) | None | None |
|  | Connectivity topology | Topology (4) | Topology (4) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | Unlicensed (e.g. sub-GHz) |
|  | Coexistence with existing 3GPP technologies | NA | NA |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device A or Device B or Device C | Device A or Device B |

4.2.2.4	Deployment scenario 4: Device outdoors, basestation outdoors

With Ambient IoT device outdoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.1-4.

Table 4.2.2.1-4: Characteristics of deployment scenario 4

| Applicable representative use cases | Characteristics | Description | Recommendation for first WG study |
| --- | --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS | Macro-cell |
|  | Connectivity topology | Topology (1), (2), (3) | Topology (1) |
|  | Spectrum | Licensed FDD or licensed TDD | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site | Co-site |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device C for Topology (1), (2), (3), Device A or Device B may support Topology (2) | Device C |

4.2.2.5	Deployment scenario 5: Device outdoors, UE-based reader

With Ambient IoT device outdoors and UE-based reader, this deployment scenario is characterized according to Table 4.2.2.1-5.

Table 4.2.2.1-5: Characteristics of deployment scenario 5

| Applicable representative use cases | Characteristics | Description | Recommendation for first WG study |
| --- | --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor | Outdoor |
|  | Basestation characteristic (if any) | None | None |
|  | Connectivity topology | Topology (4) | Topology (4) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | Unlicensed (e.g. sub-GHz) |
|  | Coexistence with existing 3GPP technologies | NA | NA |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device A or Device B or Device C | Device C |

# <Unchanged text is omitted>