3GPP TSG RAN Meeting #100                                                          RP- 231297

# Taipei, June 12 - 14, 2023

Agenda Item:	9.2.5

Source:	Apple Inc.

Title:	View on AI/ML for NR air interface on CSI prediction use case

Document for:	Discussion/Decision

# Introduction

The R18 RAN1 led study item mainly focuses on three use cases: CSI enhancement, beam management and positioning. In RAN1 #113, the initial draft TR was approved [1].

In this contribution, we discuss the scope of CSI enhancement sub-use case.

# Discussion

For CSI enhancement, two sub-use cases have been studied:

- Spatial-frequency domain CSI compression using two-sided AI model.

- Time domain CSI prediction using UE sided model

Performance evaluation is done for both sub-use cases. For CSI prediction sub-use case, observations in terms of intermediate KPI, mean UPT and 5% UPT gain were agreed in RAN1 113, are copied below for reference.

Observation

For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 10km/h UE speed, 4 sources [Fujitsu, Samsung, Xiaomi, InterDigital] observe 1.03%~6% gain, 1 source [CMCC] observes 21.93% gain.

- For 30km/h UE speed, 2 sources [OPPO, ETRI] observes 6%~10.43% gain, 5 sources [ZTE, Fujitsu, Apple, Xiaomi, Spreadtrum] observe 18.72%~31.3% gain, and 3 sources [InterDigital, MediaTek, CMCC] observe 35%~ 41.75% gain, which are in general larger than 10km/h UE speed.

- For 60km/h UE speed, 2 sources [Fujitsu, InterDigital] observe -3%~5% gain, 4 sources [Huawei, Samsung, vivo, CMCC] observe 11.2%~19.98% gain, which are in general smaller than 30km/h UE speed.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - No spatial consistency is considered by 11 sources [Fujitsu, Samsung, Xiaomi, InterDigital, CMCC, OPPO, ETRI, ZTE, Apple, Huawei, Spreadtrum]. 1 source [vivo] provides both results with spatial consistency and results w/o spatial consistency.

  - Note: Results refer to Table 5.1-1 of R1-2306059

Observation

For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 1 source [Huawei] observes 1.2%~4.2% gain;

    - 1 source [Apple] observes 7.6%~8.5% gain;

    - 1 source [vivo] observes 9.7%~17.2% gain.

    - 1 source [MediaTek] observes 22.6%~ 48.6% gain.

  - For full buffer traffic:

    - 1 source [Nokia] observes 2%~3% gain;

    - 1 source [vivo] observes 8.7% gain.

    - 1 source [MediaTek] observes 1.01% gain

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 1 source [Huawei] observes 0.7%~3.1% gain;

    - 1 source [vivo] observes 3.4%~7.0% gain.

  - For full buffer traffic:

    - 1 source [vivo] observes 8.1% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-8 of R1-2306059

Observation

For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of 5% UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 2 sources [Huawei, vivo] observes 4.5%~9.3% gain;

    - 3 sources [Huawei, Apple, vivo] observes 11.3%~20.1% gain;

  - For full buffer traffic:

    - 2 sources [Nokia, vivo] observe 6%~17.5% gain;

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 2 sources [Huawei, vivo] observes 0.5%~16% gain;

  - For full buffer traffic:

    - 1 source [vivo] observes 11% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-9 of R1-2306059

In RAN1 112bis-e, whether RAN1 should further discuss the potential specification of CSI prediction sub-use case were discussed.

Agreement

In CSI prediction using UE-side model use case, whether to address the potential spec impact of CSI prediction depends on RAN#100 final conclusion, focusing on the following

- data collection procedure, mainly including RS configuration, measurement and report configuration, resusing as much as possible what is defined for UE side use cases

- monitoring procedure and metric for AI-based CSI prediction.

- Model/functionality selection/switching and finetuning procedure.

- Note: Discussion on potential specification impact is limited to aspects which would NOT duplicate the work in Rel-18 MIMO WI.

- Note: Minimize LCM related potential specification impact discussion that follow the high-level principle of other one-sided model sub-cases.

Comparing to CSI compression, CSI prediction is a much simpler use case as it uses a UE-side model while achieve similar gain comparing to CSI compression. It is common sense that large specification impact should be justified by large gains. The life cycle management for AI-CSI prediction, including model training and model deployment, is far simpler than that for AI-CSI compression. For example, as the UE side model is a single-sided model, complications from multiple vendor operations as in AI-CSI compression is circumvented. Since R19 will be the first release of a specification with AI/ML related features, we propose to focus on the use cases using one sided model with the highest observed gain. Therefore in Rel-18, it is important to study the potential specification impact of CSI prediction for preparation of R19 WI on AI/ML.

Proposal 1: RAN1 study the potential specification impact of CSI prediction starting RAN1 114.

Proposal 2: To limit the scope, study potential specification impact of CSI prediction sub use case under collaboration level Y, focusing on

- Data collection procedure, mainly including RS configuration, measurement and report configuration, reusing as much as possible what is defined for UE side use cases.

- Monitoring procedure and metric for AI-based CSI prediction.

- Model/functionality selection/switching and finetuning procedure.

# Conclusion

In this contribution, we discussed aspects on potential specification impact on R18 CSI prediction sub-use case. Based on the discussion, the following proposals have been proposed.

Proposal 1: RAN1 study the potential specification impact of CSI prediction starting RAN1 114.

Proposal 2: To limit the scope, study potential specification impact of CSI prediction sub use case under collaboration level Y, focusing on

- Data collection procedure, mainly including RS configuration, measurement and report configuration, reusing as much as possible what is defined for UE side use cases.

- Monitoring procedure and metric for AI-based CSI prediction.

- Model/functionality selection/switching and finetuning procedure.

# Reference

[1] TR 38.843, v0.1.0, Study on AI/ML for NR air interface, June 2023