3GPP TSG-RAN Meeting #100	RP-231330

Taipei, June 15 - 16, 2023                                                                         (revision of RP-230161)

# Source:	Huawei, HiSilicon

# Title:	Revised WID for 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC

# Document for:	Approval

Agenda Item:	9.4.5.6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC

## Acronym: 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC

## Unique identifier: 	981041

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## 1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

{Not applicable for Feature nor for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title.

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |
|  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

## 3	Justification

Low frequency bands are valuable for their large coverage, and have been deployed in many countries as the baseline network. However, it has the drawback of lower throughput due to smaller BW (typically not larger than 30MHz) and less antennas in handheld UE (now only 2Rx) which makes it less attractive in high throughput services compared with mid/high frequency bands with 100MHz or larger CBW and 4Rx antennas.

To make the low bands more valuable and achieve higher throughput, increasing BW/spectrum or introducing 4Rx in handheld UE could be two approaches. Between them, introducing 4Rx handheld UE is more realistic and promising approach since the spectrum is up to regulations.

- To support more antennas, the feasibility to implement 4 antennas in handheld UE needs to be studied. However, today 4Rx antennas have already been supported by some normal size commercial handheld UEs on the market, and their antenna abilities (efficiencies, isolation, correlations and throughputs) have justified the feasibility and performance of 4Rx antennas in normal size handheld UE.

Therefore, it is time for 3GPP to study and enhance the low band capabilities with 4Rx feature to increase the throughputs and benefit the whole ecosystem.

Another point is for the enhancement of inter-band UL CA or EN-DC band combination which is a popular NW deployment scenario today.

- For this kind of band combination, typically UE has 3 independent PAs implemented in the RFFE. However, for a long time, the Tx RF architecture assumption for handheld UE is 2 concurrent Tx chains. This leads to the situation that UE works with 2PAs in single high frequency band, but when it is configured with a CA/EN-DC band combination, then only one PA is activated in each band (one mid/high band PA is not activated), the total UL throughput is degraded due to smaller total BW.

- Similar problem also exists in the dual SIM UE, i.e. UE can work with 2Tx of single SIM, but when two SIMs are activated then there is only 1Tx per SIM which leads to performance degradation for each SIM.

Therefore, it is reasonable to consider extending the current 2Tx chain architecture to 3Tx chain for inter-band UL CA or EN-DC band combination scenario.

Besides, nowadays, there are many large size handheld UEs on the market. And one of them is the foldable smartphone. Comparing to normal smartphone, these large size smartphones can easily support higher Tx and Rx capabilities.

Overall, it is time for 3GPP to enhance the handheld UE performance with 4Rx in low frequency bands, and also the 3Tx in inter-band UL CA or EN-DC.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to enhance UE performance.

- Enhancements for 4Rx at low frequency band (<1GHz)

  - Study if feasible Specify requirements for handheld UE with 4Rx antennas including delta RIB,4R and ∆TRxSRS

    - 4Rx is optional feature for handheld UE in low frequency bands

    - Signalling impact will be considered if any.

  - Low frequency bands considered in this WI is in table 4.1-1 with the contact information inside.

- Enhancements of 3Tx for band combinations with two bands

  - Two bands with inter-band UL CA or EN-DC are considered with below limitations:

    - In each band only 1CC included. The Tx capability considered is 1Tx in one band, and 2Tx in the other band

    - The following power capabilities will be considered

      - CA power class or EN-DC power class is PC2

        - PC3 FDD band 1Tx + PC2 TDD band 2Tx (UL MIMO and TxD)

        - PC3 FDD band 1Tx + PC3 TDD band 2Tx (UL MIMO)

        - PC3 TDD band 1Tx + PC2 TDD band 2Tx (UL MIMO)

      - CA power class or EN-DC power class is PC1.5

        - PC3 FDD band 1Tx + PC1.5 TDD band 2Tx (UL MIMO and TxD)

  - Targeting UE type:

    - Specify UE RF requirements for FWA.

    - Study the applicable requirements for handheld UE but no normative work in Rel-18

  - Specify requirements for 3Tx, e.g. clarify the applicable requirements for the band which support UL MIMO in inter-band UL CA or inter-band EN-DC

    - Note 1: Increase UE power high limit feature is not included

  - Frequency band combinations considered in this WI is in table 4.1-2 and table 4.1-3 with the contact information inside.

    - Note 2: Different phases can be considered for the bands and band combinations in this WI. For the bands or band combinations which cannot be completed in Rel-18 timeframe will be removed from the WID before WI close.

Table 4.1-1: Low frequency bands with 4Rx for handheld UE

| Band | Contact name, company | Contact email | Other Supporting company (min. 3) | Additional information | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- |
| n28 | Xiaoran Zhang, CMCC | zhangxiaoran@cmcc.com | OPPO, Huawei, Hisilicon | 4Rx is for handheld UE | New |
| n20 | Meriem, Orange | meriem.mhedhbi@orange.com | OPPO, Huawei, Hisilicon | 4Rx is for handheld UE | New |
| n8 | Basaier, China Unicom | basejld@chinaunicom.cn | OPPO, Huawei, Hisilicon, CHTTL | 4Rx is for handheld UE | New |
| n5 | Shan Yang, China Telecom | yangshan@chinatelecom.cn | OPPO, Huawei, Hisilicon | 4Rx is for handheld UE | New |
| n26 | Alex, Telstra | Alex.Bladenis@team.telstra.com | OPPO, Oneplus, ZEKU | 4Rx is for handheld UE | New |
| n71 | Bill, T-Mobile USA | bill.shvodian@t-mobile.com | OPPO, Oneplus, ZEKU | 4Rx is for handheld UE | New |

Table 4.1-2: Band combinations for 3Tx with CA/EN-DC power class 1.5

| Band combination | UL configuration | Power class | Contact name, company | Contact email | Other Supporting company (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n71A | CA_n41A-n71A | PC3@n71 1Tx; PC1.5@n41 2Tx; CA power class PC1.5 | Bill, T-Mobile USA | bill.shvodian@t-mobile.com | OPPO, Oneplus, ZEKU | New |

Table 4.1-3: Band combinations for 3Tx with CA/EN-DC power class 2

| Band combination | UL configuration | Power class | Contact name, company | Contact email | Other Supporting company (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n41A | CA_n28A-n41A | PC3@n28 1Tx; PC2@n41 2Tx; CA power class PC2 | Xiaoran Zhang, CMCC | zhangxiaoran@cmcc.com | OPPO Huawei, Hisilicon | New |
| CA_n28A-n78A | CA_n28A-n78A | PC3@n28 1Tx; PC3/2@n78 2Tx; CA power class PC2 | Yuta Oguma, NTT DOCOMO | yuuta.oguma.yt@nttdocomo.com | OPPO, Huawei, Hisilicon | New |
| CA_n8A-n78A | CA_n8A-n78A | PC3@n8 1Tx; PC2@n78 2Tx; CA power class PC2 | Basaier, China Unicom | basejld@chinaunicom.cn | OPPO, Huawei, Hisilicon | New |
| CA_n41A-n71A | CA_n41A-n71A | PC3@n71 1Tx; PC2@n41 2Tx; CA power class PC2 | Bill, T-Mobile USA | bill.shvodian@t-mobile.com | OPPO, Oneplus, ZEKU | New |
| CA_n41A-n77A | CA_n41A-n77A | PC2@n41 1Tx; PC2@n77 2Tx; CA power class PC2 PC2@n41 2Tx; PC2@n77 1Tx; CA power class PC2 | Bill, T-Mobile USA | bill.shvodian@t-mobile.com | OPPO, Oneplus, ZEKU | New |
| CA_n26A-n78A | CA_n26A-n78A | PC3@n26 1Tx; PC2@n78 2Tx; CA power class PC2 | Alex, Telstra | Alex.Bladenis@team.telstra.com | OPPO, Oneplus, ZEKU | New |
| DC_3A_n78A | DC_3A_n78A | PC3@n3 1Tx; PC2@n78 2Tx; EN-DC power class PC2 | Bo-Han Hsieh, CHTTL | pohanhsieh@cht.com.tw | OPPO, Huawei, Hisilicon | New |
| DC_40A_n78A | DC_40A_n78A | PC3@n40 1Tx; PC3/2@n78 2Tx; EN-DC power class PC2 | Ye Liu, Huawei | leo.liuye@huawei.com | OPPO, Hisilicon, Oneplus | New |
| DC_7A_n78A | DC_7A_n78A | PC3@B7 1Tx; PC3/2@n78 2Tx; EN-DC power class PC2 | Ye Liu, Huawei | leo.liuye@huawei.com | OPPO, Hisilicon, Oneplus | New |
| DC_8A_n78A | DC_8A_n78A | PC3@B8 1Tx; PC3/2@n78 2Tx; EN-DC power class PC2 | Ye Liu, Huawei | leo.liuye@huawei.com | OPPO, Hisilicon, Oneplus | New |
| DC_20A_n78A | DC_20A_n78A | PC3@B20 1Tx; PC3/2@n78 2Tx; EN-DC power class PC2 | Ye Liu, Huawei | leo.liuye@huawei.com | OPPO, Hisilicon, Oneplus | New |
| DC_28A_n78A | DC_28A_n78A | PC3@B28 1Tx; PC3/2@n78 2Tx; EN-DC power class PC2 | Ye Liu, Huawei | leo.liuye@huawei.com | OPPO, Hisilicon, Oneplus | New |
| DC_41A_n78A | DC_41A_n78A | PC3@B41 1Tx; PC3/2@n78 2Tx; EN-DC power class PC2 | Ye Liu, Huawei | leo.liuye@huawei.com | OPPO, Hisilicon, Oneplus | New |

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | Introduce 4Rx requirements for low band handheld UE; Clarify/specify applicable UE RF requirements for inter-band UL CA with 3Tx. | #102 | Core part |
| 38.101-3 | Clarify/specify applicable UE RF requirements for inter-band EN-DC with 3Tx. | #102 | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Jinqiang Xing, OPPO

xingjinqiang@oppo.com

## 7	Work item leadership

Leading WG: RAN WG4

## 8	Aspects that involve other WGs

None

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

AT&T

China Unicom

China Telecom

CHTTL

CMCC

Deutsche Telekom

Huawei

Hisilicon

LG Electronics

NTT DoCoMo

OPPO

Orange

Qualcomm Incorporated

SoftBank Corp

Spark NZ Ltd

SK Telecom

Telecom Italia

Telstra

T-Mobile USA

Verizon

vivo

Vodafone

Xiaomi