3GPP TSG-RAN WG4 Meeting # 107	R4-2310420

# Incheon, South Korea, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5963 | rev | - | Current version: | 15.21.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.101 on relative humidity condition for normal temperature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2023-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current relative humidity of 25% to 75% is inconsistent with other specifications while RAN5 and ETSI are removing some humidity limit for conformance test, as indicated in R5-221604. Given no RF performance dependency with various humidity condition, it is not necessary to maintain a humidity range for normal condition in core specification which brings additional burden to industry. In RAN4#106 meeting, two CRs in R4-2302504 and R4-2302505 were agreed to remove the lower limit on relative humidity condition for NR FR1 and FR2. To make it consistent with other RF specifications, it is also suggested to unify the test environment for humidity in different RATs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the explicit lower humidity range for normal test environment in E-UTRA specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relative humidity condition for normal temperature test is inconsistent with other specifications and it brings additional burden to industry. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

Annex E (normative): 
Environmental conditions

E.1		General

This normative annex specifies the environmental requirements of the UE. Within these limits the requirements of the present documents shall be fulfilled.

E.2		Environmental

The requirements in this clause apply to all types of UE(s).

E.2.1	Temperature

The UE shall fulfil all the requirements in the full temperature range of:

Table E.2.1-1

| +15C to +35C | for normal conditions (with relative humidity up to 75 %) |
| --- | --- |
| -10C to +55C | for extreme conditions (see IEC publications 68-2-1 and 68-2-2) |

Outside this temperature range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in clause 6.2 for extreme operation.

E.2.2	Voltage

The UE shall fulfil all the requirements in the full voltage range, i.e. the voltage range between the extreme voltages.

The manufacturer shall declare the lower and higher extreme voltages and the approximate shutdown voltage. For the equipment that can be operated from one or more of the power sources listed below, the lower extreme voltage shall not be higher, and the higher extreme voltage shall not be lower than that specified below.

Table E.2.2-1

| Power source | Lower extreme voltage | Higher extreme voltage | Normal conditions voltage |
| --- | --- | --- | --- |
| AC mains | 0,9 * nominal | 1,1 * nominal | nominal |
| Regulated lead acid battery | 0,9 * nominal | 1,3 * nominal | 1,1 * nominal |
| Non regulated batteries: Leclanché Lithium Mercury/nickel & cadmium | 0,85 * nominal 0,95 * nominal 0,90 * nominal | Nominal 1,1 * Nominal | Nominal 1,1 * Nominal Nominal |

Outside this voltage range the UE if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in clause 6.2 for extreme operation. In particular, the UE shall inhibit all RF transmissions when the power supply voltage is below the manufacturer declared shutdown voltage.

E.2.3	Vibration

The UE shall fulfil all the requirements when vibrated at the following frequency/amplitudes.

Table E.2.3-1

| Frequency | ASD (Acceleration Spectral Density) random vibration |
| --- | --- |
| 5 Hz to 20 Hz | 0,96 m2/s3 |
| 20 Hz to 500 Hz | 0,96 m2/s3 at 20 Hz, thereafter –3 dB/Octave |

Outside the specified frequency range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 36.101 for extreme operation.

==============================================================

<< End of changes >>