3GPP TSG-RAN5 Meeting #100	R5-234724

# Toulouse, France, Aug 21st - Aug 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0396 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for new NR slice test cases 8.1.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_slice-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New NR slice test cases are added into 38.523-1, thus relevant applicability definition needs to be added in 38.523-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicability of following test case were added in table 4.1-3a - 8.1.1.3.9 RRC connection release / Success / With slice specific cell reselection information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of test case applicability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.1	Protocol conformance test cases applicability

< Unchanged sections omitted >

Table 4.1-3a: Applicability of Protocol conformance RRC test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 8 | RRC |  |  |  |
| 8.1 | NR RRC |  |  |  |
| 8.1.1 | RRC connection management procedures |  |  |  |
| 8.1.1.1 | Paging |  |  |  |
| 8.1.1.1.1 | RRC / Paging for connection / Multiple paging records | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.1.2 | RRC / Paging for connection / Shared network environment | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.1a | Paging Early Indication and Subgrouping |  |  |  |
| 8.1.1.1a.1 | Paging Early Indication with Subgrouping / RRC_IDLE / lastUsedCellOnly not configured / Subgroup ID selection | Rel-17 | C224 | UEs supporting 5G Core and PEI |
| 8.1.1.1a.2 | Paging Early Indication with Subgrouping / RRC_INACTIVE / lastUsedCellOnly configured | Rel-17 | C239 | UEs supporting 5G Core and RRC_INACTIVE and PEI |
| 8.1.1.1a.3 | Paging Early Indication without Subgrouping / RRC_IDLE | Rel-17 | C224 | UEs supporting 5G Core and PEI |
| 8.1.1.2 | RRC connection establishment |  |  |  |
| 8.1.1.2.1 | RRC connection establishment / Return to idle state after T300 expiry | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.2.2 | Void |  |  |  |
| 8.1.1.2.3 | RRC connection establishment / RRC Reject with wait time | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.2.4 | RRC connection establishment / Extended and spare fields in SI | Rel-15 and Rel-16 only | C21 | UEs supporting 5G Core |
| 8.1.1.3 | RRC release |  |  |  |
| 8.1.1.3.1 | RRC connection release / Redirection to another NR frequency | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.3.2 | RRC connection release / Redirection from NR to E-UTRA | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 8.1.1.3.3 | RRC connection release / Success / With priority information | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.3.4 | RRC connection release / Success / With priority information / E-UTRA | Rel-15 | C26 | UEs supporting 5GS and E-UTRA |
| 8.1.1.3.5 | Void |  |  |  |
| 8.1.1.3.6 | Void |  |  |  |
| 8.1.1.3.7 | RRC connection release / Success / Deprioritisation / Frequency / T325 expiry | Rel-15 | C133 | UEs supporting 5G Core and RRC connection release with Deprioritisation |
| 8.1.1.3.7a | RRC connection release / Success / Deprioritisation / NR / T325 expiry | Rel-15 | C148 | UEs supporting 5G Core and E-UTRA and RRC connection release with Deprioritisation |
| 8.1.1.3.7b | RRC connection release / Success / Deprioritisation / Deletion of Stored deprioritisation request | Rel-15 | C161 | UEs supporting 5G Core and RRC connection release with Deprioritisation and ManualModeNetworkSelectionException |
| 8.1.1.3.8 | RRC connection release / Redirection to another NR frequency / MPS Priority Indication | Rel-16 | C274 | UEs supporting 5G Core and RRC Connection release with MPS priority indication |
| 8.1.1.3.9 | RRC connection release / Success / With slice specific cell reselection information | Rel-17 | C240 | UEs supporting 5G Core and slice based cell reselection |

<End of modified section 1>