3GPP TSG RAN Meeting #101	RP-231810

Bangalore, India, September 11-15, 2023

Source:	vivo

Title:	Remaining issues on Ambient IoT SI

Agenda Item:	9.2.2

Document for:	Discussion and decision

Introduction

In previous RAN plenary meetings, device categorization, grouping method of SA use cases, and characteristics for deployment scenarios were discussed. In this contribution, we will continue discussing the remaining issues for this study item.

RAN design targets and feasibility

According to SID, RAN should formulate RAN design targets for the Ambient IoT use cases. In RAN#99, the following design targets were agreed for further study, and some of the targets were concluded. In this section, we provide further analysis on the design targets.

- Power consumption and complexity

In previous meeting, the following design targets for power consumption are concluded.

For Device A, the power consumption target during transmitting/receiving is [≤ 1 μW] or [≤ 10 μW], For Device B, the target during transmitting/receiving is such that: - Device A power consumption ≪ Device B power consumption < Device C power consumption; or - Device A power consumption ≤ Device B power consumption < Device C power consumption. The device power consumption for Device C is ≤ 1 mW to ≤ 10 mW.

And following working assumption for design targets for complexity of Ambient IoT device were made.

For Device A, the complexity target is to be comparable to UHF RFID ISO18000-6C (EPC C1G2). For Device B, the target is such that: - Device A complexity < Device B complexity < Device C complexity. For Device C, the complexity target is to be orders-of-magnitude lower than NB-IoT.

Considering device A can only be powered by RF signals and there is no energy storage in passive device, the allowed power consumption during transmitting/receiving highly depends on the instantaneous harvested power from RF signal as the device is quickly de-energized without illuminating by the RF signal. Considering the passive tags in UHF RFID, which is already widely commercialized can achieve the power consumption of 1uw and less [1], it is feasible for ambient IoT device A to achieve ≤ 1μW power consumption target with the complexity target that is comparable to the passive tags in UHF RFID.

Observation 1: It is feasible for ambient IoT device A to achieve complexity target comparable to UHF RFID ISO18000-6C (EPC C1G2), and power consumption target ≤ 1μW.

For device C, lower power consumption should be achieved for both transmitter and receiver. For receiver architecture using heterodyne low IF envelop detector, which is studied in R18 LP-WUS SI [2], can be used for DL reception. For active transmitter with reduced transmission power, and simple low order modulation, it does not require complex circuitry. Then less than 1mW power consumption can be achieved at -10~-20dBm output power according to literatures [3][4][5] for active transmitter. Higher output power can be achieved if the design target is 10mW power consumption, e.g., 0 dBm output power.

Chip area is a simple and direct measure of device complexity. For device C, die size for active tag is 1.65mm2 using 180nm CMOS technology according to [5]. While for NB-IoT, the die size is 24.4mm2 using 28nm CMOS technology according to [6]. Hence, it is feasible for device C to achieve complexity orders of magnitude lower than NB-IoT.

Observation 2: For device C, it is feasible to achieve the complexity target of orders-of-magnitude lower than NB-IoT.

Observation 3: For device C, it is feasible to achieve power consumption target lower than 1mw if the output power is -10 ~ -20 dBm. It is possible to have higher output power if the design target forpower consumption is up to 10mW.

- Coverage

The design targets of coverage are provided in SA1 TR. The coverage targets are different for indoor and outdoor deployments, and also different for different connection topologies, as summarized below.

- 10-30m for Indoor and topologies with UE communication with A-IoT device

- 30-50m for Indoor and topologies with BS communication with A-IoT device directly or indirection

- 100-500m for outdoor and topologies with BS communication with A-IoT device directly or indirection

RAN can take SA design targets as starting point and study the feasibility of these targets.

Observation 4: According to SA, the design targets for coverage are summarized as follows:

- 10-30m for Indoor and topologies with UE communication with A-IoT device directly

- 30-50m for Indoor and topologies with BS communication with A-IoT device directly or indirectly

- 100-500m for outdoor and topologies with BS communication with A-IoT device directly or indirectly

The coverage of A-IoT device depends on the bottleneck between DL link and reverse link. We provide preliminary link budget analysis on the BS/UE<-> device A/B/C. The detailed assumptions for different devices are provided in the following tables.

For Device C, the signal is generated by A-IoT device independently without relying on carrier wave, and the transmission power is typically between {-10, -20} dBm [3][4][5]. In this contribution, -20dBm transmission power is assumed for evaluation. Regarding the receiver sensitivity, a value comparable to LP-WUR in R18 SI can be considered. It is observed from many inputs that WUR can achieve similar or better coverage compared with PUSCH eMBB . Therefore, -80dBm DL receiver sensitivity is assumed for Device C for coverage evaluation [2], and -130 dBm receiver sensitivity is assumed for gNB. Meanwhile, we assume that the penetration loss in UMa-NLOS O2I scenario is 20 dB.

Table 1: The assumptions for path loss in different scenarios

| Scenario | Path Loss |
| --- | --- |
| UMa-LOS | PL = 28.0 + 22*lg(d) + 20*lg(f) |
| UMa-NLOS | PL = 32.4 + 30*lg(d) + 20*lg(f) |
| InH-LOS | PL = 32.4 + 17.3*lg(d) + 20*lg(f) |

Table 2: The assumptions for coverage evaluation (Topology 1: BS <-> A-IoT device)

| Parameters/ Assumptions | Device A | Device B | Device C |
| --- | --- | --- | --- |
| Carrier frequency (GHz) | 0.9 |  |  |
| gNB power(dBm) | 40dBm for UMA, 23dBm for InH |  |  |
| gNB antenna gain(dB) in UMa | 6 | 6 | 6 |
| gNB Rx sensitivity(dBm) | -102 | -102 | -130 |
| Receiver at AIoT device |  |  |  |
| A-IoT Rx sensitivity(dBm) | -24 | -34 | -80 |
| LNA (dB) | 0 | 10 | NA |
| AIoT device parameters for backscatter |  |  | AIoT device Tx power |
| Tag power gain(dB) | 0 | 10 | -20dBm or 0dBm |
| Return loss (dB) | 8 | 8 |  |
| Propagation path | UMa-LOS and UMa-NLOS(O2O and O2I) |  |  |

Table 3: The assumptions for coverage evaluation (Topology 4: UE <-> A-IoT device)

| Parameters/ Assumptions | Device A | Device B | Device C |
| --- | --- | --- | --- |
| Carrier frequency (GHz) | 0.9 |  |  |
| UE power(dBm) | 23 |  |  |
| UE antenna gain(dB) | 0 |  |  |
| UE Rx sensitivity(dBm) | -92 |  |  |
| A-IoT Rx sensitivity(dBm) | -24 | -34 | -80 |
| LNA (dB) | 0 | 10 | NA |
| AIoT device parameters for backscatter |  |  | AIoT device Tx power |
| Tag power gain(dB) | 0 | 10 | -20dBm or 0dBm |
| Return loss (dB) | 8 | 8 |  |
| Propagation path | InH_LOS |  |  |

Figure 1. The coverage ranges between gNB and AIoT devices A for BS to A-IoT link and reverse link

As shown in the Figure 1, the achieved coverage ranges for gNB and AIoT devices are given as below for Device A, in different scenarios, respectively:

- 47 meters coverage can be achieved for BS<-> device A in UMa-LOS scenario;

- 12 meters coverage can be achieved for BS<-> device A in UMa-NLOS scenario;

- 8 meters coverage can be achieved for BS<-> device A in InH-LOS scenario.

Figure 2. The coverage ranges between gNB and AIoT devices B for BS to A-IoT link and reverse link

As shown in the Figure 2, the achieved coverage ranges for gNB and AIoT devices are given as below for Device B, in different scenarios, respectively:

- More than 200 meters coverage can be achieved for BS<-> device B with amplifier in UMa-LOS scenario;

- 42 meters coverage can be achieved for BS<-> device B with amplifier in UMa-NLOS scenario;

- 112 meters coverage can be achieved for BS<-> device B with amplifier in InH-LOS scenario.

Figure 3. The coverage ranges between gNB and AIoT devices C for BS to A-IoT link and reverse link

As shown in the Figure 3, the achieved coverage ranges for gNB and AIoT devices are given as below for Device C, in different scenarios, respectively:

- More than 800 meters coverage can be achieved for BS<-> device C in UMa-LOS scenario; 650 meters coverage can be achieved for BS<-> device C in UMa-NLOS scenario;

- Only 100 meters coverage can be achieved for BS<-> device C in UMa-NLOS O2I scenario, if transmission power of device C is -20dBm.

- If transmission power is 0dBm (considering PA efficiency and circuitry power, the total power may be between 5mw ~ 10mw), about 130 meters coverage can be achieved in UMa-NLOS O2I scenario.

In UMa-NLOS O2I scenario, the coverage between BS and device C is only 100 meters when the transmission power of device C is -20 dBm. Even with increased transmission power to 0 dBm (total power may between 5mw~10mw), the coverage is only 130 meters, and coverage is limited by DL coverage in this case.

The typical ISD of typical urban scenario is 500 meters, which should be considered as target for scenarios with BS outdoor. For BS outdoor and device outdoor (O2O), the communication range is greater than 500 meters which is typical ISD for urban.  While 500 meters coverage range cannot be achieved for BS outdoor and device indoor (O2I). Considering coverage of BS to UE link is more reliable than BS to AIoT link, Topology 2 with UE as intermediate node between the BS and device C can be considered to improve the coverage for BS outdoor and device indoor (O2I).

Observation 5: For device C, the coverage of 500 meters, which is typical ISD for Urban, can be achieved in UMa-NLOS O2Oscenario (Deployment scenario 4: BS outdoor and device outdoor).

Observation 6: For device C, the coverage of 500 meters, which is typical ISD for Urban, may not achieved in UMa-NLOS O2I scenario (Deployment scenario 2: BS outdoor and device indoor).

- Topology 2 with UE as intermediate node between the BS and device C can be considered for coverage extension.

Figure 4. The coverage range between UE and AIoT devices for UE to A-IoT link and reverse link

As shown in Figure 4, the achieved coverage ranges for UE and AIoT devices are given as below for Device A, Device B, and Device C in different scenarios, respectively:

- 8 meters coverage can be achieved for UE<-> device A in InH-LOS scenario;

- 70 meters coverage can be achieved for UE<->device B with amplifier in InH-LOS scenario;

- A few hundred meters coverage can be achieved for UE<->device C.

As shown by the evaluation results, it is observed that the achieved coverage ranges are different for different device types and connection topologies. Hence, besides indoor/outdoor, we suggest to define the coverage design targets separately for different connection topologies and device types. Based on the evaluation, the coverage can be achieved for different device types and connection topologies are summarized as follows.

- For BS connection directly with A-IoT device

  - Device A: less than 10 meters coverage can be achieved for indoor; 47 meters coverage can be achieved for outdoor LOS scenario and 12 meters coverage can be achieved for outdoor NLOS scenario.

  - Device B: 112 meters coverage can be achieved for indoor; more than 200 meters coverage can be achieved for outdoor LOS scenario and 42 meters coverage can be achieved for outdoor NLOS scenario.

  - Device C

    - More than 800 meters coverage can be achieved for outdoor LOS scenario, and 650 meters coverage can be achieved for outdoor NLOS scenario.

    - For NLOS O2I scenario, when the transmission power of Device C is -20 dBm, only 100 meters coverage can be achieved; when the transmission power of Device C is 0 dBm, 130 meters coverage can be achieved.

- For UE connection directly with A-IoT device

  - Device A: less than 10 meters coverage can be achieved for indoor

  - Device B: 70 meters coverage can be achieved for indoor

  - Device C: a few hundred meters coverage can be achieved.

The coverage is typically less than 10 meters for UHF RFID, hence 10 meters can also be target coverage of device. The typical ISD for Indoor scenario is 20 meters for NR, we still assume 20 meters as design targets for coverage even at least for device B and device C. For design targets for device B, the above coverage evaluation is obtained assuming device with reflection amplifier.

Proposal 1: Define the coverage design targets separately for different connection topologies and device types.

- For BS connection directly to A-IoT device

  - Device A: less than 10 meters

  - Device B: Typical ISD for Indoor, i.e., 20 meters

  - Device C:  Typical ISD for Urban, i.e., 500 meters

- For UE connection directly to A-IoT device

  - Less than 10meters for device A；

- Typical ISD for indoor for device B and device C, i.e., 20 meters.

Proposal 2: If coverage target cannot be achieved by topology 1, e.g., for deployment scenario 2: BS outdoor and device indoor, UE intermediate node can be considered to achieve the design target.

- Maximum message size (or maximum ‘TB’ size)

According to the SA use cases, the message size is generally in the range of [96bits~1000bits]. In addition, some additional bits for control / other data (e.g., location information, IP address of server) are required which depend on several factors e.g., scenario, service type, user-experienced data rate etc. Given that, the maximum message size is approximately one thousand bits.

Proposal 3: The maximum message size is approximately 1kbits.

- Latency

By analyzing the SA use cases, the latency level of hundreds of milliseconds is observed for medical instrument inventory management, positioning in parking and shopping areas. Specifically, the latency requirement of a few use cases is around 1 or 2 seconds. For most of the use cases, the latency up to 10 seconds is allowed. Especially for the use case of Airport Terminal/ Shipping Port, the Max. allowed end-to-end latency can even be larger than 1 minute.

Proposal 4: The latency requirements are in the order of second e.g., 10 seconds.

Data rate, message size, and signaling procedure have impacts on the latency. As analyzed above, the typical message size for Ambient IoT use cases is around 1kbits. Assuming a data rate of 5 kbps, the period of data transmission for 1kbits will be roughly 200ms. Thus, the minimum latency of 1 second is feasible for Ambient IoT.

Proposal 5: It is feasible to achieve the minimum latency, i.e., <=1s, for Ambient IoT use cases.

- Positioning accuracy

For both indoor and outdoor scenarios, there are use cases requiring positioning functionality. Through the existing literature research, some achieved positioning accuracy based on actual measurement data can be summarized as follows.

In [7],  the summary of UHF RFID localization methods and positioning accuracy in indoor scenarios are provided. For various AIOT positioning methods, the positioning accuracy within 3m can be achieved for indoor scenarios.

In [8]-[9], the different positioning methods of ultra-narrow IoT in outdoor scenarios are provided, which can reach positioning accuracy of [<=50m]@90% using RSSI-based method[8][9], and that of [<=150m]@90% using TDOA-based method for NB-IoT systems[10]. Ambient IoT may be deployed with narrower bandwidth and lower transmitter power compared with assumptions in the literatures. By further optimizing deployments for positioning of Ambient IoT devices, the positioning accuracy of several 10 meters or a few hundred meters can be expected for outdoor scenarios.

Proposal 6:  Positioning accuracy design target is:

- Indoor: [<=3m] @ 90%

- Outdoor: [<=100m] @ 90% or [a few hundred of meters] @ 90%

- Connection/device density

The connection/device density highly depends on the application scenarios and varies greatly among different SA use cases. For example, for logistics/supply or smart farming use cases, the device density can be up to hundreds of thousands per km2 or several million per km2 because of million units of materials are used in the manufacturing or farm areas. Besides, for the other use case such as indoor positioning or smart home etc., tens or hundreds of devices per 100m2 are common, which is usually denser than that of the outdoor use case. Moreover, there are also <5 or 10 per 100m2 device density for personal belongings positioning cases.

The connection density highly depends on the capacity of the network. Since the high connection density mainly happens in indoor scenarios, by considering the typical topologies of indoor deployment (e.g., ISD of around 30 meters for indoor micro or pico BS) and the Ambient IoT traffic with packet arrival every 1 message/15 minutes/device, there are 1.67 paging per second which is far below the maximum paging upper bound of the system. Thus, the required connection density i.e., 1.5 device per m2 can be fulfilled by the network capacity. In other words, the requirement on connection density can be satisfied by the network capacity, as all the devices can be read in each period.

Proposal 7: The connection density target for Ambient IoT is: 1 or 1.5 devices per m2.

- Device speed

Based on the SA use cases, the device speed is in the range of [3~20]km/h for three use cases. And the other are mostly stationary or low speed moving cases.

Observation 7: The oriented device speed of SA use cases is [3~20]km/h or stationary.

The doppler shift caused by device speed [3~20]km/h is quite low, the impact to reliability is limited, especially when non-OFDM and low data rate modulation is used for AIoT devices. Hence, it is feasible to support device speed for AIoT device in range of [3-20]km/h.

Observation 8: It is feasible to support device speed for AIoT device in the range of 3-20km/h.

Required functionality for the use cases

The required functionalities are provided in Table 4 and Table 5.

Table 4: Required functionality set #1: for supporting RAN design target

| Design target | Functionality |
| --- | --- |
| Device power and complexity | - Low power Tx and Rx Architecture at AIoT device, - Transmitting based on backscattering for Device A and Device B. - Transmitting based on lower power active transmitter for Device C. - Receiving based on envelop detection for Device A, Device B and Device C, and the signal/channel design should be suitable for envelope detector. - Tx and Rx at gNB/UE, - Full duplex capability, including self-interference suppressing, is required for gNB/UE to communicate with device A and Device B, if carrier wave transmission and backscatter reception is performed simultaneously by the same gNB/UE device. - Signal/Channel Design, e.g., channel structure, waveform/modulation, robust to frequency error and timing error. - Compact protocol stack and signaling procedure with low power consumption (e.g. less signaling interactions), low complexity (e.g. less signaling storage) |
| Coverage | - Coverage enhancement techniques with low device complexity |
| User experienced data rate | - Compact protocol stack and signaling procedure - Flexible MCS/data rate |
| Maximum message size | - Signal/Channel design which can deliver the maximum message size. |
| Latency | - Compact protocol stack and signaling procedure |
| Positioning accuracy | - Signal/channel design and procedures for positioning/ranging. |
| Connection/Device density | - RACH-like schemes, and mechanisms to flexibly control the collision rate. |
| Moving speed of device | - Signal/channel design robust to doppler shift |

Table 5: Required functionality set #2: for supporting other requirements

| Requirement | Functionality |
| --- | --- |
| Device management | - Activation and deactivation of devices for device A/B - Communicate with all, (selected) a subset, or specific one of the Ambient IoT devices present |
| Security | - Authentication of the Ambient IoT device, Reader is UE or BS. - Authorization of the Ambient IoT device, Reader is UE. - Encryption, data integrity. |
| Mobility | - Measurements capability at AIoT device - Mobility management |
| Interference management and coexistence | - Flexible resource allocation for DL/UL transmission for AIoT devices to co-exist with NR deployments. |
| CN connectivity | - Protocol stack supports the deployment with or without CN connectivity. |
| Forward compatibility | - Unified L1 signal/channel design for different topologies to reduce implementation complexity of AIoT device. |

Proposal 8: Endorse required functionalities for AIoT use cases summarized in Table 4 and Table 5 in RP-231810.

Conclusions

In this paper, we discussed several aspects on ambient IoT study, and we have the following conclusions.

Observation 1: It is feasible for ambient IoT device A to achieve complexity target comparable to UHF RFID ISO18000-6C (EPC C1G2), and power consumption target ≤ 1μW.

Observation 2: For device C, it is feasible to achieve the complexity target of orders-of-magnitude lower than NB-IoT.

Observation 3: For device C, it is feasible to achieve power consumption target lower than 1mw if the output power is -10 ~ -20 dBm. It is possible to have higher output power if the design target forpower consumption is up to 10mW.

Observation 4: According to SA, the design targets for coverage are summarized as follows:

- 10-30m for Indoor and topologies with UE communication with A-IoT device directly

- 30-50m for Indoor and topologies with BS communication with A-IoT device directly or indirectly

- 100-500m for outdoor and topologies with BS communication with A-IoT device directly or indirectly

Observation 5: For device C, the coverage of 500 meters, which is typical ISD for Urban, can be achieved in UMa-NLOS O2Oscenario (Deployment scenario 4: BS outdoor and device outdoor).

Observation 6: For device C, the coverage of 500 meters, which is typical ISD for Urban, may not achieved in UMa-NLOS O2I scenario (Deployment scenario 2: BS outdoor and device indoor).

- Topology 2 with UE as intermediate node between the BS and device C can be considered for coverage extension.

Proposal 1: Define the coverage design targets separately for different connection topologies and device types.

- For BS connection directly to A-IoT device

  - Device A: less than 10 meters

  - Device B: Typical ISD for Indoor, i.e., 20 meters

  - Device C:  Typical ISD for Urban, i.e., 500 meters

- For UE connection directly to A-IoT device

  - Less than 10meters for device A；

- Typical ISD for indoor for device B and device C, i.e., 20 meters.

Proposal 2: If coverage target cannot be achieved by topology 1, e.g., for deployment scenario 2: BS outdoor and device indoor, UE intermediate node can be considered to achieve the design target.

Proposal 3: The maximum message size is approximately 1kbits.

Proposal 4: The latency requirements are in the order of second e.g., 10 seconds.

Proposal 5: It is feasible to achieve the minimum latency, i.e., <=1s, for Ambient IoT use cases.

Proposal 6:  Positioning accuracy design target is:

- Indoor: [<=3m] @ 90%

- Outdoor: [<=100m] @ 90% or [a few hundred of meters] @ 90%

Proposal 7: The connection density target for Ambient IoT is: 1 or 1.5 devices per m2.Observation 7: The oriented device speed of SA use cases is [3~20]km/h or stationary.

Observation 8: It is feasible to support device speed for AIoT device in the range of 3-20km/h.

Proposal 8: Endorse required functionalities for AIoT use cases summarized in Table 5 and Table 6 in RP-231810.

- References

- Liu, V., Talla, V., & Gollakota, S. (2014). Enabling instantaneous feedback with full-duplex backscatter. Proceedings of the 20th Annual International Conference on Mobile Computing and Networking - MobiCom ’14.

- TR 38.869, Study on low-power wake up signal and receiver for NR, v0.3.1.

- Kimionis, J., Georgiadis, A., Collado, A., & Tentzeris, M. M. (2014). Enhancement of RF Tag Backscatter Efficiency With Low-Power Reflection Amplifiers. IEEE Transactions on Microwave Theory and Techniques, 62(12), 3562–3571.

- K. Tang et al., "A 75.3 pJ/b Ultra-Low Power MEMS-Based FSK Transmitter in ISM-915 MHz Band for Pico-IoT Applications," 2021 IEEE International Symposium on Circuits and Systems (ISCAS), 2021, pp. 1-4, doi: 10.1109/ISCAS51556.2021.9401715

- J. Bae and H. Yoo, "A low energy injection-locked FSK transceiver with frequency-to-amplitude conversion for body sensor applications," 2010 Symposium on VLSI Circuits, 2010, pp. 133-134, doi: 10.1109/VLSIC.2010.5560325.

- Lee, J., Han, J., Lo, C., Lee, J., Kim, W., Kim, S., … Kang, I. “NB-IoT and GNSS All-In-One System-On-Chip Integrating RF Transceiver, 23-dBm CMOS Power Amplifier, Power Management Unit, and Clock Management System for Low Cost Solution”, IEEE JSSC, Apr. 2020

- Chenyang Li , Lingfei Mo, and Dongkai Zhang, “Review on UHF RFID Localization Methods”, IEEE Journal of Radio Frequency Identification, VOL. 3, NO. 4, DECEMBER 2019

- Lam K H, Cheung C C, Lee W C, “RSSI-based LoRa localization systems for large-scale indoor and outdoor environments”, IEEE Transactions on Vehicular Technology, VOL. 68, No. 12, December 2019

- Sallouha H, Chiumento A, Pollin S, “Localization in long-range ultra-narrow band IoT networks using RSSI”, 2017 IEEE International Conference on Communications (ICC), IEEE, May 2017

- Xingqin Lin, Johan Bergman, Fredrik Gunnarsson, etc., “Positioning for the Internet of Things: A 3GPP Perspective”, IEEE Communications Magazine, VOL. 55, NO. 12, DECEMBER 2017