3GPP TSG RAN #102 Meeting 	RP-232861

Edinburgh, December 11-15, 2023

Title:	Candidate topics for a RAN2 led Rel-19 IoT-NTN-evolution Work Item

Source: 	Thales, ETRI, Telit, CEWiT, SES, Fraunhofer IIS, Fraunhofer HHI, Lockheed Martin, NEC, Gatehouse Satcom, Airbus DS, Novamint, Sateliot, Eutelsat, Qualcomm, ESA, MITRE, Ligado, JSAT, OQ Technology, Hispasat, TNO, Magister Solutions, ST Engineering Idirect, Reliance Jio, DLR, Radisys, Hughes, Leonardo, Satellite Applications Catapult, Inmarsat, Viasat, Gilat, Omnispace, Terrestar Solutions, Intelsat, AccelerComm

Type:	discussion

Document for:	discussion

Agenda Item:	 10.1.2.1	NTN (Non-Terrestrial Networks) evolution for IoT

# Introduction

This document entails a list of candidate topics for a potential RAN2 led Rel-19 IoT-NTN-evolution WI as per the structure in RP-232745 (page 29).

Note that the yellow highlighted text refer to some proposals that are still subject to discussions.

Table of content

1	Introduction	1

2	RAN1-centric candidate topics for IoT-NTN-evolution WI	3

2.1	Capacity enhancements for uplink	3

2.1.1	Justification	3

2.1.2	Objectives	4

2.1.3	Working Groups	4

3	RAN2-centric candidate topics for IoT-NTN-evolution WI	5

3.1	Mobility enhancements	5

3.1.1	Justification	5

3.1.2	Objectives	5

3.1.3	Working Groups	6

4	RAN3-centric candidate topics for IoT-NTN-evolution WI	7

4.1	Support of S&F Satellite operation	7

4.1.1	Justification	7

4.1.2	Objectives	7

4.1.3	Working groups	7

5	Conclusion and Way forward	8

6	References	8

# RAN1-centric candidate topics for IoT-NTN-evolution WI

## Capacity enhancements for uplink

### Justification

NB-IoT NTN is already being deployed live at this very moment. In these early and upcoming deployments, it is clearly emerging that IoT-NTN, in particular NB-IoT, will have to support massive capacity in terms of number and types of UE, some of which with worse characteristics than others (e.g. low cost devices, wearables, etc).

What has also clearly emerged by capacity analysis is that the current NB-IoT NTN UL system capacity is actually limited by the corresponding system DL capacity, due to the larger signalling overhead in the downlink (up to ~53%), and the tight coupling between UL and DL signalling. This impacts also predominantly UL-driven traffic, such as Mobile Originated (MO) transmissions, which are the primary target of massive IoT and initial emergency messaging use cases supported by IoT NTN.

Morevoer, every time a UE needs to transmit UL data, it requires an approximately equal number of DL control messages from the Network. This severely limits the UL capacity, which, by itself, is expected to be at least double due to more efficient resource multiplexing and lower UL signalling overhead.

Therefore, in order to unlock the additional UL capacity potential, there is a need to identify methods to de-couple the UL from the DL as much as possible.

Some techniques, such as Preconfigured Uplink Resource (PUR) with Shared Resource and the adoption of a Random Access procedure similar to 2-step RACH, already supported in 5G NR Small Data Transmission (SDT), could be used to achieve this and thus significantly increase the UL system capacity by reducing the reliance on DL signalling. However:

- NB-IoT does not currently support 2-step RACH or similar procedures;

- Dedicated PUR schemes rely on persistent periodic resources that have to be reserved for specific UE, thus significantly reducing the network capacity available to other users.

- Current Contention Free Shared (CFS) PUR schemes currently only allow a limited number of pre-configured UL sequences (up to 2 for 12 tones, or less), thus significantly limiting the potential gains.

- In addition, for single tone transmissions, NB-IoT does not allow the possibility of multiplexing multiple UEs in the same tone at all, which significantly reduces capacity in power-limited scenarios.

In the past 3GPP had considered the specification of Contention-Based schemes for PUR with Shared resources (CBS PUR), with the specification of additional signatures, based on CDM orthogonal cover codes, additional DMRS configurations, etc, which had shown the potential for massive gains in UL capacity.

Deployment perspectives: All satellite operators deploying NB-IoT NTN would need this feature

### Objectives

Assumptions

- NOMA techniques are not considered in this topic

Objectives

- Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to 4) in a single 3.75kHz or 15 kHz subcarrier via orthogonal cover codes for NPUSCH and NPRACH [RAN1]

  - Multi-tone support should also be considered

  - NOTE: The enhancements in this objective can be combined with the enhancements in the following objectives.

  - NOTE : Impact of frequency offset shall be taken into account

- Study then specify, if beneficial, enhancements to existing PUR/EDT procedure to minimize the amount of necessary downlink/uplink signaling to complete an PUR/EDT transaction [RAN1, RAN2], e.g.

  - RRC early data complete delivery

  - Msg3 contention-based operation without PRACH and random access response

  - Consider primarily for Mobile Originated (MO) data transmissions

- Note that PUR/EDT enhancement may be used also independently from the use of OCC

### Working Groups

RAN1, RAN2

# RAN2-centric candidate topics for IoT-NTN-evolution WI

## Mobility enhancements

### Justification

Load balancing between NB-IoT anchor carriers mapped to the same or different satellite beams.

NB-IoT NTN is being deployed commercially as we speak. Satellite beam coverage areas are very large, in some cases with different, variable beam sizes or tiers and potentially large areas of overlap.  This will likely imply multiple NB-IoT NTN cells deployed as different carriers in the same satellite beam footprint, and as a minimum one anchor carrier per-beam. In addition, a combination of multiple NB-IoT NTN Anchor and Non-anchor carriers may be deployed within any given satellite beam coverage area across one or more beams.

NTN operators may however have constraints of being able to deploy only Anchor carriers in adjacent beams or within the same beam.  It is also expected that additional Non-Anchor carriers associated with a given Anchor carrier can only be deployed within the same beam, and that adjacent beams will mostly be associated with different Anchor carriers.

As deployments scale out, there are emerging very common situations in which the network may need to re-locate one or more UE between carriers both within the same beam, as well as across beams in adjacent or co-coverage, for load-balancing, coverage, or frequency management purposes.

In addition, since NB-IoT NTN is expected to be used for more use cases than just pure IoT and messaging, there are use cases in which some basic level of mobility between beams might need to be supported.

However, since NB-IoT currently does not support CONNECTED_MODE mobility or any kind of Network-controlled mobility or load-balancing, the NTN operator is severely limited in the ability to manage such scenarios.

UE Dedicated RRC signalling CarrierConfigDedicated-NB can be used to specify a new Non-Anchor Carrier for the UE to switch to, or for the UE to return from a Non-Anchor Carrier to its original Anchor carrier, however, this does not support switching between different Anchor carriers.

The only other mechanism available to the Network to influence UE switching between carriers is via RRC Release with Redirection and RRCEarlyDataComplete-NB , which, however, can only be used once after the UE has acquired into the initial Anchor carrier or after an EDT procedure has completed, and forces the UE to restart the initial access procedure from scratch in the new Anchor carrier, leading to potentially severe network capacity impact.

An additional aspect to improve is the signaling overhead upon TN to NTN mobility. According to current specifications, the UE shall provide the capabilities during the TAU procedure, which will result in large overhead If the UE moves frequently from TN to NTN. RAN WGs (in collaboration potentially with SA2) should provide a solution to avoid the capability transfer in these cases.

Also, the objective is to allow for idle/inactive/connected mode inter-RAT (TN-)NTN mobility (such as idle/inactive/connected mode mobility from IoT NTN to NR NTN).

### Objectives

Objectives:

- Study solutions to avoid sending UE capability update during TAU update upon TN to NTN mobility and if needed, specify enhancements to UE capability update procedure [RAN2]

- FFS : Specify NB-IoT enhancements to support Network-controlled anchor to anchor carrier switching for UE [RAN2]

  - Note : No impact on connected measurements

- FFS : study, and define if needed, NB-IoT enhancements (e.g. SIB signalling) to support Network assisted carrier switching for UE  [RAN2]

- FFS: Study and specify solutions for inter-RAT (TN-)NTN mobility [RAN2]

### Working Groups

RAN2

Coordination with SA2 and CT1 as needed

# RAN3-centric candidate topics for IoT-NTN-evolution WI

## Support of S&F Satellite operation

### Justification

Benefits specifically associated with the support of Store and Forward (S&F) Satellite operation with regenerative payload:

- S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas)

- Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure.

- The support of S&F puts the 3GPP IoT NTN solution on par with other non-3GPP solutions intended for massive satellite IoT, which natively already supports S&F services

Deployment perspectives: The lack of support of S&F Satellite operation is a current limitation of Rel-17 & 18 for IoT NTN via NGSO. All NGSO IoT NTN satellite operators are expected to implement S&F operation.

### Objectives

- Specify the support of eNB on board in TS 36.300 and define if needed, any necessary enhancements related to S1 protocol to address the feeder link switch over issue

- Define the necessary enhancements into RAN to support store and forward (S&F) operation for delay-tolerant services. Specific aspects at least to be considered if applicable:

  - Network and UE Capabilities on S&F (FFS)

  - Signalling procedures for S&F operation

  - Data transmission under S&F operation (e.g. enforcement of specific S&F QoS requirements such as maximum message size and acknowledgement policy)

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite), etc.

### Working groups

RAN2, RAN3

# Conclusion and Way forward

Proposal 1: Consider the following topics as part of a RAN2 led Rel-19 IoT-NTN-evolution WI:

- Capacity enhancements for uplink

- Mobility enhancements

- Support of S&F Satellite operation

Proposal 2: Consider the following TU estimates per candidate topics as part of a RAN2 led Rel-19 IoT-NTN-evolution WI:

| Candidate topics | RAN1 | RAN2 | RAN3 | RAN4 RF | RAN4 RRM |
| --- | --- | --- | --- | --- | --- |
| Capacity enhancements for Uplink | 2,25 | 0 |  |  |  |
| Mobility enhancements | 0,25 | 1 | 0,5 |  |  |
| Store and Forward (requiring regenerative payload) |  | 4,5 | 1,5 |  |  |

# References

[1] RP-223519; R18 WID on IoT NTN enhancements

END