3GPP TSG RAN Meeting #102												RP-233093

Edinburgh, Scotland, December 11-15, 2023

Agenda Item:	9.1.1.4

Source:	Huawei (moderator, RAN1 Vice-Chair)

Title:	Moderator summary on new SI Study on solutions for Ambient IoT (Internet of Things) in NR

Document for:	Discussion and decision

# Introduction

Contributions on the scope of the Rel-19 study on solutions for Ambient IoT were provided to RAN#102 as listed in the reference section. A moderator’s draft WID was provided in RP-233094 accounting for the contributions in those references. This document provides a discussion for progressing on scoping of the SID, with questions provided in section 2 for companies to consider. Feedback on the questions can be provided on a voluntary basis using the link below, to help before the drafting session scheduled for 7:30-9:30 am on Wednesday 13 December.

Feedback can be provided until Monday midnight UTC on 11 December. For convenience before and during the meeting, an NWM document was created for collecting voluntary responses. Please go to https://nwm-trial.etsi.org/ and search for RAN102_R19_Ambient_IoT.

# Question on the proposed SID general scope

The intention is to use discussion during RAN to perform the down-selections implied by the […] in the General Scope in the objective section of RP-233094. To help with this, a few questions are provided below.

Square brackets on rUCs and traffic types are expected to be resolved once decisions are made on deployment scenarios and topologies, so no questions are provided on rUCs and traffic types.

## Deployment scenarios and topologies

Question: If we focus on only one Deployment + Topology pairing from the recommendations of TR38.848, which one should be the focus of Rel-19?

| Company | Response |
| --- | --- |

Question: If we add one more Deployment + Topology pairing from the recommendations, which one? You can indicate “no additional”.

| Company | Response |
| --- | --- |

## Device types

A joint proposal in RP-233380 proposes that Rel-19 is scoped to Device A and Device B only, and shows a majority support for scoping Rel-19 only to Device A and Device B. This is the only joint proposal submitted to this meeting, which may serve as a good basis for a start.

Question: If you also support or can accept this proposal, please indicate.

| Company | Response |
| --- | --- |

Question: If Rel-19 includes Devices A & B as per RP-233880, do you think Rel-19 can also include Device C? If so, to what extent?

| Company | Response |
| --- | --- |

Question: If Rel-19 includes Device C, do you think Rel-19 can also include Devices A & B? If so, to what extent?

| Company | Response |
| --- | --- |

Question: As a proponent of including Device C in Rel-19, can you live with an understanding that 3GPP will take Device C forward in Rel-20 and Device A/B only in Rel-19, including by e.g. also preparing a SID text for Device C during this RAN#102 meeting?

| Company | Response |
| --- | --- |

## Spectrum

Question: Do you agree with the RAN chair’s proposal to have only licensed FDD in FR1 in Rel-19?

| Company | Response |
| --- | --- |

Question: It seems clear Ambient IoT operating in-band to NR and guard-band to NR spectrum should be included in Rel-19. Do you think spectrum standalone from NR should also be included in Rel-19?

| Company | Response |
| --- | --- |

## Positioning

SID excerpt: “Study of positioning in Rel-19 is limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation)”

Question: Is the statement in the draft moderator SID suitable for scoping positioning or locating the Ambient IoT device in this Release of Ambient IoT? If not, please explain, including WG workload considerations.

| Company | Response |
| --- | --- |

# Question on the proposed SID objective 1 (evaluation assumptions)

Objective 1 is about remaining design targets, assumptions of deployment scenarios for coverage and coexistence evaluations, and assessment for power consumption and coverage, with the following note:

NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically.

## 1a

- Conclude the aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1].

This part intends to point at the parts where WGs are mentioned in Section 5.3 of TR 38.848.

Question: Do you think this objective is suitable?

| Company | Response |
| --- | --- |

## 1b

- Define necessary further assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4]

This part intends to produce assumptions such as typical layout of Ambient IoT BS with potential carrier wave node for Device A/B, transmit power of Ambient IoT BS and carrier wave node, and distance between NR and Ambient IoT BS if not co-sited.

Question: Do you think this objective is suitable?

| Company | Response |
| --- | --- |

## 1c

- Identify “reference” basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity.

Question: Do you think this objective relating to whether to look at Ambient IoT device architectures is (1) necessary and, (2) if so, appropriately written?

| Company | Response (1): Is it necessary? (2) Is it appropriately written? |
| --- | --- |

## 1d

- Define link budget calculation for coverage [, including whether/how to model carrier wave for Device A/B from node(s) inside or outside the connectivity topology].

Possibly this objective could be taken inside Objective 1a. The current suggestion is to keep it separate to allow further explanation in the SID of what is expected of RAN1.

Question: Do you agree with this approach and the wording?

| Company | Response |
| --- | --- |

# Question on the proposed SID objective 2 (study on solutions)

The high-level description of objective 2 is:

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed for [Device A/B] [and Device C], and ensuring at least the required functionalities in Section 6.2 of TR 38.848.

- [The node providing the carrier wave to Device A/B is assumed to transmit the carrier wave in UL spectrum.]

- Study of positioning in Rel-19 is in RAN3 only, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation).

Question: Taking into account the content proposed next per WG, does anything need to be changed here? (Note that the [..] content would be decided by other questions).

| Company | Response |
| --- | --- |

## RAN1

- RAN1-led:

  - Frame structure, synchronization and timing, random access (Moderator note: Grouped as issues related to timing)

  - Numerologies, bandwidths, and multiple access

  - Waveforms and modulations

  - Channel coding

  - Downlink channel/signal aspects

  - Uplink channel/signal aspects

  - Scheduling and timing relationships

  - [Device A/B carrier-wave aspects]

Question: Is everything needed in the SI included for RAN1?

| Company | Response |
| --- | --- |

## RAN2

- RAN2-led:

  - Study and decide which control-plane and user-plane functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable [DO-DTT and DT] data transmission.

At least including

    - Paging

    - Random access

    - Data transmission

    - [For Device C: Additional necessary RRC aspects]

For control plane and user plane functionalities not listed above, they are studied only if found essential.

  - [Study control of carrier-wave node, when used for Devices A and B, according to RAN1 progress.]

Question: Is everything needed in the SI included for RAN2?

| Company | Response |
| --- | --- |

## RAN3

- RAN3-led:

  - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable:

    - Paging

    - Connection management

    - Data transport, e.g. CP-like solution

  - Identify RAN architecture aspects

  - Identify potential solutions for locating an Ambient IoT device with no, or minimal, specification impact

  - Decide if split architecture is needed for Ambient IoT, and any F1 impacts

Question: Is everything needed in the SI included for RAN3?

| Company | Response |
| --- | --- |

## RAN4

- RAN4-led:

  - Coexistence study of Ambient IoT and NR[/LTE].

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Device [A/B/C] transmission and reception

    - [Carrier wave node, if needed]

Question: Is everything needed in the SI included for RAN4? Can [LTE] be deleted?

| Company | Response |
| --- | --- |

# Conclusions

To be completed once feedback is received.

# References

- RP-232745	Summary for RAN Rel-19 Package: RAN1/2/3-led	RAN chair (Qualcomm)

- RP-232755	Rel-19 study on Ambient IoT	Nokia, Nokia Shanghai Bell

- RP-232757	Study on Ambient IoT for Rel 19	Lekha Wireless Solutions

- RP-232768	Consideration on Rel-19 Ambient IoT	New H3C Technologies Co., Ltd.

- RP-232867	Ambient IoT Devices Complexity and Power Comparison	Wiliot Ltd.

- RP-232870	Views on Ambient IoT	NEC

- RP-232897	Views on Rel 19 Ambient IoT Scope	Semtech Neuchatel SA

- RP-232902	Discussion on ambient IoT in Rel-19	OPPO

- RP-232944	Considerations for Ambient IoT devices	Wiliot Ltd.

- RP-232960	Discussion of R19 work on Ambient IoT	Futurewei

- RP-232968	Views on Rel-19 Ambient IoT	Fujitsu Limited.

- RP-232977	Ambient IoT considerations in Rel-19	Spreadtrum Communications

- RP-233014	Ambient IoT in Rel-19	CATT

- RP-233045	Considerations on Rel-19 Ambient IoT	Ruijie Network Co. Ltd

- RP-233051	Views on Rel-19 Ambient IoT	vivo

- RP-233093	Moderator's summary on new SI Study on solutions for Ambient IoT (Internet of Things) in NR	Huawei (moderator, RAN1 VC)

- RP-233094	New SID: Study on solutions for Ambient IoT (Internet of Things) in NR	Huawei (moderator, RAN1 VC)

- RP-233123	Support of AIoT Study in Rel-19	SHARP Corporation

- RP-233138	Views on Ambient IoT for R19	Xiaomi EV Technology

- RP-233154	Proposals on Ambient IoT in Rel-19	LG Electronics

- RP-233162	Views on Ambient IoT for Rel-19	Apple

- RP-233212	Views on Rel-19  Ambient IoT	Haier

- RP-233268	Rel-19 Ambient IoT: Device Types and Topologies	MediaTek Inc.

- RP-233298	Proposal on Ambient IoT ‘Devices’ in Release 19	Vodafone

- RP-233378	Views on scope for Ambient-IOT in Rel-19	Qualcomm Incorporated

- RP-233398	On Rel-19 Ambient IoT	InterDigital, Inc.

- RP-233420	Views on Rel-19 Ambient IoT	CMCC

- RP-233471	Scope on Ambient IoT	Samsung

- RP-233504	Ambient IOT: aspects of study and prioritization	CEWiT

- RP-233518	Multiple access and backscatter consideration  for Rel-19 ambient IoT	BJTU

- RP-233523	Study on Ambient IoT 	Kyocera

- RP-233535	Views on Ambient IoT for Rel-19	NTT DOCOMO, INC.

- RP-233571	Views on Rel-19 Ambient IoT	China Telecom

- RP-233597	Views on Ambient IoT in R19	China Unicom

- RP-233611	Views on Rel-19 Ambient IoT	ZTE, Sanechips

- RP-233642	Views on Ambient IoT study for Rel-19	Lenovo

- RP-233676	Considerations on Ambient IoT	EURECOM

- RP-233739	On the scope of Rel-19 Ambient IoT SI	Ericsson

- RP-233778	Views on Release 19 Ambient IOT	IIT Kanpur, CEWiT, Indian Institute of Technology Madras

- RP-233803	Ambient IoT scope in Rel-19	Huawei, HiSilicon

- RP-233807	Considerations for R19 Work on Ambient IoT	Philips International B.V.

- RP-233866	Operator perspective on Rel-19 Ambient IoT	AT&T, FirstNet

- RP-233880	Proposal on Ambient IoT ‘Devices’ in Release 19	Vodafone, China Mobile, China Unicom, China Telecom, Telefónica, T-Mobile USA, CAICT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Spreadtrum, TCL, Kyocera, H3C, Philips, Quanray, Maxwave Micro, Futurewei, Xiaomi, T-Mobile USA