3GPP TSG RAN #102 Meeting 	RP-233230

Edinburgh, December 11-15, 2023

Title:	Candidate topics for RAN4 led Rel-19 NR-NTN-evolution WI(s)

Source: 	Thales, Novamint, Airbus, Ligado, Fraunhofer IIS and HHI, Sateliot, Hughes, Eutelsat Group, TTP, TerreStar Solutions, ESA

Type:	discussion

Document for:	discussion

Agenda Item:	9.1.4.1	General (Proposals led by RAN4 (non spectrum related))

# Introduction

This document entails a list of candidate topics for potential RAN4 led Rel-19 NR-NTN-evolution WI(s).

# RAN4 led candidate topics for NR-NTN-evolution WI(s)

## Enhanced TE-emulated channel model

### Justification

To ensure that the 3GPP tests are sufficient for an operation with NGSO networks i.e. dynamic conditions

### Objectives

Define delay and Doppler shifts matching the satellite propagator model [RAN4]

update core RF requirements (e.g. frequency accuracy error) and RRM requirements (e.g. timing error) to be valid/applicable for dynamic channel conditions for FR1 and FR2-NTN.

Define OTA test methodologies for frequencies between 7 and 24 GHz (or at least Ka and Ku bands).

### Working groups

RAN4

## NR Channel BW less than 5 MHz

### Justification

This feature is currently defined for Terrestrial Networks to cater for use-cases where there are spectrum limitation/constraints. Satellite deployments in L-Band and S-Band face the same constraints and would benefit from the work done in the Rel-18 WID « NR support for dedicated spectrum less than 5 MHz for FR1 » by extending support for NTN.

This feature should as much as possible build on the work done in the Rel-18 NR support for dedicated spectrum less than 5 MHz for FR1 WID , by extending support for NTN.

The NTN extension should support the 3 MHz NR bandwidth as agreed in the Rel-18 TN WID.

This feature will enable more efficient deployment of the FR1 NTN bands where spectrum for satellite use is limited, for example:

- Facilitate deployment of NR NTN within the context of existing non-3GPP satellite services

- Lower bandwidth for NR will enable better use of spectrum resource by allowing higher frequency re-use between beams, increasing capacity.

  - Maximizing capacity for a given region requires the deployment of many spot beams, within satellite power constraints. To mitigate the inter-beam interference requires high frequency re-use which is more difficult to achieve with 5 MHz beams.

- Enable deployment of both NR and standalone NB-IoT/eMTC carriers in the same 5 MHz Bandwidth to enable parallel support of both eMBB and IoT services.

- Allowing more flexibility in deployment where spectrum allocation is irregular (not in the nX5 MHz)

- Smaller channel BW also helps with link budget in situations where the payload is EIRP-limited

- Allow for optimizing capacity vs demand in each spot beam.

### Objectives

The aim will be to re-use the work done in TN Rel-18 NR support for spectrum less than 5 MHz and focus on the RAN4 requirements for NTN:

- Specify support for UE channel bandwidth less than 5 MHz for NR NTN FR1 bands [RAN4]

  - Support for transmission with 12 PRB and 15 PRB, with priority for 12 PRB.

  - Re-use the same assumptions as adopted for terrestrial, in respect to maintaining current PSS/SSS without puncturing, reusing the PBCH design with puncturing and appropriate CORESET formats supporting 12 and 15 PRB transmission;

  - Allow support of reduced bandwidth SSB and CORESET within a wider total system bandwidth, to allow the network to flexibly assign reduced bandwidth cells within a wider system BW

- If necessary, specify any additional RF requirements for supporting 3 MHz channel bandwidth in NTN FR1 bands n255, n256, n254 and upcoming n253 (Extended L-band with UL: 1668-1675 MHz, DL: 1518-1525 MHz) [RAN4]

### Working groups

RAN4

# Conclusion and Way forward

Proposal: Consider the following topics as part of RAN4 led NR-NTN-evolution WI(s):

- Enhanced TE-emulated channel model

- NR channel bandwidth below 5 MHz

# References

[1] RP-232669; WID: NR NTN (Non-Terrestrial Networks) enhancements

END