3GPP TSG-RAN WG4 Meeting #109	 R4-2318085

# Chicago, America, November 13 – November 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0423 | rev |  | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_newRAT-Perf] CR to TS38.101-4 Corrections to test parameters for CSI test cases (Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the agreed Cat-A CR R4-2009059, the sub-field “PDCCH & PDCCH DMRS Precoding configuration” should be under the filed PDCCH, not under PDSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move the sub-field “PDCCH & PDCCH DMRS Precoding configuration” from PDSCH to PDCCH. - Correct the typo “an” to “and” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not implemented correctly. It should be implemented based on the agreed CR R4-2009059. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

Table 6.1.2-1: Test parameters for CSI test cases

| Parameter |  |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  |  |  | Transmission scheme 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) |  |  |  | RBs | 0 |
|  | Subcarrier spacing |  |  |  | kHz | 15 or 30 |
| DL BWP configuration #1 | Cyclic prefix |  |  |  |  | Normal |
|  | RB offset |  |  |  | RBs | 0 |
|  | Number of contiguous PRB |  |  |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
| Additional PDCCH Configuration for Aperiodic Reporting (Note 4) | Slots for PDCCH monitoring |  |  |  |  | Each slot |
|  | Symbols with PDCCH |  |  |  |  | 0,1 |
|  | Number of PDCCH candidates and aggregation levels |  |  |  |  | 1/AL8 |
|  | DCI format |  |  |  |  | 0_1 |
|  | TCI state |  |  |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  |  |  | Multi-path fading propagation conditions: Single Panel Type I, Random per slot with equal probability of each applicable i1, i2 combination, and with REG bundling granularity for number of Tx larger than 1 |
| Active DL BWP index |  |  |  |  |  | 1 |
| Common serving cell parameters |  | Physical Cell ID |  |  |  | 0 |
|  |  | SSB position in burst |  |  |  | First SSB in Slot #0 |
|  |  | SSB periodicity |  |  | ms | 20 |
| PDCCH configuration |  | Slots for PDCCH monitoring |  |  |  | Each slot |
|  |  | Symbols with PDCCH |  |  |  | 0,1 |
|  |  | Number of PDCCH candidates and aggregation levels |  |  |  | 1/AL8 |
|  |  | DCI format |  |  |  | 1_1 |
|  |  | TCI state |  |  |  | TCI state #1 |
|  |  | PDCCH & PDCCH DMRS Precoding configuration |  |  |  | Multi-path fading propagation conditions: Single Panel Type I, Random per slot with equal probability of each applicable i1, i2 combination, and with REG bundling granularity for number of Tx larger than 1 Static propagation conditions: Single Panel Type I, Random precoder chosen from precoder index 0 and 2, selection updated per slot |
| Cross carrier scheduling |  |  |  |  |  | Not configured |
| PDSCH configuration |  | Mapping type |  |  |  | Type A |
|  |  | k0 |  |  |  | 0 |
|  |  | Starting symbol (S) |  |  |  | 2 |
|  |  | Length (L) |  |  |  | 12 |
|  |  | PDSCH aggregation factor |  |  |  | 1 |
|  |  | PRB bundling type |  |  |  | Static |
|  |  | PRB bundling size |  |  |  | 2 |
|  |  | Resource allocation type |  |  |  | type 0 |
|  |  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |
|  |  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |
| PDSCH DMRS configuration |  | DMRS Type |  |  |  | Type 1 |
|  |  | Number of additional DMRS |  |  |  | 1 |
|  |  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |
|  |  | DMRS ports indexes |  |  |  | {1000} for Rank1 {1000,1001} for Rank2 {1000,1001,1002} for Rank3 {1000,1001,1002,1003} for Rank4 |
|  |  | Number of PDSCH DMRS CDM group(s) without data |  |  |  | 2 |
| PTRS configuration |  | Frequency density (KPT-RS) |  |  |  | N/A |
|  |  | Time density (LPT-RS) |  |  |  | N/A |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 4 for CSI-RS resource 1 and 3 8 for CSI-RS resource 2 and 4 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  |  |  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  |  |  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  |  |  | CSI-RS periodicity |  | slot | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource |
|  |  |  | CSI-RS offset |  | slot | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  |  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  |  |  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition |  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  |  |  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition |  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
| TCI state #0 |  |  | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  |  |  | QCL Type |  | Type C |
|  |  |  | Type 2 QCL information | SSB index |  | N/A |
|  |  |  |  | QCL Type |  | N/A |
| TCI state #1 |  |  | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  |  |  | QCL Type |  | Type A |
|  |  |  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  |  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  |  |  | 4 For FDD 8 for TDD |
| HARQ ACK/NACK bundling |  |  |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  |  |  | {0,2,3,1} |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  |  |  |  | 2 for FDD For FR1.30-1: 8 if mod(i,10) = 0 6 if mod(i,10) = 2 5 if mod(i,10) = 3 5 if mod(i,10) = 4 4 if mod(i,10) = 5 3 if mod(i,10) = 6 Where i is slot index per radio frame with 0~19 For FR1.30-7: 8 if mod(i,10) = 0 7 if mod(i,10) = 1 6 if mod(i,10) = 2 5 if mod(i,10) = 3 4 if mod(i,10) = 4 3 if mod(i,10) = 5 2 if mod(i,10) = 6 Where i is the slot index of all slots in every 5ms i = {0,…,9} |
| Symbols for unused REs |  |  |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Physical signals, channels mapping and precoding |  |  |  |  |  | As specified in Annex B.4.1 |
| Note 1: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. Note 2: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 3: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. Note 4: Additional PDCCH configuration for aperiodic reporting is only for test cases with aperiodic CSI reporting configured. |  |  |  |  |  |  |

END OF CHANGE 1