3GPP TSG RAN Meeting #102							                           RP-233622

# Edinburgh, Scotland, December 11-15, 2023

Agenda Item:	9.1.3.1

Source:	ZTE, Sanechips

Title:	View on Rel-19 AI/ML for NG-RAN

Document for:		Discussion and Approval

- Introduction

The Rel-18 AI/ML for NG-RAN work item (WI) has been successfully completed in RAN3. Throughout the RAN3 meeting, companies exhibited a positive inclination towards presenting their normative designs and signaling solutions for AI/ML use cases. It is evident that all companies unanimously recognize AI/ML techniques as powerful tools that provide significant assistance to operators in enhancing network management and optimization.

Following the completion of the normative work for the R18 WI, the subsequent step will be the R19 study item (SI) and the associate work item (WI). The Rel-19 SI will focus on exploring new AI use cases, and procedures enhancements to support the agreed use cases. The following WI is expected after 6 months study and finalize the corresponding normative work. The starting point can be seen in RP-232745[1] as below:

Potential objectives:

  - 6 months SI + 12 months WI

  - Rel-18 leftovers (depends on Rel-18 outcome), e.g.:

    - Mobility optimization

      - NR-DC

      - UE Trajectory prediction

    - Energy saving

      - Prediction for energy cost

    - Continuous MDT

  - At least 1 At most 2 potential new use case/scenario which benefits from inference-based techniques

    - Coverage and Capacity Optimization

    - Potential Rel-18 use case enhancements (e.g. new scenarios for ES)

    - QoE

    - Slicing

    - Note: Model training/inference at the DU and/or F1 enhancements must be justified by the use case

  - Study: Consider available functionality specified in 5GC (NWDAF, MDAS) in order to align AI/ML framework between RAN and CN

Potential objectives:

  - 6 months SI + 12 months WI

  - Rel-18 leftovers (depends on Rel-18 outcome), e.g.:

    - Mobility optimization

      - NR-DC

      - UE Trajectory prediction

    - Energy saving

      - Prediction for energy cost

    - Continuous MDT

  - At least 1 At most 2 potential new use case/scenario which benefits from inference-based techniques

    - Coverage and Capacity Optimization

    - Potential Rel-18 use case enhancements (e.g. new scenarios for ES)

    - QoE

    - Slicing

    - Note: Model training/inference at the DU and/or F1 enhancements must be justified by the use case

  - Study: Consider available functionality specified in 5GC (NWDAF, MDAS) in order to align AI/ML framework between RAN and CN

In this contribution, we present our perspectives on the study scope for the Rel-19 SI concerning AI/ML in NG-RAN and its associated WI.

- Justification of Rel-19 AI RAN

During the Rel-17 SI, the concepts of AI/ML-based energy saving, AI/ML-based load balancing, and AI/ML-based mobility optimization were documented and included in the TR 37.817 [2]. Furthermore, the potential standard impacts related to these use cases were thoroughly discussed during the Rel-18 WI phase, drawing from the outcomes of the SI. Here, we provide our potential AI/ML based use cases to be discussed in Rel-19.

2.1 New AI/ML based use cases

2.2.1 AI/ML based Network Slicing

Network slicing is indeed a critical aspect of 5G technology as it enables the provisioning of various new services across multiple industries. By utilizing network slicing, 5G networks can support a wide range of applications, including manufacturing, autonomous driving, gaming, and more. This is achieved by providing specific slices tailored to the unique requirements of each application. AI/ML techniques can be applied to optimize the resource allocation for network slicing.

AI/ML algorithms can analyze network performance, UE lever and cell level traffic patterns, and UE level service requirements to determine the optimal placement of network slices and allocate resources more efficiently. By considering factors such as UE level traffic latency, bandwidth requirement, and service demands, AI/ML models can make intelligent decisions on how to allocate resources to UEs among different slices dynamically, including the scenarios of non-mobility and mobility, ensuring optimal performance and resource utilization to fulfill the requirement of SLA (Service Level Agreement).

- AI/ML-based network slicing brings benefits such as optimized resource allocation, improved dynamic slice deployment, automated and flexible slice management, contributing to efficient and dynamic support for diverse services and applications in modern networks.

Besides, during Rel-18 WI, it was acknowledged that predicted resource status as one kind of information is useful for the agreed three use cases, e.g., load balancing, energy saving and mobility enhancement. However, the predicted resource status for now is limited to total PRB usage, instead of PRB usage per slice level.

- Predicted Resource status per slice-level for current use cases (e.g., ES, LB, ME) is not discussed during Rel-17 & Rel-18 phase.

Following is the examples on AI/ML based network slicing to be studied:

- Slice-level Resource status prediction: Accurate predictions of resource status per slice level are crucial for efficient network slicing. AI/ML algorithms can analyze historical data and make predictions about future Slice-level resource status. This information helps in determining the appropriate allocation of network resources and the placement of network slices to meet anticipated demand.

- Feedback information for each slice: After resource allocation of each slice based on the prediction, nodes should know the performance of these decision. Firstly, resource status per slice information, as one type of information, can be collected through the existing resource status procedure. Another form of feedback information is UE performance feedback for each slice, including UE performance feedback and user traffic, which explicitly reflects the performance.

According to the benefits of the AI/ML based Network Slicing, it is proposed to study the solutions, measurements, signaling, and procedures for AI-based Network slicing in Rel-19.

- Study the benefits and scope of AI/ML based network slicing, and the input/output/feedback and its potential solutions during Rel-19 SI (followed by a WI) in RAN3.

2.2.2 AI/ML based Coverage and Capacity Optimization (CCO)

CCO stands out as one of the standard operational tasks essential for optimizing the RAN. The primary objective of CCO is to ensure the provision of the necessary capacity in targeted coverage areas while simultaneously minimizing the interference and maintaining an acceptable quality of service in an autonomous manner. Notably, there exists a crucial trade-off between coverage and capacity information, wherein capacity enhancements often come at the expense of service coverage degradation, and vice versa.

In addressing this challenge, the application of AI/ML techniques proves instrumental. AI/ML algorithms have the capacity to analyze UE distributions, resource allocation patterns, and the current CCO configuration. Leveraging the analysis, the most appropriate CCO configurations can be predicted and generated, thereby optimizing the performance of network and user experience.

Following is the examples on AI/ML based CCO to be studied:

- Predicted CCO information: It can be generated by the NG-RAN node through the analysis of historical data, UE distributions, and other relevant factors using AI/ML algorithms. By leveraging these advanced algorithms, the NG-RAN node can forecast and generate optimal CCO information for future scenarios. The predicted CCO information can then be shared proactively among neighbouring NG-RAN nodes. This collaborative sharing mechanism serves to address potential coverage issues or cell edge capacity concerns for UEs within the network, enhancing the overall efficiency and performance of the NG-RAN.

- Feedback information: UE performance feedback, and related KPIs of network is the essential of feedback information to evaluate whether the predicted CCO information bring benefits to the network optimization.

According to the benefits of the AI/ML based CCO, it is proposed to study the solutions, measurements, signaling, and procedures for AI-based CCO in Rel-19.

- Study the benefits and scope of AI/ML based CCO, and the input/output/feedback and its potential solutions during Rel-19 SI (followed by a WI) in RAN3.

  - Impacts over split architecture

Following is the WI objectives of Rel-18 AI/ML for NG-RAN:

Normative work is based on the conclusions captured in TR37.817. The detailed objectives of the WI are listed as follows:

•	Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3)

A point of contention arose during the discussion regarding whether to adopt a new procedure or rely on the legacy procedure for transferring information to support AI/ML. However, the consensus could not be reached so for Rel-18 there would be no pursuit of enhancements to support AI/ML within the split architecture. Actually, it is recognized that the impacts on split architecture should be considered on a case-by-case basis. Each AI/ML-based use case, including new ones proposed in Rel-19, may exist potential standard impacts.

In light of this, we propose to persist with discussions on the impacts of split architecture for the support of AI/ML function in Rel-19 based on all agreed AI/ML based use cases.

- Continue to study and specify the enhancements and signaling support within split architecture for the support of AI/ML function in Rel-19.

  - Essential Rel-18 Leftovers

Some essential Rel-18 leftovers, which has heated discussion during Rel-18 phase, can be studied and specified in Rel-19:

AI/ML based Mobility Optimization:

Currently, in RAN3, it has been agreed upon that cell-level UE trajectory prediction can be transferred from the source node to the target node via the HANDOVER REQUEST message. However, mobility optimization extends beyond cell-level UE trajectory prediction in NG-RAN nodes, the following open issues can be further studied in Rel-19.

- AI/ML based mobility optimization enhancements in NR-DC scenarios: While traffic steering in NR-DC scenarios is covered in TR 37.817 [2], NR-DC scenarios have been given lower priority in Rel-18. The suggested focus is to investigate AI/ML-based solutions for optimizing mobility in NR-DC scenarios. Some of essential parts can be transferring the predicted/measured UE trajectory and UE performance feeback between MN and SN nodes.

- Multiple-hop target node UE trajectory: The UE trajectory prediction is only limited to the next one hop target NG-RAN node, and RAN3 will not pursue enhancements for one gNB to request UE trajectory from more than one hop gNBs in Rel-18. However, the limitation restricts the usage of predicted UE trajectory, and capability of AI/ML algorithms. Multiple hop UE trajectory can provide more information for subsequent mobility decision. The essential part to be discussed in Rel-19 is how to retrieve the measured UE trajectory from more than one hop gNBs.

- Finer granularity level UE trajectory: Only cell-level UE trajectory is considered in Rel-18. Actually, geographic location level UE trajectory, beam-level UE trajectory can provide the detailed information for mobility optimization for NG-RAN node.

AI/ML based Network Energy Saving:

For network energy saving, it was agreed to introduce the new metrics – Energy Cost (EC), and transfer EC between NG-RAN nodes, which means the predicted information is generated in the requesting node, instead of transferring predicted EC over Xn interface. The controversial part is the definition of “additional load” for EC. Therefore, it can be further discussed how to support the transfer of predicted EC for network energy saving cases.

Continuous MDT enhancement:

Continuous collection of MDT traces is beneficial for AI/ML training. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive). The solution to continuous MDT collection could not reach consensus in Rel-18. To resolve this issue above, there seems several kinds of solutions on the table proposed in Rel-18:

- Solution #1: Reuse Trace ID to identify the UE across various state transitions

- Solution #2: Allocate the persistent UE ID from CN/AMF

- Solution #3: Reuse the method proposed in SON/MDT (UE pointer solution). Transmitting a 'continuous flag' to the UE. The UE will then store this indication and include it in the report sent back to the RAN.

- Solution #4: Introduce an indication in the logged MDT measurement configuration to indicate UE whether to collect continuous information.

Therefore, solution down-selection can be further discussed in Rel-19.

- Following the Rel-18 leftovers can be further studied and specified in Rel-19:

- Mobility Optimization:

-  AI/ML based mobility optimization enhancements in NR-DC scenarios

-  Multiple-hop target node UE trajectory

-  Finer granularity level UE trajectory, e.g., beam-level

-  Network Energy Saving

-  Predicted Energy Cost

- Continuous MDT collection

  - Coordination between CN and RAN

In Rel-18, coordination between CN and RAN for support AI/ML function is not considered. Upon checking the progress of AI/ML in other groups, SA2 has specified NWDAF in 5GC to support AI/ML functionality, and RAN3 has also specified support of AI/ML NG-RAN function based on following framework:

Figure 1. Functional Framework for RAN Intelligence

For now, model training and model inference is only considered to be deployed in either OAM or NG-RAN in Rel-18. In Rel-19, AI/ML information coordination between different network entities can be considered as one of Rel-19 objectives, e.g., between CN and RAN.

The benefits of AI/ML interaction between the CN and RAN sides are evident. Taking mobility as an example, the CN can provide UE trajectory analysis reports and/or predictive information to the RAN side as assistance information for better training and prediction in RAN side. Or vise versa, the RAN side can offer its own AI/ML predicted information to the CN side. Currently, the expected UE behavior including predictable activity and/or mobility behaviour, potentially generated by NWDAF, is already being conveyed from CN to RAN over NGAP on an optional basis.

This existing provision allows for the exchange of AI/ML related information between the CN and RAN. Take another example, NWDAF has the capability to generate slice-level load predictions, and can provide its to the RAN, facilitating optimized management of network slices. Therefore, the exchange of AI/ML information between the CN and RAN sides can be further studied based on use case.

It is meaningful and benefitial to study the potential AI/ML related data coordination/interaction between CN and NG-RAN for support of AI/ML function in Rel-19.

- Study the benefits and potential coordination/interaction between CN and NG-RAN for support of AI/ML function in Rel-19.

- Conclusion

Following is our observations and proposals:

- AI/ML-based network slicing brings benefits such as optimized resource allocation, improved dynamic slice deployment, automated and flexible slice management, contributing to efficient and dynamic support for diverse services and applications in modern networks.

- Predicted Resource status per slice-level for current use cases (e.g., ES, LB, ME) is not discussed during Rel-17 & Rel-18 phase.

- Study the benefits and scope of AI/ML based network slicing, and the input/output/feedback and its potential solutions during Rel-19 SI (followed by a WI) in RAN3.

- Study the benefits and scope of AI/ML based CCO, and the input/output/feedback and its potential solutions during Rel-19 SI (followed by a WI) in RAN3.

- Continue to study and specify the enhancements and signaling support within split architecture for the support of AI/ML function in Rel-19.

- Following the Rel-18 leftovers can be further studied and specified in Rel-19:

- Mobility Optimization:

-  AI/ML based mobility optimization enhancements in NR-DC scenarios

-  Multiple-hop target node UE trajectory

-  Finer granularity level UE trajectory, e.g., beam-level

-  Network Energy Saving

-  Predicted Energy Cost

- Continuous MDT collection

- Study the benefits and potential coordination/interaction between CN and NG-RAN for support of AI/ML function in Rel-19.

- To capture following objectives in Rel-19:

The aim of this study item is to further investigate new AI/ML base use cases and identify enhancements to support AI/ML functionality.

The detailed objectives of the SI are listed as follows:

- Study the following new AI/ML based use cases, i.e., Network Slicing, Coverage and Capacity Optimization, and the corresponding solution for the identified use cases within the existing architecture. [RAN3]

- Study the data collection enhancements and signalling supports of AI/ML function over split architecture for Rel-18 and Rel-19 identified AI/ML based use cases. [RAN3]

- Study the benefits and potential coordination/interaction between CN and NG-RAN for support of AI/ML function. [RAN3]

- Continue to work on the following Rel-18 leftovers:

-   Mobility Optimization [RAN3]:

-   AI/ML based mobility optimization enhancements in NR-DC enhancements scenarios

-   Multiple-hop target node UE trajectory

-   Finer granularity level UE trajectory, e.g., beam-level

-   Network Energy Saving [RAN3]:

-   Predicted Energy Cost

-   Continuous MDT collection [RAN3, RAN2]

Note: Coordination based on LSs with other groups, if needed, e.g. RAN2, SA2, SA5.

4. Reference

[1] RP-232263, Moderator's summary for Rel-19 RAN3 topic AI/ML for NG-RAN, Ericsson

[2] TR37.817, Study on enhancement for Data Collection for NR and EN-DC