3GPP TSG-RAN WG1 Meeting #115	R1-2312737

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0163 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on NR positioning enhancement in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the agreements and TPs achieved in RAN1#114bis and RAN1#115 for NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the specification to reflect the following: - DCI size alignment between DCI format 3_0, 3_1 and 3_2 - SCI 2-D size determination - Terminology alignment for resource pool - Update of RRC parameters - Update the I to be the total number of dedicated SL PRS resource pools for DCI 3_2 - Incorporate DCI 0_3 and 1_3 into the table 7.3.1.1.2-24 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for Rel-18 NR positioning enhancement is incomplete or incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.0.1, 7.3.1.1.2, 7.3.1.4.1, 7.3.1.4.2, 7.3.1.4.3, 8.3.1.2, 8.4.1.4, 8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.0.1	DCI size alignment for DCI formats for scheduling of sidelink

If DCI format 3_0, and/or DCI format 3_1, and/or DCI format 3_2 is monitored on a cell, DCI size alignment for DCI format 3_0, DCI format 3_1, and DCI format 3_2 is performed as described in this clause after performing the DCI size alignment described in Clause 7.3.1.0. The size(s) of the DCI formats configured to monitor for a cell in this clause refers to that after performing the DCI size alignment described in Clause 7.3.1.0.

If DCI format 3_0, and/or DCI format 3_1, and/or DCI format 3_2 is monitored on a cell and the total number of DCI sizes of the DCI formats configured to monitor for the cell and DCI format 3_0, and/or DCI format 3_1, and/or DCI format 3_2 is more than 4, zeros shall be appended to DCI format 3_0 if configured, to DCI format 3_1 if configured, and to DCI format 3_2 if configured, until the payload size of DCI format 3_0, DCI format 3_1, and DCI format 3_2 equals that of the smallest DCI format configured to monitor for the cell that is larger than DCI format 3_0, DCI format 3_1, and DCI format 3_2.

The UE is not expected to handle a configuration that results in:

-	the total number of different DCI sizes configured to monitor for the cell and DCI format 3_0, and/or DCI format 3_1, and/or DCI format 3_2 is more than 4; and

-	the payload size of DCI format 3_0, and/or DCI format 3_1, and/or DCI format 3_2 is larger than the payload size of all other DCI formats configured to monitor for the cell.

< Unchanged parts are omitted >

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field, or value of ‘SRS request’ index for each cell in the scheduled cell set indicated by SRS request field in DCI 0_3 or 1_3 | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 0_2, 0_3, 1_1, 1_2, 1_3, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 1 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 1st set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 or an entry in aperiodicSRS-ResourceTriggerList set to 2 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 2 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 or an entry in aperiodicSRS-ResourceTriggerList set to 3 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 3 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |

< Unchanged parts are omitted >

#### 7.3.1.4	DCI formats for scheduling of sidelink

##### 7.3.1.4.1	Format 3_0

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell, or scheduling of NR PSCCH, NR PSSCH and NR SL PRS for a shared SL PRS resource pool in one cell.

The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI:

-	Resource pool index –log2I bits, where I is the total number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured.

-	Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214]

-	HARQ process number – 4 bits.

-	New data indicator – 1 bit.

-	Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits as defined in clause 8.1.2.2 of [6, TS 38.214]

-	SCI format 1-A fields according to clause 8.3.1.1:

-	Frequency resource assignment.

-	Time resource assignment.

-	PSFCH-to-HARQ feedback timing indicator –log2Nfb_timing bits, where Nfb_timing is the number of entries in the higher layer parameter sl-PSFCH-ToPUCCH, as defined in clause 16.5 of [5, TS 38.213]

-	PUCCH resource indicator – 3 bits as defined in clause 16.5 of [5, TS 38.213].

-	Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 3 bits as defined in clause 8.1.2 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI.

-	Counter sidelink assignment index – 2 bits

-	2 bits as defined in clause 16.5.2 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic

-	2 bits as defined in clause 16.5.1 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static

-	Padding bits, if required

If the total number of transmit resource pools provided in sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured, is larger than one, zeros shall be appended to the DCI format 3_0 until the payload size is equal to the size of a DCI format 3_0 given by a configuration of the transmit resource pool resulting in the largest number of information bits for DCI format 3_0.

If the UE is configured to monitor DCI format 3_1 and/or DCI format 3_2 and the number of information bits in DCI format 3_0 is less than the larger payload size of DCI format 3_1 if configured and DCI format 3_2 if configured, zeros shall be appended to DCI format 3_0 until the payload size equals the larger payload size of DCI format 3_1 if configured and DCI format 3_2 if configured.

##### 7.3.1.4.2	Format 3_1

DCI format 3_1 is used for scheduling of LTE PSCCH and LTE PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_1 with CRC scrambled by SL Semi-Persistent Scheduling V-RNTI:

-	Timing offset – 3 bits determined by higher layer parameter sl-TimeOffsetEUTRA-List, as defined in clause 16.6 of [5, TS 38.213]

-	Carrier indicator –3 bits as defined in 5.3.3.1.9A of [11, TS 36.212].

-	Lowest index of the subchannel allocation to the initial transmission -  bits as defined in 5.3.3.1.9A of [11, TS 36.212].

-	Frequency resource location of initial transmission and retransmission, as defined in 5.3.3.1.9A of [11, TS 36.212]

-	Time gap between initial transmission and retransmission, as defined in 5.3.3.1.9A of [11, TS 36.212]

-	SL index – 2 bits as defined in 5.3.3.1.9A of [11, TS 36.212]

-	SL SPS configuration index – 3 bits as defined in clause 5.3.3.1.9A of [11, TS 36.212].

-	Activation/release indication – 1 bit as defined in clause 5.3.3.1.9A of [11, TS 36.212].

If the UE is configured to monitor DCI format 3_0 and/or DCI format 3_2 and the number of information bits in DCI format 3_1 is less than the larger payload size of DCI format 3_0 if configured and DCI format 3_2 if configured, zeros shall be appended to DCI format 3_1 until the payload size equals the larger payload size of DCI format 3_0 if configured and DCI format 3_2 if configured.

##### 7.3.1.4.3	Format 3_2

DCI format 3_2 is used for scheduling of NR SL PRS for a dedicated SL PRS resource pool in one cell.

The following information is transmitted by means of the DCI format 3_2 with CRC scrambled by SL-PRS-RNTI or SL-PRS-CS-RNTI:

-	Resource pool index –log2I bits, where I is the total number of dedicated SL PRS resource pools for transmission configured by the higher layer parameter sl-PRS-TxPoolScheduling, if configured.

-	Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.2.4.1.1 of [6, TS 38.214]

-	First SL PRS indicator - log2NSL-PRS bits indicating the SL PRS resource ID for the first SL PRS transmission, where the value NSL-PRS is the total number of SL PRS resources within a slot in a dedicated SL PRS resource pool and provided by the higher layer parameter sl-PrsResources-Dedicated-SL-PRS-RP.

-	SCI format 1-B fields according to clause 8.3.1.2:

-	Time resource assignment

-	Resource ID indication

-	Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_2 with CRC scrambled by SL-PRS-CS-RNTI; otherwise 3 bits as defined in clause 8.2.4.1 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_2 with CRC scrambled by SL-PRS-CS-RNTI, this field is reserved for DCI format 3_2 with CRC scrambled by SL-PRS-RNTI.

-	Activation/release indication – 0 bit if the UE is not configured to monitor DCI format 3_2 with CRC scrambled with SL-PRS-CS-RNTI; otherwise 1 bit, where value 0 indicates release and value 1 indicates activation. If the UE is configured to monitor DCI format 3_2 with CRC scrambled with SL-PRS-CS-RNTI, this field is reserved for DCI format 3_2 with CRC scrambled by SL-PRS-RNTI.

-	Padding bits, if required.

If the total number of transmit resource pools provided in sl-PRS-TxPoolScheduling, if configured, is larger than one, zeros shall be appended to the DCI format 3_2 until the payload size is equal to the size of a DCI format 3_2 given by a configuration of the transmit resource pool resulting in the largest number of information bits for DCI format 3_2.

If the UE is configured to monitor DCI format 3_0 and/or DCI format 3_1 and the number of information bits in DCI format 3_2 is less than the larger payload size of DCI format 3_0 if configured and DCI format 3_1 if configured, zeros shall be appended to DCI format 3_2 until the payload size equals the larger payload size of DCI format 3_0 if configured and DCI format 3_1 if configured.

< Unchanged parts are omitted >

#### 8.3.1.2	SCI format 1-B

SCI format 1-B is used for the scheduling of SL PRS for a dedicated SL PRS resource pool.

The following information is transmitted by means of the SCI format 1-B:

-	Priority – 3 bits as specified in clause x.x of [12, TS 23.586] and clause x.x of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Source ID – 12 or 24 bits determined by higher layer parameter sl-SRC-ID-Len-Dedicated-SL-PRS-RP, as defined in clause 16.4A of [5, TS 38.213].

-	Destination ID – 24 bits as defined in clause 16.4A of [5, TS 38.213].

-	Cast type indicator – 2 bits as defined in Table 8.3.1.2-1 and in clause 16.4A of [5, TS 38.213].

-	Resource reservation period – log2Nrsv_period bits as defined in clause 16.4A of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP, if higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP is configured; 0 bit otherwise.

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve-Dedicated-SL-PRS-RP is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve-Dedicated-SL-PRS-RP is configured to 3, as defined in clause 8.2.4.3 of [6, TS 38.214].

-	Resource ID indication – log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserve-Dedicated-SL-PRS-RP  is configured to 2; otherwise 2log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserve-Dedicated-SL-PRS-RP is configured to 3. The value NSL-PRS is the total number of SL PRS resources within a slot in a dedicated SL PRS resource pool and provided by the higher layer parameter sl-PrsResources-Dedicated-SL-PRS-RP.

-	SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-based-SL-PRS-Tx-Trigger-SCI1-B  is provided; 0 bit otherwise.

-	Reserved - Nreserved bits as configured by higher layer parameter sl-NumReservedBits-SCI1B-Dedicated-SL-PRS-RP, with value set to zero.

Table 8.3.1.2-1: Cast type indicator

| Value of Cast type indicator | Cast type |
| --- | --- |
| 00 | Broadcast |
| 01 | Groupcast |
| 10 | Unicast |
| 11 | Reserved |

< Unchanged parts are omitted >

#### 8.4.1.4	SCI format 2-D

SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared SL PRS resource pool.

The following information is transmitted by means of the SCI format 2-D:

-	SL PRS resource ID – log2NSL-PRS bits, where the value NSL-PRS is the total number of SL PRS resource IDs within a slot in a shared SL PRS resource pool and provided by the higher layer parameter sl-PrsResources-Shared-SL-PRS-RP.

-	SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-based-SL-PRS-Tx-Trigger-SCI2-D is provided; 0 bit otherwise.

-	Embedded SCI format – 2 bits. This field indicates the embedded SCI format as defined in Table 8.4.1.4-1.

-	Embedded SCI format payload – number of bits determined according to Table 8.4.1.4-1. This field is set to the associated payload of the embedded SCI format indicated by the ‘Embedded SCI format’ field as defined in Table 8.4.1.4-1.

Table 8.4.1.4-1: Embedded SCI format and payload

| Value of the Embedded SCI format field | Embedded SCI format | Embedded SCI format payload |
| --- | --- | --- |
| 00 | SCI format 2-A | Set to all fields included in SCI format 2-A. Padding bits, if necessary, are appended to the 'Embedded SCI format payload' field untill the bitwidth equals the larger payload size of SCI format 2-A and SCI format 2-B. |
| 01 | SCI format 2-B | Set to all fields included in SCI format 2-B. Padding bits, if necessary, are appended to the 'Embedded SCI format payload' field untill the bitwidth equals the larger payload size of SCI format 2-A and SCI format 2-B. |
| 10 | Reserved | Reserved |
| 11 | Reserved | Reserved |

< Unchanged parts are omitted >

### 8.4.4	Rate Matching

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols generated for 2nd-stage SCI transmission prior to duplication for the 2nd layer if present, denoted as QSCI2', is determined as follows:

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2QmSCI2∙R, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

where

-	OSCI2 is the number of the 2nd-stage SCI bits

-	LSCI2 is the number of CRC bits for the 2nd-stage SCI, which is 24 bits.

-	βoffsetSCI2 is indicated in the corresponding 1st-stage SCI.

-	MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission, expressed as a number of subcarriers.

-	MscPSCCH(l) is the number of subcarriers in OFDM symbol l that carry PSCCH and PSCCH DMRS associated with the PSSCH transmission.

-	MscSCI2(l) is the number of resource elements that can be used for transmission of the 2nd-stage SCI in OFDM symbol l, for l=0,1,2⋯,NsymbolPSSCH-1 and for NsymbolPSSCH=Nsymbsh-NsymbPSFCH-NsymbSL PRS, in PSSCH transmission, where NsymbshNsymbslot = sl-lengthSymbols - 2, where sl-lengthSymbols is the number of sidelink symbols within the slot provided by higher layers as defined in [6, TS 38.214]. NsymbSL PRS is the number of symbols for SL PRS provided by the higher layer parameter numSym-SL-PRS-2ndStageSCI if the 2nd-stage SCI is SCI format 2-D, and NsymbSL PRS = 0 otherwise. If startingSymbolFirst and startingSymbolSecond are provided for the SL-BWP, NsymbshNsymbslot = numRefSymbolLength - 2, where numRefSymbolLength is provided by higher layers. If higher layer parameter sl-PSFCH-Period = 2 or 4, NsymbPSFCH = 3 if "PSFCH overhead indication" field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise. If higher layer parameter sl-PSFCH-Period = 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

-	MscSCI2(l) = MscPSSCH(l) - MscPSCCH(l)

-	γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs.

-	R is the coding rate as indicated by "Modulation and coding scheme" field in SCI format 1-A.

-	α is configured by higher layer parameter sl-Scaling.