## Compliance template for 3GPP 5G NTN RIT (Release 17 and beyond)

The compliance templates provided by 3GPP are for the assessment of the compliance of 5G satellite access developed by 3GPP. It includes one compliance template for SRIT, encompassing component RITs for NR NTN (NR satellite access) and IoT NTN (consisting of NB-IoT satellite access and eMTC satellite access), and one compliance template for NR NTN (NR satellite access) RIT.

This document provides the compliance template for the RIT based on 3GPP Rel-17 work.

#### Compliance template for services

| Service-related minimum capabilities within the satellite radio interface(s) | Evaluator’s comments |
| --- | --- |
| Support of a wide range of services Does the proposal support a wide range of services? YES The NR satellite radio interface RIT can support eMBB-s, HRC-s, and mMTC-s usage scenarios. | The assessment of service requirement follows the evaluation method as defined in Section 8.2 in Report ITU-R M.2514. |

#### Compliance template for spectrum

| Spectrum capability requirements | Evaluator’s comments |
| --- | --- |
| Spectrum bands Is the proposal able to utilize at least one band identified for IMT? YES Specify in which band(s) the candidate satellite radio interface(s) can be deployed The supported frequency bands are provided in item 8.2.5.1.8.3 in characteristics template for NR satellite radio interface RIT. |  |

#### Compliance template for technical performance

##### For NR satellite radio interface RIT:

NOTE : The following values are derived based on the employed evaluation configurations. Higher performance might be achieved by using other evaluation configurations.

See self evaluation report for detailed analysis, results and specific assumptions (e.g. Frequency reuse factor, antenna configurations, etc.).

| Minimum technical requirements items | Category |  |  | Required value | Value | Requirement met? | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Usage scenario | Test environment | Downlink or uplink |  |  |  |  |
| Peak data rate | eMBB-s | N/A | Uplink | 2 Mbit/s | 2.67 | Yes |  |
|  |  |  | Downlink | 70 Mbit/s | 111 | Yes |  |
| Peak spectral efficiency | eMBB-s | N/A | Uplink | 1.5 bit/s/Hz | 1.85 | Yes |  |
|  |  |  | Downlink | 3 bit/s/Hz | 3.71 | Yes |  |
| User experienced data rate | eMBB-s | Rural | Uplink | 100 kbit/s | 130~280 | Yes | Frequency reuse factor 3 (FRF3) is used to achieve higher user experienced data rate |
|  |  |  | Downlink | 1 Mbit/s | 0.85~1.43 | Yes |  |
| 5th percentile user spectral efficiency | eMBB-s | Rural | Uplink | 0.003 bit/s/Hz | 0.006~0.010 | Yes | Both configurations with frequency reuse factor equal to 1 and evaluation configuration with frequency reuse factor equal to 3 are evaluated |
|  |  |  | Downlink | 0.03 bit/s/Hz | 0.029~0.047 | Yes | Both configurations with frequency reuse factor equal to 1 and evaluation configuration with frequency reuse factor equal to 3 are evaluated |
| Average spectral efficiency | eMBB-s | Rural | Uplink | 0.1 bit/s/Hz | 0.145~0.233 | Yes |  |
|  |  |  | Downlink | 0.5 bit/s/Hz | 0.562~0.783 | Yes |  |
| Area traffic capacity | eMBB-s | Rural | Uplink | 1.5 kbit/s/km² | 3.06~4.87 | Yes |  |
|  |  |  | Downlink | 8 kbit/s/km² | 11.30~16.60 | Yes |  |
| User Plane latency | eMBB-s | N/A | N/A | 10 ms | Uplink : 4.65~6.93 Downlink : 4.65~6.93 | Yes |  |
| Control Plane latency | eMBB-s | N/A | N/A | 40 ms | 19.2~23.4 | Yes |  |
| Connection density | mMTC-s | Rural | N/A | 500 devices/km² | 600/180kHz~ 7205/180 kHz 2277/540 kHz ~ 27357/540 kHz | Yes | A bandwidth of 180kHz is considered for FRF1 and bandwidth 540kHz for FRF3 |
| Energy efficiency | eMBB-s | N/A | N/A | High sleep ratio and long sleep duration | Sleep ratio: 80%~99.87% Sleep duration: Up to 159ms | Yes | Network side |
| Reliability | HRC-s | Rural | N/A | 1-10−3 | Uplink: 99.97% Downlink: 99.96%~99.98% | Yes | Both configurations with frequency reuse factor equal to 1 and evaluation configuration with frequency reuse factor equal to 3 are evaluated |
| Mobility – UE speed | eMBB-s | Rural | N/A | 250 km/h | 250 km/h | Yes |  |
| Mobility - Traffic channel link data rate | eMBB-s | Rural | N/A | 0.005 bit/s/Hz | 0.07~0.14 | Yes |  |
| Mobility interruption time | eMBB-s | N/A | N/A | 50 ms | 0 ms | Yes |  |
| Bandwidth | N/A | N/A | N/A | At least up to and including 30 MHz | 30 MHz | Yes |  |

Developed by 3GPP as 5G NTN, Release 17 and beyond.