3GPP RAN TS Meeting #103	RP-240062

Maastricht, The Netherlands, March 18th – 22nd, 2024

Title:	Candidate non spectrum related NTN topics for Rel-19 work plan

Source: 	Thales, Gilat, Terrestar, CATT, CeWIT, Sateliot, ESA, Inmarsat, Viasat, Mitre, Sateliot, ESA, OQ technology, Fraunhofer IIS and HHI, Airbus, Novamint, Magister solution, ETRI, Iridium, Satellite Applications Catapult, Omnispace, SES

Type:	discussion

Document for:	discussion

Agenda Item:	 9.1.4.6 - Other topics (e.g. covering multiple))

# Introduction

This document entails a list of candidate non spectrum related NTN topics (not in priority order) for the Rel-19 RAN4 work plan.

# Candidate non spectrum related NR-NTN topics for Rel-19 RAN4 work

## Support of High power UE in FR1-NTN for NR-NTN

### Justification

Increasing the UE maximum transmit power will significantly improve UL performance in terms of coverage, availability and throughput, also needed to meet the market demands associated with fixed wireless, public safety and automotive usage.

For a number of NTN use cases, including automotive, maritime, logistics, outdoor industrial monitoring and other scenarios not reliant on battery power, such that the higher power requirements of PC2, PC1.5 and PC1 are not a power issue.

Lastly, for the satellite community, this should also be a good opportunity to better and more clearly align with the existing regulation and specs used by current satellite services (primarily MSS), such as ECC/CEPT and FCC, but not only, which typically allow significantly higher UE transmit powers even for handheld  (e.g. satellite phones and trackers, which are different from smartphones) and automotive UE, and operate in narrow-band spectrum. Therefore, regulation is not expected to be a barrier.

Some UE (e.g. smartphones) may be subject to SAR restriction and hence require duty cycle enhancement.

### Objectives

Assumptions

- Support HPUE in FR1-NTN bands (e.g. n256, n255, n254 and the upcoming n253)

- Support of UE with PC2 (+26 dBm)

  - Note that it applies to handheld terminals with omni antenna and automotive terminals with characteristics as per LS by 5GAA [RP-232733].

- Support of UE with PC1 (+31 dBm)

  - Note that it can apply to non handheld terminals with omnidirectional or non omnidirectional antenna

- Support of UE with PC1.5 (+29 dBm)

- Coexistence studies, shall be carried out considering same conditions (i.e. geographical separation between NTN UE and Terrestrial Networks (TN)) as in Rel-17 and 18

Objective include

- Update, if needed, coexistence analysis based on TR 38.863 defined methodology

- Specify RF aspects of the targeted UE

- Specify transmission enhancement for NTN high power UE, e.g. duty cycle enhancement subject to Specific Absorption Rate restriction

### Working groups

RAN4

## Less than 5 MHz NR Channel Bandwidth in FR1-NTN

### Justification

This feature is currently defined for Terrestrial Networks to cater for use-cases where there are spectrum limitation/constraints. Satellite deployments in L-Band and S-Band face the same constraints and would benefit from the work done in the Rel-18 WID “NR support for dedicated spectrum less than 5 MHz for FR1” by extending support for NTN.

This feature should as much as possible build on the work done in the Rel-18 NR support for dedicated spectrum less than 5 MHz for FR1 WID, by extending support for NTN.

The NTN extension should introduce support for the 12 PRB and 15 PRB operation and new 3 MHz NR channel bandwidth.

This feature will enable more efficient deployment of the FR1 NTN bands where parts of the spectrum for satellite use is limited, for example:

- Facilitate deployment of NR NTN within the context of existing non-3GPP satellite services

- Lower bandwidth for NR will enable better use of spectrum resource by allowing higher frequency re-use between beams, increasing capacity.

  - Maximizing capacity for a given region requires the deployment of many spot beams, within satellite power constraints. To mitigate the inter-beam interference requires high frequency re-use which is more difficult to achieve with 5 MHz beams.

- Enable deployment of both NR and standalone NB-IoT/eMTC carriers in the same 5 MHz Bandwidth to enable parallel support of both eMBB and IoT services.

- Allowing more flexibility in deployment where spectrum allocation is irregular (not in the nX5 MHz)

- Smaller channel BW also helps with link budget in situations where the payload is EIRP-limited

- Allow for optimizing capacity vs demand in each spot beam.

### Objectives

The aim will be to re-use the work done in TN Rel-18 NR support for spectrum less than 5 MHz and focus on the RAN4 requirements for NTN:

- Specify SAN and UE RF requirements for supporting 3 MHz channel bandwidth in NTN FR1 bands n255, n256, n254 and upcoming n253 (Extended L-band with UL: 1668-1675 MHz, DL: 1518-1525 MHz)   [RAN4/RF]

- Specify RRM requirements for supporting 12 and 15 PRB operation in NTN FR1 bands [RAN4/RRM]

Note:

- Re-use the same assumptions as adopted for terrestrial, in respect to maintaining current PSS/SSS without puncturing, reusing the PBCH design with puncturing and appropriate CORESET formats supporting 12 and 15 PRB transmission, and consider reusing the 3 MHz sync raster defined for TN at least for 3 MHz channel BW. Hence no additional work is expected in RAN1 and RAN2.

- GSO and NGSO to be considered

### Working groups

RAN4

## Support of Mobile VSAT with NGSO

### Justification

In Rel-18 NR-NTN-enh WI, the support of the Mobile VSAT to NGSO use case in frequency bands above 10 GHz was not included due to regulatory uncertainties. Given that WRC-2023 confirmed the support of Mobile VSAT in Ka-band [R4-2400511], this should now be addressed in Rel-19.

Mobile services provided by VSAT connecting to satellite, in particular NGSO satellite, is a rapidly growing market segment. Providing 5G NR connectivity to aircraft and maritime vessels (referred to in ITU-R as ESIM-A and ESIM-M respectively), as can be provided by existing proprietary systems, requires full mobility to be supported. Further, automotive, non-handheld  terminals, as defined in the LS by 5GAA [RP-232733], are considered an important emerging market [R4-2400516].

### Objectives

Objectives include

- Update if necessary, RRM timing requirements [RAN4/RRM]

Note:

- It is assumed that the existing NR numerology in FR2-NTN will remain unchanged.

- The RF parameters of already defined UE types in FR2-NTN, are to be reused

- The exemplary Ka band may be considered in priority

### Working groups

RAN4

# Candidate non spectrum related IoT-NTN topics for Rel-19 RAN4 work

## Support of High power UE in FR1-NTN

### Justification

Increasing the UE maximum transmit power will enable to improve UL performance in terms of coverage, availability and throughput performance, to meet the market demands associated with fixed wireless, public safety and automotive usage.

For a number of NTN use cases, including automotive, maritime, logistics, outdoor industrial monitoring and other scenarios not reliant on battery power, such that the higher power requirements of PC2, PC1.5 and PC1 are not a power issue.

Lastly, for the satellite community, this should also be a good opportunity to better and more clearly align with the existing regulation and specs used by current satellite services (primarily MSS), such as ECC/CEPT and FCC, but not only, which typically allow significantly higher UE transmit powers even for handheld  (e.g. satellite phones and trackers, which are different from smartphones) and automotive UE, and operate in narrow-band spectrum. Therefore, regulation is not expected to be a barrier.

Some UE (e.g. smartphones) may be subject to SAR restriction and hence require duty cycle enhancement.

### Objectives

Assumptions

- Support high power feature for eMTC IoT NTN UEs (Cat M1) and NB-IoT NTN UEs (Cat NB1, NB2)

- Support HPUE on FR1-NTN bands (e.g. 256, 255, 254 and 253)

- Support of UE with PC2 (+26 dBm)

  - Note that it applies to handheld terminals with omni antenna and automotive terminals with characteristics as per LS by 5GAA [RP-232733].

- Support of UE with PC1 (+31 dBm)

  - Note that it can apply to non handheld terminals with omnidirectional or non omnidirectional antenna

Objective include

- Update, if needed, coexistence analysis based on TR 38.863 defined methodology

  - Possibly revisiting geographical separation between NTN UE and Terrestrial Networks (TN) at least for certain use cases and scenarios (e.g. automotive)

- Specify RF aspects of the targeted UE

  - Possibly revisiting spectrum emission mask (SEM)

- Specify transmission enhancement for NTN high power UE, e.g. duty cycle enhancement subject to Specific Absorption Rate restriction

### Working groups

RAN4

## In-band IoT-NTN

### Justification

NTN NB-IoT has been defined in Rel-17 and there has been successful demonstration of NTN capable devices. To support growth of NTN NB-IoT it is necessary to further improve and encourage the network deployment with NTN-NR operation. NB-IoT in-band operation co-existence with NR allows the configuration of the DL/UL resource reservation on non-anchor carriers for unicast transmission to avoid resource overlapping with NR channels/signals. In leveraging the NB-IoT and NR co-existence work in TN (captured in TR 37.824), at least two system scenarios should be considered:

- For NB-IoT operation in NR in-band

- For NB-IoT operation in NR guard band, baseline assumption is to re-use the conclusions from TR 37.824, where this is treated the same as in-band deployment.

- The 7.5 kHz UL shift required on the NR NTN carrier is already specified from Rel-17 and Rel-18 version of the NR NTN spec.

### Objectives

Specify for NTN NB-IoT in-band operation co-existence with NTN NR.

- Check whether the assumptions for terrestrial deployment in TR 37.824 can be directly applied for NTN

- Update NB-IoT UE RF spec (TS 36.102) to include support for in-band NB-IoT deployment, including additional M_dl and M_ul values, if needed

- Update NR SAN RF and test requirements to support NTN NB-IoT, if needed.

  - FFS whether update to MSR requirements is required

Note: Assumption is that NB-IoT NTN and NR NTN carrier are deployed from within the same SAN.

### Working groups

RAN4 (Only RF)

# Candidate non spectrum related IoT & NR-NTN topics for Rel-19 RAN4 work

## Enhanced Test Equipment-emulated channel model for NGSO

### Justification

To ensure that the 3GPP tests cover the operation of UE with NGSO networks i.e. dynamic conditions.

### Objectives

Assumptions

- Consider FR1-NTN and FR2-NTN

- Consider NR-NTN and IoT-NTN

Objectives

- Add reference in the relevant specification(s) of the joint delay and Doppler variation model related to NGSO of TR 38.811 [RAN4/RRM]

- Define demod requirements under these delay and Doppler variation conditions for respectively NR-NTN and IoT-NTN [RAN4/Demod]

- Update RRM requirements (e.g. timing), as applicable [RAN4/RRM]

- FFS: Update RF requirements (e.g. frequency accuracy), as applicable [RAN4/RF]

### Working groups

RAN4

# Conclusion and Way forward

Proposal: Consider the following non spectrum related NR-NTN topics as part of Rel-19 RAN4 work plan:

- Support of High power UE (NR-NTN) in FR1-NTN

- 3 MHz NR channel bandwidth in FR1-NTN

- Enhanced Test Equipment-emulated channel model for NGSO (NR-NTN)

- Support of Mobile VSAT with NGSO (examplary frequency band can be considered, i.e. Ka band)

Proposal: Consider the following non spectrum related IoT-NTN topics as part of Rel-19 RAN4 work plan:

- Support of High power UE (IoT-NTN) in FR1-NTN

- Enhanced Test Equipment-emulated channel model (IoT-NTN)

- In band NB-IoT NTN deployment with NR NTN in the same SAN

# References

[1] RP-233918 Moderator's summary for RAN4 Candidate RF/OTA Topics for Rel-19, RAN4 chair (Huawei)

[2] RP-233920 Moderator's summary for RAN4 Candidate Topics Impacting both RF/OTA and Demod/RRM for Rel-19, Apple

END