3GPP TSG RAN Meeting #103								RP-240169

Maastricht, Netherlands, March 18-21, 2024

## Status Report to TSG

Agenda item:			9.3.1.5

| WI / SI Name | Enhancements of network energy savings for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | Netw_Energy_NR_enh |  |  |  |  |
| Unique ID | 1020095 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234065 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 5% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Ajit Nimbalker |
|  | Company | Ericsson |
|  | Email | Ajit.Nimbalker@ericsson.com |
| Rapporteur (RAN2) | Name | Peng Cheng |
|  | Company | Apple |
|  | Email | pcheng24@apple.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#116, the following agreements were made.

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

- RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

- For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

- Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options.

On target cell of UL WUS transmission:

- Option 1: UE transmits UL WUS to NES Cell

- Option 2: UE transmits UL WUS to Cell A

On configuration provision for UL WUS transmission

- Option A: UE obtains the UL WUS configuration from NES Cell

- Option B: UE obtains the UL WUS configuration from Cell A

Other options are not precluded

Agreement

For further study of achievable NES gain with on-demand SIB1 for idle/inactive mode UE,

- Assume the following for network energy evaluation of non-NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case A: 20ms SSB period with 20ms SIB1 period;

    - Case C: 20ms SSB period with 160ms SIB1 period;

    - Case D: 20ms SSB period with 40ms SIB1 period;

Note: Other SSB/SIB1 periodicity assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms or 160ms PRACH monitoring period

- Assume the following for network energy evaluation of NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case 1: 20ms SSB period with no SIB1 transmitted;

Note: Other SSB/SIB1 assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms/160ms UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1 is used

Agreement

- For study of UL WUS design, consider at least PRACH as a starting point

  - FFS: Whether there is dedicated PRACH resource for SIB1 request

- Other option(s) not precluded

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS.

Agreement

For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

Agreement

For the further study on UL WUS configuration among the following options:

- Option 1: Pre-defined UL WUS configuration

- Option 2: UL WUS configuration that applies to multiple NES cell

- Option 3: UL WUS configuration that applies to a single NES cell

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

#### 2.1.2	Remaining Open issues

- Details of specifying procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

- Details of the study of procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode.

- Details of specifying adaptation of common signal/channel transmissions, including the study of adaptation of PRACH in spatial domain.

## 2.2	RAN2

#### 2.2.1	Agreements

- Not yet started

#### 2.2.2	Remaining Open issues

- Not yet started

## 2.3	RAN3

#### 2.3.1	Agreements

- Not yet started

#### 2.3.2	Remaining Open issues

- Not yet started

## 2.4	RAN4

#### 2.4.1	Agreements

- Not yet started

#### 2.4.2	Remaining Open issues

- Not yet started

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

#### 4.1	RAN1#116

R1-2400062	Discussion on on-demand SSB SCell operation	Spreadtrum Communications

R1-2400063	Discussion on on-demand SIB1 for idle/inactive mode UEs	Spreadtrum Communications

R1-2400064	Discussion on adaptation of common signal/channel transmissions	Spreadtrum Communications

R1-2400097	Discussion of on-demand SSB Scell operation	FUTUREWEI

R1-2400098	Discussion of on-demand SIB1 for idle/inactive mode UEs	FUTUREWEI

R1-2400099	Discussion of the adaptation of common signal/channel transmissions	FUTUREWEI

R1-2400119	On-demand SSB SCell operation for NES	Huawei, HiSilicon

R1-2400120	Discussion on on-demand SIB1 for NES	Huawei, HiSilicon

R1-2400121	Adaptation of common signal/channel transmissions	Huawei, HiSilicon

R1-2400176	On-demand SSB SCell operation	Ericsson

R1-2400180	On-demand SSB Scell Operation	Nokia, Nokia Shanghai Bell

R1-2400181	On-demand SIB1 for Idle/Inactive mode UEs	Nokia, Nokia Shanghai Bell

R1-2400182	Adaptation of common signal/channel transmissions	Nokia, Nokia Shanghai Bell

R1-2400208	Discussion on On-Demand SSB SCell operation	Transsion Holdings

R1-2400209	Discussion on on-demand SIB1 transmission	Transsion Holdings

R1-2400210	Discussion on adaptive transmission of common signal or common channel	Transsion Holdings

R1-2400250	Discussions on on-demand SSB Scell operation	vivo

R1-2400251	Discussions on on-demand SIB1 for idle/inactive mode UEs	vivo

R1-2400252	Discussions on adaptation of common signal/channel transmissions	vivo

R1-2400335	Discussion on on-demand SSB SCell operation	CMCC

R1-2400336	Discussion on-demand SIB1 for UEs in idle/inactive mode	CMCC

R1-2400337	Discussion on adaptation of common signal/channel transmissions	CMCC

R1-2400369	Design of on-demand SSB SCell operation	Intel Corporation

R1-2400370	Study of on-demand SIB1 for idle/inactive mode UEs	Intel Corporation

R1-2400371	Adaptation of common signal/channel transmissions for Network Energy Saving	Intel Corporation

R1-2400399	On-demand SSB SCell Operation	Google

R1-2400400	On-demand SIB1 for Idle/Inactive Mode UE	Google

R1-2400401	Adaptation of Common Signals	Google

R1-2400441	Discussion on on-demand SSB SCell operation	CATT

R1-2400442	Discussion on on-demand SIB1	CATT

R1-2400443	Discussion on adaptation of common signal/channel transmissions	CATT

R1-2400492	Discussion on on-demand SSB for NES	ZTE, Sanechips

R1-2400493	Discussion on on-demand SIB1 for UEs in idle or inactive mode	ZTE, Sanechips

R1-2400494	Discussion on common signal_channel for NES	ZTE, Sanechips

R1-2400515	On-demand SSB delivery of NES cells	Dell Technologies

R1-2400525	Discussion on on-demand SSB SCell operation	Honor

R1-2400526	Discussion on adaptation of common signal channel transmissions	Honor

R1-2400566	Discussion on on-demand SSB SCell operation	xiaomi

R1-2400567	Discussion on on-demand SIB1 for idle/inactive mode UEs	xiaomi

R1-2400568	Discussion on adaptation of common signal and channel transmissions	xiaomi

R1-2400599	Discussion on the enhancement to support on demand SSB SCell operation	OPPO

R1-2400600	Discussion on the enhancement to support on demand SIB1 for idle/inactive mode UE	OPPO

R1-2400601	Discussion on adaptation of common signal/channel transmission	OPPO

R1-2400667	Discussion on on-demand SSB operation for SCell	China Telecom

R1-2400668	Discussion on on-demand SIB1 for idle/inactive mode UEs	China Telecom

R1-2400669	Discussion on common signal/channel adaptation	China Telecom

R1-2400740	On-demand SSB SCell operation	Samsung

R1-2400741	On-demand SIB1 for idle/inactive mode UEs	Samsung

R1-2400742	Adaptation of common signal/channel transmissions	Samsung

R1-2400806	Discussion on on-demand SSB for SCell operation	NEC

R1-2400807	Discussion on on-demand SIB1 for UEs in idle/inactive mode	NEC

R1-2400808	Discussion on adaptation of common signal/channel transmissions for Cell DTX/DRX	NEC

R1-2400821	Discussion on On-demand SSB SCell operation	InterDigital, Inc.

R1-2400822	Discussion on On-demand SIB1 for idle/inactive mode UEs	InterDigital, Inc.

R1-2400823	Discussion on adaptation of common signal/channel transmissions	InterDigital, Inc.

R1-2400861	Considerations on on-demand SIB1 for idle/inactive mode UEs	Sony

R1-2400862	Considerations on adaptation of SSB in time domain	Sony

R1-2400901	Discussion on on-demand SSB SCell operation	Panasonic

R1-2400902	Discussion on on-demand SIB1 for idle/inactive mode UEs	Panasonic

R1-2400903	Discussion on adaptation of common signal/channel transmission	Panasonic

R1-2400927	On-demand SSB SCell operation for NES	Fraunhofer IIS, Fraunhofer HHI

R1-2400928	On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI

R1-2400929	Adaptation of Common Signals and Channels for NES	Fraunhofer IIS, Fraunhofer HHI

R1-2400979	Adaptation of common signals and channels	Lenovo

R1-2401020	On-demand SSB SCell operation	Apple

R1-2401021	On On-demand SIB1 for IDLE/INACTIVE mode UEs	Apple

R1-2401022	On adaptation of common signal/channel for NES enhancements	Apple

R1-2401053	Discussion on on-demand SSB SCell operation	NTPU

R1-2401054	Scope of on-demand SIB study	NTPU

R1-2401085	On-demand SSB SCell operation	Lenovo

R1-2401086	On-demand SIB1 for idle/inactive mode UEs	Lenovo

R1-2401123	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.

R1-2401124	Discussion on on-demand SIB1 for idle/inactive mode UEs	NTT DOCOMO, INC.

R1-2401125	Discussion on adaptation of common signal/channel transmissions	NTT DOCOMO, INC.

R1-2401143	Workplan for Rel-19 network energy savings WI	Rapporteurs (Ericsson, Apple)

R1-2401144	Study of on-demand SIB1 for UEs in idle/inactive mode for NES	Ericsson

R1-2401145	Adaptation of common signal/channel transmissions for NES	Ericsson

R1-2401149	Views on adaptation of SS/PBCH blocks	KT Corp.

R1-2401176	Discussion on on-demand SIB1 transmission for idle UEs	Sharp

R1-2401177	Discussion on adaptation of common signal/channel transmissions	Sharp

R1-2401190	Discussion on On-demand SSB SCell operation	ITRI

R1-2401196	On-demand SSB for SCell	ASUSTeK

R1-2401197	Triggering of on-demand SIB1	ASUSTeK

R1-2401204	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu

R1-2401205	Discussion on adaptation of common signal / channel transmissions	Fujitsu

R1-2401206	Discussion on on-demand SSB SCell operation	Fujitsu

R1-2401212	Discussion of on-demand SSB SCell operation	CAICT

R1-2401232	Discussion on On-demand SSB SCell operation	ETRI

R1-2401233	Discussion on on-demand SIB1 for idle/inactive mode UEs	ETRI

R1-2401234	Adaptation of common signal/channel transmissions	ETRI

R1-2401253	Views on On-demand SIB1 operation for idle/inactive UEs	Vodafone

R1-2401279	Discussion on on-demand SSB Scell operation	CEWiT

R1-2401280	Discussion on  on-demand SIB1	CEWiT

R1-2401281	Discussion on adaptation of common signal and channel transmissions	CEWiT

R1-2401291	On-demand SSB SCell operation	MediaTek Inc.

R1-2401292	On-demand SIB1 for idle or inactive mode UEs	MediaTek Inc.

R1-2401293	Adaptation of common signal/channel transmissions	MediaTek Inc.

R1-2401332	On-demand SSB SCell operation	LG Electronics

R1-2401333	On-demand SIB1 for idle/inactive mode UEs	LG Electronics

R1-2401334	Adaptation of common signal/channel transmissions	LG Electronics

R1-2401449	On-demand SSB operation for Scell	Qualcomm Incorporated

R1-2401450	On-demand SIB1 procedure	Qualcomm Incorporated

R1-2401451	Adaptation of common channel transmissions	Qualcomm Incorporated

R1-2401473	Discussion on on-demand SSB SCell operation	NTPU

R1-2401474	Scope of on-demand SIB study	NTPU

R1-2401536	Discussions on on-demand SIB1 for idle/inactive mode UEs	vivo

R1-2401565	Discussions on on-demand SIB1 for idle/inactive mode UEs	vivo

R1-2401661	FL summary 1 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2401662	FL summary 2 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2401663	FL summary 3 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2401673	Summary #1 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2401674	Summary #2 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2401675	Summary #3 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2401728	Summary#1 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2401821	Summary#2 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2401840	Summary #4 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2401856	Summary#3 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2401870	Final summary of AI 9.5.3 for R19 NES	Moderator (Ericsson)