3GPP TSG-RAN5 Meeting #102	R5-241615

# Athens, Greece, 26 February-1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5279 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SENSE NB-IoT TC 22.2.15-PLMN selection Exception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SENSE-UEContest |  |  |  |  |  | Date: |  |  | 2024-2-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 22.2.15 is added to test UE may select EHPLMN(not RPLMN) when signal quality of EHPLMN is higher than the threshold configured in USIM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of SENSE NB-IoT TC 22.2.15 NB-IoT / SENSE/ PLMN selection of RPLMN or (E)HPLMN / Automatic mode |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SENSE-UEContest will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.15 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-2 CR 1437 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | revised from R5-240422r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

22.2.15	NB-IoT / SENSE/ PLMN selection of RPLMN or (E)HPLMN / Automatic mode

22.2.15.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and signal level enhanced network selection is applicable and UE is fitted with a USIM containing the EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected and received signal quality of RPLMN is equal to or greater than the "Operator controlled signal threshold per access technology" and received signal quality of EHPLMN is lower than the "Operator controlled signal threshold per access technology" }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN }

}

(2)

with { UE in Automatic network selection mode and signal level enhanced network selection is applicable and UE is fitted with a USIM containing the EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected and received signal quality of RPLMN and EHPLMN are equal to or greater than than the "Operator controlled signal threshold per access technology" }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN or EHPLMN }

}

(3)

with { UE in Automatic network selection mode and signal level enhanced network selection is applicable and UE is fitted with a USIM not containing or containing empty EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected and received signal quality of RPLMN and VPLMN are equal to or greater than than the "Operator controlled signal threshold per access technology" and received signal quality of HPLMN is lower than the "Operator controlled signal threshold per access technology" }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN }

}

(4)

with { UE in Automatic network selection mode and signal level enhanced network selection is applicable and UE is fitted with a USIM not containing or containing empty EHPLMN list and the USIM indicates RPLMN or (E)HPLMN should be selected and received signal quality of RPLMN, HPLMN and VPLMN are equal to or greater than the "Operator controlled signal threshold per access technology" }

ensure that {

when { UE is switched on }

then { UE selects a cell of the RPLMN or HPLMN }

}

22.2.15.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 4.4.3.1. Unless otherwise stated these are Rel-18 requirements.

[TS 23.122, clause 4.4.3.1]

EXCEPTION: As an alternative option to this, if the MS is in automatic network selection mode and it finds coverage of an EHPLMN, the MS may register to that EHPLMN and not return to the registered PLMN or equivalent PLMN. If the EHPLMN list is not present or is empty, and the HPLMN is available, the MS may register on the HPLMN and not return to the registered PLMN or equivalent PLMN. The operator shall be able to control by SIM configuration whether an MS that supports this option is permitted to perform this alternative behaviour. If signal level enhanced network selection is applicable (see clause 3.11), the MS may register to that EHPLMN or HPLMN only over an access technology for which the received signal quality is equal to or greater than the "Operator controlled signal threshold per access technology" stored in the USIM.

…

22.2.15.3	Test description

22.2.15.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN cells as specified in TS 36.508 clause 8.1.4.1.2 are configured broadcasting PLMNs as indicated in Table 22.2.15.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 22.2.15.3.1-1.

Table 22.2.15.3.1-1: PLMN identifiers

| Ncell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is registered to PLMN1 with USIM A inserted before it is switched off.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.8.3.1-2 and Table 6.1.1.8.3.1-3 will be used.

-     The UE is configured for using SENSE as specified in 3GPP TS 24.368 which could be done by MMI or AT command.

Table 22.2.15.3.1-2: USIM A configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFEPSLOCI |  | PLMN1 |  |
| EFUST |  | Service n°71, n°74, n°85 and n°148 are "available" |  |
| EFEHPLMN | 1 | PLMN2 |  |
| EFLRPLMNSI |  | 01 |  |
| EFOCST |  | 1’H(SENSE enabled by operator) 58’H(Operator controlled signal threshold for E-UTRAN is -91dBm) | E-UTRAN |

Table 22.2.15.3.1-3: USIM B configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFEPSLOCI |  | PLMN1 |  |
| EFUST |  | Service n°74, n°85 and n°148 is "available" |  |
| EFEHPLMN |  | Empty |  |
| EFLRPLMNSI |  | 01 |  |
| EFOCST |  | 1’H (SENSE enabled by operator) 58’H(Operator controlled signal threshold for E-UTRAN is -91dBm) | E-UTRAN |

Preamble:

-	The UE is made to camp on Ncell 12 and then Switched OFF (State 1).

22.2.15.3.2	Test procedure sequence

Table 22.2.15.3.2-1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2” etc are applied at the points indicated in the Main behaviour description in Table 22.2.15.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 22.2.15.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 12 | Ncell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | “Off” | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | -85 | -85 | -97 |  |
| T2 | NRS EPRE | dBm/15kHz | -85 | -85 | -85 |  |
| T3 | NRS EPRE | dBm/15kHz | -97 | -85 | -85 |  |
| T4 | NRS EPRE | dBm/15kHz | -85 | -85 | -85 |  |

Table 22.2.15.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts Ncell levels according to row T1 of table 22.2.15.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 12? | --> | RRCConnectionRequest-NB | 1 | P |
| 4-15 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 16a1 to 16b1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 16a1-16a4 | If pc_SwitchOnOff then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 16b1 | Else power off the UE |  |  |  |  |
| 17 | SS adjusts Ncell levels according to row T2 of table 22.2.15.3.2-1 | - | - | - | - |
| 18 | Power on the UE. | - | - | - | - |
| 19 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 12 or Ncell 13? | --> | RRCConnectionRequest-NB | 2 | P |
| 20-31 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12 or Ncell 13. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 32a1 to 32b1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 32a1-32a4 | If pc_SwitchOnOff or pc_USIM_Removal then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 32b1 | Else power off the UE. |  |  |  |  |
| 33 | The SS adjusts Ncell levels according to row T0 of table 22.2.15.3.2-1. | - | - | - | - |
| 34 | The UE is brought back to operation with USIM B inserted. | - | - | - | - |
| 35-46 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 47a1 to 47b1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 47a1-47a4 | If pc_SwitchOnOff then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 47b1 | Else power off the UE. |  |  |  |  |
| 48 | SS adjusts Ncell levels according to row T3 of table 22.2.15.3.2-1 | - | - | - | - |
| 49 | Power on the UE. | - | - | - | - |
| 50 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 12? | --> | RRCConnectionRequest-NB | 3 | P |
| 51-62 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| - | EXCEPTION: Step 63a1 to 63b1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 63a1-63a4 | If pc_SwitchOnOff then switch off procedure defined in TS 36.523-3 Table 10.3.2.1-1 Steps 2a1-2a4 is performed. | - | - | - | - |
| 63b1 | Else power off the UE. |  |  |  |  |
| 64 | SS adjusts Ncell levels according to row T4 of table 22.2.15.3.2-1 | - | - | - | - |
| 65 | Power on the UE. | - | - | - | - |
| 66 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 12 or Ncell 1? | --> | RRCConnectionRequest-NB | 4 | P |
| 67-78 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 12 or Ncell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

22.2.15.3.3	Specific message contents

None

<End of modified section 1>