3GPP TSG-RAN Meeting #103 						                   RP-240329

Maastricht, Netherlands, March 18-21, 2024

Source:	LG Electronics, Volkswagen AG

Title:	Motivation on Rel-19 further NR sidelink evolution

Agenda item:	9.1.4.6

Document for:	Discussion

1. Introduction

In this paper, we provide our views and the motivation of Rel-19 New WI on sidelink.

2. Motivation

In Rel-18, RAN4 specified the following NR sidelink requirements:

- NR sidelink CA (n47)

  - Intra-band contiguous CA with power class 3

- NR sidelink single carrier in unlicensed spectrum (n46/n96/n102)

  - Power class 5 for 1 Tx with 5 NS values

- NR sidelink inter-band con-current operation

  - NR Uu band with power class 3 + SL-U band with power class 5

For NR sidelink CA, automotive industry shows strong interest towards power class 2 contiguous and non-contiguous intra band CA to account for the fragmented utilization of different channels of 10MHz and 20MHz in n47 and to achieve sufficient communication range.

Recently, 5GAA WG4 sent LS [1] to RAN #103 on SL CA, the main message of which is copied as below:

From an automotive perspective, further work on sidelink carrier aggregation (CA) is needed: - Introduction of intra-band non-contiguous CA in band n47 (ITS spectrum), including the Component Carrier (CC) combinations 10MHz + 10MHz and 10MHz + 20MHz, for UE Power Class 2 (26 dBm) and Power Class 3 (23 dBm). This requirement is to account for a fragmented utilization of different channels within band n47 in some parts of the world. - Introduction of Power Class 2 (26 dBm) for intra-band contiguous sidelink CA in band n47. The Power Class 2 is important to maintain a sufficient communication range when employing CA. 5GAA would highly appreciate if 3GPP TSG RAN could consider these requirements in their upcoming Rel-19 work in particular in RAN4.

The power class 2 was not covered in the Rel-18 NR sidelink intra-band contiguous CA due to the time constraints even though the power class 2 is supported for single carrier operation in n47. Considering the request from the automotive industry, completeness of standard and other activities on the high power UE in 3GPP, intra-band CA along with power class 2 support should be specified in Rel-19.

When it comes to NS values, total of 17 NS values are supported for NR-U while, only 5 out of 17 NS values in NR-U are defined in Rel-18 SL-U. In order to achieve the global footprint, the missing 12 NS values should also be specified. In RAN4#110, it was agreed to specify remaining NS values either in Rel-19 SL WI or in Rel-18 maintenance pending the inclusion of SL WI in Rel-19 package. It is proposed to specify the missing NS values in Rel-19 WI since this would be more stable and efficient way of specifying the missing NS values.

- Proposal 1: It is proposed that the following objectives  should be considered in Rel-19 RAN4-led NR sidelink WI.

- Objectives

- Specify the following requirements for NR sidelink CA in n47.

- Intra-band non-contiguous CA with power class 2 and power class 3

- Intra-band contiguous CA with power class 2

- Intra-band contiguous CA with non-contiguous RB allocation

- Specify the missing requirements for NR sidelink in unlicensed spectrum.

- Remaining 12 NS values

- Specify the corresponding RRM core requirements for the above features.

3. Conclusion

In this contribution, our views and the motivation are provided for Rel-19 RAN4-led NR sidelink WI.

- Proposal 1: It is proposed that the following objectives  should be considered in Rel-19 RAN4-led NR sidelink WI.

- Objectives

- Specify the following requirements for NR sidelink CA in n47.

- Intra-band non-contiguous CA with power class 2 and power class 3

- Intra-band contiguous CA with power class 2

- Intra-band contiguous CA with non-contiguous RB allocation

- Specify the missing requirements for NR sidelink in unlicensed spectrum.

- Remaining 12 NS values

- Specify the corresponding RRM core requirements for the above features.

The example WID on Rel-19 further NR sidelink evolution is attached in the annex for infomation.

References

[1] RP-240023 LS on extending Sidelink Carrier Aggregation support in Rel-19,  5GAA_WG4

[2] R4-2401167 Rel-19 New WID on further NR sidelink evolution, LG Electronics

Annex :

Example WID on further sidelink evolution is attached as below for information