3GPP TSG RAN Meeting #103									           RP-240367

Maastricht, Netherlands, March 18-21, 2024

Status Report to TSG

Agenda item:			9.4.4.5

| WI / SI Name | Enhanced NR support for high speed train scenario in frequency range 2 (FR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_HST_FR2_enh |  |  |  |  |
| Unique ID | 950079 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222272 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2023 | Performance part: 06/2024 (extended for 1Q from original completion data 03/2024) | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 90% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | WANG, He (Jackson) |
|  | Company | Samsung |
|  | Email | h0809.wang@samsung.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

RAN4 #110 (Athens, Greece, 26 Feb - 1 March, 2024)

1. RRM requirement

Big CR (R4-2402556) to TS 38.133 on Enhanced HST FR2 Core Requirements Maintenance is agreed with the following endorsed dCRs contained:

- R4-2403354, Draft CR on FR2 HST inter-frequency measurement requirements in Idle mode

- R4-2403355, draftCR to TS 38.133 on SCell Activation Requirement in HST FR2 Enhanced

Big CR (R4-2403320) to TS 38.133 on Rel-18 HST FR2 RRM performance requirements is endorsed with the following endorsed dCRs contained:

- R4-2403404, Test case for SA event triggered reporting for Rel-18 FR2 HST inter-frequency measurement with SSB time index detection when DRX is not used (Pcell in FR2)

- R4-2403405, Test case requirements for cell reselection to FR2 inter-frequency NR carrier for FR2 HST

- R4-2403406, Draft CR on draft CR on SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA without SSB time index detection when DRX is not used

- R4-2403407, Draft CR to 38.133 on test case for SSB based L1-RSRP for FR2 PC6 UE with multi-Rx

- R4-2403408, draftCR to TS 38.133 on TCI State Switch, UL Timing and RTD Test Case for HST FR2 Enhanced

WF on FR2 HST RRM performance requirements is approved in R4-2403319 (part 2), with details provided as follows:

Way forwards on NR HST FR2 performance part are provided as:

Sub-topic 1-1 How to design the TC for SSB based L1-RSRP in Rel-18 FR2 HST

Issue 1-1-1 AoA setup for multi-Rx chain DL reception in Rel-18 FR2 HST

Agreements in Ad-hoc minutes:

- For FR2 PC6 UE,

  - 2AoA Setup 3, the Relative angular offset between active probes:

    - Only keep 150 degree as option

- Whether to use 2AoA setup for SSB based L1-RSRP test case

Agreement:

- For the test design, configuration set 2 with 2AoA setup should be applied for SSB based L1-RSRP test case in FR2 HST.

- Whether to define a new 2AoA setup for multi-Rx chain DL reception for SSB based L1-RSRP test case

Way Forward:

- Option 1: Yes.

Define a new AoA setup to support the SSB based L1-RSRP measurement test on FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18 - AoA Setup X: 2 AoAs, both AoAs are in Rx beam peak direction - The details on how to define the AoA Setup X for test case, the conclusion from Rel-18 Multi-RX WI could be considered

- Option 2: No.

Issue 1-1-2 Whether to configure higher layer parameter timeRestrictionForChannelMeasurements

Agreements:

- For the SSB based L1-RSRP test case in Rel-18 FR2 HST with highSpeedMeasFlagFR2-r17 is configured to set2, the higher layer parameter timeRestrictionForChannelMeasurements should be configured.

  - Only one sample (M=1) is defined as the measurement period

Sub-topic 1-2 Scope of RRM Performance Requirements for intra-band CA in Rel-18 FR2 HST

Issue 1-2-1 Whether to define test case for enhancement on SCell measurement

Agreement:

- No need to define test case for SCell enhanced measurement in Rel-18 FR2 HST.

Issue 1-2-2 Whether to define test case for enhancement on intra-frequency measurements with/without measurement gaps

Agreement:

- RAN4 to define a new test case for A.7.6.1.X SA event triggered reporting test without gap under non-DRX for UE supporting [measurementEnhancementCAInterFreqFR2-r18]

Issue 1-2-3 Whether to design the test case for SCell activation delay

Agreement:

- No need to define new test case (s) for SCell activation delay for Rel-18 FR2 HST PC6.

Issue 1-2-4 Whether to define test case for enhancement on inter-frequency measurement with MGs considering DRX in connected mode

Agreement:

- No need to define test case (s) for enhancement on inter-frequency measurement with MGs considering DRX in connected mode

Issue 1-2-5 How to design the test case for enhancement on SCell re-selection requirement: Inter-frequency measurement in Idle mode

Agreement:

- For the general test parameters in the test case on cell reselection to FR2 inter-frequency NR for Rel-18 FR2 HST, the highSpeedMeasFlagFR2-r17 is set to Set 2.

Sub-topic 1-3 How to design the TC for UL timing adjustment in Rel-18 FR2 HST

Issue 1-3-1 Whether to introduce simultaneous multi-panel operation at UL timing adjustment in Rel-18 FR2 HST

Agreement from Tuesday RRM session:

Issue 1-3-1: The necessity of introducing simultaneous multi-panel operation at UL timing adjustment in Rel-18 FR2 HST

Agreement: No

Issue 1-3-2 Discussion on the new R18 UL timing adjustment TC design for FR2 HST

It is encouraged companies to discuss how to design the MAC-CE indication based TCI state switch delay TC in Rel-18 FR2 HST at CR stage directly.

Sub-topic 1-4 RRM TC configuration

Issue 1-4-1 Channel model and propagation condition

Agreement in Ad-hoc minutes:

- RAN4 to adopt bi-directional assumption (i.e., AWGN (serving cell) and AWGN with 19444 Hz frequency offset (neighbour cell)) in each test case discussion.

Issue 1-4-2 Test setup of AoA configuration for all new test cases

Way Forward:

- Address directly at the CR stage.

Sub-topic 1-5 Test case list agreed for information

Table 2 The RRM performance test cases list (for information)

| TC index | Corresponding Core Requirement | Necessity of New Test Case | Company for CR volunteer |
| --- | --- | --- | --- |
| TC1 | Requirements for Cell Re-selection | New test case needed. (New test case for A.7.1.1.X Cell reselection to FR2 inter-frequency NR case for Rel-18 FR2 HST intra-band CA) | Intel |
| TC2 | Requirements for SCell Activation Delay for Deactivated SCell | No new test case needed. |  |
| TC3-TC6 | Requirements for Inter-frequency measurement with measurement gaps | No new test case needed. |  |
|  |  | No new test case needed. |  |
|  |  | New test case needed. (New test case for A.7.6.2.X3 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA with SSB time index detection when DRX is not used (Pcell in FR2)) | Huawei |
|  |  | New test case needed. (New test case for A.7.6.2.X4 SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA without SSB time index detection when DRX is not used (Pcell in FR2)) | Ericsson |
| TC7 | Requirements for Inter-frequency measurement without measurement gaps | No new test case needed. |  |
| TC8 | Requirements for One shot large UL timing adjustment for FR2 Power Class 6 UE | New test case(s) needed. | Nokia |
|  | Requirements for MAC-CE based TCI state switch delay in HST FR2 scenarios |  |  |
|  | New requirements for MRTD with simultaneous two-panel reception for FR2 Power Class 6 UE |  |  |
| TC9 | Requirements for SSB based radio link monitoring | No new test case needed. |  |
| TC10 | Requirements for SSB based beam failure detection | No new test case needed. |  |
| TC11 | Requirements for SSB based L1-RSRP | New test case needed. (New test case for A.7.6.3.X SSB based L1-RSRP measurement when DRX is used for FR2-1 PC6 UEs supporting SimultaneousReceptionFR2HST-r18) | Samsung |
| TC12 | Requirements for Intra-frequency measurements with/without measurement gaps | New test case needed. (New test case for A.7.6.1.X SA event triggered reporting test without gap under non-DRX for UE supporting [measurementEnhancementCAInterFreqFR2-r18]) | Nokia |

UE capability in the feature list:

Issue 1-1-1: UE features for simultaneous two-panel reception

Agreement:

Remove the description for demod for now, and further discuss whether the following component can be added taking into account the discussion in demod.

- 3) Support of enhanced demodulation processing to support FR2-1 PC6 UEs with simultaneous DL signals reception associated with two different QCL TypeD RSs

Further discuss whether 16-2c is the prerequisite UE feature if the component for demod will be added.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34.NR_HST_FR2_enh | 34-1 | Support of NR FR2 HST with simultaneous DL signals reception associated with two different QCL TypeD RSs | 1) Support of enhanced RF requirement to support FR2-1 PC6 UEs with simultaneous DL signals reception associated with two different QCL TypeD RSs. 2) Support of enhanced RRM requirement to support FR2-1 PC6 UEs with simultaneous DL signals reception associated with two different QCL TypeD RSs | 22-1 | Yes | N/A | UE does not support FR2 high speed train scenario with simultaneous DL signals reception associated with two different QCL TypeD RSs | Per Band | No | FR2 only | N/A | # A single indication element is used to indicate for the components for 34-1. The UE capability reported in this feature is applied when highSpeedDeploymentTypeFR2-r17 is configured by network as bidirectional | Optional with capability signaling |

Issue 1-1-2: UE features for CA

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34.NR_HST_FR2_enh | 34-2 | Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode | 1) Support of the RRM requirement for intra-band CA operation in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 2) Support of the RRM requirement for enhanced inter-frequency measurements in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 | 22-1 | Yes | N/A | The performance of intra-frequency measurement on SCC and/or inter-frequency measurements in connected mode for NR FR2 HST scenario cannot be guaranteed | Per Band | No | FR2 only | N/A |  | Optional with capability signaling |
| 34.NR_HST_FR2_enh | 34-3 | Enhanced FR2 HST RRM requirements for inter-frequency measurement in Idle and Inactive mode | Support of the RRM requirement for inter-frequency measurements in idle and Inactive mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 | 22-1 | No | N/A | The performance of inter-frequency measurement in idle and Inactive mode for FR2 HST scenario cannot be guaranteed | N/A | No | FR2 only | N/A |  | Optional without capability signaling |

Issue 1-1-3: UE features for enhanced MAC CE

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34.NR_HST_FR2_enh | 34-4 | Support of enhanced MAC CE for TCI state switch indication for FR2 HST | 1. Support of enhanced one shot large UL transmit timing adjustment requirement to support FR2-1 PC6 UEs, as specified in TS 38.133 based on [the cross-RRH TCI state indication for UE-specific PDCCH MAC CE] 2. Support of enhanced TCI state switching delay requirements based on new MAC CE for TCI state switch indication named as [the cross-RRH TCI state indication for UE-specific PDCCH MAC CE] in HST FR2 scenario, as specified in TS 38.133. | 22-1 | Yes | N/A | UE does not support enhanced MAC CE for TCI state switch indication for FR2 HST | Per Band | No | FR2 only | N/A | The naming of MAC CE for [the cross-RRH TCI state indication for UE-specific PDCCH] depends on RAN2 | Optional with capability signaling |

2. Demodulation requirement

WF (R4-2402861) on FR2 HST demodulation requirements is approved, with the following agreements and way forwards contained

Way forwards on FR2 HST demodulation requirements are provided as:

Sub-topic 1-1: Test setup for PDSCH requirement with multi-Rx reception

Issue 1-1-1: MCS pair for PDSCH requirement for multi-Rx chain reception

Agreement:

- Define one set of requirements with {MCS19, MCS11}

- Companies are encouraged to check and update results in the next meeting, and define final SNR requirements according to the usual procedure with target span for simulation alignment of 2.5dB.

Issue 1-1-2-1:   FFS on considering Tfirst SSB after TRS and TSBB proc in the PDSCH allocation timeline for FR2 HST with multiple Rx panels

Agreement:

- The existing formula in Rel-17 for the TCI switching time can be reused, i.e., THARQ+TMAC Proc+TfirstSSB + TSSB proc +TfirstTRSafterSSB+ TTRS proc

Issue 1-1-2-2:   TCI state switching scheduling for left-facing panel (backward-looking Rx panel or left panel)

Agreement:

- Configure TCI state switching command scheduled by MAC CE in the last available slot according to the channel delay profile and RRH coverage, following the approach used in FR1 HST DPS

- TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy mod(i, n) = Nl (i≠0) for left-facing panels, n = 57600 and Nl= 16457

Issue 1-1-2-3:   TCI state switching scheduling for right-facing panel (forward-looking Rx panel or right panel)

Agreement:

- Configure TCI state switching command scheduled by MAC CE in the last available slot according to the channel delay profile and RRH coverage, following the approach used in FR1 HST DPS

- TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy mod(i, n) = 0 (i≠0) for right-facing panels, n = 57600

Issue 1-1-2-4:   common value of THARQ + TMAC Proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc

Agreement:

- For left-facing panel (backward-looking Rx panel or left panel)

  - THARQ = 4

  - TMAC Proc =24

  - TSSB proc =16

  - TTRS proc =16

  - TfirstSSB = 155 (= SSB period (160) – THARQ – TMAC Proc – 16457 )

  - TfirstTRSafterSSB = 69 (= TRS period (80) – TRS offset from SSB (5) – TTRS proc)

- For right-facing panel (forward-looking Rx panel or right panel)

  - THARQ = 4

  - TMAC Proc =24

  - TSSB proc =16

  - TTRS proc =16

  - TfirstSSB = 132 (= SSB period (160) – THARQ – TMAC Proc)

  - TfirstTRSafterSSB = 69 (= TRS period (80) – TRS offset from SSB (5) – TTRS proc)

Issue 4-1-1:  UE feature lists for FR2 PDSCH requirements with CA

Agreement:

- Following the UE feature list in main session discussion on Rel-18 FR2 HST, the components of support of the enhanced demodulation requirements specified for CA for HST FR2 UE is not captured into the UE feature list of 34-2

Issue 4-1-2:  UE feature lists for FR2 PDSCH requirements with multi-Rx chain reception

Agreement:

- Following the UE feature list in main session discussion on Rel-18 FR2 HST, the components of support of enhanced demodulation processing to support FR2-1 PC6 Ues with simultaneous DL reception with two different QCL TypeD RSs] is removed in the feature list 34-1

2.4.2	Remaining Open issues

Core Part:

- No remaining work expected.

Performance Part:

- Complete RRM test cases for performance requirements:

  - Continue work on the test cases with endorsed CRs.

  - Specify new test cases agreed to be introduced in RAN4#110 for enhancement on intra-frequency measurements with/without measurement gaps.

  - Specify the definition AoA configurations for test setup for all new test cases.

- Complete demodulation performance requirements for simultaneous multi-panel reception, based on the endorsed CR.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#110 contributions:

- R4-2401423	Feature list proposals for HST FR2 enhancements                                                         Intel Corporation

- R4-2402556	bigCR to TS 38.133 on Enhanced HST FR2 Core Requirements Maintenance              Nokia, Nokia Shanghai Bell

- R4-2401422	Maintenance CR on IDLE mode HST FR2 UE mobility                                                    Intel Corporation

- R4-2403354	Draft CR on FR2 HST inter-frequency measurement requirements in Idle mode	Samsung

- R4-2401897	Discussion on feature list for Rel-18 FR2 HST						Samsung

- R4-2402554	On HST FR2 Enhanced RRM Requirements Maintenance 			Nokia, Nokia Shanghai Bell

- R4-2403355	draftCR to TS 38.133 on SCell Activation Requirement in HST FR2 Enhanced       Nokia, Nokia Shanghai Bell

- R4-2403320	Big CR to TS 38.133 on Rel-18 HST FR2 RRM performance requirements                Samsung, Nokia

- R4-2401349	Discussion on FR2 HST RRM performance                                                                    Huawei, HiSilicon

- R4-2403404	Test case on SA event triggered reporting for Rel-18 FR2 HST inter-frequency measurement with SSB time index detection when DRX is not used (Pcell in FR2)                                                                                  Huawei, HiSilicon

- R4-2401424	Test cases list for HST FR2 UE multi-panel simultaneous reception and CA                     Intel Corporation

- R4-2403405	Test case requirements for cell reselection to FR2 inter-frequency NR carrier for FR2 HST    Intel Corporation

- R4-2401600	RRM performance requirements for train roof-mounted FR2 high power devices                     Ericsson

- R4-2403406	draft CR  to 38.133 on SA event triggered reporting tests for Rel-18 FR2 HST intra-band CA without SSB time index detection when DRX is not used                                                                                                         Ericsson

- R4-2401898	Discussion on test case for SSB-based L1-RSRP for PC6 UE with multi-Rx                            Samsung

- R4-2403407	Draft CR to 38.133 on test case for SSB based L1-RSRP for FR2 PC6 UE with multi-Rx        Samsung

- R4-2402557	On HST FR2 Enhanced RRM Performance Requirements                                              Nokia, Nokia Shanghai Bell

- R4-2403408	draftCR to TS 38.133 on TCI State Switch, UL Timing and RTD Test Case for HST FR2 Enhanced  Nokia, Nokia Shanghai Bell

- R4-2402808	RRM performance test cases for FR2 HST enhancements                                 Qualcomm Incorporated

- R4-2401571      Draft CR for channel model on Rel-18 FR2 HST demodulation requirement                             Samsung

- R4-2401572      Draft big CR for TS 38.101-4 on Rel-18 FR2 HST demodulation requirement                           Samsung

- R4-2401573      Simulation results summary for Rel-18 FR2 HST demodulation requirement                            Samsung

- R4-2402716	HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Carrier Aggregation       Nokia, Nokia Shanghai Bel

- R4-2401570      Discussion and simulation results for PDSCH requirements with multi-Rx reception                Samsung

- R4-2401699      Discussion on UE multi-Rx demodulation requirements for HST FR2                         Huawei, HiSilicon

- R4-2402984    	Draft CR on PDSCH requirement with multi-Rx reception for FR2 HST (TS38.101-4, Rel-18)  Huawei, HiSilicon

- R4-2403087     draft CR to 38.101-4: FRC of PDSCH demodulation requirements for FR2 HST    		Ericsson

- R4-2401757     UE demodulation requirements for FR2 HST multi-Rx reception      				Ericsson

- R4-2402717     HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Multi-Rx Chain DL Reception   Nokia, Nokia Shanghai Bel

- R4-2402718      Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception    Nokia, Nokia Shanghai Bel

- R4-2402749	Discussion on PDSCH requirements with multi-Rx Chain DL reception      			Qualcomm

- R4-2402750      DraftCR for FR2 HST Enhancements – Applicability Rules       					Qualcomm

- R4-2402754      Simulation Results for FR2 HST with multi-Rx Chain DL reception      				Qualcomm

- R4-2403090      Draft CR on PDSCH requirement with multi-Rx reception for FR2 HST (TS38.101-4, Rel-18), Huawei, HiSilicon

- R4-2402861	WF on FR2 HST demodulation requirements							Samsung

- R4-2402656	Topic summary for [108][324] NR_HST_FR2_enh_Demod				Moderator (Samsung)

- R4-2402981	Ad-hoc minutes for FR2 HST demodulation							Samsung

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template