3GPP TSG RAN meeting #103										RP-240406

Maastricht, Netherlands, March 18-21, 2024

Status Report to TSG

Agenda item:			9.4.4.11

| WI / SI Name | Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_OTA_enh |  |  |  |  |
| Unique ID | 970081 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-233742 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Dec/2023 | Performance part: Jun/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 70% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Zhu, Siting |
|  | Company | CAICT |
|  | Email | zhusiting@caict.ac.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

RAN4 #110 (Athens, Greece, 26 Feb – 01 Mar, 2024)

General

- Topic summary for NR_MIMO_OTA_enh is in R4-2402672 [1]

- Ad-hoc meeting minutes for NR_MIMO_OTA_enh is in R4-2403011 [2]

- WF on NR_MIMO_OTA_enh was approved in R4-2402876 [3]

- Template for 3GPP Rel-18 FR1 MIMO OTA Measurement Campaign was approved in R4-2403019 [4]

- Template for 3GPP FR2 MIMO OTA Measurement Campaign was approved in R4-2403014 [5]

- CR to TS 38.151 on update to FR1 Channel model validation was agreed in R4-2400534 [6]

- CR to TS 38.151 on update to FR1 Calibration Procedure was agreed in R4-2403016 [7]

- CR to TR 38.761 on update to the FR1 channel model validation results was agreed in R4-2400198 [12]

- CR to TR 38.761 on updating the lab 5 channel model validation for n28 was agreed in R4-2402949 [20]

- CR to TR 38.761 on the remaining lab 5 channel model validation for n41 and n78 was agreed in R4-2402950 [21]

- CR to TS 38.761 on FR1 MIMO OTA channel model validation results was agreed in R4-2403015 [22]

Topic #1: FR1 MIMO OTA

Issue 1-1-1: UE positioning for FR1 MIMO OTA

Agreement:

- To avoid misalignment and ambiguity, align the three coordinate systems, i.e. DUT coordinate system, Channel model coordinate system and chamber coordinate system, for FR1 MIMO OTA test methodology.

Issue 1-1-2: Power validation systematic offset

Agreement:

- Remove the statement in TS 38.151 C3.6 that allows the power validation result to be considered a systematic offset.

Issue 1-2: Completing channel model validation for band n1 4x4 MIMO

Agreements:

- Encourage Rel-17 aligned labs to complete CDL-C UMa channel model validation for 4x4 MIMO at band, and contribute to Measurement Campaign for band n1.  The deadline for channel model validation and data submission for measurement campaign is RAN4 #111 (May 2024).  Labs are allowed to submit channel model validation results for band n1 together with the measurement data for performance requirements development at RAN4 #111 (May 2024).

- Each volunteer lab shall use the same measurement setup and equipment for channel model validation, lab alignment, and measurement campaign.

Issue 1-3-1: Updated pass/fail limit for Rel-18 FR1 MIMO OTA lab alignment

Agreement:

- The pass/fail limit (defined as 0.75*MU) for Rel-18 FR1 MIMO OTA lab alignment is updated based on the latest MU agreed in RAN5. For bands≤3GHz, the value is updated to 0.75*2.61=1.96 dB.

Issue 1-3-2: Effect of power validation systematic offset on lab alignment

Agreement:

- With the agreement in Issue 1-1-2, the lab alignment outcome should be based on the measurement results without power validation systematic offsets.

Issue 1-3-3: Outcome of the 1st round Rel-18 FR1 MIMO OTA lab alignment

Agreements:

- Based on the analysis in R4-2400033, the 1st round Rel-18 FR1 MIMO OTA lab alignment is concluded with the following outcome:

  - 5 labs are aligned in the 1st round.

- The remaining lab can resubmit the PAD measurement result at the next meeting and be confirmed as an aligned lab, without affecting the conclusion that the 5 labs are aligned.

Issue 1-4-1: Thresholds of data pool for specifying FR1 MIMO OTA requirements

Agreements:

- Thresholds for specifying FR1 MIMO OTA performance requirement:

  - Minimum number of data: 15

  - Minimum number of UE models: 12

- As long as the thresholds are met, all the data submitted by aligned labs will be taken into the data pool for specifying FR1 MIMO OTA performance requirements.

- Volunteer labs should submit the information of the UEs that are planned to be measured using the template in R4-2321191 to RAN4 secretary as soon as possible, in order to monitor if the above thresholds can be met.

Issue 1-4-3: Effect of power validation systematic offset on the data pool for performance requirements

Agreement:

- With the agreement in Issue 1-1-2, the performance requirements should be based on the measurement data with no systematic offset applied.

Issue 1-4-4: Whether IEs can be used to identify 2Rx UE and 4Rx UE

Agreement:

- Not to use IE srs-TxSwitch or srs-TxSwitch-v1610 to differentiate 2Rx capable UE from 4Rx capable UE in a target testing RF band for FR1 MIMO OTA testing.

Topic #2: FR2 MIMO OTA

Issue 2-1: FoM of FR2 MIMO OTA for PC1 UE

Agreement:

- For FR2 PC1 MIMO OTA, the performance metric is to be specified with 5 of 36 test points, i.e.,

MASC70=10log5110P70,110+110P70,210+⋅⋅⋅+110P70,510

where {P70,1, …, P70,5} are the best 5 sensitivity values from all the 36 constant density measurement points.

Issue 2-2: FR2 MIMO OTA lab alignment activity

Agreements:

- Complete the 1st round lab alignment at the next meeting with the results from at least three labs. Allow remaining labs to submit measurement results afterwards.

- Return the PADs to ETS-L by RAN4 #110bis (Apr. 2024).

- All labs can submit data for Measurement Campaign, whether the data can be included in the data pool for specifying requirements depends on lab alignment outcome.

- The deadline to submit lab alignment results and the data for Measurement Campaign is RAN4 #111 (May 2024).

Issue 2-3-1: Devices provision for FR2 MIMO OTA Measurement Campaign

Agreement:

- Further encourage companies to volunteer FR2 devices to test lab before measurement campaign is finished. It is suggested for companies to provide devices at least two weeks before April meeting starts.

2.4.2	Remaining Open issues

Core part WI:

- None

Perf. part WI:

- FoM and criteria for FR2 MIMO OTA

- Refine the additional criteria of FR2 MIMO OTA for PC3 UE, if necessary

- Conclude FR2 MIMO OTA lab alignment and the Measurement campaign, specify FR2 MIMO OTA performance requirements

- Conclude the 2nd round FR1 MIMO OTA lab alignment and the Measurement campaign, specify FR1 MIMO OTA performance requirements

- Specify recommended test tolerance (TT) for FR1 and FR2 MIMO OTA performance requirements

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

None

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R4-2402672	Topic summary for [110][332] NR_MIMO_OTA_enh	Moderator (CAICT)

- R4-2403011	Ad-hoc meeting minutes for [110][332] NR_MIMO_OTA_enh	CAICT

- R4-2402876	Way forward on [110][332] NR_MIMO_OTA_enh	CAICT

- R4-2403019	Template for 3GPP Rel-18 FR1 MIMO OTA Measurement Campaign	CAICT

- R4-2403014	Template for 3GPP FR2 MIMO OTA Measurement Campaign	CAICT

- R4-2400534	Update to FR1 Channel model validation	MVG Industries, Spirent, Apple, ETS-Lindgren, Telecom Italia, Sony

- R4-2403016	Update to FR1 Calibration Procedure	MVG Industries, Keysight, Spirent

- R4-2400032	CDL-C UMa channel model validation results for band n1	CAICT

- R4-2400033	Analysis of 3GPP Rel-18 FR1 MIMO OTA lab alignment results	CAICT

- R4-2400194	On FR2 MIMO OTA Lab Alignment PADs measurement results	Apple

- R4-2400195	On FR1 MIMO OTA channel model validation	Apple

- R4-2400198	Update to the FR1 channel model validation results	Mediatek India Technology Pvt.

- R4-2400267	on 2Rx vs 4Rx performance in 2x2 MIMO	Huawei Tech.(UK) Co.. Ltd

- R4-2400535	Discussion Paper on 38.151 C3.6	MVG Industries, Apple, Spirent, ETS-Lindgren, Telecom Italia, Sony

- R4-2401184	Channel model validation results for Band n1,n5 and n8	xiaomi

- R4-2401804	UE positioning for FR1 MIMO OTA	OPPO

- R4-2401805	Consideration on FR1 MIMO OTA measurement campaign	OPPO

- R4-2402255	Discussion of FR2 MIMO OTA on PC3 campaign and PC1 metric	Samsung

- R4-2402310	Discussion on FR1 MIMO OTA testing method	Google Inc.

- R4-2402949	Update the lab 5 channel model validation for n28	Xiaomi

- R4-2402950	On the remaining lab 5 channel model validation for n41 and n78	Xiaomi

- R4-2403015	CR to TS 38.761 on FR1 MIMO OTA channel model validation results	CAICT, CMCC, BUPT

- R4-2403017	On FR1 MIMO OTA Lab Alignment PADs measurement results	Apple

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template