3GPP TSG RAN meeting #103										RP-240445

Maastricht, Netherlands, March 18-21, 2024

## Status Report to TSG

Agenda item:			9.7.4.7

| WI / SI Name | Further enhancements on NR and MR-DC measurement gaps and measurements without gaps |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MG_enh2 |  |  |  |  |
| Unique ID | 950082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-232440 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall: 100% RAN4: 100% | Performance Part: Overall: 35% RAN4: 35% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Tsang-Wei (Ato) YU, Meng ZHANG |
|  | Company | MediaTek Inc., Intel Corporation |
|  | Email | ato.yu@mediatek.com, meng.zhang@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#110 (Feb, 2024, Athens, Greece)

- 1st version of feature list were agreed in [105] attached in an LS [106] to RAN2

- RRM core maintenance and performance part for pre-configured MGs, multiple concurrent MGs and NCSG

  - A set of maintenance CRs were endorsed [107][108][109][110] and merged into the Big CRs [103]

  - The following agreements for performance part are captured in the agreed WF [101]

  - Case 1 (Pre-configured MG and concurrent MG)

    - The following scenarios be considered in the test cases:

      - Pre-MG + Type-2 MG

      - Pre-MG + Pre-MG

        - Under the following conditions:

          - Simultaneous multiple activation, FFS non-simultaneous multiple activation

          - Without dynamic collision

      - Dynamic collision with Pre-MG + Type-2 MG

      - FFS other scenarios, options for further discussion:

        - Pre-MG + Type-1 MG

    - Tentative test case list as a starting point for CR drafting

| No | Test case category | Test purpose | Volunteering company |
| --- | --- | --- | --- |
| Con-Pre-MG TC1 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap and autonomous activation/deactivation of Pre-MG + Type-2 | - When pre-MG being deactivated at the beginning of testing, UE can report the results of Cell2 within the required period - Pre-MG activation/deactivation delay - After pre-MG being activated by UE autonomously, UE can report the results of Cell2 and Cell3 within the required period - Verify the gap collision rule | MediaTek |
| Con-Pre-MG TC2 | Event triggered reporting test on intra-frequency in FR2 with concurrent gap and network-controlled activation/deactivation of Pre-MG + Type-2 | - When pre-MG being deactivated at the beginning of testing, UE can report the results of Cell2 within the required period - Pre-MG activation/deactivation delay - After pre-MG being activated by network-control, UE can report the results of Cell2 and Cell3 within the required period - Verify the gap collision rule | Huawei |
| Con-Pre-MG TC3 | Event triggered reporting test on intra-frequency in FR2 with concurrent gap with Pre-MG and autonomous activation/deactivation of two Pre-MG for FR2 | - Verify that the UE correctly activates and deactivates the pre-MG and makes correct measurement and reporting of an event with activated and deactivated pre-MG - Multiple Pre-MG activation/deactivation delay - Verify that the UE makes correct reporting of an event: - After pre-MG being activated by UE autonomously, UE can report the results of Cell2 and Cell3 within the required period. | Xiaomi |
| Con-Pre-MG TC4 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap with Pre-MG and network-controlled activation/deactivation of two Pre-MG for FR1 | - Verify that the UE correctly activates and deactivates the pre-MG and makes correct measurement and reporting of an event with activated and deactivated pre-MG - Multiple Pre-MG activation/deactivation delay - Verify that the UE makes correct reporting of an event: - After pre-MG being activated by UE network-controlled, UE can report the results of Cell2 and Cell3 within the required period. | CMCC |

  - Case 2 (NCSG and concurrent MG)

    - Tentative test case list as a starting point for CR drafting

| No | Test case category | Test purpose | Volunteering companies |
| --- | --- | --- | --- |
| Con-NCSG TC1 | Event triggered reporting test on inter-frequency in FR1 with concurrent gap and NCSG | - Inter-frequency cell search/measurement delay is met for Cell2 in NCSG and Inter-frequency cell search/measurement delay for Cell3 in NCSG, and - UE receives data in Cell1 meeting scheduling restriction requirements, and - Verify the gap collision rule | Ericsson |
| Con-NCSG TC2 | Event triggered reporting test on intra-frequency in FR2 with concurrent gap and NCSG | · Same as Con-NCSG TC 1, yet for intra-f in FR2. | CATT |

  - Test case principles common to both Case 1 and Case 2

    - Only define test case in NR SA in both FR1 and FR2:

    - Do not introduce the test for L1 impact:

    - Define a minimum set of test cases for SSB-based measurement:

    - Only define test case under non-DRX:

    - Define test case without SBI reporting:

    - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations:

    - Do not define test cases with simultaneously FR1 and FR2 gaps configured:

    - Test cases are limited to single serving carrier:

    - Only use mandatory gap patterns to define test cases:

    - Verify gap dropping behaviour without introducing additional test cases:

    - For case 1 and case 2, Define test for partial overlap, and fully non-overlap is used if no core requirements for partial overlap in some cases.

- RRM core requirements and performance part for measurements without gaps

  - A set of maintenance CRs were endorsed [111][112][113][114] and merged into the Big CRs [103][104]

  - The following agreements are captured in the ad-hoc meeting minutes [100] and the agreed WF [102]

  - Requirements of NeedforGap

    - No need to establish the mapping between UE’s indication for NeedForGaps and NCSG

    - RAN4 not to define the need for interruption feature for NFG

  - Inter-RAT gapless measurements

    - For Case a-1,

      - Tentative test case list as a starting point for CR drafting

| No. # | Item of core requirements | Type of test cases | Frequency range of serving cell | SMTC configuration | MG configuration | DRX configuration | Subclause | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NFG1 | Event triggered reporting and interruptions | Intra-frequency measurements without gap config but with DRX | FR1 | 20ms | No gap config | [DRX pattern TBD] | A.6.6.1.X | Nokia |
| NFG2 |  | Intra-frequency measurements without gap or DRX configuration | FR2 | 160ms | No gap config | No DRX | A.7.6.1.X | Intel |
| NFG3 |  | Inter-frequency measurements with gap config but without DRX | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | QC |
| NFG4 |  |  | All in FR2 | 20ms | [Pattern TBD] | [DRX pattern TBD] | A.7.6.1.X | HW |
| NFG5 | Event triggered reporting [without interruption] | Inter-frequency measurements without gap or DRX configuration | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | Ericsson |
| NFG6 |  |  | All in FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X | CATT |
| NFG7 |  | Intra-frequency measurements without gap or DRX configuration | FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | CMCC |
| NFG8 |  |  | FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X | MTK |

    - For Case b-2,

      - Introduce the feature for Inter-RAT EUTRAN measurement without gap [and within active DL BWP]

      - Introduce the feature for Case b-2: “Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology”

      - Tentative test case list as a starting point for CR drafting

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | EMW configuration | MG configuration | DRX configuration | Subclause | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IR1 | Event triggered reporting and interruptions | Inter-RAT EUTRAN measurements case b-1 with MG and DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | [DRX pattern TBD] | A.6.6.3.X | CATT |
| IR2 |  | Inter-RAT EUTRAN measurements case b-1 with MG but no DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | No DRX config | A.6.6.3.X | xiaomi |
| IR3 | Event triggered reporting [without interruption] | Inter-RAT EUTRAN measurements case b-2 without MG or DRX configuration | NR FR1 | [Pattern TBD] | No gap configuration | No DRX configuration | A.6.6.3.X | CMCC |
| IR4 | Event triggered reporting | Inter-RAT NR measurements case a-1with MG | NR FR2 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | HW |
| IR5 | Event triggered reporting and interruptions | Inter-RAT NR measurements case a-1 with MG | NR FR1 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | Nokia |

    - Modification of test case list and configuration is possible based on RAN4 consensus.

    - For issues common to Case b-1 and b-2

      - Effective measurement window (EMW)

        - #0 and #1 are mandatory, if UE supports EMW feature.

          - FFS other conditions, e.g., UE supports Case b-1 or b-2

        - Other patterns are optional

#### 2.4.2	Remaining Open issues

- The remaining open issues in UE feature list are to be further concluded in the maintenance part of the work item

- RRM core maintenance for measurements without gaps

  - Requirements of NeedforGap

    - The UE behavior of interruption-based measurements when DRX is configured are to be discussed in the maintenance part of the work item.

- RRM performance requirements

  - Final test cases details and CRs

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

- R4-2401427	Feature list proposals for measurement gap enhancements	Intel Corporation

- R4-2401202	Rel-18 RAN4 UE feature list for enhancements on NR and MR-DC measurement gaps and measurements without gaps	xiaomi

- R4-2401867	Discussion on UE feature list for NR_MG_enh2 WI	Nokia, Nokia Shanghai Bell

- R4-2402879	Discussion on feature list for MGE-2 part 1	MediaTek inc.

- R4-2402880	Big CR to TS 38.133 on Further enhancements on NR and MR-DC measurement gaps and measurements without gaps	MediaTek inc.

- R4-2402881	Big CR to TS 36.133 on inter-RAT NR measurement without gap	MediaTek inc.

- R4-2402882	Draft CR to Rel-18 TS 38.133: on MGE maintenance in TS 38.133	MediaTek inc.

- R4-2403358	Draft CR to Rel-18 TS 38.133: on MGE maintenance in TS 38.133	MediaTek inc.

- R4-2400374	Discussion on case 1 requirements of R18 gap enhancement	Apple

- R4-2400375	Draft CR for case 1 requirements	Apple

- R4-2400872	On maintenance issues  for Pre-MG and concurrent MGs requirements	vivo

- R4-2400967	Remaining issues on PreMG and ConMGs	Ericsson

- R4-2400998	Discussion on case 1 requirements for Pre-MG and conMG	OPPO

- R4-2401025	(NR_MG_enh2-Core) Discussion on Pre-MG MG and concurrent MG (case 1)	CMCC

- R4-2401028	(NR_MG_enh2-Core) draft CR on concurrent gap with Pre-MG and concurrent gap with NCSG	CMCC

- R4-2401195	Discussion on RRM requirements for combinations of pre-configured MGs and multiple concurrent MGs	xiaomi

- R4-2401200	Draft CR on activation deactivation delay for Pre-MG within the concurrent MGs	xiaomi

- R4-2401237	Discussion on UE capabilities for Case 1 Pre-MG and concurrent MG	China Telecom

- R4-2401637	Discussion on Case 1 RRM requirements	ZTE Corporation

- R4-2401863	Discussion on Case 1 requirements	Nokia, Nokia Shanghai Bell

- R4-2401864	CR 38.133 Corrections to Case 1 core requirements for NR_MG_enh2	Nokia, Nokia Shanghai Bell

- R4-2402163	Discussion on remaining issues for Case 1	Huawei, HiSilicon

- R4-2402164	draftCR on RRM requirements for con-MG + pre-MG	Huawei, HiSilicon

- R4-2403359	(NR_MG_enh2-Core) draft CR on concurrent gap with Pre-MG and concurrent gap with NCSG	CMCC

- R4-2403360	CR 38.133 Corrections to Case 1 core requirements for NR_MG_enh2	Nokia, Nokia Shanghai Bell

- R4-2403541	CR 38.133 Corrections to Case 1 core requirements for NR_MG_enh2	Nokia, Nokia Shanghai Bell

- R4-2400108	Discussion on maintenance issue for case 2 requirements	CATT

- R4-2400376	Discussion on case 2 requirements of R18 gap enhancement	Apple

- R4-2400377	Draft CR for case 2 requirements	Apple

- R4-2400873	On maintenance issues for NCSG and concurrent MGs requirements	vivo

- R4-2400968	Remaining issues on NCSG and ConMGs	Ericsson

- R4-2400999	CR on NCSG collision	OPPO

- R4-2401000	Discussion on case 2 requirements NCSG and conMG	OPPO

- R4-2401024	(NR_MG_enh2-Core) Discussion on NCSG and concurrent MG (case 2)	CMCC

- R4-2401197	Discussion on RRM requirements for combinations of NCSG and multiple concurrent MGs	xiaomi

- R4-2401238	Discussion on UE capabilities for Case 2 NCSG and concurrent MG	China Telecom

- R4-2401638	Discussion on Case 2 RRM requirements	ZTE Corporation

- R4-2401865	Discussion on Case 2 requirements	Nokia, Nokia Shanghai Bell

- R4-2401866	CR 38.133 Corrections to Case 2 core requirements for NR_MG_enh2	Nokia, Nokia Shanghai Bell

- R4-2402165	Discussion on remaining issues for Case 2	Huawei, HiSilicon

- R4-2402883	Discussion on case 2 requirements (NCSG and concurrent MG)	MediaTek inc.

- R4-2403361	Draft CR for case 2 requirements	Apple

- R4-2403362	CR on NCSG collision	OPPO

- R4-2402884	Discussion on feature list for MGE-2 part 2	MediaTek inc.

- R4-2400277	[NR_MG_enh2-Core] CR 38.133 Correction of normative text in requirements	BeammWave

- R4-2400378	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	Apple

- R4-2400379	Draft CR for NeedForGaps requirements	Apple

- R4-2400542	Remaining issues on R18 NFG	Qualcomm Incorporated

- R4-2400874	On maintenance issues for NFG requirements	vivo

- R4-2400969	Remaining issues on NeedForGaps measurement	Ericsson

- R4-2401001	CR on interruption requirements for NFG measurements	OPPO

- R4-2401002	On interruption requirements for NFG measurements	OPPO

- R4-2401026	(NR_MG_enh2-Core) Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	CMCC

- R4-2401027	(NR_MG_enh2-Core) DraftCR on intra-frequency measurement delay for NFG	CMCC

- R4-2402013	Discussion on measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2402014	Draft CR interruption requirements for measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2402166	Discussion on remaining issues for NeedForGaps	Huawei, HiSilicon

- R4-2402167	draftCR on measurement requirements for NeedForGaps	Huawei, HiSilicon

- R4-2402885	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	MediaTek inc.

- R4-2403263	(NR_MG_enh2-Core) DraftCR on intra-frequency measurement delay for NFG	CMCC

- R4-2400109	Discussion on maintenance issues for inter-RAT measurement without gaps	CATT

- R4-2400380	Discussion on R18 inter-RAT measurement without gap	Apple

- R4-2400381	CR for measurement delay for nogap-noncsg	Apple

- R4-2400492	(NR_MG_enh2-Core) CR on UE Scheduling availability for inter-RAT NR measurement without gap	Apple

- R4-2400875	On maintenance issues  for inter-RAT measurement without gap requirements	vivo

- R4-2400970	Remaining issues on Inter-RAT measurement without gap	Ericsson

- R4-2401023	(NR_MG_enh2-Core) Discussion on UE capabilities for measurements without gap	CMCC

- R4-2401187	draftCR on introduction of interruprion requirements for inter-RAT NR measurement without gap (case a-1)	xiaomi

- R4-2401426	Maintenance CR on interruption requirements for measurements without gap	Intel Corporation

- R4-2402015	Draft CR interruption requirements for interRAT measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2402168	Discussion on remaining issues for inter-RAT MG-less measurement	Huawei, HiSilicon

- R4-2402169	draftCR on requirements for inter-RAT LTE measurement without gap	Huawei, HiSilicon

- R4-2402436	Discussion on interRAT measurements without gaps	Nokia, Nokia Shanghai Bell, BT Plc., Vodafone, Charter Communications Inc., Deutsche Telekom, TELECOM ITALIA S.p.A., Verizon

- R4-2402886	Discussion on inter-RAT measurements	MediaTek inc.

- R4-2403262	Maintenance CR on interruption requirements for measurements without gap	Intel Corporation

- R4-2403264	draftCR on requirements for inter-RAT LTE measurement without gap	Huawei, HiSilicon

- R4-2403265	draftCR on introduction of interruprion requirements for inter-RAT NR measurement without gap (case a-1)	xiaomi

- R4-2400382	Discussion on performance requirement for pre-configured MGs, multiple concurrent MGs and NCSG	Apple

- R4-2400876	Discussion on performance part for Pre-MG, NCSG and concurrent MGs	vivo

- R4-2400971	Discussion on Pre-MG, Con-MGs and NCSG test cases	Ericsson

- R4-2401003	Discussion on test case design for Pre-MG, conMG and NCSG	OPPO

- R4-2401030	(NR_MG_enh2-Perf) Discussion on RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG	CMCC

- R4-2401196	Discussion on test cases for pre-configured MGs within concurrent MGs	xiaomi

- R4-2401218	Scope of RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG	Qualcomm Incorporated

- R4-2401239	Discussion on RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG	China Telecom

- R4-2401639	Discussion on the performance requirements for the joint MG considerations	ZTE Corporation

- R4-2401868	Discussion on RRM performance requirements for Pre-MG, concurrent MG and NCSG	Nokia, Nokia Shanghai Bell

- R4-2402170	Discussion on test cases for Case 1 and Case 2	Huawei, HiSilicon

- R4-2402887	Discussion on RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG	MediaTek inc.

- R4-2400543	View on RRM performance requirements for R18 NFG measurements without gaps	Qualcomm Incorporated

- R4-2400972	Discussion on measurement without gap test cases	Ericsson

- R4-2401004	Discussion on test case design for measurement without gaps	OPPO

- R4-2401029	(NR_MG_enh2-Perf) Discussion on RRM performance requirements for measurements without gaps	CMCC

- R4-2401428	Test cases discussions for measurements without gaps	Intel Corporation

- R4-2402016	Discussion on performance requirements for measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2402171	Discussion on test cases for measurement without MG	Huawei, HiSilicon

- R4-2402888	Discussion on RRM performance requirements for measurements without gaps	MediaTek inc.

- R4-2400745	Topic summary for [110][209] NR_MG_enh2_part1	Moderator (MediaTek)

- R4-2400746	Topic summary for [110][210] NR_MG_enh2_part2	Moderator (Intel)

- R4-2403312	Ad-hoc minutes for NR_MG_enh2 WI	MediaTek

- R4-2403542	WF on NR_MG_enh2_part1	MediaTek

- R4-2403543	WF on NR_MG_enh2_part2	Intel

- R4-2402880 Big CR to TS 38.133 on Further enhancements on NR and MR-DC measurement gaps and measurements without gaps	MediaTek inc.

- R4-2402881 Big CR to TS 36.133 on inter-RAT NR measurement without gap, MediaTek inc.

- R4-2403842	RAN4 Rel-18 feature list		CMCC

- R4-2403636	LS on RAN4 Rel-18 feature list	CMCC

- R4-2403358	Draft CR to Rel-18 TS 38.133: on MGE maintenance in TS 38.133	MediaTek inc.

- R4-2403359	(NR_MG_enh2-Core) draft CR on concurrent gap with Pre-MG and concurrent gap with NCSG	CMCC

- R4-2403541	CR 38.133 Corrections to Case 1 core requirements for NR_MG_enh2	Nokia, Nokia Shanghai Bell

- R4-2403362	CR on NCSG collision	OPPO

- R4-2403263	(NR_MG_enh2-Core) DraftCR on intra-frequency measurement delay for NFG	CMCC

- R4-2403265	draftCR on introduction of interruprion requirements for inter-RAT NR measurement without gap (case a-1)	Xiaomi

- R4-2403262	Maintenance CR on interruption requirements for measurements without gap	Intel Corporation

- R4-2403264	draftCR on requirements for inter-RAT LTE measurement without gap	Huawei, HiSilicon

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template