3GPP TSG RAN Meeting #103		RP-240509

Maastricht, The Netherlands, March 18 – 22, 2024

Agenda item:	9.1.4.1

Source:	Apple, Ligado Networks, Inmarsat, Hughes/Echostar, Globalstar Inc., Skyworks Solutions Inc., Viasat, Thales, IITH

Title:	High-power classes UE for NTN NR FR1 bands

Release:	Rel-19

Document for:	Discussion

# Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments. During the Rel-17 technical discussions an issue of applicable power classes was discussed, and it was agreed to focus on PC3 (+23dBm) as a common terrestrial power class. However, there are more and more terrestrial bands that have higher power classes with the corresponding support at the UE side, which could also benefit the satellite communication. This issue was raised during the RAN#97 meeting, and it was concluded that TSG RAN would consider enabling high-power classes also for the NTN links.

In addition to the NR NTN technology, RAN#92 approved a Rel-17 Work Item to enable so-called IoT communication over the satellite links.  Despite the IoT NTN Work Item late start during the second half of Rel-17 [1], after an accelerated Study Phase [2], IoT NTN core and RF specifications were completed offering NB-IoT and eMTC technological components for the satellite communication. However, similarly to the NR NTN track, existing IoT NTN specifications do not allow operation with the Tx power higher than +23dBm.

In this paper we present our further technical analysis of existing power classes for the NR/IoT NTN systems providing motivation for enabling higher power classes for these technologies.

# High-power classes for NTN

When compared to the existing terrestrial deployments, satellite communication becomes critical in adverse conditions such as severe weather events or natural disasters, where or when the terrestrial deployments are not present or temporally unavailable. However, satellite communication involves transmitting signals over long distances, which results in larger path loss. Despite the link budget closing and additional enhancements introduced to the 3GPP NTN systems, link budgets are still very much power starved in the UL and are extremely tight, with almost no margins, making it difficult to satisfy all use cases and usage conditions without impacting system capacity. And this problem is valid for both GEO and NGSO deployment scenarios. Furthermore, when considering personal handheld or IoT scenarios, there is a very high probability of blockage of the signal. In emergency situations, such as for personal distress beacons for mountain excursions and wilderness use, the effects of debris or snow (such as in case of a user being victim of an avalanche) can significantly attenuate the signal.

One of the approaches to ensure more reliable communication over the satellite channel is to rely upon (HARQ) repetitions. However, due to long propagation delays HARQ is not the best approach for NTN systems. Furthermore, in TN deployments UEs are expected to experience extreme link budgets primarily when in deep-indoor coverage scenarios. It is also assumed that, within a given cell, only a limited subset of the UE population will experience extreme coverage conditions requiring a high number of repetitions, therefore this mechanism alone is considered acceptable both from a UE battery drain and system capacity perspective. For the NTN deployments this is not true.  Since the link budget is primarily dominated by the path loss between the UE and the satellite, most of UEs in a cell will experience a similarly challenging link budget, therefore most if not all the UE may have to employ repetitions.   This impacts the UE battery life and cell capacity, size of which is larger when compared to TN. Higher power UE TX, on the other hand, maximizes the probability of single-shot UL transmission being successfully decoded by the network without the need for repetitions.

As mentioned in the Introduction part, current 3GPP specifications limit maximum NTN UE Tx power to the level of +23dBm not allowing to use the full potential of satellite communication. At the same time, existing satellite communication systems and spectrum bands already use high transmission powers from user devices. Furthermore, existing spectrum usage regulations for such applications already account for UL TX powers of up to 15 dBW (45 dBm) even whilst operating in very narrow-band channels. Thus, to provide deployment flexibility for the different NTN use cases and to address existing satellite communications use cases, the 3GPP NTN specifications should consider higher power classes already specified for 3GPP TN systems. In the following sections we will consider separately high-power UE classes for the NR NTN and IoT NTN tracks.

## 2.1	HPUE for NR NTN

As per the latest version of the 3GPP NTN specifications, more precisely TS 38.101-5, the highest Tx power available for the NTN UE is PC3, i.e. +23dBm. At the same time, NR terrestrial UEs can already use PC2, i.e. +26dBm. In fact, while initial use case for the PC2 power class was mostly TDD bands, recent changes enabled the PC2 power class also for the FDD bands. Since the FR1 NTN bands are also FDD bands, the 3GPP core specifications already have all the basic functional components to enable PC2 for the satellite FDD bands.

The NR core specifications also define even higher power classes, such as PC1.5 (+29dBm) and PC1 (+31dBm), which can be adopted for the NTN system. Nevertheless, referring specifically to the PC1.5 power class, it has been specified only for certain TDD bands, such as band n41, n77, n78 and n79. Furthermore, according to TS 38.101-1, +29dBm Tx power is achieved through the dual Tx architecture. So, while the 3GPP core specifications already define the PC1.5 power class for some TDD bands, additional specification efforts will be needed to adapt it to the satellite FDD bands potentially requiring single Tx architecture.  On the contrary to it, PC1 power class has been already specified for several NR FDD bands, such as 14, 71, 85, with an additional clarification that it is not for smartphone form factors and for the public safety scenarios only.

Based on these considerations our proposal is to enable the corresponding high-power classes for the NR NTN based satellite communication.

Proposal 1:	Enable PC2 and PC1 power classes for the NR NTN FR1 bands.

## 2.2	HPUE for IoT NTN

As identified early in the study phase, IoT NTN is a very good fit for satellite deployment due to the efficient spectrum utilization and ease of deployment, as well as very good baseline coverage capabilities, thanks in large part to its narrow-band nature.  There are also several scenarios, such as (but not limited to) automotive, maritime, logistics, outdoor industrial monitoring where the NTN IOT systems are more suitable.

According to the latest version of TS 36.102, which specifies UE radio transmission and reception for satellite access, the highest power class for NTN M1 and NB1/NB2 devices is PC3 (+23dBm). At the same time TS 36.101 already defines PC2 (+26dBm) power class and PC1 (+31dBm) power class, whereupon the latter has an additional clarification that it is not for smartphone form factors.

Based on these considerations our proposal is to enable the corresponding high-power classes for the IoT NTN based satellite communication.

Proposal 2:	Enable PC2 and PC1 power classes for the IoT NTN FR1 bands.

# 3	Conclusions

In this paper we have presented the motivation to consider enabling higher Tx power for the NR and IOT UEs that implement the NTN functionality. For the reasons and observations described in this paper, we consider beneficial to the industry to introduce the corresponding UE high-power classes for the FR1 NR NTN and IoT NTN bands.

Proposal 1:	Enable PC2 and PC1 power classes for the NR NTN FR1 bands.

Proposal 2:	Enable PC2 and PC1 power classes for the IoT NTN FR1 bands.

The corresponding draft WI proposal with the corresponding objectives can be found in Annex.

# 4	References

- [1] RP-21160, WID on NB-IoT/eMTC support for NTN

- [2] 3GPP TR 36.763; Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)

- [3] R4-2400236, "Draft new WID on High-power classes for NTN NR FR1 bands", Apple Inc.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: High-power classes for NTN NR FR1 bands

Acronym: [NTN_HPUE]

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Target Release:	Rel-19

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
|  |  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

# 3	Justification

RAN#86 meeting approved the Rel-17 WI that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. In addition to that, RAN#95 meeting approved a new spectrum WI to introduce a new NTN band as a combination of L- and S-bands.

The common aspect of all existing NTN FR1 bands is that the UE power class is limited to PC3 (+23dBm). As extensively discussed during previous RAN and RAN WG4 meetings, higher power classes are quite essential for the FR1 NTN bands to ensure appropriate UL coverage and performance. Thus, this WI aims to add support of higher power classes for the FR1 NTN bands.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective of the core part is to:

-	For NR NTN:

-	Specify support of UE Power class 2 (+26dBm);

-	Specify support of UE Power class 1 (+31dBm);

-	Specify the band specific RF requirements for all considered power classes

-	For NB-IOT NTN:

-	Specify support of UE Power class 2 (+26dBm);

-	Specify support of UE Power class 1 (+31dBm);

-	Specify the band specific RF requirements for all considered power classes

NOTE: New NTN UE power classes are applicable to all FR1 NTN bands.

NOTE: NTN UE power class 1 will not aim at smartphone form factors.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

The objective of the performance part is to:

-	Define conformance requirements for SAN.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 38.xxx | High power classes for the NTN FR1 FDD bands | TSG#xx | TSG#xx | TBD Rapporteur |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.101-5 | Support for new power classes | RAN#106 (Dec 2024) | Core |
| TS 38.108 | Support for new power classes | RAN#106 (Dec 2024) | Core |
| TS 36.102 | Support for new power classes | RAN#106 (Dec 2024) | Core |
| TS 36.108 | Support for new power classes | RAN#106 (Dec 2024) | Core |
| TS 38.101-5 | Support for new power classes | RAN#107 (Mar 2025) | Perf |
| TS 38.181 | Support for new power classes | RAN#107 (Mar 2025) | Perf |
| TS 36.102 | Support for new power classes | RAN#107 (Mar 2025) | Perf |
| TS 36.181 | Support for new power classes | RAN#107 (Mar 2025) | Perf |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

TBD Rapporteur

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

RAN WG4

# 8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Apple Inc.

Ligado Networks

Inmarsat

Hughes/Echostar

Globalstar Inc.

Skyworks Solutions Inc.

Viasat

Thales