3GPP TSG-RAN4 Meeting #110	R4-2402351

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | 0426 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI17) CR to TS 38.141-1 - BS spurious receiver protection note generalization R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 2017, the CR R4-1710754 was agreed adding a note to table 6.6.4.2-1 of TS 36.104 and specifying that for E-UTRA Band 28 BS operating in regions where Band 28 is only partially allocated, the requirement on BS Spurious emissions limits for protection of the BS receiver only apllies in the UL frequency range which is partially allocated. This CR was proposed to address the European situation where band 28 UL overlaps with band 67: as the full band 28 was not possible to operate in Europe, the protection of the BS receiver was only needed to the relevant part of band 28. We have noticed that similar situations exist for other bands as well, e.g. in Australia for Band 5 and Band 26, where the uppermost 4MHz is not used since there is 900MHz (Band 8) operation. This note should then be generalized to address each regional spectrum allocation and apply this requirement on the relevant part of the band which is effectively allocated and not to the full 3GPP band which is not necessary. It should be noted that this note was missing in TS 38.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add and generalize the note of the table on the spurious limit for protection of the BS receiver to all bands and not only to band 28 as it’s done currently. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BSs operating in regions, where only a portion of the operating band is allocated by regulations, need to meet unnecessary requirements in the rest of the operating band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.5.5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.104, 37.104, 37.105, 38.104 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.141, 37.141, 38.141-2, 37.145-1, 37.145-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

###### 6.6.5.5.1.2	Protection of the BS receiver of own or different BS

This requirement shall be applied for NR FDD operation in order to prevent the receivers of the BSs being desensitised by emissions from a BS transmitter. It is measured at the transmit antenna connector for BS type 1-C or at the TAB connector for BS type 1-H for any type of BS which has common or separate Tx/Rx antenna connectors / TAB connectors.

The basic limits are provided in table 6.6.5.5.1.2-1.

Table 6.6.5.5.1.2-1: BS spurious emissions basic limits for protection of the BS receiver

| BS class | Frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- | --- |
| Wide Area BS |  | -96 dBm |  |
| Medium Range BS | FUL_low – FUL_high | -91 dBm | 100 kHz |
| Local Area BS |  | -88 dBm |  |
| NOTE 1: For BS operating in band n104, the basic limit is increased by 1dB. NOTE 2: For BS operating in regions where a band is only partially allocated for NR operations (e.g. band n28), this requirement only applies in the UL frequency range of the partial allocation. |  |  |  |

<End of the change>