3GPP TSG-RAN4 Meeting #110	R4-2400451

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4014 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_feMIMO-Core) CR for active state list switching delay requirements for UTCI state -R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The active TCI state list update delay requirements suggest that UE should be ready to receive or tranmsit with the target TCI state after the list update. But UE needs to receive DCI after the list update to change the target TCI state. Details provided in R4-2400450. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the requirements to reflect the TCI state list update delay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The active TCI state switching delay requirements for unified TCI will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.5, 8.16.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change 1

### 8.15.5	Active Downlink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one DL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n + THARQ +3Nslotsubframe,µ +  TOk*(Tfirst-SSB_List + TSSB-proc) / NR slot length.

If a subset of the target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP_list +TOuk*(Tfirst-SSB_List+ TSSB-proc)) / NR slot length.

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Where

-	TL1-RSRP_List is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states, where TL1-RSRP is specified in clause 8.15.3

-	If the number of cells associated with the target TCI states in the active TCI list is 2, and time to first SSBs associated to the TCIs are overlapped in FR2,

-	Tfirst-SSB_List = Tfirst-SSB_SC + min(TSSB_SC , TSSB_CDP).

-	If the number of cells associated with the target TCI states in the active TCI list is 2, and time to first SSBs associated to the TCIs are not overlapped in FR1 or FR2,

-	Tfirst-SSB_List = max(Tfirst-SSB_SC , Tfirst-SSB_CDP)

-	Otherwise,

-	Tfirst-SSB_List = Tfirst-SSB_SC.

-	TSSB_SC is the SSB periodicity of serving cell.

-	TSSB_CDP is the SSB periodicity of cell with different PCI from serving cell.

- 	Tfirst-SSB_SC is the Tfirst-SSB from serving cell

-	Tfirst-SSB_CDP is the Tfirst-SSB from cell with different PCI from serving cell.

-	THARQ, Tfirst-SSB, TSSB-proc , TOk, TOuk are defined in clause 8.15.3. TSSB is the SSB periodicity.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state list update.

Change 2

### 8.16.5	Active Uplink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one UL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n + THARQ + 3Nslotsubframe,µ + NM * (Tfirst_target-PL-RS_List + 4 * Ttarget_PL-RS_List + 2ms) / NR slot length,

If a subset of target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot

n+THARQ + 3Nslotsubframe,µ+ (TL1-RSRP_List + Tfirst_target-PL-RS_List + 4*Ttarget_PL-RS_List + 2ms) / NR slot length

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Where

-	If all TCIs are known,

-	if the target PL-RS associated with or included in any UL TCI is not maintained, NM = 1;  Where maintained PL-RS is defined in clause 8.16.3

-	if a subset of target TCI states in the active TCI state list are unknown,

-	TL1-RSRP_List = is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states , where TL1-RSRP is specified in clause 8.16.3

-	if the number of cells associated with the target TCI states in the active TCI list is 2, and time to first PL-RS associated to the TCIs are overlapped in FR2,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS_SC + min(Ttarget-PL-RS_SC , Ttarget-PL-RS_CDP) ;

-	Ttarget-PL-RS_List = 2 * Ttarget-PL-RS_SC, if Ttarget-PL-RS_SC = Ttarget-PL-RS_CDP

-	Ttarget-PL-RS_List = max(Ttarget-PL-RS_SC , Ttarget-PL-RS_CDP), if  Ttarget-PL-RS_SC  ≠ Ttarget-PL-RS_CDP

-	if the number of cells associated with the target TCI states in the active TCI list is 2, and time to first PL-RS associated to the TCIs are not overlapped in FR2,

-	Tfirst_target-PL-RS_List = max (Tfirst_target-PL-RS_SC, Tfirst_target-PL-RS_SC);

-	Ttarget-PL-RS_List = max(Ttarget-PL-RS_SC , Ttarget-PL-RS_CDP)

-	Otherwise,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS_SC

-	Ttarget-PL-RS_List = Ttarget-PL-RS_SC.

-	Tfirst_target-PL-RS_SC is Tfirst_target-PL-RS for serving cell

-	Tfirst_target-PL-RS_CDP is Tfirst_target-PL-RS for cell with different PCI from serving cell

-	Ttarget-PL-RS_SC is Ttarget-PL-RS for serving cell

-	Ttarget-PL-RS_CDP is Ttarget-PL-RS for cell with different PCI from serving cell-	THARQ, Tfirst_target-PL-RS, Ttarget-PL-RS are defined in clause 8.16.3.

The requirements specified in this clause do not apply if more than 4 different RSs are activated as PL-RS per serving cell among all active UL (or joint) TCI states.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to transmit on UL based on the target TCI before UE completes the DL and UL TCI list update.

The requirements in this clause are applicable when the source RS of the active UL TCI state is a DL-RS and this DL-RS is included as one of the source RSs in the DL active TCI list.