3GPP TSG-RAN4 Meeting #110	R4-2402000

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4211 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ext_to_71GHz) CR corrections RRM core requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Removal of square brackets and fixing TBDs |  |  |  |  |  |  |  |  |
| Summary of change: |  | TBD for beam sweeping scaling factor fixed, as it has been agreed to use N=12 for FR2-2. Since there was no further discussion on the topic, the values in square brackt can be considered final and removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2A.2.3, 4.2a.2.4, 8.1A.2.2, 8.5A.2.2, 8.5A.5.2, 9.2.5.3.3, 9.5A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change #1>

#### 4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell

The UE shall be able to identify new intra-frequency cells with CCA and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

When the intra-frequency measurement is performed on a neighbour cell in FR2-2 with shared spectrum channel access, UE shall determine the CCA mode of the neighbour cell according to channelAccessMode2-r17of the cell configured in SIB3. If channelAccessMode2-r17 of the cell is enabled, UE shall assume that CCA applies to the cell and perform measurement accordingly, and the requirements in clause 4.2A.2.3 shall apply; otherwise, UE shall assume that CCA does not apply to the cell and perform measurement accordingly, and requirements in 4.2.2.3 shall apply.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 within Tdetect,NR_Intra_CCA when that Treselection= 0 . An intra frequency cell is considered to be detectable according to the conditions defined in Annex B. 2. 8  for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra_CCA (see table 4.2A.2.3-1) for intra-frequency cells that are identified and measured according to the measurement rules.  For a cell that is already identified, after 2 unsuccessful measurement attempts due to exceeding the maximum number of SMTC occasions not available at the UE, the UE shall detect cells on any of the configured serving- and/or non-serving carriers.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra_CCA/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Tevaluate,NR_Intra_CCA when Treselection = 0 as specified in table 4.2A.2.3-1 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5 dB better ranked in FR2-2.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3dB better ranked in FR1 or 4.5 dB better ranked in FR2-2 if the current serving cell is among them.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non-zero value and the intra-frequency cell is satisfied with the reselection criteria, which are defined in TS38.304 [1], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

## <End of Change #1>

## <Start of Change #2>

#### 4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

When the inter-frequency measurement is performed on a neighbour cell in FR2-2 with shared spectrum channel access, UE shall determine the CCA mode of the neighbour cell according to channelAccessMode2-r17of the cell configured in SIB4. If channelAccessMode2-r17 of the cell is enabled, UE shall assume that CCA applies to the cell and perform measurement accordingly, and the requirements in clause 4.2A.2.4 shall apply; otherwise, UE shall assume that CCA does not apply to the cell and perform measurement accordingly, and requirements in 4.2.2.4 shall apply.

If Srxlev > SnonIntraSearchP  and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2A.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 within Kcarrier * Tdetect,NR_Inter + Kcarrier_CCA * Tdetect,NR_Inter_CCA  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 or 6.5dB in FR2-2 for reselections based on ranking or 6dB in FR1 or 7.5 dB in FR2-2 for SS-RSRP reselections based on absolute priorities or 4dB in FR1 and 4 dB in FR2-2 for SS-RSRQ reselections based on absolute priorities. The parameter Kcarrier is the number of NR inter-frequency carriers on licensed band and Kcarrier_CCA is the number of NR inter-frequency carriers on unlicensed band indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B. 2. 9  for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter_CCA. If after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter + Kcarrier_CCA * Tmeasure,NR_Inter_CCA  for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

For a cell that is already identified, after 2 unsuccessful measurement attempts due to exceeding the maximum number of SMTC occasions not available at the UE, the UE shall detect cells on any of the configured serving- and/or non-serving carriers.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter_CCA/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 within Kcarrier * Tevaluate,NR_Inter + Kcarrier_CCA * Tevaluate,NR_Inter_CCA when Treselection = 0 as specified in table 4.2A.2.4-1 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

when rangeToBestCell is not configured:

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2-2 or.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2-2 if the current serving cell is among them. or

-	6dB in FR1 or 7.5dB in FR2-2 for SS-RSRP reselections based on absolute priorities or

-	4dB in FR1 or 4dB in FR2-2 for SS-RSRQ reselections based on absolute priorities.

## <End of Change #2>

## <Start of Change #3>

#### 8.1A.2.2	Minimum Requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB,CCA [ms] period becomes worse than the threshold Qout_SSB,CCA within TEvaluate_out_SSB,CCA [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB,CCA [ms] period becomes better than the threshold Qin_SSB,CCA within TEvaluate_in_SSB,CCA [ms] evaluation period. During the in-sync evaluation procedure, layer 1 of the UE shall not send any in-sync indication for the cell to the higher layers when Lin exceeds Lin,max, where Lin and Lin,max are defined in Table 8.1A.2.2-1.

TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA are defined in Table 8.1A.2.2-1 for FR1.

TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA are defined in Table 8.1A.2.2-2 for FR2-2 with scaling factor N = 12.

For a UE supporting concurrentMeasGap-r17 and when concurrent gaps are configured,

-	P value for an RLM-RS resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with measurement gap occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

Otherwise, for a UE not supporting concurrentMeasGap-r17 or when concurrent gaps are not configured,

For FR1,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, and these GAPs are overlapping with some but not all occasions of the SSB RLM-RS resources; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB RLM-RS resources.

When a measurement gap is configured and the measurement gap is not NCSG,

-	an RLM-RS resource is considered to be overlapped with the GAP f it overlaps a measurement gap occasion, and

-	xRP = MGRP

Otherwise, when NCSG measurement gap is configured,

-	an RLM-RS resource is considered to be overlapped with the GAP if it overlaps the VIL1 or VIL2 of NCSG, and

-	xRP = VIRP

If the UE is configured with Pre-MG, an RLM-RS resource is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

When concurrent gaps are configured, an RLM-RS resource is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

For FR2-2,

-	P=11-TSSBTSMTCperiod, when RLM-RS resource is not overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when the RLM-RS resource is not overlapped with measurement gap and RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P=11-TSSBMGRP - TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P=Psharing factor1-TSSBMGRP, when the RLM-RS is partially overlapped with measurement gap and the RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5 × TSMTCperiod

-	P=11-TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P=Psharing factor1-TSSBMRGP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 1, if the RLM-RS resource outside measurement gap is

- not overlapped with the SSB symbols indicated by SSB-ToMeasure and K data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and K data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier,  and K is defined in clause 9.2.5.3.3, and,

- not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and K data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and K data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured, and K is defined in clause 9.2.5.3.3.

-	Psharing factor = 3, otherwise.

where,

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2-2 band, provided the SMTC offset of all CCs in FR2-2 have the same offset.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

Table 8.1A.2.2-1: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA for FR1

| Configuration | TEvaluate_out_SSB,CCA (ms) |  | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- | --- |
|  | RLM-RS SSB Es/IotNote4 ≥-7 dB | RLM-RS SSB Es/Iot Note4 <-7 dB |  |
| no DRX | Max(200, Ceil(17*P)*TSSB) | Max(200, Ceil(24*P)*TSSB) | Max(100, Ceil((5+Lin)*P)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*15*P)*Max(TDRX,TSSB)) | Max(200, Ceil(1.5*20*P)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil(13*P)*TDRX | Ceil(16*P)*TDRX | Ceil((5+Lin)*P)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, Lin is the number of RLM-RS SSB occasions which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. When DRX is configured, Lin is the number of DRX cycles in which at least one RLM-RS SSB occasion is not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. The UE is not required to determine the availability of SSB occasions more frequent than Once per Max(10ms, P * TSSB) if no DRX is used, Once per Max(10ms, Ceil(1.5 * P) * Max(TDRX, TSSB)) if DRX cycle ≤ 320ms, Once per P * TDRX if DRX cycle > 320ms. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: RLM-RS SSB Es/Iot is the averaged Es/Iot over the most recent previous out-of-sync evaluation period. |  |  |  |

Table 8.1A.2.2-2: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA for FR2-2

| Configuration | TEvaluate_out_SSB,CCA (ms) | TEvaluate_in_SSB,CCA (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(12*P* N)*TSSB) | Max(100, Ceil((5 +Lin)*P* N)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*10*P* N)*Max(TDRX,TSSB)) | Max(100, Ceil(1.5*(5 +Lin)*P* N)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil(10*P* N)*TDRX | Ceil((5+Lin)*P* N)*TDRX |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, Lin is the number of RLM-RS SSB occasions groups which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. A RLM-RS SSB occasions group consists of N consecutive RLM-RS SSB occasions, and the RLM-RS SSB occasions group is not available at the UE when at least one RLM-SSB occasion in the group is not transmitted by the gNB. When DRX is configured, Lin is the number of DRX cycles groups which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. A DRX group consists of N DRX cycles, and the DRX group is not available when there is at least one DRX in which at least one RLM-RS SSB occasion is not available. The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. |  |  |

## <End of Change #3>

## <Start of Change #4>

#### 8.5A.2.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured BFD-RS SSB resource in set q0 estimated over the last TEvaluate_BFD_SSB_CCA ms period becomes worse than the threshold Qout_LR_SSB,CCA within TEvaluate_BFD_SSB_CCA ms period.

The value of TEvaluate_BFD_SSB_CCA is defined in Table 8.5A.2.2-1 for FR1.

The value of TEvaluate_BFD_SSB_CCA is defined in Table 8.5A.2.2-2 for FR2-2 with scaling factor N=12.

For a UE supporting concurrentMeasGap-r17 and when concurrent gaps are configured,

-	P value for a BFD-RS resource to be measured is defined as Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with measurement gap occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Otherwise, for a UE not supporting concurrentMeasGap-r17 or when concurrent gaps are not configured,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the BFD-RS SSB.

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the BFD-RS SSB.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a BFD-RS resource is considered to be overlapped with the  GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG measurement gap is configured,

-	a BFD-RS resource is considered to be overlapped with the  GAP if it overlaps the VIL1 or VIL2 of NCSG, and

-	xRP = VIRP

-	If the UE is configured with Pre-MG, a BFD-RS resource is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps are configured, a BFD-RS resource is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8.

For FR2-2,

-	P=11-TSSBTSMTCperiod, when BFD-RS resource is not overlapped with measurement gap and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = Psharing factor, when the BFD-RS resource is not overlapped with measurement gap and the BFD-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P=11-TSSBMGRP - TSSBTSMTCperiod, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P=Psharing factor1-TSSBMGRP, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5*TSMTCperiod

-	P=11-TSSBTSMTCperiod, when the BFD-RS resource is partially overlapped with measurement gap (TSSB <MGRP) and the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P=Psharing factor1-TSSBMGRP, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

Where,

-	Psharing factor = 1, if the BFD-RS resource outside measurement gap is

-	not overlapped with  the SSB symbols indicated by SSB-ToMeasure and K data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and K data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and K is defined in clause 9.2.5.3.3, and;

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and K data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and K data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured, and K is defined in clause 9.2.5.3.3.

-	Psharing factor = 3, otherwise.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2-2 band, given the SMTC offset of all CCs in FR2-2 provided the same offset.

Longer evaluation period would be expected if the combination of BFD-RS SSB resource, SMTC occasion and  GAP configurations does not meet pervious conditions.

Table 8.5A.2.2-1: Evaluation period TEvaluate_BFD_SSB_CCA for FR1

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |  |
| --- | --- | --- |
|  | BFD-RS SSB Es/Iot Note2 ≥ -7 dB | BFD-RS SSB Es/Iot Note2 < -7 dB |
| no DRX | Max(50, Ceil((10 P) TSSB)) | Max(50, Ceil((12 P) TSSB)) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 8 P) Max(TDRX,TSSB)) | Max(50, Ceil(1.5 10 P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(7 P) TDRX | Ceil(8 P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. Note 2: BFD-RS SSB Es/Iot is the averaged BFD-RS SSB Es/Iot over the most recent previous evaluation period. |  |  |

Table 8.5A.2.2-2: Evaluation period TEvaluate_BFD_SSB_CCA for FR2-2

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |
| --- | --- |
| no DRX | Max(200, Ceil(12*P* N)*TSSB) |
| DRX cycle ≤ 320ms | Max(200, Ceil(1.5*10*P* N)*Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(10*P* N)*TDRX |
| Note 1: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. |  |

## <End of Change #4>

## <Start of Change #5>

#### 8.5A.5.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CBD-RS SSB resource in set  estimated over the last TEvaluate_CBD_SSB_CCA ms period becomes better than the threshold Qin_LR,CCA provided SSB_RP and SSB Ês/Iot are according to Annex Table B.2.4.1 for a corresponding band.

The UE shall monitor the configured SSB resources using the evaluation period in table 8.5A.5.2-1 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_SSB_CCA is defined in Table 8.5A.5.2-1 for FR1.

The value of TEvaluate_CBD_SSB_CCA is defined in Table 8.5A.5.2-2 for FR2-2 with scaling factor N=12.

## <End of Change #5>

## <Start of Change #6>

##### 9.2.5.3.3	Scheduling availability of UE performing measurements on FR2

The following scheduling restriction applies due to SS-RSRP or SS-SINR measurement on an FR2 intra-frequency cell

-	If deriveSSB-IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on K data symbol(s) before each consecutive SSB symbols to be measured and K data symbol(s) after each consecutive SSB symbols to be measured within SMTC window duration.

-	If deriveSSB-IndexFromCell is not enabled and the SCS of data and SSB symbols are smaller than 960kHz, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration.

-	If deriveSSB-IndexFromCell is not enabled and the SCS of data or SSB symbols is 960kHz, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI SSB symbols to be measured, and on K’ data symbol(s) before each consecutive SSB symbols to be measured and K’ data symbol(s) after each consecutive SSB symbols to be measured within SMTC window duration.

The following scheduling restriction applies to SS-RSRQ measurement on an FR2 intra-frequency cell

-	If deriveSSB-IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, RSSI measurement symbols, and on K data symbol(s) before each consecutive SSB to be measured/RSSI symbols and K data symbol(s) after each consecutive SSB to be measured/RSSI symbols within SMTC window duration

-	If deriveSSB-IndexFromCell is not enabled and the SCS of data and SSB symbols are smaller than 960kHz, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration.

-	If deriveSSB-IndexFromCell is not enabled and the SCS of SSB symbols is 960kHz, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, RSSI measurement symbols, and on K’ data symbol(s) before each consecutive SSB to be measured/RSSI symbols and K’ data symbol(s) after each consecutive SSB to be measured/RSSI symbols  within SMTC window duration.

where

-	If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

-	The signaling deriveSSB-IndexFromCell is always enabled for FR2-1 and FR2-2 when SSB is using 120 kHz SCS and 480 kHz SCS.

-	K=1 for a serving cell with data symbols of 120 kHz SCS

-	K=4 for a serving cell with data symbols of 480 kHz SCS and SSB symbols of 120kHz or 480kHz SCS

-	K=3 for a serving cell with data symbols of 480 kHz SCS and SSB symbols of 960kHz SCS

-	K=7 for a serving cell with data symbols of 960 kHz SCS and SSB symbols of 120kHz or 480kHz SCS

- 	K=4 for a serving cell with data symbols of 960 kHz SCS and SSB symbols of 960kHz SCS

-	K’=2 for a serving cell with data symbols of 120 kHz SCS and SSB symbols of 960kHz SCS

-	 K’=4 for a serving cell with data symbols of 480 kHz SCS and SSB symbols of 960kHz SCS

-	 K’=7 for a serving cell with data symbols of 960 kHz SCS and SSB symbols of 960kHz SCS

## <End of Change #6>

## <Start of Change #7>

### 9.5A.4	L1-RSRP measurement requirements

#### 9.5A.4.1	SSB based L1-RSRP Reporting

The value of TL1-RSRP_Measurement_Period_SSB_CCA is defined in Table 9.5A.4.1-1 for FR1, and in Table 9.5A.4.1-2 for FR2-2, where,

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N = 8

## <End of Change #7>