3GPP TSG-RAN WG4 Meeting #110						                 					     R4-2403533

Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4006 | rev | 1 | Current version: | 16.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on combination of HST and RRM relaxation - R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST, NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2024-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 received an LS from RAN2 (R2-2311435) on combination of HST and RRM relaxation. After discussion, RAN4 agreed to clarify UE measurement behavior when both HST flag and low mobility relaxation are enabled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify UE measurement behavior when both HST flag and low mobility relaxation are enabled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When both HST flag and low mobility relaxation are enabled, UE measurement behavior would still be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.3, 4.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### 4.2.2.3	Measurements of intra-frequency NR cells

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [1] within Tdetect,NR_Intra when that Treselection= 0 . An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1 or table 4.2.2.3-2) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1 or table 4.2.2.3-2 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2 if the current serving cell is among them.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non zero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [1], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For UE not configured with highSpeedMeasFlag-r16, Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra are specified in Table 4.2.2.3-1. For UE configured with highSpeedMeasFlag-r16, Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra are specified in Table 4.2.2.3-2.

For a UE configured with highSpeedMeasFlag-r16, and configured with relaxed measurement criterion (lowMobilityEvaluation or both lowMobilityEvaluation and cellEdgeEvaluation), it is up to UE implementation whether to apply the relaxed measurements requirements specified in clause 4.2.2.9.2 or high speed requirements specified in this clause provided that the relaxed measurement criteria in section 4.2.2.9.2 are fulfilled.

The requirements in Table 4.2.2.3-2 apply only when the UE supports measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. For UE not supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16, the UE is not required to meet the requirements specified in Table 4.2.2.3-2.

Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. |  |  |  |  |  |

Table 4.2.2.3-2: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with highSpeedMeasFlag-r16 (Frequency range FR1)

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |  |  |  |

End of Change 1

Start of Change 2

#### 4.2.2.5	Measurements of inter-RAT E-UTRAN cells

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

The requirements in this clause apply for inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this clause.

The parameter NEUTRA_carrier is the total number of configured E-UTRA carriers indicated to meet non high speed requirements in the neighbour frequency list. The parameter NEUTRA_carrier_HST is the total number of configured E-UTRA carriers indicated to meet high speed requirements in the neighbour frequency list. If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, an inter-RAT E-UTRAN layer is indicated to meet high speed requirements if highSpeedMeasFlag-r16 is configured and the carrier to be measured is configured with highSpeedEUTRACarrier-r16 and UE supports the enhanced inter-RAT E-UTRAN measurement requirements. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE is required to meet non high speed requirements no matter whether highSpeedMeasFlag-r16 or highSpeedEUTRACarrier-r16 is configured or not.

For a UE configured with highSpeedMeasFlag-r16, and configured with relaxed measurement criterion (lowMobilityEvaluation or both lowMobilityEvaluation and cellEdgeEvaluation), it is up to UE implementation whether to apply the relaxed measurements requirements specified in clause 4.2.2.11.2 or high speed requirements specified in this clause provided that the relaxed measurement criteria in section 4.2.2.11.2 are fulfilled.

The parameter NEUTRA_carrier for a UE configured with idle mode DC measurements (while T331 is running), is the combined number of configured E-UTRA carriers in the neighbour frequency list and E-UTRA carriers configured for idle mode DC measurements, excluding the configured E-UTRA carriers indicated to meet high speed requirements in the neighbour frequency list.

Note:	combined total number means that if a carrier is an E-UTRA carrier indicated by the serving cell for mobility and additionally a carrier configured for idle mode CA measurements, it only counts as one carrier.

The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

-	the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band, and

-	the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band.

-	SCH conditions specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 [1] within NEUTRA_carrier_HST * Tdetect,EUTRAN_HST + NEUTRA_carrier  * Tdetect,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

Cells which have been detected shall be measured at least every NEUTRA_carrier_HST * Tmeasure,EUTRAN_HST + NEUTRA_carrier  * Tmeasure,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT E-UTRAN carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [1] within NEUTRA_carrier_HST * Tevaluate,EUTRAN_HST + NEUTRA_carrier  * T evaluate,EUTRAN when Treselection = 0 as speficied in table 4.2.2.5-1 and 4.2.2.5-2 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

If Treselection timer has a non zero value and the inter-RAT E-UTRA cell is satisfied with the reselection criteria which are defined in TS 38.304 [1], the UE shall evaluate this E-UTRA cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2.2.5-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32(25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

Table 4.2.2.5-2: Tdetect,EUTRAN_HST, Tmeasure,EUTRAN_HST, and Tevaluate,EUTRAN_HST for UE configured with highSpeedMeasFlag-r16

| DRX cycle length [s] | Tdetect,EURAN_HST [s] (number of DRX cycles) | Tmeasure,EUTRAN_HST [s] (number of DRX cycles) | Tevaluate,EUTRAN_HST [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 (13) | 0.64 (2) | 0.96 (3) |
| 0.64 | 7.68 (12) | 1.28 (2) | 1.92 (3) |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [interRAT-MeasurementEnhancement-r16]. |  |  |  |

The requirements in Table 4.2.2.5-2 apply only when the UE supports measurementEnhancement-r16 or interRAT-MeasurementEnhancement-r16. For UE not supporting either measurementEnhancement-r16 or interRAT-MeasurementEnhancement-r16, the UE is not required to meet the requirements specified in Table 4.2.2.5-2.

End of Change 2